SECTION I:PROJECT DESCRIPTION ( Seoni-Bina 765 KV Transmission System )

- BACKGROUND:

In India, major energy resources for electrical power are concentrated in Eastern Region (mainly thermal resources) and North-Eastern Region (mainly hydro resources), whereas power demand is spread all over the country. To transfer power from these resource-rich Regions to distant part of the country, it is envisaged to develop high capacity National Grid by interconnecting Eastern, Northern and Western Regions through high capacity transmission links. In this direction, 2x765 kV Sipat - Seoni lines are being constructed as a part of transmission system associated with Sipat stage-I generation project. This corridor shall also be used to transfer power imported at Sipat from Eastern Region projects through Ranchi-Sipat 400kV D/c line, planned with Eastern Region generation projects. To disperse power from Seoni to Northern part of Western Region and to complete the transmission corridor of National Grid in Western Region, Seoni-Bina 765 kV line has been envisaged.

1.1	BENEFITS OF THE PROJECT:

The objective of this line is to connect major load centre in Northern part of M.P. to power pool point at Seoni as well as to complete the 765kV kV ring interconnecting Eastern, Western and Northern Regions as part of development of National grid. This will help export of surplus power from Eastern Region to Western & Northern Regions and strengthen the transmission system within Western Region for secure & reliable supply of power.

1.2     PROJECT DESCRIPTION

In Western Region, Seoni, in the central part of Madhya Pradesh is being developed as a major power pooling station at 765kV level. Seoni shall be directly connected with Sipat generation complex through two (02) nos. 765kV line between Sipat and Seoni. Also, power from Kahalgaon-II generation project in Eastern Region would be imported at Sipat generation complex which would be transferred to Western Region beneficiaries mainly through Seoni substation. In this way, Sipat-Seoni would be a major transmission corridor in Western Region (WR) over which large quantum of power would flow. To disperse power to different load centres in WR from Seoni onwards, there is a necessity to develop transmission corridor of adequate capacity to meet the immediate as well as future requirement.

A high capacity 765kV line between Bina and Gwalior is proposed, as a part of Sipat-II Transmission system to supply power towards Gwalior area. The interconnection at Gwalior would be further extended upto Agra by a 765kV link for power exchange between Western and Northern Regions as a part of transmission system of Kahalgaon-II generation project. Therefore, Seoni-Bina along with Bina-Gwalior-Agra line would be a part of 765kV ring interconnecting ER, NR and WR. In this way, this transmission line would enable exchange of power between NR and WR.

In addition, a parallel transmission corridor between eastern part of the region and Gujarat in western part of the region is being developed through 400kV Bina-Nagda - Dehgam line.  The Seoni-Bina line would interconnect the two parallel transmission corridors between eastern and western part of WR. In case of outage of lines in any one of these corridors, the Seoni – Bina inter-connection would provide alternate path for power transfer from eastern part to western part of WR.

Considering above network configuration and long-term power transfer requirement from eastern part of the region towards western part, construction of 765kV Seoni – Bina S/c line is proposed. This shall develop a high capacity transmission corridor right from Seoni towards Gujarat as well as towards northern part of M.P. It would also facilitate development of transmission corridor between Agra in NR and Seoni in WR.

The Transmission line has been designed with 765kV parameters whereas the associated bay extension would be at 400kV level. This type of construction has been designed as the transmission line is a part of 765kV ring interconnecting ER-NR-WR. The power flow over this corridor would progressively increase with more generation addition in ER and NER having beneficiaries in WR. As the power flow over this line initially would be less, the operation of the line has been proposed at 400kV level initially. This would defer the investment for 765kV equipment and would conserve the Right-of-Way (ROW). With the increase in power flow requirement in future, the line shall be operated at 765kV level.

Project Highlights

| a) | Project Name | : | Seoni-Bina 765 KV Transmission System |
| --- | --- | --- | --- |
| b) | Location | : | Madhya Pradesh |
| c) | Beneficiary States | : | Constituents of Western and Northern Region |
| d) | Project Cost | : | US $ 79.41 million (including IDC) |

Scope of work

Transmission Lines

- Seoni – Bina 765kV S/C (initially to be operated at 400kV) – 292 km.

Substations:

Extension of existing 800 KV substation at Seoni and 400 KV at Bina

(Not in the scope of The World Bank funding to be funded by POWERGRID).

A power map showing the transmission grid of Western Region highlighting the above scope of works is placed at Exhibit-1.

SECTION II: BASE LINE DATA

2.0  	The project is located in the State of Madhya Pradesh in India. The basic details of the area under project are given below:

2.1   MADHYA PRADESH:

PHYSIOGRAPHY: Madhya Pradesh as its name implies, lies in the heart of India.It consists of a geographical area of 3,08,300 sq. km which constitutes 9.38% of the land area of the country & bordering the states - Uttar Pradesh, Chhattisgarh, Maharashtra, Gujarat and Rajasthan. It lies between lat.210 04’&260 52’N and long.740 02’ & 820 49’ E.

Climate:  The climatic condition is generally Sub-Tropical wet and dry. MP state has three main seasons:

- Winter (November through February);

- Summer (March through May); and

- Monsoon season (June through September).

Temperature:  During the winter average temperatures range from 10° to 27° C. Summers are hot, with an average temperature of 29° C and a high temperature that at times reaches 48° C. During the monsoon season temperatures average 19° to 30° C.

Rainfall:  Annual rainfall tends to decrease from south to north and from east to west.   The average rainfall in the different regions of the state ranges from 450 to 900 mm.    The annual mean total rainfall recorded at Ujjain is 934.1 mm (1960 – 1980 data period) with almost 45 rainy days.  The heaviest rainfall in 24 hours has been recorded as 239 mm at Ujjain.

Wind: The mean wind speed at Ujjain is 10 km/hr though the wind speeds are normally recorded to exist between 4 km/hr to 20 km/hr.

Soil: Red, Yellow and black soils are generally available in the state.

Mineral Resources: Madhya Pradesh is one of the major mineral producing states of the country.  It has large deposits of a variety of minerals.  Important among them are bauxite, copper, manganese, coal, dolomite and limestone.

Water Resources: The important rivers of the state are Chambal, the Betwa, the Sone and the Narmada.

Ecological   Resources: The forest cover of the state based on the satellite data of October-December, 1999, is 77,265 Sq. km. which constitutes 25.07% of the geographic area (Map-1).  Out of these dense forest accounts for 44,384 sq.km. having  crown  density  of  more  than  40% and  open  or  degraded  forest  of  32,881 sq.km. having crown  density  ranging  between  10-40%. By legal status Reserve Forest constitutes 61.69%, Protected Forest 37.36% and unclassified Forest 0.95%. The forests are the main source of supply of fodder and fuel and subsistence for the poorest sections of the people and tribal population in the interior under-developed areas of the state. There are four forest types:

- Tropical Moist Deciduous

- Tropical Dry Deciduous

- Tropical Thorn and

- Sub Tropical Broadleaved Hill Forests.

Forests are largely distributed in central, southern and eastern parts of the undivided state of Madhya Pradesh. Northern and western parts of the state are deficient in forest vegetation. Teak and Sal are the two most important forest formations of the state, covering 18.0% and 16.7% forest area, respectively while miscellaneous forests cover 65.3%. Madhya Pradesh forest reserves are logged for Teak, Sal, Bamboo and Salai. These forests catered to the needs of the people and cattle for generations, largely because they contained good cropping species. However, population explosion and developmental needs have exerted a steadily increasing demand on the ever-diminishing extent of forests. Over-exploitation resulted in reduction of area under forests. The maximum forest cover is recorded in the district Sheopur having 56.75% of forest cover and minimum in the Ujjain district having only 0.61%.    There are 11 National Parks (NP) and 32 Wild Life Sanctuaries in the state however the Kanha tiger reserve in Jabalpur district is the most important National Park of the state.

The lines of proposed transmission system shall pass through mainly five district of this state having forest cover ranging from 25 % to 38 %. It may be noted from the table below that in all five districts the forest cover is a good mixture of both dense and open/degraded forest nature this warrant extra precaution in routing of line through forest area. Details of forest cover of these districts are as follows:

In sq. Km.

| District | Geographic area | Dense forest | open forest | Total | % forest Cover |
| --- | --- | --- | --- | --- | --- |
| Seoni | 8,758 | 2,237 | 910 | 3,147 | 35.93 |
| Narshimhapur | 5,133 | 812 | 500 | 1,312 | 25.56 |
| Chindwara | 11,815 | 2,631 | 1,864 | 4,545 | 38.47 |
| Raisen | 8,466 | 1,589 | 1,107 | 2,696 | 31.85 |
| Sagar | 10,252 | 1,292 | 1,525 | 2,817 | 27.48 |

SECTION III:   POLICY, LEGAL & REGULATORY FRAMEWORK

- 3.0     POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and directives and guidelines with respect to environmental and social considerations of Funding Agencies.

- 3.1     ENVIRONMENTAL

3.1.1 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under The Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under The Forest (Conservation) Act, 1980

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded, hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/reconditioner or at the designated collection centers – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)Amendment Rules, 2003:

MOEF vide its notification dt. 20th May, 2003 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned / sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organization in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalizing the route alignment.

    - FUNDING AGENCIES:

- WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM)- F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.3   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- Applicable Legislations

- 3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement

- on Environment and Development, 1992

- Policy statement for Abatement of pollution, 1992

- 3.2.0   SOCIAL

- 3.2.1    MANDATORY REQUIREMENTS (NATIONAL)

- National Policy on Resettlement and Rehabilitation for Project Affected Families: Ministry of Rural Development, Government of India has notified a National policy on R&R for Project Affected Families (PAFs) in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the assetless rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

- When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups.

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan/Tribal Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.3    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- National and State-wide Laws and Policies Relating

to Land Acquisition and Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.4   RELEVANT POLICIES

- The Orissa Resettlement and Rehabilitation of Project Affected Persons Policy, 1994 (water resources projects)

- Resettlement & Rehabilitation Policy – NTPC

# SECTION-IV: MAJOR FEATURES OF FINAL ROUTE & ENVIRONMENTAL IMPACT

- 4.0     ROUTE SELECTION

- Environmental impact of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. To minimize these possible impact POWERGRID at the system planning stage itself try to avoid ecological sensitive areas like forest Exhibit-2.   Wherever such infringements are substantial, different alternative options are considered to select most viable route alignment. For further optimization of route modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS in route selection result in access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. The route/ site selection criteria followed by POWERGRID is detailed below:

- 4.1    Environmental Criteria for Route Selection

- For selection of optimum route, the following points are taken into consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

- Any monument of cultural or historical importance is not affected by the route of the transmission line.

- The proposed route of transmission line does not create any threat to the survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion Exhibit-3.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

- In addition, care is also taken to avoid National parks and sanctuaries and any other forest area rich in wild life.

- Keeping above in mind the routes of line under this transmission System has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps etc. to arrive at most optimum route which can be taken up for detailed survey using GIS/GPS and assessment of environmental & social impacts for their proper management (Annexure-1).

- Similarly the TOR for detailed survey using modern tool like GIS/GPS also contained parameters to avoid/reduce environmental impact while deciding the final route alignment.  The major objectives for detailed survey that are part of contract are summarized below:

- The alignment of transmission line shall be most economical from the point of view of construction and maintenance.

- Routing of transmission line through protected the reserved forest area should be avoided.  In case it is not possible to avoid the forest or areas having large trees completely then keeping in view of the overall economy, the route should be aligned in such a way that cutting of trees is minimum.

- The route should have minimum crossing of major river, railway lines, and national/state high ways, overhead EHP power lines and communication lines.

- The number of angle point shall be kept to a minimum.

- The distance between the terminal points specified shall be kept shortest possible, consistent with the terrain that is encountered.

- Marshy and low line areas, river beds and earth slip zones shall be avoided to minimum risk to the foundations.

- It would be preferable to utilize level ground for the alignment.

- Crossing of power line shall be minimum.  Alignment will be kept at a minimum distance of 300 meters from power lines to avoid induction problems on the lower voltage lines.

- Crossings of communication lines shall be minimized and it shall be preferably at right angle, proximity and paralyses with telecom lines shall be eliminated to avoid danger of induction to them.

- Area subjected to flooding searches streams shall be avoided.

- Restricted areas such as civil and military airfield shall be avoided.  Care shall also be taken to avoid the aircraft landing approaches.

- All alignment should be easily accessible both in dry and rainy seasons to enable maintenance throughout the year.

- Certain areas such as query sites, tea, tobacco and saffron fields and rich plantation, gardens and nurseries that will present the owner problems in acquisition of right of way and way leave clearance during construction and maintenance should be avoided.

- Angle point should be selected such that shifting of the point within 100 m radius is possible at the time of construction of the line.

- The line routing should avoid large habitation densely populated areas to the extent possible

- The area requires special foundations and those prone to flooding should be avoided

- For examination of the alternatives and identification of the most appropriate route, besides making use of information/data/details available/extracted through survey of India topographical maps and computer aided processing of NRSA satellite imagery, the contractor shall also carry out reconnaissance/preliminary survey as may be required for the verification and collection of additional information/data/details.

- The contractor shall submit his preliminary observation and suggestion along with various information/data/details collected and also processed satellite imagery data, topographical map data marked with alternative routes etc.  The final evaluation of the alternative routes shall be conducted by the contractor in consultations with owners representatives and optimal route alignment shall be proposed by the contractor.  Digital terrain modeling using contour data from topographical maps as well as processed satellite data shall be done by the contractor for the selected route.  A flythrough perspective using suitable software(s) shall be developed or further refinement of the selected route.  If required site visit and field verification shall be conducted by the contractor jointly with the owners representatives for the proposed route alignment.

- Final digitized route alignment drawing with the latest topographical and other details/features including all river railway lines, canals, roads etc. up to 8 Km on both side of selected route alignment shall be submitted by the contractors for owners approval along with report containing other information / details as mentioned above.

- 4.2   765 KV S/C Seoni-Bina transmission Line

- Based on above, the most optimum route alignments are finalized. In the instant project also such studies have been carried out and final route alignment has been finalized and chosen for construction.  The major features of transmission line covered under this project are as follows:

Seoni and Bina are located in Madhya Pradesh and the distance between the two Cities (Bee Line) is 265 Km .On both sides of the bee line vast amount of forest exist. Careful route selection has avoided these sections, however small amount of forest that could not be avoided completely is getting affected, details of which are described separately.

Road network is present all throughout the corridor and at some remote places un-metalled roads and Cart-tracks exist and are motorable in dry seasons .The major roads that exist are NH-7, NH-12,NH-86 and other State Highways. Rail Network also exists in the corridor primarily the Itarsi – Jabalpur and Bina - Sagar of West Central Railway.The main rivers that flow in this corridor between Seoni to Bina are:

- Wain ganga River

- Narmda River

- Bewas River

None of these rivers have a larger width at crossing points or higher water level that requires special attention.

The nearest airport to the transmission line is Jabalpur at about 90 KM. Railway Station exist all along the route, the prominent stations would be Seoni , Narsinghpur, Sagar and Bina.

Both unskilled and semi-skilled labour for construction purposes are available locally  in nearby villages along the line. Local labours are quite conversant with the normal construction activity. The prevailing rate of unskilled labour is Rs 101 per day and that of semi-skilled labour is Rs 108 per day for a fair days work.

Details of final route alignment along with map are placed as Annexure-2.

Details of Forest involvement:

The subject line involves 111.98 ha in Madhya Pradesh. Forest proposal for obtaining clearance of MOEF submitted to concern forest authorities (Annexure-3). Affected forest comprises of Rerserve/Protected forest. Major sp. that  will be affected are Tectona grandis, Anogeissus latifolia, Terminalia chebula, Diospyros melonoxylon, Lagerstroemia Parvirlora, Semecarput anacardium, Buchanania lanzan, Butea monosperama(Lamk)  and other miscellaneous sp. The total no. of trees coming in the entire ROW is 39650. However, felling of trees has been restricted to barest minimum and rests are only lopped/pruned to maintain electrical clearance.  A detailed compensatory afforestation scheme for carrying out afforestation in lieu of above diversion is prepared by forest authorities at a cost of Rs. 1,60,20,663/- (Annexure-4)

Forest Clearance: Proposal after inspection by Chief Conservator of Forest (CCF) Central, Regional MOEF, Bhopal on 09.07.07, was forwarded to MOEF, New Delhi on 20.07.07. The proposal was discussed in Forest Advisory Committee (FAC) on --- and recommended for approval. The FAC recommendation was put up before Central Empowered Committee (CEC) a body constituted by Hon’ble Supreme Court of India for its review and recommendation to Hon;ble Court for approval. The CEC vide its report dt.14.11.07 to Hon’ble Court recommended it for approval. The Hon’ble court has granted its permission for approval vide its order dt. 15.2.08. In-principle approval for diversion of 111.98 ha. has been obtained from MOEF vide its letter dt. 12.5.08 (Annexure-5)

(Environmental Impact Matrix)

| S. No. | PARAMETERS | Extent of Impact |
| --- | --- | --- |
| 1. A. | Line length- 292 Kms. Madhya Pradesh -292 Km | -- |
| B. | Terrain:- Plain - 274 km (93.84%) Hilly - 18 Km (06.16%) |  |
| 2. | Type of line, 765 KV S/C | -- |
| 3. | Forest land traversed (Km) | 17.497 |
| 4. | Forest land traversed (Ha) | 111.98 |
| 5. | Forest type | Reserve forest ( 14.464 ha ), Protected forest ( 97.516 ha .) |
| 6. | Forest density | 0.0 to 0.4 |
| 7. | Rare/ endangered flora | Nil |
| 8. | Rare/ endangered fauna | Nil |
| 9. | Migrating Wild life/ breeding ground | Nil |
| 10. | National Park / sanctuaries | None |
| 11. | Amt. of wet land traversed | Nil |
| 12. | Soil erodability | low |
| 13. | Historical / Cultural Monument | None |
| 14. | Relocation of villagers | None |
| 15. | Loss/ Hindrance to Public Utilities | Negligible, restricted to construction phase only. |

SECTION-V:  POTENTIAL ENVIRONMENTAL IMPACT, EVALUATION AND ITS

MANAGEMENT

- 5.0	Impact Due to Project Location

Although, all possible measures have been taken during the finalization of route alignment as described in the earlier chapter for the proposed transmission system but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to possible environmental impact and measures taken to minimize the same are as follows:

- (i)	Resettlement

- As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be noted from the above description that final route alignments do not impact habitation. Moreover, keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

- The proposed project do not envisages construction of any new substation and extension of only two existing substation is involved where sufficient land is already available for proposed bays. Hence no fresh land acquisition or R&R issues are involved in the present projects.

- (ii)	 Land value depreciation

- Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

- (iii) Historical/cultural monuments/value

- As per the assessment carried out during finalization of route alignment in consultation with State revenue authorities and ASI, no such monuments are getting affected in the instant project.

- (iv)	Encroachment into precious ecological areas

- As already explained all precautions have been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries. However, complete avoidance of forest area was not possible though National Park/Sanctuary or any other protective areas have been completely avoided.  But the routes of proposed transmission line are so finalized that it affects minimum forest area, which has also been certified by concerned Divisional Forest Officer (DFO)  Annexure- 3 . In order to minimize forest involvement and subsequent environmental impact special tower design for 765 KV line has been developed which has reduced the requirement of ROW by 21 m (from 85 m to 64 m) thereby saving large area of forest.  It may be  seen  from  the  earlier description   that  out of total transmission line length of about 292 Km about 17.49 Km (5.98% only) length shall  pass  through  forest  land  consisting  of  111.98 Ha. forest area  in  the  states  of   Madhya Pradesh. A total of 57 towers are coming in the forest area. Prior approval of GOI/MOEF has been obtained for affected forest under Forest (Conservation) Act, 1980 (Annexure- 5). Most  of  the forests to  be  traversed  by  the  lines  are  already  heavily  degraded  and the  wildlife species  present  are  those  who  have  been  adapted  to  open  or  disturbed habitat. Therefore with provision of Compensatory Afforestation on equivalent area 112 ha. of non forest land , the  overall  forest status  will  in  many  cases  improve (refer Annexure-4). Nonetheless, to  mitigate  losses  to  existing forests, clearing  of  the  transmission  line  Right-of-way  will  be done  under supervision  of  Forest  Department, and  some  low  canopy  seed  trees  and  shrubs may  be  kept  intact  if  they  do not  interfere  with  tower  erection and  line installation. The wood will be sold by the Forest Department, who will also retain the sale proceeds. Three-meter wide strips of land under each conductor will be cleared and maintained as maintenance rows, but the remaining land will be allowed to regenerate. Lopping of trees to maintain line clearance will be done under the direction of Forest Department. POWERGRID will provide construction crews with fuel wood or alternative fuels as a precaution against collection of fuel wood from nearby forest.

- Transmission lines can serve as new access routes into previously inaccessible or poorly accessible forests, thereby accelerating forest and wildlife loss. In such cases, POWERGRID cannot take action itself, but local Forest Department personnel will normally assess the dangers and take appropriate action, such as establishing guard stations at  the  entrance  to  the  forest etc. cost of which is borne by POWERGRID and is included in the compensatory afforestation scheme.  Given the already easy access and degraded conditions at the proposed projects sites, this problem is not expected to be encountered.  Nonetheless, POWERGRID staff will report to the Forest Department any noticeable encroachment induced by the Projects.

- (v)     Encroachment into other valuable lands

- Impacts on agricultural land are restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1).

- In areas where lines will traverse agricultural land, compensation will be paid to owners for any crop damage incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation.  Revenue department, after evaluating the loss due to construction activity and productivity of land, arrives at the compensation cost that is paid to farmer.   Agricultural activities will be allowed to continue  following the

- construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner's satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss of products from the tree over an eight year period (for fruit bearing trees) and for other trees compensation is finalized in consultation with local forest authorities. Out of total tower location of 783, 726 tower locations are coming either on private land or revenue land.  Agricultural lands under private ownership are being identified, and in accordance with normal POWERGRID procedures compensation is paid to the affected villagers towards crop and trees damages.  It is estimated that total compensation towards crop/trees shall be the tune of Rs. 200 lakh. Budgetary provision Rs. 292 lakh @ Rs. 1.00 Lakh per Km is made in the cost estimate to meet these expenses (Annexure-6).

- (vi)	 Interference with other utilities and traffic

- As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission lines affect nearby telecommunication circuits by causing electrical interference.  A standing committee -- Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like rerouting of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

- The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50000/- per km for POWERGRID. Provision to meet these expenses has been made in the cost estimate for the same.

- Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission lines pass by the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights are placed atop these towers. All necessary clearance have already been obtained or in the advance stage of processing with relevant authorities.

- (vii) Interference with drainage pattern

- As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings which is very small, there is little possibility

- of affecting drainage pattern.  Since in the instant project most of the line is being constructed in the plain area no such impact is encountered till date and  during further implementation if such  instances are reported,  flow will be trained and guided to safe zones.

- 5.1	Environmental Problems Due to Design

- (i)	Escape of polluting materials

- The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials. To avoid/minimize during construction phase a clause has been included in the contract document and is being monitored regularly by the site engineers (refer EMP).

- (ii)	Explosion/fire hazards

- During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires. Fires due to flashover from lines can be a more serious problem in forest. However, adequate safety measures shall be taken to avoid such incidence besides this forest authorities also incorporate measures like making fire lines to prevent spreading of fire in the affected forest area.

- (iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

- Adequate measures are taken to re-surface the area where excavation works are done.Topsoil disturbed during the development of sites is used to restore the surface of the platform. Infertile and rocky material are removed carefully and used as fill for tower foundations and rest is disposed at selected dumping areas.

- (iv)	 Environmental aesthetics

- Since spacing between the towers in case of 765/400 KV lines is approx. 400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible.   POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted grey or green to merge with the background.

- (v)	 Noise/vibration nuisances

- The equipment installed at sub-station are mostly static and are so designed that the noise level  always  remains  within  permissible  limits  i.e. 85  dB  as  per   Indian standards. The noise levels reported during normal operating conditions are about 60 to 70 dB at 2 m. distance from the equipment.  To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipment are undertaken.  In addition, plantations of sound absorbing species like Casuarinas, Tamarind, and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.  Actual noise levels measured at perimeters of existing Substations are 20 to 30 dB.

- Noise during construction phase is periodically monitored and due maintenance of equipments are ensured to keep the noise level well within the prescribed limit.

- (vi)  Blockage of wildlife passage

- Since the line is passing through mostly agricultural, wasteland and the affected forest area is also not a migration path of wild life hence, possibility of disturbance to wild life are nil/remote.

- 5.2	Environmental Problems During Construction Phase

- (i)	Uncontrolled silt runoff

- The Project involves only small scale excavation for tower foundations at scattered locations that are re-filled with excavated material as the major portion of project area (94%) is in plain hence uncontrolled silt run off is not expected.

- (ii)  Nuisance to nearby properties

- As already described in preceding paras, during site selection due care is taken to keep the transmission line and substations away from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour therefore nuisance to the nearby properties if any, is not expected. The construction activities are normally undertaken in lean period and post harvesting to avoid/minimize such impact Exhibit-4.

- (iii)  Interference with utilities and traffic and blockage of access way

- Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures. Using details collected using GIS and GPS during  route alignment,  It may be noted that access road ( Metalled / Non-metalled /cart roads) are existing to access all angle points and construction of no new road is envisaged (taking 250 m as buffer zone which can always be accessed through head load) for this line.   Even if at some places it is found that access road is not available than existing field/path is upgraded/ augmented for utilization and compensation for any damage to crop or field is paid to the owner. In many areas such improvement in the access road is highly appreciated by the local population.

- As and when a transmission line crosses any road/ railways line, the short span angle (DT) towers are located at a distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

- (iv)	Inadequate resurfacing for erosion control

- Since proposed line is to be constructed in plain area where erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained above adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is generally undertaken outside the rainy season. As the proposed lines are mostly passing through plain (94%0 areas no such problems are anticipated.

- (v)	Inadequate disposition of borrow area

- As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area. As such acquisition/opening  of  borrow area is not needed.

- (vi)  Protection of Worker's health/safety

- The Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents will guide provisions for workers’ health and safety. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the Erection Conditions of Contract (ECC), a part of bidding documents (Annexure-7).

POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Deptt. POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. These guidelines/checklist include work permits and safety precautions for work on the transmission lines both during construction and operation and is monitored regularly by site in-charge and corporate Operation Services (Annexure-8). In addition training is imparted to the workers in fire fighting and safety measures (Exihibit-5).  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act. Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses are  disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID conduct awareness building programs on such issues for the construction workers.

- 5.3	Environmental Problems Resulting from Operation

- (i)     O&M Staff/Skills less than acceptable resulting in variety of adverse effects

- The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However in respect of the long distance transmission lines there are monitoring offices that are located at various points en-route. Monitoring measures employed include patrolling and thermo-vision scanning. The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

- A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of  PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard.  Because of issues relating to need to ensure health and safety relating to the line such as fire safety, safe voltages on metallic parts of buildings, and safety clearances to avoid flashover, the transmission lines will not pass directly over any residential properties and as such the potential for EMF effects to occur will be further diminished. Given that it will be necessary to ensure that there are no properties in the ROW beneath and to the sides of the overhead line, automatic mitigation against EMF will be provided between the source of potentially high strengths (the transmission line) and the residential properties.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India :

“ Power Grid was the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid do not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

5.4     CRITICAL ENVIRONMENTAL REVIEW CRITERIA

(i)	Loss of irreplaceable resources

The transmission projects do not involve any large scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As only 17.49 Km. (111.98 ha.) of the lines in the subject project are passing through forest area some loss of vegetation is anticipated but with the provision of compensatory afforestation on equivalent  area affected (112 ha.) will not only compensate the loss of vegetation but will increase the forest area. The non forest land identified for such plantation by forest department is in the Katni  forest divisions of the concerned State (MP). The cost of such plantation paid to forest department is Rs. 160.26 lakhs (refer Annexure-4).

(ii)	 Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as in the affected  forest  thus there is no possibility of endangering/causing extinction of any species.

(iv)	Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

5.6    PUBLIC CONSULTATION:

Public consultation/information is an integral part of the project implementation. Public is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted.

Apart from this, public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting  as  per  Environmental Social Policy  & Procedures of POWERGRID  (ESPP) shall also be carried out during different activities of project cycle. During such consultation the public will be informed about the project in general and in particular about the following:

- complete project plan (i.e. its route and terminating point and substations, if any, in between);

- POWERGRID design standards in relation to approved international standards;

- Health impacts in relation to EMF;

- Measures taken to avoid public utilities such as school, hospitals, etc.;

- Other impacts associated with transmission lines and POWERGRID’s approach to minimizing and solving them;

- Land acquisition details, proposed R&R measures and compensation packages in line with POWERGRID’s policy;

- Trees and crop compensation process.

Apart from organising many informal group meetings in different villages a public meeting was also organised in the village Harri of Chindwara distt. of Madhya Pradesh on 28.9.04.  To get the maximum participation during the Public consultation Programme a notice was served well in advance to the villagers .  The   details of line and its importance was explained to the villagers by the Chief Manager/Seoni.  About sixty villagers including Surpanchs, M/s. P. Prabhakar Reddy (Survey Agency) representative and POWERGRID representative participated in the programme.  Pamphlets in local language were circulated during the programme amongst the participants. The programme was arranged in interactive way and queries like crop compensation etc. were replied.  The programme was appreciated by the villagers including Sarpanch and they assured to extend their cooperation for construction of transmission project. The process of such consultation shall continue during project implementation and even during O&M stage. The details of consultation, Pamphlet etc. are placed at Annexure-9.

5.7	CONCLUSIONS :

From the above discussion, it would seem that the area is rich in physical resources. But careful route selection has avoided involvement of any National Park/Sanctuary or protected area but involvement of forest could not be avoided due to terrain and other physiographical reasons. Thus, routes selected for detailed survey are the most optimum route alignment and involved minimum forest and no R&R issues are anticipated as no fresh land acquisition is involved and land for extension of existing sub-staion is already available. Hence, based on the criteria agreed with The Bank the project is coming under the P2 category or non sensitive as only 5.9% of total length is passing through forest which is already degraded in nature and no impact on wild life is envisages as the forest is not the habitat of any endangered or endemic sp. of fauna and flora.  The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts while on the one hand underline the need for implementation of the project for overall development of the area and on another hand suggest that a detailed E.I.A. may not be necessary.

# SECTION-VI:  MONITORING AND ORGANISATIONAL SUPPORT STRUCTURE

6.0      ENVIRONMENTAL MONITORING PROGRAM IN POWERGRID:

- Monitoring is a continuous process for POWERGRID projects at all the stages be it the site selection, construction or maintenance.

- The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan :

- 6.1    Corporate Level

- An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and Site to carry out environmental and social surveys for new projects.

- Assisting RHQs and site to finalize routes of entire power transmission line considering environmental and social factors that could arise enroute

- Help RHQs and Site to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & site Officials on environment and social issues and their management plan.

- 6.2     Regional Level

- At its Regional Office POWERGRID has a Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the Corporate level and the Construction Area Office (CAO). The key functions envisaged for ESMC are :

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and CAO to finalize routes of entire power transmission lines considering the environmental and social factors that could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and CAOs on various environmental and social aspects.

6.3      Site Office

- At the Site level , POWERGRID has made  the head of the CAOs responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social  Management Team (ESMT) . Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalise the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorised Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

- As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park , it  is generally  done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties. A monitoring system (done by the Forest Department) is also in place for compensatory forests established as part of the Project.  A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn and is being implemented at site during various stage of project execution. The updated EMP with compliance status for each line is enclosed as Annexure-10. The budgetary estimates for EMP implementation has been made based on past experience and practice and are placed at Annexure-11.

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID’s CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan. The copy of recent  annual audit report is placed as Annexure-12.