SEONI-BINA 765 KV S/C TRANSMISSION SYSTEM

Environment Management Plan

| Project activity /stage | Potential Impact |  | Proposed mitigation measures | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule | Compliance Report |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre construction |  |  |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks |  | Setback of dwelling to c\overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Tower location and line alignment selection with respect to nearest dwellings | Setback distance to nearest houses – once | POWERGRID | Part of tower siting survey and detailed alignment survey and design | Careful route alignment had ensured that no house/dwelling unit is coming in the ROW. |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors(air, water, land) |  | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformer stated in tender specifications –Once | POWERGRID | Part of tender specifications for the equipment | Compiled and included in tender document |
|  |  |  | Process, equipment and system not to use chlorofluorocarbons(CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirement of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – Once | POWERGRID | Part of tender specifications for the equipment | Compiled and included in tender document |
|  |  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design | Not applicable |
| Transmission line design | Exposure to electromagnetic |  | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Electromagnetic field strength for proposed line design | Line design compliance with relevant standards – Once | POWERGRID | Part of detailed alignment survey and design | Designs are in compliance with international standards as certified by PTI,USA,CPRI Banglore |
| Location of transmission towers and transmission line alignment and design | Impact on water bodies and land |  | Consideration of tower location at where they could be located to avoid water bodies | Tower location and line alignment selection (distance) to water bodies | Consultation with local authorities and avoiding tower foundation in water bodies. | POWERGRID | Part of tower siting survey and detailed alignment survey and design | Complied and no tower is placed in water bodies. |
|  | Social inequities |  | Careful route selection to avoid existing settlements | Tower location and line alignment selection (distance to nearest dwelling or social institutions) | Consultation with local authorities and land owners – ONCE | POWERGRID | Part of tower siting survey and detailed alignment survey and design | Careful route selection and provision of adequate extensions has avoided the habituated area to the extent possible. |
|  |  |  | Minimize need to acquire agriculture land | Tower location and line alignment selection (distance to agricultural land) | Consultation with local authorities and land owners – once | POWERGRID | Part of tower siting survey and detailed alignment survey and design | No agricultural land is acquired for tower location and people are consulted at every stage. |
| Encroachment into precious ecological area | Loss of precious ecological values/damages to precious species |  | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest designated ecological protection area) | Consultation with local forest authorities to avoid/minimize forest involvement – once | POWERGRID | Part of detailed siting and alignment survey / design | Careful route selection has avoided these crossings and 111.98 Ha. (17.49 Km.)of forest land in the State of Madhya Pradesh is involved in the line. |
| Transmission line through forestland | Deforestation and loss of biodiversity |  | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest protected or reserved forest) | Consultation with local authorities – once | POWERGRID | Part of detailed siting and alignment survey /design | Adequate extensions and reduction in ROW from 85 to 64 m resulted in minimization of trees cutting and 39650 trees are getting affected in forest area calculated in complete ROW. Tree cutting shall be further reduced during construction. |
|  |  |  | Minimize the need by using existing towers, tall towers, and ROW, wherever possible |  | Consultation with local authorities and design engineers- once |  |  |  |
|  |  |  | Obtain statutory clearances from the Government | Statutory approvals from Government | Compliance with regulations-once for each sub project |  |  | Forest clearance obtained from Ministry of Env. & Forests, Govt. of India. |
| Encroachment into farmland Loss of agricultural productivity |  |  | Use existing tower footings/towers wherever possible | Tower location and line alignment selection | Consultation with local authorities and design engineer- once | POWERGRID | Part of detailed alignment survey and design | Foundations cast during lean period to avoid damage to the crops during harvest. |
|  |  |  | Avoid siting new towers on farmland wherever feasible | Tower location and line alignment selection | Consultation with local authorities and design engineers – once |  | Part of detailed siting and alignment survey / design | Due care taken to avoid the damage to the extent possible. |
|  |  |  | Farmers compensated for any permanent loss of productive land | Design of Implementation of Crop Compensation (based on affected area) | Consultation with affected parties – once in a quarter |  | Prior to construction phase | Compensation being paid for all damages if any. |
|  |  |  | Farmers / land owners compensated for significant trees that need to be trimmed / removed along ROW. | Design of implementation of Tree compensation (estimated area to be trimmed / removed) | Consultation with affected parties – once in a quarter |  | Prior to construction phase | Tree compensation being paid for all damages during construction. |
|  |  |  |  | Statutory approvals for tree trimming / removal | Compliance with regulations-once for each sub project |  | Part of detailed siting and alignment survey/ design | Being complied with regularly. |
| Interference with drainage patterns/Irrigation channels | Flooding hazards / loss of agricultural production |  | Appropriate siting of towers to avoid channel interference | Tower location and line alignment selection (distance to nearest flood zone) | Consultation with authorities and design engineers-once | POWERGRID | Part of detailed alignment survey and design | Complied and no blocking of any water channel is anticipated. |
| Construction |  |  |  |  |  |  |  |  |
| Equipment layout and installation |  | Noise and vibration | Construction techniques and machinery selection seeking to minimize ground disturbance | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance – once at the start of each construction phase | POWERGRID (Contractor through contr act provisions) | Construction period | Complied and periodic monitoring of machinery is done. |
| Physical construction |  | Disturbed farming activity | Construction activities on cropping land timed to avoid disturbance of field crops(within one month of harvest wherever possible) | Timing of start of construction | Crop disturbance – Post harvest as soon as possible but before next crop-once par site | POWERGRID (Contractor through contract provisions) | Construction period | Foundation being planned in lean period or avoided during harvest. (Exhibit-4) |
| Mechanized construction |  | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment-estimated noise emissions | Complaints received by local authorities – every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | All equipments being checked regularly. |
|  |  | Noise vibration, equipment wear and tear | Turning off plant not in use | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities – every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Complied. |
| Construction of roads for accessibility |  | Increase in airborne dust particles | Existing roads and tracks used for construction and maintenance access to the line wherever possible | Access roads, routes (length and width of new access roads to be constructed) | Use of established roads wherever possible – every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | No new roads are being constructed in the present project rather some existing road/path has been strengthened/repaired. |
| Temporary blockage of utilities |  | Over flows, reduced discharge | Temporary placement of fill in drains/canals not permitted | Temporary fill placement | Absence of fill in sensitive drainage areas – every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Nil |
| Site clearance |  | Vegetation | Marking of vegetation to be removed prior to clearance, and strict control on clearing activities to ensure minimal clearance | Vegetation marking and clearance control ( area in m2) | Clearance strictly limited to target vegetation – every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Included in contract provisions and being monitored regularly. An area of 10-15 x 10-15 m2 is being cleared tower foundation at each location depending on the type of tower. In rest of ROW trees that are coming in the electrical clearance zone are cleared. |
|  |  |  |  |  | Presence of target species in ROW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period | Parameter is being monitored and taken care during const. |
|  |  | Trimming/cutting of trees within ROW | Fire Hazards | Trees allowed growing up to a height within the ROW by maintaining adequate clearance between the top of tree and the conductor as per the regulations | .Species – specific tree retention as approved by statutory authorities (average and maximum tree height at maturity in meters) | POWERGRID (Contractor through contract provisions) | Construction period | Parameter is being monitored and taken care during const. Controlled felling coupled with other safety measures applied restrict any such incident. No such case of fire has been reported till date. |
| Wood /vegetation harvesting |  | Loss of vegetation and deforestation | Construction workers prohibited from harvesting wood in the project area during their employment (apart from locally employed staff continuing current legal activities) | Illegal wood / vegetation harvesting (area in m3 number of incident reported) | Complaints by local people or other evidence of illegal harvesting every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Provision for providing fire wood/ fuels etc. are included in contract. All such provisions are complied by contractor and is being monitored and taken care during const by the site in charge. |
| Surplus earthwork / soil |  | Runoff to cause water pollution, solid waste disposal | Soil excavated from tower footings disposed of by placement along roadside, or at nearby house blocks if requested by landowners. | Soil disposal locations and volume (m3) | Acceptable soil disposal sites – every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Excavated earth is being used for refilling. The top/ fertile soil is kept separately for resurfacing and other earth is used for refilling . Approx. 100-120 m3 earth is excavated at each tower location and 90-95% of this is used for refilling/resurfacing and rest is being disposed off along with other debris at selected location. |
| Tower construction-disposal of surplus earthwork/fill |  | Waste disposal | Excess fill from tower foundations excavation disposed of next to roads or around houses, in agreement with the local community or landowner | Location and amount (m3) of fill disposal | Appropriate fill disposal locations- every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | --Do--- These provisions are strictly complied and recorded during construction. |
| Storage of chemicals and materials |  | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports &type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles – every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Complied and condition is being taken care during storage. |
| Construction schedule |  | Noise nuisance to neighboring properties | Construction activities only undertaken during the day and local communities informed of the construction schedule |  | Weeks | POWERGRID (Contractor through contract provisions) | Construction period | It is ensured by site in-charge that construction activities takes place during day time and villagers are informed in advance and affected villagers are even served notice in advance. |
| Provision of facilities for construction workers |  | Contamination of receptors(land, water, air) | Construction work force facilities to include proper sanitation, water supply and waster disposal facilities. | Amenities for workforce faculties | Presence of proper sanitation, water supply and waste disposal faculties - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period | Complied and included in the contract provision. Adequate sanitation facilities are provided in the labour camp. |
| Encroachment into farmland |  | Loss of agricultural productivity | Use existing access roads wherever possible | Usage of existing utilities | Complaints received by local people / authorities every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period | Mitigation measures are followed and monitored regularly. No new roads are being constructed in the present project rather some existing road/path has been strengthened/repaired. |
|  |  |  | Ensure existing irrigation facilities are maintained in working condition | Status of existing facilities |  |  |  | No irrigation facilities is affected or blocked. |
|  |  |  | Protect/ preserve topsoil and reinstate after construction completed | Status of facilities (earthwork in m3) |  |  |  | All measures to resurface the excavated area by top soil is adopted as described above. |
|  |  |  | Repair/reinstate damaged bunds etc after construction completed | Status of facilities in m3) |  |  |  | Complied. |
|  |  | Social inequities | Compensation for temporary loss in agricultural production | Implementation of crop compensation (amount paid, dates, etc) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction | Compensation paid regularly Annexure-6. |
| Uncontrolled erosion / silt runoff |  | Soil loss, downstream siltaion; | Need for access tracks minimized, use of existing roads. Limit site clearing to work areas. | Design basis and construction procedure | Incorporating good design and construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period | Parameter is being monitored and taken care during construction. |
|  |  |  | Regeneration of vegetation to stabilize works areas on completion (where applicable) |  |  |  |  | Regenration / cultivation is allowed in the complete ROW and even in the area below tower after completion of construction activities. |
|  |  |  | Avoidance of excavation in wet seasons |  |  |  |  | It is ensured by the site in-charge that no excavation is carried out during monsoon/rainy season. |
|  |  |  | Water courses protected form siltation through use of bunds and sediment ponds |  |  |  |  | N.A. |
| Nuisance to near by properties |  | Losses to neighboring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices- once for each site | POWERGRID (Contractor through contract provisions) | Construction period | Complied and included in contract conditions. |
|  |  |  | As much as possible existing access ways will be used | Design basis and layout | Incorporating good design engineering practice- once for each site |  |  | Parameter is being monitored and taken care during construction. |
|  |  |  | Productivity land will be reinstated following completion of construction | Reinstatement of land status(area affected, m2) | Consultation with affected parties-twice – immediately after completion of construction and after the first harvest |  |  | Parameter is being monitored and taken care during construction. |
|  |  | Social inequities | Compensation will be paid for loss of production, if any | Implementation of Tree/crops compensation (amount paid) | Consultation with affected parties – Once in a quarter | POWERGRID | Prior to construction | Crop compensation paid regularly and taken care during construction. |
| Health and safety |  | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work day caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period | Complied and included in contract conditions. No incident of accident/injury reported till date. |
|  |  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  | All health and safety plan are in place and monitored regularly Annexure-8 / Exhibit-5. |
|  |  |  | Contractor to arrange for health and safety training sessions |  |  |  |  | Regular briefing / training for labour is organized by contractor/POWERGRID Exhibit-5. |
| Inadequate construction stage monitoring |  | Likely to maximize damages | Training of POWERGRID environmental monitoring personnel. | Training schedule | Number of program attended by each person – once a year | POWERGRID | Routinely throughout construction period | Under implementation as periodic training programs are being organized for such persons. |
|  |  |  | Implementation of effective environmental monitoring and reporting system using check list of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof | Submission of duly completed checklists of all contracts for each site-once |  |  | Complied. Regular monitoring by site, RHQ and Corporate is being organized. |
|  |  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures | Compliance report related to environmental aspect for the contract | Submission of duly completed checklists of all contracts for each site-once |  |  | All provisions are complied and monitored regularly by Site/RHQ. |
| Operation and Maintenance |  |  |  |  |  |  |  | N.A. YET |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks |  | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Compliance with setback distance (“as built” diagram) | Setback distances to nearest houses – once in quarter | POWERGRID | During operation |  |
| Oil spillage | Contamination of land / nearby water bodies |  | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks | Substain bundign (oil sump) (“as built” diagram) | Bunding (Oil sump) capacity and permeability – once | POWERGRID | During operation |  |
| Inadequate Provision of staff/workers health and safety during operations | Injury and sickens of staff / workers |  | Careful design using appropriate technologies to minimize hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |  |
|  |  |  | Safety awareness raising for staff | Training / awareness program and mock drills | Number of program and percent of staff /worker covered – once each year |  |  |  |
|  |  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |  |
| Inadequate provision of staff / workers health and safety during operation | Injury and sickness of staff workers |  | Careful design using appropriate technologies to minimize hazards | Usage of appropriate technologies (lost work day due to illness and injuries) | Preparedness level for using these technologies in crisis- once each year | POWERGRID | Design and operation |  |
|  |  |  | Safety awareness raising for staff | Training/awareness program and mock drills | Number of programs and percent of staff / workers covered – once each year |  |  |  |
|  |  |  | Preparation of fire emergency action plan and training given to staff on implanting emergency action plan |  |  |  |  |  |
| Electric shock hazards | Injury / mortality to staff and public |  | Careful design using appropriate technologies to minimize hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |  |
|  |  |  | Security fences around substation | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |  |
|  |  |  | Barriers to prevent climbing on /dismantling of transmission towers | Maintenance of Barriers |  |  |  |  |
|  |  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |  |
|  |  |  | Electricity safety awareness raising in project areas | Training / awareness programs and mock drills for all concerned parties |  |  |  |  |
| Equipment specifications and design parameters | Release of chemicals and gases in receptor(air, water, land) |  | Processes, equipment and systems using chlorofluorocarbons(CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |  |
| Transmission line maintenance | Exposure to electromagnetic interference |  | Transmission line design to comply with the limits of electromagnetic interference overhead power lines | Required ground clearance(meters) | Ground clearance – once | POWERGRID | Operations |  |
| Noise related | Nuisance to neighboring properties |  | Substation sited and designed to ensure noise will not be a nuisance | Noise level (dB(a)) | Noise level at boundary nearest to properties and consultation with affected parties if any – once | POWERGRID | Operations |  |