SECTION I:  PROJECT DESCRIPTION

(East–West Transmission Corridor Strengthening Scheme)

- Background

In Eastern Region, a large number of generation schemes like Kahalgaon Stage-II (1500 MW), Barh (1980 MW), North Karanpura (1980 MW), Maithon (1000 MW), Nabinagar (1000 MW) etc. have been planned with beneficiaries located in Western and other Regions. While transmission system for the respective generation capacity has been planned to reliably evacuate and transfer the power to the beneficiaries, due consideration needs to be given to strengthen the inter-regional corridors between Eastern and Western Regions.

Presently Western region is connected to Eastern region through a 400kV  Rourkela-Raipur D/c line in addition to 3 nos. of 220kV lines between Budhipadar and Korba/Raigarh, which is being utilized to import power from Eastern Region to Western Region. Further, to enhance the capacity of this inter-regional link, series compensation along with TCSC is under implementation. This link is also proposed to be LILO at 400/220kV Raigarh substation in Chattisgarh as a part of Vindhyachal-III (1000MW) Tr. system.

As per the projected load-generation scenario of Western Region in next 5-6 years, it is envisaged that a large quantum of power need to be imported from Eastern to Western Region. Proposed 400kV Rourkela-Raigarh-Raipur D/c line shall strengthen the existing Rourkela-Raipur inter-regional Tr. corridor between Eastern and Western Regions and would enable increase in the level of import to Western Region. However, to pool surplus power from various generation projects in Eastern Region for onward transmission to Rourkela, a 400kV D/c line between Ranchi, a pooling point in eastern Region, and Rourkela is proposed. This would provide a parallel Tr. corridor from central part of the region towards Rourkela.

1.1	Project Highlights

| a) | Project Name | : | East–West Transmission Corridor Strengthening Scheme ( Inter-regional) |
| --- | --- | --- | --- |
| b) | Location | : | Jharkhand,Orissa and Chhatishgarh |
| c) | Beneficiary States | : | Constituent States of Western Region Grid |

1.2	Scope of work

The scope of work covered under the project broadly includes the implementation of following transmission lines, substation and other associated works.

#### Transmission Lines

- Ranchi - Rourkela 400kV D/c, 143.54 Kms

ii)	Rourkela – Raigarh 400kV D/c, 210 Kms

iii)	Raigarh – Raipur 400kV D/c, 214 Kms

Substations

a)       400/220kV Ranchi (POWERGRID) Substation (Extension)

400kV  Line Bays: 2

b)      400/220 KV Rourkela (POWERGRID) Substation (Extension)

400kV	:Line Bays: 4

c)	400/220 KV Raigarh (POWERGRID) Substation (Extension)

400kV:Line Bay: 4

d)      400/220 KV Raipur (POWERGRID) Substation (Extension)

400kV:Line Bays: 2

1.3    Benefits of the project

The system will strengthen the inter-connection between Eastern and Western Regions.  It will help export of surplus power from Eastern Region to Western Region as well as to maintain system stability, security of the combined grid (comprising Eastern, North-eastern, Northern and Western Regions) under all operating conditions. The system will also increase the capacity of National Grid

1.4	Beneficiaries

The target beneficiaries of this project are the Constituent States of Western Region Grid.

A power map showing the East–West Transmission Corridor Strengthening Scheme is placed as Exihibit-1.

SECTION II: BASE LINE DATA

2.0  	The project is an inter-state one and is spread/located in the States of Jharkhand, Orissa, Chhattishgarh. The basic details of the area under project are given below:

  - Jharkhand

Jharkhand, carved out from Bihar, accounts for 45% of former Bihar’s geographical area.  The State  is bordered by Bihar, Madhya Pradesh, Orissa and West Bengal to its north, south and east respectivelyThe geographical area of the state is 79,714 Km2.  About 28 % of the population of this area is tribal. Jharkhand state was made on Nov.1, The new State Jharkhand formerly a part of Bihar state, was formed on November 15th, 2000 with Ranchi as its capital. Jharkhand is the 28th state of the Indian Union.

Pysiographically it consist of both plateau and hills.  Santhalparganas and Chhotanagpur is main plateau about 3000 feet above sea level. The highest part of the plateau is Netarhat which has an elevation of 3600 ft. The Parasnath Hill is the highest point with an elevation of 4800 feet. The plateau is full of mountain ranges covered with dense forests. A number of rivers and rivulets flow down through the hilly terrains and valleys.

Climate:  The climatic condition is generally Sub-Tropical wet and dry.

Rainfall:  The mean annual rainfall varies between 100-200 Cms.

Temperature: The annual temperature varies between 20-35 0 C.

Soil: Generally mixed red and black soil along with yellow soils in few pockets is encountered in the state.

Minerals: Jharkhand is endowed with vast natural resources specially the vast variety of minerals ranging from Iron ore, Coal, Copper ore, Mica, Bauxite, Fire clay, Graphite, Kyanite, Sillimanite, Lime stone, Uranium & other minerals. Jharkhand is the leading producer of mineral wealth in the country. The total value of mineral production is amounted to over Rs. 3000crores.

Water Resources:- Rivers, tanks and groundwater are the main sources of water in State Jharkhand.Damodar, Mayurakshi, barakar, Koyal, sankh, Son, Auranga, More, Karo, Bansloi, South Koel, Kharkai, Swarna Rekha, Ganga, Gumani, Batane are some of the perennial river veins meandering through the State.

Forest: - Total geographic area of state is 79,714   Km2 covered forest area in state 28.4% of total area .Dense forest is 11,787Km2 & open forest 10,850 Km2. Total forest covered area is 22637 Km2. The State has following three forest types:

- Tropical Moist Deciduous

- Tropical Dry Deciduous

- Sub Tropical Broadleaved Hill Forests

Forest covers slightly more than one third of the geographical area of Jharkhand (Map-1). The state is rich in forest resources having Sal and Teak as the two most important forest formations.

The Ranchi-Rourkela 400KV D/C, transmission line (Jharkhand portion) will pass through following districts of Jharkhand. Details of forest cover of these districts are as follows:

Area in Km2

| S. No. | District | Geographic Area | Dense Forest | Open Forest | Total Forest | (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Ranchi | 7698 | 735 | 997 | 1732 | 22.50 |
| 2. | Simdega | 9077 | 1231 | 1255 | 2486 | 27.39 |
|  | Total | 16775 | 1966 | 2252 | 4218 | 25.14 |

Ranchi-Roukela transmission line will involve 27 Ha.  forest in these two districts, which is about 0.065% of the total forest area.

Human and Economic Development

Jharkhand’s economy is sustained by mining and heavy industry.  It is India’s second most important source of coal, mining 26 % of the total.  In minerals Jharkhand is probably India’s richest state.  Jharkhand has rich reserves of iron ore and coal and several industries. Minerals, forest produce and agricultural land constitute the principal resource base of Jharkhand. Other factors adding to strength of the economy are moderate climate, adequate availability of surface and ground water, presence of a number of Industrial giants, technical institutions and cheap labour. Nearly 80 per cent of the State's population depends on agriculture. With varied agro-climatic conditions, Jharkhand provides scope for production of a wide range of agricultural/horticultural crops, spices and medicinal plants

The largest four cities are Jameshedpur, Dhanbad, Ranchi and Bokaro Steel City.  The two major steel plants of India are located in Jharkhand: at Jamshedpur and Bokaro.  Road length: 4, 310 km, including 1,500 km NH and 2,711km SH.  The state has a well developed railway system. Ranchi, Bokaro, Dhanbad, Jamshedpur are some of the major railway stations.  Agriculture is poorly developed.  Irrigation is negligible.  Drought therefore is a periodic threat.

2.2	Orissa

The geographic area of Orissa is 15.57 million ha, which constitutes 4.74 % area of the country. It lies in the tropical zone between lat. 170 47’ and 220 34’ N and long. 810 22’ and 870 29’ E.

Physiography

The state can be divided into four distinct physiographic regions viz. Northern Plateau, Eastern Ghat, Central table land and coastal plains. The state is drained by three major rivers, the Mahanadi, the Brahmani and the Baitarni.

On the basis of homogeneity, continuity and physiographical characteristics, Orissa has been divided into five major morphological regions: the Orissa Coastal Plain in the east, the Middle Mountainous and Highlands Region, the Central plateaus, the western rolling uplands and the major flood plains.

Rainfall

The average annual rain fall varies between 1,200 mm to 1,600 mm.

Temperature

The average annual temperature ranges between 250 C to 27.50 C.

Climate

Orissa has an equable climate, neither too hot nor too cold. In some places, however, extremes of climate are experienced, namely in the Western district like Bolangir, Sambalpur and Sundergarh. Summer temperature varies 20ºC-45ºC. Winter 13ºC-32ºC.

Mineral resources

About 80% of the state's landmass is underlain by pre- cambrian rocks, the oldest rocks in the earth's crust. The remaining area is composed of rocks belonging to Gondwanas, Quarternary & Recent formations. The geological set up of the state is not only favourable for discovery of wide variety of valuable ores and minerals but there is distinct possibility of substantial increase in the resources. The main mineral resources  found in Orissa are  coal, Iron & Manganese, Limestone & Dolomite,Bauxite,Nickel, Manganese, Graphite,Chromite, Serilite Quartzite, Quartz Serilite schist, Serilite schist, Andalusite schist and genesis,Beach sand, Sand stone and decorative stone etc

Water resources

Orissa is endowed with water in abundance. As per Ground Water Board, the total ground water resources in Orissa amounted to 2,101,128 hectare meter. There are 4 groups of rivers which flow through Orissa in to bay of Bengal and thus make Orissa water rich state. The main rivers are Subarnarekha, Brahmani, Mahanadi, Budhbalanga, Salandi, Kolab and Indravati etc.

Forest resources

The recorded forest area is 58,136 km (sq.) which constitutes 37.34% of the geographic area of the state. The forests are uniformly distributed in the state except in coastal districts where they are scarce. By legal status, the Reserved Forest constitutes 26,329 km , Protected Forest 15,525 km. and  Unclassed Forest  16,282 km. Foest Map of Orissa  is enclosed as Map-2.
 The State has following  forest types:

- Tropical Semi Evergreen,

- Tropical Moist Deciduous,

- Tropical Dry Deciduous,

- Littoral and Swamp Forests.

Protected areas

There are 2 National Parks and 18 Wildlife Sanctuaries covering an area of 0.80 million ha, which is 5.11% of the geographic area of the state. Similipal National Park, also a Tiger Reserve has been declared as a Biosphere Reserve. The Gahirmatha Wildlife sanctuary, a mangrove eco-system, is a famous nesting ground of the Olive Ridely sea turtle. In the Orissa the actual forest cover is about 48,366 sq. km. Out of which 28170 sq. km is dense and  20,196 sq. km is open forest

The Ranchi-Rourkela 400KV D/C, transmission line (Orissa portion) and Rourkela-Raigarh transmission line (Orissa portion) will pass through following districts of Orissa. Details of forest cover of these districts are as follows:

Area in Km2

| Sr. No. | District | Geographic Area | Dense Forest | Open Forest | Total Forest | Percent |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Sundargarh | 9,712 | 2,722 | 1,303 | 4,025 | 41.44 |
| 2. | Jharsuguda | 2,081 | 157 | 132 | 289 | 13.89 |

The above two line  will involve 108.33 Ha. forest in these two district of Orissa, which is about 0.19% of the total forest area of both districts.

Human and economic development

In Orissa about 76% population is dependent on agriculture. The major crops are rice, pulses, oil seeds, jute, sugarcane, coconut and turmeric. The state contributes about 1/10th of rice production in India. But Orissa’s agro based economy is always upset by frequent natural calamities like floods, drought, tornado and cyclone. Orissa’s vast mineral, marine and forest resources provide adequate opportunities for industrial growth. About 170 large and medium industries along with 9000 small scale industries are running in the state. Forests have an important role in the economy of the State, particularly, amongst rural S.C. and Tribal masses. Forestry activities contribute a lot to their food security and day-to-day livelihood.  Forests also sustain the large sized livestock of about 29.3 million.

The infrastructure for the development of industry in Orissa is available. A combination of coal, iron ore, limestone, bauxite and a host of other minerals on the one hand and port facilities on the other are the unique features in Orissa. In addition, the bountiful forest resources and agricultural products provide ample scope for the development of forest-based and agro-based industries.

Iron and steel industry, aluminum, cement, ferro-maganese, ferro-chrome, galvanised pipe units, steel rolling mills and fertilizer plants.

The manufacture of paper pulp is dependent on bamboo pulp as raw material. Orissa's forests abound in bamboo resources. Other Small Scale Industries

Among the other industries beverages, tobacco, gur and molasses are important. The tobacco industry is wide spread and provides employment to large number of persons. In bidi production, the old districts of sambalpur, Sundergarh, Bolangir,

Keonjhar ,Cuttack and Balasore are important. 
Although Orissa has a large cattle population, it imports leather goods .The principal centres of leather products are Baudh, Bhanjanagar, Bolangir and Keonjhar. Modern tanneries have been set up at Cuttack and Khariar Road. This industry is run primarily on family lines, the 'Mochis ' and 'Chamars' being the two castes engaged in it. Oil pressing is a traditional occupation run on caste and family line. Mustard, groundnut, castor and til seeds are the principal raw materials used.

Some villages specialize in brass products, but owing to competition from the large manufacturing concerns, especially those manufacturing aluminium utensils, this cottage industry is decaying. The high price of brass is another handicap.

  - Chhattisgarh

Chhattisgarh has a geographic area of 1, 35,200 Sq. Km   which constitutes 4.11% of the land area of the country. It lies between lat.17047’ & 240 06’ N and long. 800 15’ & 840 24’ E. Physiographically the state is mainly plain. It consists of about one third of former Madhya Pradesh’s geographic area and population. It consists of 44 percent of MP’s total forest land.

Physiography

Located in the Central India, Chhattisgarh has been carved out of Madhya Pradesh to become the 26th State of the Indian Union. Uttar Pradesh and Jharkhand bound the State in north, in the east by Orissa, in the south by Andhra Pradesh and in the West by Madhya Pradesh and Maharashtra.

Climate:  The climatic condition is generally Sub-Tropical wet and dry

Rainfall:  The mean annual rainfall varies between 100-200 Cms.

Temperature: The annual temperature varies between 20 -35 0 C.

Soil: Generally mixed red and black soil along with yellow soils in few pockets is encountered in the state.

Mineral Resources: Chhattisgarh is rich in mineral resources like coal, iron ore & bauxite and the mines are located respectively in the districts as mentioned below:

Coal-in Bilaspur & Raipur, Bauxite-in Sarguja; Iron Ore-in Bastar, Durg & Rajnandgaon.

Water Resources: Rivers, Tanks and groundwater are the main sources of water in Chhattisgarh State. Mahanadi, Indrawati, Sheonath, Rinand Hasdeo, Mand, Eib, Pairi, Sabari and Arpa are some of the perennial river veins meandering through the State. The irrigated area in the State is only sixteen percent of the total cultivable area, while potential exists to raise the irrigated area to seventy five percent.

Forest Resources: Chhattisgarh having a geographical area of 1, 35,200 sq. km.  has  about  59,285 Sq. Km. of  recorded forest  area  which  is  about  43.93%  of  total  geographic  area. However, actual forest cover in the state is 56,448 Sq. Km, out  of  these  dense  forest  accounts  for  37,880 Sq. Km. having  crown  density  of

more  than  40% and  open  or  degraded  forest  of  18,568  Sq. Km. having crown  density  ranging  between  10-40% thereby covering 41.8% of State geographical area (Map-3). The State has following three forest types:

- Tropical Moist Deciduous

- Tropical Dry Deciduous

- Sub Tropical Broadleaved Hill Forests

Forest covers slightly less than half of the geographical area of Chhattisgarh. The state is rich in forest resources having Sal and Teak as the two most important forest formations. The maximum forest cover is recorded from Tribal district Bastar having 59.72%of forest and minimum cover of only 8.52% has been recorded from   Durg district. There are 10 Wild Life Sanctuaries and three National Parks namely the Sanjay Gandhi Udayn, Indrawat Tiger project and Kanger valley in the State covering approx. 3.61% of geographical area of the State.

Rourkela-Raigarh transmission line (Chhatishgarh portion) and Raigarh-Raipur transmission line will pass through following districts of Chhatishgarh Details of forest cover of these districts are as follows:

Area in Km2.

| S. No. | District | Geographic Area | Dense Forest | Open Forest | Total | (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Raigarh | 7086 | 1339 | 1175 | 2514 | 35.48 |
| 2. | Raipur | 16,468 | 3912 | 1542 | 5454 | 33.12 |
| 3. | Jajangir-Champ | 3,852 | 72 | 72 | 144 | 3.74 |
|  |  | Total | 5323 | 2789 | 8112 |  |

Rourkela-Raigarh and Raigarh-Raipur transmission line will not involve forest in Chhatishgarh. The forest have been avoided during routing of transmission line

Human and Economic Development

Chhattisgarh is rich in natural resources, mainly minerals, power and forest products.  Revenue from forest products and mining is (bauxite, coal and iron ore) Rs.300 Crore and Rs.400 Crores respectively.  The well endowed Baldia mines, numerous power projects, the steel plant at Bhilai and many cement factories had given a great   boost to Chhatisgarh’s economy.

The Mahanadi Basin portion of Chhattisgarh has considerable potential as irrigation facilities expand.  It has been dubbed as the country’s rice bowl. In the mid nineties, most of Chhattisgarh was still a mono-crop belt. Only one fourth to one fifth of the sown area was double cropped. When a very substantial portion of the population is dependent on agriculture, a situation where nearly 80 percent of a states area is covered only by mono cropping needs immediate attention to turn them into double crop areas. Also there are very few cash crops grown in Chhattisgarh, and there is a need to diversify the agriculture produce towards oilseeds and other cash crops.

The total agriculture production taken from two years can give some idea of the general share between Chhattisgarh and Madhya Pradesh. In 1995-96, out of a total agricultural production of 180.73 lakh metric tones, Chhattisgarh produced 54.09 lakh tones or 29.9 percent. The total gross value of agricultural output of undivided Madhya Pradesh was Rs. 18023 crores, of which Chhattisgarh produced Rs. 4102 crores or 22.8 percent. In 1997-98, out of the total agricultural production of undivided Madhya Pradesh of 252.19 lakh tones, valued at Rs. 28362 crores, Chhattisgarh produced 60.53 lakhs tones or 24 percent valued at Rs. 6854 crores or 24.2 percent. The coalfields in the north are among India’s most productive.

SECTION III:   POLICY, LEGAL & REGULATORY

FRAMEWORK

- 3.0     POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and directives and guidelines with respect to environmental and social considerations of Funding Agencies.

- 3.1     ENVIRONMENTAL

3.1.1 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under The Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for POWERGRID to take up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under The Forest (Conservation) Act, 1980

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded, hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/reconditioner or at the designated collection centers – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)Amendment Rules, 2003:

MOEF vide its notification dt. 20th May, 2003 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned / sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organization in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalizing the route alignment.

    - FUNDING AGENCIES:

- WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM)- F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative / management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.3   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- Applicable Legislations

- 3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement

on Environment and Development, 1992

- Policy statement for Abatement of pollution, 1992

- 3.2.0   SOCIAL

- 3.2.1    MANDATORY REQUIREMENTS (NATIONAL)

- National Policy on Resettlement and Rehabilitation for Project Affected Families: Ministry of Rural Development, Government of India has notified a National policy on R&R for  PAFs in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the assetless rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups.

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan/Tribal Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement  is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require neither resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.3    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- National and State-wide Laws and Policies Relating

to Land Acquisition and Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.4   RELEVANT POLICIES

- The Orissa Resettlement and Rehabilitation of Project Affected Persons Policy, 1994 (water resources projects)

- Resettlement & Rehabilitation Policy – NTPC

# SECTION-IV: MAJOR FEATURES OF FINAL ROUTE &                             ENVIRONMENTAL IMPACT

- 4.0     ROUTE SELECTION

- Environmental impact of transmission line projects are not significant and are mostly localized to Right of Way (ROW) .  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. To minimize these possible impact POWERGRID at the system planning stage itself try to avoid ecological sensitive areas like forest.  Wherever such infringements are substantial, different alternative options are considered to select most viable route alignment. For further optimization of route  modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS in route selection result in access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. In the instant project also these techniques have been used for studied of 3 alternatives route . Details of three alternative routes is enclosed as Annexure-I. .Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. The route/ site selection criteria followed by POWERGRID is detailed below:

- Environmental Criteria for Route Selection

- POWERGRID’s   route selection criteria are based on the principle of

- Avoidance

- Minimisation

- Mitigation

- For selection of optimum route, the following points are taken into consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

- Any monument of cultural or historical importance is not affected by the route of the transmission line.

- The proposed route of transmission line does not create any threat to the survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns,

whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not

possible, a route is selected in consultation with the local Divisional Forest

Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both

financial and environmental reasons.

- In addition, care is also taken to avoid National parks and sanctuaries and any other forest area rich in wild life.

- Keeping above in mind the routes of line under this transmission System has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps etc. to arrive at most optimum route which can be taken up for detailed survey using GIS/GPS and assessment of environmental & social impacts for their proper management.

- Similarly the TOR for detailed survey using modern tool like GIS/GPS also contained parameters to avoid/reduce environmental impact while deciding the final route alignment.  The major objectives for detailed survey that are part of contract are summarized below:

i.   The alignment of transmission line shall be most economical from the point of view of construction and maintenance.

ii.   Routing of transmission line through protected the reserved forest area should be avoided.  In case it is not possible to avoid the forest or areas having large trees completely then keeping in view of the overall economy, the route should be aligned in such a way that cutting of trees is minimum.

iii. The route should have minimum crossing of major river, railway lines, and national/state high ways, overhead EHP power lines and communication lines.

ii. The number of angle point shall be kept to a minimum.

v. The distance between the terminal points specified shall be kept shortest possible, consistent with the terrain that is encountered.

vi.    Marshy and low line areas, river beds and earth slip zones shall be

avoided to minimum risk to the foundations.

vii.    It would be preferable to utilize level ground for the alignment.

Crossing of power line shall be minimum.  Alignment will be kept at a  minimum distance of 300 meters from power lines to avoid induction problems on the lower voltage lines, crossings of communication lines shall be minimized and it shall be preferably at right angle, proximity and paralyses with telecom lines shall be eliminated to avoid danger of induction to them.

viii . Area subjected to flooding searches streams shall be avoided.

Restricted areas such as civil and military airfield shall be avoided.  Care shall also be taken to avoid the aircraft landing approaches.

ix. All alignment should be easily accessible both in dry and rainy seasons to enable maintenance throughout the year.

Certain areas such as query sites, tea, tobacco and saffron fields and rich plantation, gardens and nurseries that will present the owner problems in acquisition of right of way and way leave clearance during construction and maintenance should be avoided.

x.   Angle point should be selected such that shifting of the point within 100 m radius is possible at the time of construction of the line.

xi   The line routing should avoid large habitation densely populated areas to the extent possible

xii   The area requires special foundations and those prone to flooding should be avoided

xiii For examination of the alternatives and identification of the most appropriate route, besides making use of information/data/details available/extracted through survey of India topographical maps and computer aided processing of NRSA satellite imagery, the contractor shall also carry out reconnaissance/preliminary survey as may be required for the verification and collection of additional information/data/details.

xiv The contractor shall submit his preliminary observation and suggestion along with various information/data/details collected and also processed satellite imagery data, topographical map data marked with alternative routes etc.  The final evaluation of the alternative routes shall be conducted by the contractor in consultations with owners representatives and optimal route alignment shall be proposed by the contractor.  Digital terrain modeling using contour data from topographical maps as well as processed satellite data shall be done by the contractor for the selected route.  A flythrough perspective using suitable software(s) shall be developed or further refinement of the selected route.  If required site visit and field verification shall be conducted by the contractor jointly with the owners representatives for the proposed route alignment.

xv Final digitized route alignment drawing with the latest topographical and other details/features including all river railway lines, canals, roads etc. up to 8 Kms on both side of selected route alignment shall be submitted by the contractors for owners approval along with report containing other information / details as mentioned above.

Based on above the most optimum route alignments are finalized. In the instant project also such studies have been carried out and final route alignment has been finalized and chosen for construction. Final route alignment map of Ranchi-Rourkela,Roukela-Raigarh and Raigarh-Raipur 400KV, D/C transmission lines are   placed as Annexure-II. In case of Rajgarh-Raipur no forest is involved and survey was done by conventional method by avoiding environmentally sensitive area.

- Ranchi-Rourkela, 400 KV, D/C  line

- Ranchi is located in Southern part of Jharkhand and Rourkela is located in northern part of the  Orissa and the distance between two cities is about  128.9 Kms. On both sides of bee line  forests  are available. Careful route selection has avoided environmentally sensitive area, however small amount of forest could not be avoided completely, details of which are describe separately. Total length of line is 143.54 Kms.

In the vicinity of the Rourkela Sub-Station, the available corridor was over crowded with Transmission Lines. A special solution has been worked out for this location. The South Koel River Crossing near Rourkela is somewhat populated at the location where the river was its narrowest. A special solution was worked out here. Selection of suitable corridor at Ranchi-Khunti State Highway has been a bit difficult as the most optimal route selected passed through built up area. The alignment had to be specifically moved a bit in order to get a hindrance free corridor. There is only one location near the Ranchi Sub-Station where the proposed alignment could cross NH33. Should this alignment be blocked or made unavailable, it may result in increasing the alignment length by about four kilometers.

The nearest airport to the transmission line would be Ranchi and Bhubneswar

Both unskilled and semi-skilled labour for construction purposes are available locally in nearby villages along the line. Local labours are quite conversant with the normal construction activity. The minimum wages of unskilled labour and that of semi-skilled labour is notified by state govts. And payment of wages not less than the notified minimum wages is made.

The route details including available cart track, river, pond, electric line crossing, mettaled road, forest crossing etc is enclosed as Annexure-III

The major towns in (Ranchi-Rourkela 400KV D/C, transmission line) alignments area Ranchi, Khuti, Torpa, Bano,Raniya, Manoharpur, Rourkela

Rourkela –Raigarh 400 KV, D/C line

The starting point from Rourkela is the Gantry point near Power Grid Corporation Sub Station in Sector-21, Rourkela, Orissa located in  the eastern side of the Rourkela city. As per the bee line or orientation of the proposed transmission line, the movement of the line should be towards south or south-west but there are number of power lines including three 400 KV transmission lines, main railway line, roads and thickly population area besides forest area. Otherwise also there is very limited scope for alternative routes due to number of forest areas, hilly terrain and other physical features besides number of existing power lines. These constraints enforce to move towards north side and then towards west thereby encircling the Rourkela city in the north and ultimately orienting towards south-west.  The route alignment moves almost parallel to the existing transmission line. From Dairy Farm it orients towards south-west direction crosses Railway line of Rourkela-Birmitrapur Branch under SER in south of Girjatola. The alignment near AP-26 passes in the south of  Jugsai Reserved Forest. After crossing Sankh River between AP-30 and AP-31, it passes north of Rajgangpur. After traversing about 100 km distance, it crosses Sapai River and then IB River which is the major river across the alignment. It continues towards south-west direction and crosses SER railway line again the south of Rajpur   Reserved Forest. After AP-101 it moves towards west thereby crossing in the north of Hirakund Reservoir. After AP-135 it moves towards northern direction and ends up at AP-138 in the south-west of Raigarh in Thakurpall and Jampall area. The overall route is aligned almost saving reserve forest areas or covering minimum area and avoiding problematic points.

The proposed 400kV D/C Transmission Line from existing Sub-Station at Barkanna in the East of Rourkela to the proposed sub-station at Thakurpall/Janpall area in the south-west of Raigarh has to negotiate plain as well as undulated terrain with agricultural land, forest areas as well as the built up areas thus necessitating to go for suitable, economical and safe corridor. Efforts have been made to keep the alignment safe and economical for utilization of the existing facilities during the construction and maintenance work. The alignment starts from Barkanna located in the East of Rourkela and moves towards north-east then north, then west and then south-west direction thereby encircling the Rourkela city in the northern side. So the alignment is easily accessible from Rourkela. It crosses Rourkela-Birmitrapur railway line as well as National/state highways. The entire route moves almost parallel to the existing Rourkela-Raipur 400kV transmission line. It also moves almost parallel to the South-Eastern Railway Line from Rourkela to Raigarh. Therefore, the route alignment is approachable with roads and railways. The route details including village road, river crossing, State High way and NH, forest crossing crossing, type of terrain etc. is enclosed as Annexure-IV

Road net work is also available. Light as well as heavy vehicles for transportation for men and material are available at Rourkela, Bargaon, Sundergarh, Jarsuguda, Hirakund and Raigarh. In addition to above, it is also required to carry the material at different locations and transport is available all along the route alignment.

Raigarh-Raipur 400 KV, D/C,  line

Raigarh is located in eastern part of Chattishgarh  and Raipur is located in central part of Chattishgarh. The Raigarh-Raipur 400 KV, D/C, transmission line passes through normal plain at Chhattisgarh with nominal undulation.

The complete area is approachable either by national Highway and different State Highways. The route details including village road, river crossing, State High way and NH, forest crossing crossing,  type of terrain etc. is enclosed as Annexure-V

The transmission line crosses Howrah-Mumbai BG railway line of S.E. Central Railway at a point between Siliary Rly. Station and Boikunth Railway Station at a distance about 1Km. from  Siliary Rly Station. The line crosses NH-200 at a point 21 Km from Raipur and State Highway no.72 at a point 72 Km. from Raipur near village Amera. The nearest airport is situated at Ranchi and  Raipur.

- The Topography of the area is plain (87.5%) and 12.5% hilly.

- Details of Forest involvement

- The Ranchi-Rourkela 400KV,D/C, transmission  line involves  18.837ha. in Jharkhand . The total no. of trees coming in the entire ROW is 4932. However, felling of trees has been restricted to barest minimum and rests are only lopped / pruned to maintain electrical clearance.

- Rourkela –Raigarh 400 KV,D/C transmission line involves 111.39 Ha. forest in Orissa. The total no. of trees coming in the entire ROW is 9444. However, felling of trees has been restricted to barest minimum and rests are only lopped / pruned to maintain electrical clearance.

- Raigarh-Raipur 400 KV,D/C, transmission  line involve no forest  in complete route of line.

- The district wise forest area and details of trees are given below:

| S.N. | Name of Line | Type of forest | Forest Area in Ha. |  | No. of Trees | District/State Govt. | Type of Tree |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Ranchi-Rourkela | PF and Jungle Jhari | 3.94 |  | 693 | Ranchi East | Sal,Asan,Teak,Sisum,Misc |
|  |  | PF and Jungle Jhari | 8.89 |  | 3326 | Khunti | Sal,Asan,Kend,Karam,Misc |
|  |  | PF and Jungle Jhari | 6.007 |  | 853 | Simdega | Asan,Gamhar,Karam,Sal,Sisam,Misc |
| A. |  | Sub total 18.837 |  |  | 4932 |  |  |
| 2. | Rourkela-Raigarh | Revenue & RF (Rourkela ) |  | 23.269 | 1985 | Rourkela | Sal,Sissoo,Bija,Kendu,Dhaura,Asan, Char,Jamun,Harida |
|  |  | Revenue & RF (Sundargarh) |  | 20.93 | 969 | Sundargarh | do |
|  |  | Revenue, Demarcated Protected Forest, Protected Reserved Forest, DLC |  | 67.20 | 6490 | Sambalpur | do |
| B. |  | Sub total |  | 111.39 | 9444 |  |  |
| C. |  | Total A+B 130.227 |  |  | 14376 |  |  |

- A detailed compensatory afforestation scheme for carrying out afforestation in lieu of above diversion have been prepared by forest authorities. Compensatory afforestation scheme are annexed as Annexure-VI. CA cost and NPV will be paid to forest deptt. after Ist stage clearance. Forest clearance status of transmission lines  are as follows:

| Sl. No. | Name of transmission Line /Line length in Km | Forest area in(Ha.) | State. | Forest Clearance Status |
| --- | --- | --- | --- | --- |
| 1 | Ranchi – Rourkela 143.54 | 18.837 | Jharkhand | Govt. of Jharkhand forwarded the forest proposel to Regional office of Ministry of Environment & forests Bhubaneshwar. |
| 2 | Rourkela Raigarh 210 | 111.39 | Orissa | State Govt. forwarded the proposal to Ministry of Environment & Forests. |
| 3 | Raigarh Raipur 214 | Nil | Chattisgarh | No Forest |
|  | Total | 130.227 |  |  |

- Forest proposal for obtaining clearance from  MoEF submitted to concern State forest authorities ( Forest proposals are annexed as Annexure-VII).

- Forest clearance

- As per the requirement of of Forest (Conservation) Act,1980 prior approval of Government of India is being obtained following due process (refer flow chart of forest clearance) .The total no. of trees coming in the entire ROW ( in forest area) is14376. However, felling of trees has been restricted to barest minimum and rests are only lopped / pruned.

- The Ranchi-Rourkela Transmission line is passing through degraded  forest  .  Only 18.525  Ha. Reserve forest /Protected forest is involved in Jharkhand. And 8 Ha. Forest in Orissa and it is being accommodated on Multi circuit  tower of Rourkela & Raigarh transmission line near Rourkela  ( in Orissa ) portion and it has saved 8 Ha. forest in Orissa. Rourkela-Raigarh 400KV, D/C line  is passing  through degraded forest. Only 111.39 Ha. forest is involved in Orissa.

(Environmental impact Matrix)

| S. No. | PARAMETERS | Extent of Impact |
| --- | --- | --- |
| 1. A. | Line length- 567.54 Km. Jharkhand- 123.69 Km. Orissa- 210 Km. Chattisgarh 233.85 Km | -- |
| B. | Terrain . Plain 496.89 Km (87.5%) Hilly 70.65 Km (12.4%) |  |
| 2. | Type of line, 400 KV D/C | -- |
| 3. | Forest land traversed (Km) | 28.22 |
| 4. | Forest land traversed (Ha) | 129.825 |
| 5. | Forest type | PF & Jungle Jhari: 18.837 Ha Revenue &PRF : 17.25 Ha. DPF: 11.96 Ha. Revenue Forest & DPF: 82.180Ha. |
| 6. | Forest density | 0.1-0.4 |
| 7. | Rare/ endangered flora | Nil |
| 8. | Rare/ endangered fauna | Nil |
| 9. | Migrating Wild life/ breeding ground | Nil |
| 10. | National Park / sanctuaries | None |
| 11. | Amt. of wet land traversed | Nil |
| 12. | Soil erodability | low |
| 13. | Historical / Cultural Monument | None |
| 14. | Relocation of villagers | None |
| 15. | Loss/ Hindrance to Public Utilities | Negligible, restricted to construction phase only only. |

# SECTION-V: POTENTIAL ENVIRONMENTAL IMPACT, EVALUATION AND ITS MANAGEMENT

- 5.1	Impact Due to Project Location and Design

Environmental impacts of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources. To reduce environmental impacts, environmentally sensitive area like water bodies, temple, school building etc. have been avoided by putting special tower. To reduce tree cutting in orchard, extension tower has been provided at different locations.  Details of tower extension and change of tower type are annexed as Annexure-VIII. These impacts can be minimized by careful route selection. Although, all possible measures have been taken during the finalization of route alignment for the proposed transmission line but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to specific environment review criteria based on preliminary survey are as follows:

- (i) Resettlement

- As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be seen from the above description of proposed route alignment and also keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

The proposed project do not envisages construction of new sub-station . No fresh land acquisition is involve. Hence R&R is not issue in this project

- (ii) Land value depreciation

- Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

- (iii) Historical/cultural monuments/value

- As per the POWERGRID’s policy of route selection only that route alignment is finalized this avoids all the historical and cultural monuments. As per the assessment carried out with State revenue authorities and ASI, no such monuments are getting affected in the instant project.

- (iv) Encroachment into precious ecological areas

- As already explained all precautions have been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries. However, complete avoidance of forest area was not possible though National Park/Sanctuary or any other protective areas have been completely avoided.  But the route of proposed transmission line is so finalized that it affects minimum forest area, which has also been certified by all concerned Divisional Forest Officers (DFO).   It may be seen from the earlier description that out of the total transmission line length of about 567.54 Kms about 28.31 Km/130.28 Ha. (5% only) length shall pass through forest land  consisting of total 18.525 Ha./4.095 Kms forest area  in  the  states  of   Jharkhand and 111.39 Ha./24.215 Km in Orissa.

- A total of 85  towers are coming in the forest area.  Prior approval of GOI/MOEF would be obtained for affected forest under Forest (Conservation) Act, 1980. Most of the forests to be traversed by the lines are heavily degraded and the wildlife species present are those who have been adapted to open or disturbed habitat. Therefore, with provision of Compensatory Afforestation on 260 Ha. of degraded forest land, the overall forest status will in many cases improve (Refer Annexure-VI). Nonetheless, to mitigate losses to existing forests, clearing of the transmission line Right-of-way will be done under supervision of Forest Department, and some low canopy seed trees and shrubs may be kept intact if they do not interfere with tower erection and line installation. The wood will be sold by the Forest Department, who will also retain the sale proceeds. Three-meter wide strips of land under each conductor will be cleared and maintained as maintenance rows, but the remaining land will be allowed to regenerate. Lopping of trees to maintain line clearance will be done under the direction/supervision of POWERGRID & Forest Department. POWERGRID will provide construction crews with fuel wood or alternative fuels as a precaution against collection of fuel wood from nearby forest.

- Transmission lines can serve as new access routes into previously inaccessible or poorly accessible forests, thereby accelerating forest and wildlife loss. In such cases, POWERGRID cannot take action itself, but local Forest Department personnel will normally assess the dangers and take appropriate action, such as establishing guard stations at the entrance to the forest.  Given the already easy access and degraded conditions at the proposed project sites, this problem is not expected to be encountered.  Nonetheless, POWERGRID staff will report to the Forest Department any noticeable encroachment induced by the Projects.

(v) Encroachment into other valuable lands

- Impacts on agricultural land will be restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1).

In areas where lines will traverse agricultural land, compensation will be paid to owners for any crop damage incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation.  Revenue department, after evaluating the loss due to construction activity and productivity of land, arrives at the compensation cost, which is paid to farmer.  Agricultural activities will be allowed to continue following the construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner’s satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss of products from the tree over an eight-year period (for fruit bearing trees) and for other trees depending on the timber value. Out of the total tower location of, 1616 (1701-85, in forest =1616) tower locations are coming either on private land or revenue land. Agricultural lands under private ownership are identified, and in accordance with normal POWERGRID procedures crop/tree compensation is paid to the affected persons.   It is estimated that total compensation towards crop/trees shall be the tune of Rs. 539.26 lakh.  Budgetary provision Rs. 2260.70 lakh is kept to meet these expenses. Crop /tree compensation payment procedure and Budget provision is annexed as Annexure-IX.

.

(vi.) Interference with other utilities and traffic

- As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission lines affect nearby telecommunication circuits by causing electrical interference.  A standing committee – Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage, which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like rerouting of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

- The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50,000/- per KM.

- Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission lines pass by the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights are placed atop these towers. All necessary clearance have already been obtained or in the advance stage of processing with relevant authorities.

- (vii)    Interference with drainage pattern

- As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings, which is very small, there is little possibility of affecting drainage pattern. Since in the instant project most of the lines is being constructed in the plain area no such impact is encountered till date and during further implementation if such instances are repotted, flow will be trained and guided to safe zones.

- 5.2	Environmental Problems Due to Design

- Escape of polluting materials

- The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials. To avoid/minimize during construction phase a clause has been included in the contract document and is being monitored regularly by site engineers.

(ii) Explosion/fire hazards

- During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires. Fires due to flashover from lines can be a more serious problem in forest. However, adequate safety measures shall be taken to avoid such incidence besides this forest authorities also incorporate measures like making fire lines to prevent spreading of fire in the affected forest area.

(iii) Erosion hazards due to inadequate provision for resurfacing of exposed

area

- Adequate measures are taken to re-surface the area where excavation works are done.  Topsoil disturbed during the development of sites will be used to restore the surface of the platform. Infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for tower foundations and rest is disposed at selected dumping area   (Exhibit-II).

- (iv)	  Environmental aesthetics

- Since spacing between the towers in case of 400 KV, D/C lines is approx. 300-400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible.   POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted gray or green to merge with the background.

(v) Noise/vibration nuisances

- The equipment installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian standards. The noise levels reported during normal operating conditions are about 60 to 70 dB at 2 m. distance from the equipment.  To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipment are undertaken.  In addition, plantations of sound absorbing species like Casuarinas, Tamarind, and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.  Actual noise levels measured at perimeters of existing Substations are 20 to 30 dB.  Noise during construction phase is periodically monitored and due maintenance of equipments are ensured to keep the noise level well within the prescribed limit.

- (vi)   Blockage of wildlife passage

- Since the line is passing through mostly agricultural, wasteland and the affected forest area is also not a migration path of wild life hence, possibility of disturbance to wild life are nil/remote.

- 5.3	Environmental Problems During Construction Phase

(i)    Uncontrolled silt runoff

- The Project involves only small scale excavation for tower foundations at scattered locations that are re-filled with excavated material, as the major portion of project area  (87.5%)  is in plain hence uncontrolled silt run off is not expected.

(ii)    Nuisance to nearby properties

- As already described in preceding paras, during site selection due care is taken to keep the transmission line and substations away from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour therefore nuisance to the nearby properties if any, is not expected. The construction activities are normally undertaken in lean period and post harvesting to avoid/minimize such impact.

(iii)  Interference with utilities and traffic and blockage of access way

- Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures.  It may be noted that access road ( Metalled / Non-metalled /cart roads) are existing to access all angle points and construction of no new road is envisaged (taking 200 m as buffer zone which can always be accessed through head load) for this line. Refer Annexure-III,IV &V. Even if at some places it is found that access road is not available than existing field/path is upgraded/augmented for utilization and compensation for any damage to crop or field is paid to the owner. In many areas such improvement in access road is highly appreciated by the local population.

- As and when a transmission line crosses any road/ railways line, the short span angle (DT) towers are located at a distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

- (iv)	Inadequate resurfacing for erosion control

- Since proposed line is to be constructed in plain area where erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained above adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is generally undertaken outside the rainy season. As the proposed line is mostly passing through plains (87.5%)  areas no such problems are anticipated.

(v)  Inadequate disposition of borrow area

- As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area. As such acquisition/opening of borrow area is not needed.

(vi)   Protection of Worker’s health/safety

- The Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents will guide provisions for workers’ health and safety. Various aspects such as, work and safety regulations, workmen’s compensation, insurance are adequately covered under the Erection Conditions of Contract (ECC), a part of bidding documents (Annexure- X).

POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. This guidelines/checklist includes work permits and safety precautions for work on the transmission lines both during construction and operation (Annexure- XI) and is monitored regularly by site in-charge and corporate Operation Services. In addition training is imparted to the workers in fire fighting and safety measures Exhibit-III.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act. POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Deptt. Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses will be disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID will conduct awareness building programs on such issues for the construction workers.

- 5.3	Environmental Problems Resulting from Operation

(i) O&M Staff/Skills less than acceptable resulting in variety of adverse

effects

- The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However in respect of the long distance transmission lines there are monitoring offices that are located at various points en-route. Monitoring measures employed include patrolling and thermo-vision scanning. The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

- A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard.  Because of issues relating to need to ensure health and safety relating to the line such as fire safety, safe voltages on metallic parts of buildings, and safety clearances to avoid flashover, the transmission lines will not pass directly over any residential properties and as such the potential for EMF effects to occur will be further diminished. Given that it will be necessary to ensure that there are no properties in the ROW beneath and to the sides of the overhead line, automatic mitigation against EMF will be provided between the source of potentially high strengths (the transmission line) and the residential properties.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India:

“ Power Grid was the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid do not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

5.4   CRITICAL ENVIRONMENTAL REVIEW CRITERIA

(i)   Loss of irreplaceable resources

The transmission projects do not involve any large-scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As only 28.22 Km (129.82 Ha.) of the lines in the subject project are not passing through y forest area some loss of vegetation is anticipated but with provisions of compensatory afforestation on degraded forest area (260 Ha.)  will compensate the loss of vegetation. The forest land identified for such plantation by forest department is in various  Forest Divisions of the concerned. The cost of compensatory afforestation cost will be paid to forest department.

(ii)  Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short-term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as no reserve forest is getting affected thus there is no possibility of endangering/causing extinction of any species.

(iv)  Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

5.6    PUBLIC CONSULTATION

Public consultation/information is an integral part of the project implementation. Public is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted.

Apart from this, public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting  as  per  Environmental Social Policy  & Procedures of POWERGRID  (ESPP) is carried out during different activities of project cycle. During such consultation the public is informed about the project in general and in particular about the following:

- Complete project plan (i.e. its route and terminating point and substations, if any, in between);

- POWERGRID design standards in relation to approved international standards;

- Health impacts in relation to EMF;

- Measures taken to avoid public utilities such as school, hospitals, etc.;

- Other impacts associated with transmission lines and POWERGRID’s approach to minimizing and solving them;

- Land acquisition details, proposed R&R measures and compensation packages in line with POWERGRID’s policy;

- Trees and crop compensation process.

Apart from organising many informal group meetings in different villages (Table-5.1) public meeting were also organised in the routes of transmission lines. To get the maximum participation during the Public consultation Programme a notice was served well in advance to the villagers.  The   details of line and its importance was explained to the villagers.  The programme was arranged in interactive way and queries like crop compensation, route alignment etc. were replied. Most of the participants were small farmers and were worried about their land through which the line will pass.  They were informed that POWERGRID will not acquire their land for construction of transmission lines.  Only towers will be spotted in their fields where they can do farming without any fear because the tower height is very high and even tractor can pass below the tower.  Moreover, there is no risk of passing current from the above line as there is foolproof system of earthing for tower. The consultation process was appreciated by the villagers.  They were happy to know about the transparent policy of POWERGRID for execution of the project and promised to extend their cooperation during construction of the line. The detail of public consultation is enclosed as Annexure-XI.  The process of such consultation and its documentation shall continue during project implementation and even during O&M stage.

Table 5.1

| Sl. No. | Name Of transmission line | village | Date of meeting | No. of villagers | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 400 KV D/C Ranchi-Rourekla | Rudungkocha,Sarwal & Butiyo | 17.06.2006 | 15-25. | Villagers of Rudungkocha Sarwal &Butiyo participated. |
| 2. | --Do-- | Nachladag,Jordag,Tunju Khuntui,Biru &Chikor | 18.06.06 | 14-20. | Villagers of Nachladag Jordag,Tunju Khuntui,Biru & Chikor participated. |
| 3. | --Do-- | Darla,Kulap,Hututua, Khatkura,Kanaroiya & Latakhel | 19.06.2006 | 16-24. | Villagers of Darla,Kulap,Hututua, Khatkura,Kanaroiya & Latakhel participated. |
| 4. | Rorkela-Rajgarh 400 KV, D/C | Usra, | 11.11.2006 | 21 | Villagers of Usra participated. |
| 5. | --Do-- | Bhoipali | 16.11.2006 | 26 | Villagers of Bhoipali participated. |
| 6 | --Do-- | Bhanwrkhol | 20.11.2006 | 9 | Villagers of Bhanwrkhol participated |
| 7. | --Do-- | Bhikampali | 22.11.2006 | 31 | Villagers of Bhikampali participated |
| 9. | --Do-- | Lakhanpur | 23.11.2006 | 21 | Villagers of Bhikampali participated. |
| 10. | Raigarh-Raipur 400 KV, D/C | Gora | 16.12.06 | 22 | Villagers of Bhikampali participated. |

  - Conclusions

From the above discussion, it would seem that the area is not rich in physical resources. During route selection care has been taken to avoid involvement of any National Park/Sanctuary but involvement of Road/canal side forest could not be avoided due to terrain and other physiographical reasons. Thus  routes  selected for detailed survey is the most optimum route alignment  and involved minimum forest area .   R&R is not required because no fresh land was acquired. Hence, based on the criteria agreed with the Bank the project is coming under the P2 category or non sensitive as only 5% of total length is passing through forest, which is already degraded in nature and no impact on wild life is envisages as the forest is not the habit of any endangered or endemic species of fauna and flora.   The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts, while on the one hand underline the need for implementation of the project for overall development of the area and on another hand; suggest that a detailed E.I.A. may not be necessary.

# SECTION-VI:  MONITORING AND ORGANISATIONAL SUPPORT

# STRUCTURE

6.0      ENVIRONMENTAL MONITORING PROGRAM IN POWERGRID:

- Monitoring is a continuous process for POWERGRID projects at all the stages be it the site selection, construction or maintenance.

- The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan :

- 6.1    Corporate Level

- An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and Site to carry out environmental and social surveys for new projects.

- Assisting RHQs and site to finalize routes of entire power transmission line considering environmental and social factors that could arise enroute.

- Help RHQs and Site to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & site Officials on environment and social issues and their management plan.

- 6.2     Regional Level

- At its Regional Office POWERGRID has a Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the Corporate level and the Construction Area Office (CAO). The key functions envisaged for ESMC are :

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and CAO to finalise routes of entire power transmission lines considering the environmental and social factors that could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and CAOs on various environmental and social aspects.

6.3      Site Office

- At the Site level , POWERGRID has made  the head of the CAOs responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social  Management Team (ESMT) . Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalise the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorised Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

- As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park , it  is generally  done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties. A monitoring system (done by the Forest Department) is also in place for compensatory forests established as part of the Project.  A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn and is being implemented at site during various stage of project execution. The updated EMP with compliance status for East-West transmission as Annexure-XIII.

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID’s CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 are undertaken for compliance of agreed policy and management plan. The copy of recent   annual audit report is placed as Annexure-XIV.