SECTION I:  PROJECT DESCRIPTION

(Eastern Region System Strengthening Scheme-I)

- Background

The CEA study reveals that Eastern Region will be surplus in power to the tune of 3500MW by 2009-10 due to addition of power generation. It is, therefore required to strengthen the transmission grid of Eastern Region in order to transmit the additional surplus power.

Accordingly, the following transmission system has been proposed which would transmit the surplus power from Eastern part of the Eastern Region grid to the central areas from where it would get further transmitted through the existing transmission system:

- Durgapur (PG) – Jamshedpur 400 kV D/c line

- Jamshedpur – Baripada 400 kV D/c line

- Baripada – Mendasal 400 kV D/c line

1.1	Project Highlights

| a) | Project Name | : | Eastern Region Strengthening Scheme-I |
| --- | --- | --- | --- |
| b) | Location | : | West Bengal, Jharkhand, Orissa and Bihar in Eastern Region. |
| c) | Beneficiary States | : | Eastern Region constituents |
| d) | Cost | : | Rs.733.32 Crores |

1.2	Scope of work

#### The scope of work covered under the project broadly includes the construction of following transmission lines, substation and other associated works:

#### Transmission Lines

Durgapur (PG) - Jamshedpur 400 kV D/c line 			: 181 Km

Jamshedpur - Baripada 400 kV D/c line				: 135 Km

Baripada – Chandaka (Mendhasal) 400kV D/c line		: 255 Km

Re-conductoring of Siliguri - Purnea 400 kV D/c line with twin INVAR Moose conductor									: 175 Km

#### Extension of Substations

a) Jamshedpur 400/220 kV sub-station

b)  Durgapur 400/220 kV sub-station

c) Baripada 400/220/132 kV sub-station

d) Re-conductoring of 400 KV bays including dismantling & replacement of

equipment and associated work at  Siliguri and Purnea

1.3    Benefits of the project

The system will strengthen transmission system of Eastern Regions. It will facilitate inter-state trading of exportable power of State sector generation apart from bulk power from the central sector stations. It will help to maintain system stability, security of the combined grid under all operating conditions. The system will also increase the capacity of National Grid

1.4	Beneficiaries

The target beneficiaries of this project are the Constituent States of Eastern Region Grid.

A power map showing the Eastern Region System Strengthening Scheme-I, is placed as Exihibit-1.

SECTION II: BASE LINE DATA

2.0  	The project is an inter-state one and is spread/located in the States of West Bengal, Jharkhand, Orissa; the basic details of the area under project are given below:

  - West Bengal

West Bengal is strategically placed with three international frontiers - Bangladesh, Nepal and Bhutan. A hinge between the bulk of Indian territory and the north-east of the country, West Bengal is located at 21°31' and 27°14' North Latitude at the head of the Bay of Bengal and 86°35' and 89°53' East Longitude, with the Tropic of Cancer running through it. West Bengal stretches from the Himalayas to the Bay of Bengal. Its 1600-km long frontier with Bangladesh, neither natural nor well defined, has in recent years been a source of tension. It also has borders with Nepal and Bhutan. 

The State of West Bengal situated in the eastern part of the country between the Himalayas in the north and Bay of Bengal in the south where mighty river.  Ganga meets the ocean forming well-known mangrove forests in Sundarbans, mainly embraces the fertile alluvial gangetic plains.

Physiography

Its northern part comprises the sub-Himalayan region called the Terai, while the rest of the state forms part of the Indo-Gangetic plain. The course of the river Ganga, much of which lies now in Bangladesh, has for centuries been shifting eastward. Very little of its water now goes to the sea via its western distributaries, the principal one being the Hooghly that flows by Calcutta. The deep alluvial soil of the Gangetic plain in West Bengal is very suitable for agriculture. 
The Terai region has some of the finest tea plantations in India. From the Terai, the Himalayan mountain ranges rise abruptly along the state's northern boundary. Mount Kanchenjunga overlooks Darjeeling in north-west Bengal.

Climate

West Bengal has a tropical climate. The plains are hot except during the short winter season. The mountainous region in the north is cold on account of its altitude but there humidity is high. The classical tradition speaks of six seasons-springs, summer, the Rainy, autumn, mild winter and severe winter. Only four clearly marked seasons with a brief interregnum of spring are observed, namely the hot season, the rainy season, the post monsoon season corresponding to autumn and the cold season. At nights, cool southerly breeze carrying moisture from the Bay of Bengal is usually present.

Rainfall

The average rainfall in the State is 1750 mm. In the Himalayan Region i.e in northern part the average rainfall ranges from 2500 - 6000 mm. In the southern part average rainfall ranges from 1125 - 1900 mm.

Temperature

The hot season lasts from mid-March to mid-June, with the day temperature ranging from 38o C to 45oC in different parts of the state. Winter, which lasts about three months, is mild over the plains, the average minimum temperature is15o C.

Soil

The soil patterns in the state are Tarai Soil and Gangetic Alluvium - found in northern part of the state along the border of Nepal.

Minerals

Minerals, like rock phosphate, fire clay, iron, limestone, copper, silica, quartz, manganese and sandstone are found in a considerable amount in the state

Water resources

The State of West Bengal falls under 3 major River Basins:

|  | - The Brahmaputra Basin - Part of Darjeeling, Jalpaiguri and Coochbihar districts . |
| --- | --- |
|  | - The Ganga Basin - Part of Darjeeling, Jalpaiguri and all the remaining districts . |
|  | - The Subarnarekha Basin - Parts of Purulia, E&W Medinipur district. |

Forests

The recorded forest area is 11,879 km (sq.) which constitutes 13.4% of the geographic area of the state.  By legal status, the Reserved Forest constitutes 7,054 km2, Protected Forest 3,772 km2. and Unclassed Forest  1,053 km2. Forest Map of West Bengal   is enclosed as Map-1.

There are eight forest types in the West Bengal. These are eight forest types in the state.  They are Tropical Semi Evergreen, Tropical Moist Deciduous.  Tropical Dry Deciduous, Subtropical Broadleaved Hill, Subtropical Pine, Himalayan Moist Temperate,  Wet Temperate, Littoral and Swamp forests.  Forests are mainly distributed in the north, south west and south-east.

Protected area

There are 5 National Parks and 16 Wildlife sanctuaries covering an area of 0.28 million ha.  It constitutes 3.15% of the geographic area of the state.  The state has 2 tiger reserves namely Sundarbans and Buxa.  Sunderbans Biosphere Reserve is one of the 11 Biosphere Reserves of the country.  Sunderbans National Park is also a World Heritage site.

Human and Economic Development

The geographic area of the state is 88,752 km2 which is 2.7% of the country’s geographic area. The human population of the state is 80.22 million, constituting 7.8% of the country’s population, of which 72.0% is rural and 28.0% urban. The average population density is 904 persons per km. The tribal population is 5.6% of the state’s population. The livestock population is 47.09 million, which is 7.5% of the country’s livestock population. The State ranks 6th amongst States/UTs in terms of percentage of area under tree cover. Minerals, like rock phosphate, fire clay, iron, limestone, copper, silica, quartz, manganese and sandstone are found in a considerable amount in the state. As a result, a few industries related to these are being established in the state. Steel and alloy steel plants in Durgapur and another steel plant in Burnpur are some of the important industries of the state. Other industries include jute, tea, cotton textiles, automobiles, bicycle, footwear, leather, paper, pharmaceuticals, chemicals, aluminum, sugar, timber processing, ceramic, glass, bone metal, and dairy.

Agriculture dominates both the landscape and the economy. Fifty-five percent of the population of West Bengal depends upon agriculture. In fact, agriculture makes a substantial contribution to the state’s income. West Bengal accounts for 57.3% of the total jute and 24% of the total tea produced in the country. Pulses, oilseeds, barley, maize, betel leaf, tobacco and sugarcane are some of the other agricultural products of West Bengal. Moreover, there are several multipurpose irrigation schemes in West Bengal, namely, the Damodar Valley, Mayurakshi, Kangsbati and Subarnarekha Barrage. The irrigation schemes are Teesta-Mahananda link canal in Jalpaiguri district, Terageria and Turga irrigation scheme in Purulia, Hinlow irrigation scheme in Birbhum district and Sohajare scheme in Bankura district. 
 86°35'

  - Jharkhand

Jharkhand, carved out from Bihar, accounts for 45% of former Bihar’s geographical area. It lies between lat 22° 00' and 24° 37' N and long. 83° 15' and 87° 01' E. The State is bordered by Bihar, Madhya Pradesh, Orissa and West Bengal to its north, south and east respectively. The geographical area of the state is 79,714 Km2.  About 28 % of the population of this area is tribal. The new State Jharkhand formerly a part of Bihar state, was formed on November 15th, 2000 with Ranchi as its capital. Jharkhand is the 28th state of the Indian Union.

Physiographically it consists of both plateau and hills.  Santhalparganas and Chhotanagpur is main plateau about 3000 feet above sea level. The highest part of the plateau is Netarhat which has an elevation of 3600 ft. The Parasnath Hill is the highest point with an elevation of 4800 feet. The plateau is full of mountain ranges covered with dense forests. A number of rivers and rivulet flow down through the hilly terrains and valleys.

Climate:  The climatic condition is generally Sub-Tropical wet and dry.

Rainfall:  The mean annual rainfall varies between 100-200 Cms.

Temperature: The annual temperature varies between 20-35 0 C.

Soil: Generally mixed red and black soil along with yellow soils in few pockets is encountered in the state.

Minerals:- Jharkhand is endowed with vast natural resources specially the vast variety of minerals ranging from Iron ore, Coal, Copper ore, Mica, Bauxite, Fire clay, Graphite, Kyanite, Sillimanite, Lime stone, Uranium & other minerals. Jharkhand is the leading producer of mineral wealth in the country.

Water Resources:- Rivers, tanks and groundwater are the main sources of water in State Jharkhand. Damodar, Mayurakshi, Barakar, Koyal, Sankh, Sone, Auranga, More, Karo, Bansloi, South Koel, Kharkai, Swarna Rekha, Ganga, Gumani, Batane are some of the perennial river veins meandering through the State.

Forest: - Total geographic area of state is 79,714   Km2.  Covered forest area is 28.4% of total geographical  area .Dense forest is 11,787 Km2 & open forest 10,850 Km2. Total forest covered area is 22637 Km2. The State has following three forest types:

- Tropical Moist Deciduous

- Tropical Dry Deciduous

- Sub Tropical Broadleaved Hill Forests

Forest covers slightly more than one third of the geographical area of Jharkhand (Map-2). The state is rich in forest resources having Sal and Teak as the two most important forest formations.

Protected area

There is 1 National Park and 10 Wildlife Sanctuaries in the state.  They cover an area of 0.21 million ha. Constituting 2.58% of the total geographic area of the state.  Palamu tiger reserve is located in the state.

Human and Economic Development

Jharkhand’s economy is sustained by mining and heavy industry.  It is India’s second most important source of coal mining.  In minerals Jharkhand is probably India’s richest state.  Jharkhand has rich reserves of iron ore and coal and several industries. Minerals, forest produce and agricultural land constitute the principal resource base of Jharkhand. Other factors adding to strength of the economy are moderate climate, adequate availability of surface and ground water, presence of a number of Industrial giants, technical institutions and cheap labour. Nearly 80 per cent of the State's population depends on agriculture. With varied agro-climatic conditions, Jharkhand provides scope for production of a wide range of agricultural/horticultural crops, spices and medicinal plants

The largest four cities are Jameshedpur, Dhanbad, Ranchi and Bokaro Steel City.  The two major steel plants of India are located in Jharkhand: at Jamshedpur and Bokaro.  Road length: 4, 310 km, including 1,500 km National Highways and 2,711km State Highways.  The state has a well developed railway system. Ranchi, Bokaro, Dhanbad, Jamshedpur are some of the major railway stations.  Agriculture is poorly developed.  Irrigation is negligible.  Drought therefore is a periodic threat.

2.2	Orissa

The geographic area of Orissa is 15.57 million ha, which constitutes 4.74 % area of the country. It lies in the tropical zone between lat. 170 47’ and 220 34’ N and long. 810 22’ and 870 29’ E.

Physiography

The state can be divided into four distinct physiographic regions viz. Northern Plateau, Eastern Ghat, Central table land and coastal plains. The state is drained by three major rivers, the Mahanadi, the Brahmani and the Baitarni.

On the basis of homogeneity, continuity and physiographical characteristics, Orissa has been divided into five major morphological regions: the Orissa Coastal Plain in the east, the Middle Mountainous and Highlands Region, the Central plateaus, the western rolling uplands and the major flood plains.

Rainfall

The average annual rain fall varies between 1,200 mm to 1,600 mm.

Temperature

The average annual temperature ranges between 250 C to 27.50 C.

Climate

Orissa has an equable climate, neither too hot nor too cold. In some places, however, extremes of climate are experienced, namely in the Western district like Bolangir, Sambalpur and Sundergarh. Summer temperature varies between 20ºC-45ºC.and Winter temperature between 13ºC-32ºC.

Mineral resources

About 80% of the state's landmass is underlain by pre- cambrian rocks, the oldest rocks in the earth's crust. The remaining area is composed of rocks belonging to Gondwanas, Quarternary & Recent formations. The geological set up of the state is not only favourable for discovery of wide variety of valuable ores and minerals but there is distinct possibility of substantial increase in the resources. The main mineral resources  found in Orissa are  coal, Iron & Manganese, Limestone & Dolomite,Bauxite,Nickel, Manganese, Graphite,Chromite, Serilite Quartzite, Quartz Serilite schist, Serilite schist, Andalusite schist and genesis,Beach sand, Sand stone and decorative stone etc

Water resources

Orissa is endowed with water in abundance. As per Ground Water Board, the total ground water resources in Orissa amounted to 2,101,128 hectare meter. There are 4 groups of rivers which flow through Orissa in to Bay of Bengal and thus make Orissa water rich state. The main rivers are Subarnarekha, Brahmani, Mahanadi, Budhbalanga, Salandi, Kolab and Indravati etc.

Forest resources

The recorded forest area is 58,136 km (sq.) which constitutes 37.34% of the geographic area of the state. The forests are uniformly distributed in the state except in coastal districts where they are scarce. By legal status, the Reserved Forest constitutes 26,329 km , Protected Forest 15,525 km. and  Unclassed Forest  16,282 km. Foest Map of Orissa  is enclosed as Map-3.
 The State has following forest types:

- Tropical Semi Evergreen,

- Tropical Moist Deciduous,

- Tropical Dry Deciduous,

- Littoral and Swamp Forests.

Protected areas

There are 2 National Parks and 18 Wildlife Sanctuaries covering an area of 0.80 million ha, which is 5.11% of the geographic area of the state. Similipal National Park, also a Tiger Reserve has been declared as a Biosphere Reserve. The Gahirmatha Wildlife sanctuary, a mangrove eco-system, is a famous nesting ground of the Olive Ridley sea turtle. 
 In the Orissa the actual forest cover is about 48,366 sq. km. Out of which 28170 sq. km is dense and 20,196 sq. km is open forest

Human and economic development

In Orissa about 76% population is dependent on agriculture. The major crops are rice, pulses, oil seeds, jute, sugarcane, coconut and turmeric. The state contributes about 1/10th of rice production in India. But Orissa’s agro based economy is always upset by frequent natural calamities like floods, drought, tornado and cyclone. Orissa’s vast mineral, marine and forest resources provide adequate opportunities for industrial growth. About 170 large and medium industries along with 9000 small scale industries are running in the state. Forests have an important role in the economy of the State, particularly, amongst rural Scheduled castes and Tribal masses. Forestry activities contribute a lot to their food security and day-to-day livelihood.  Forests also sustain the large sized livestock of about 29.3 million.

The infrastructure for the development of industry in Orissa is available. A combination of coal, iron ore, limestone, bauxite and a host of other minerals on the one hand and port facilities on the other are the unique features in Orissa. In addition, the bountiful forest resources and agricultural products provide ample scope for the development of forest-based and agro-based industries.

Iron and steel industry, aluminum, cement, ferromanganese, ferro-chrome, galvanised pipe units, steel rolling mills and fertilizer plants. Among the other industries beverages, tobacco, gur and molasses are important. The tobacco industry is wide spread and provides employment to large number of persons
Although Orissa has a large cattle population, it imports leather goods .The principal centres of leather products are Baudh, Bhanjanagar, Bolangir and Keonjhar. Modern tanneries have been set up at Cuttack. 
Oil pressing is a traditional occupation run on caste and family line. Mustard, groundnut, castor and til seeds are the principal raw materials used.

Some villages specialize in brass products, but owing to competition from the large manufacturing concerns, especially those manufacturing aluminum utensils, this cottage industry is decaying. The high price of brass is another handicap.

Details of forest land involvement in Eastern Region Strengthening Scheme-I

Durgapur-Jamshedpur 400KV D/C transmission line will pass through following districts of WB and Jharkhand.  Details of forest cover of these districts are as follows:

Area in Km2

| S. No. | District/ State | Geographic Area | Very Dense Forest | Moderatately dense | Open dense Forest | Total Forest | Percent (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Burdwan/ WB | 7024 | 16 | 74 | 135 | 225 | 3.20 |
| 2. | Bankura/ WB | 6,882 | 101 | 295 | 584 | 980 | 14.24 |
| 3. | Purulia/ WB | 6,259 | 34 | 234 | 496 | 764 | 12.21 |
| 4. | East Singhbhum/ Jharkhand | 3,533 | 49 | 528 | 346 | 923 | 26.13 |
| 5. | West Singh bhum/ Jharkhand | 9,907 | 446 | 1,598 | 1,767 | 3,811 | 38.47 |
|  | Total |  |  |  |  | 6703 |  |

Source : Forest Survey Report 2003

The above line will involve 17.36 Ha. Statewise break up forest in these two state, which is about 0.03% of the total forest area of both state.

Jamshedpur-Baripada 400KV D/C, transmission line will pass through following districts of Jharkhand and Orissa. Details of forest cover of these districts are as follows:

Area in Km2

| S. No. | District/State | Geographic Area | Dense Forest | Moderately | Open Forest | Total Forest | Percent (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | West Singh bhum ( Saraikela is part of West Singhbhum)/ Jharkhand | 9,907 | 446 | 1,598 | 1,767 | 3811 | 38.47 |
| 2. | East Singhbhum/ Jharkhand | 3,533 | 49 | 528 | 346 | 923 | 26.13 |
| 3. | Mayurbhanj/ Orissa | 10,418 | 99 | 2,910 | 996 | 4005 | 38.44 |
|  | Total |  |  |  |  | 8739 |  |

Source: Forest survey Report 2003

The above line will involve 6.9 Ha. State wise break up forest in these two states, which is about 0.008% of the total forest area of both state.

Baripada-Mendhasal transmission line (Orissa portion) will pass through following districts of Orissa. Details of forest cover of these districts are as follows:

Area in Km2

| Sr. No. | District/State | Geographic Area | Dense Forest | Moderately | Open Forest | Total Forest | Percent (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Mayurbhanj/Orissa | 10,418 | 99 | 2,910 | 996 | 4005 | 38.44 |
| 2. | Balasore/Orissa | 3,806 | 0 | 154 | 155 | 309 | 8.12 |
| 3. | Bhadrak/Orissa | 2,505 | 0 | 22 | 7 | 29 | 1.16 |
| 4. | Jaipur/Orissa | 2,899 | 0 | 82 | 180 | 262 | 9.04 |
| 5. | Cuttack/Orissa | 3,932 | 0 | 265 | 359 | 624 | 15.87 |
| 6. | Khurda/Orissa | 2,813 | 0 | 211 | 151 | 362 | 12.87 |
|  | Total |  |  |  |  | 5591 |  |

Source: Forest survey report 2003

The above lines will involve 125.58 Ha. forest in Orissa, which is about 0.23% of the total forest area .

SECTION III:   POLICY, LEGAL & REGULATORY

FRAMEWORK

- 3.0   POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and directives and guidelines with respect to environmental and social considerations of Funding Agencies.

- 3.1     ENVIRONMENTAL

3.1.1 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under The Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under The Forest (Conservation) Act, 1980

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded; hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/reconditioner or at the designated collection centers – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)Amendment Rules,

2003:

MOEF vide its notification dt. 20th May, 2003 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned/sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organization in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalizing the route alignment.

    - FUNDING AGENCIES:

- WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM)- F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.3   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- Applicable Legislations

- 3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement

on Environment and Development, 1992

- Policy statement for Abatement of pollution, 1992

- 3.2.0   SOCIAL

- 3.2.1    MANDATORY REQUIREMENTS (NATIONAL)

- National Policy on Resettlement and Rehabilitation for Project Affected Families: Ministry of Rural Development, Government of India has notified a National policy on R&R for  PAFs in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the assetless rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups.

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan/Tribal Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require neither resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.3    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- National and State-wide Laws and Policies Relating

to Land Acquisition and Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.4   RELEVANT POLICIES

- The Orissa Resettlement and Rehabilitation of Project Affected Persons Policy, 1994 (water resources projects)

- Resettlement & Rehabilitation Policy – NTPC

# SECTION-IV: POWERGRID APPROACH FOR

# ROUTE SELECTION

- 4.0     ROUTE SELECTION - (ASSESSMENT & MANAGEMENT PROCESS)

- At the system planning stage itself one of the factors that govern the evolution of system is the possible infringement with the forest. Wherever such infringements are substantial, different alternative options are considered. The route/ site selection criteria followed by POWERGRID is detailed below:

- While identifying the transmission system for a generation project or as a part of National Power Grid, preliminary route selection is done by POWERGRID based on the topo sheets of Survey of India and Forest Atlas (Govt. of India's Publication). Further optimization is done during detailed survey using GIS/GPS. During route alignment all possible efforts are made to avoid the forest area involvement completely or to keep it to the barest minimum, whenever it becomes unavoidable due to the geography of terrain or heavy cost involved in avoiding it.

- 4.1   STUDY OF ALTERNATIVES

- Environmental Criteria for Route selection

- For selection of optimum route, the following points are taken into    consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

- Any monument of cultural or historical importance is not affected by the route of the transmission line.

- The proposed route of transmission line does not create any threat to the survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

- In addition, care is also taken to avoid National parks and sanctuaries and any other forest area rich in wild life.

- Keeping above in mind the routes of proposed line under this transmission System has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps etc. to arrive at most optimum route which can be taken up for detailed survey using GIS/GPS and assessment of environmental & social impacts for their proper management.

4.2 	Evaluation of alternatives Route Alignment of 400 KV D/C Durgapur

-Jamshedpur Transmission line

- Three different alignments (Map-4) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three alternatives are as follows:

| Sl.No. | Description | Alternative-I | Alternative-II | Alternative-III |
| --- | --- | --- | --- | --- |
| 1 | Route Particulars |  |  |  |
| i. | Length(Kms) | 170.08 | 177.00 | 178.00 |
| ii. | Terrain |  |  |  |
| a | Hilly/Plain(Kms) | 2.2/167.88 | 2.2/174.8 | 2.2/175.8 |
| b | Wet/Marshy | - | - | - |
| c | Estuarine | - | - | - |
| d | Other type of land | - | - | - |
| 2 | Environment Details |  |  |  |
| i. | Name of district/district details (through which transmission line pass) | Burdwan, Bankura, Purulia (WB) and Poorv Singhbum, Paschim Singhbhum (Jharkhand) | Burdwan, Bankura, Purulia (WB) and Poorv Singhbum, Paschim Singhbhum (Jharkhand) | Burdwan, Bankura, Purulia (WB) and Poorv Singhbum, Paschim Singhbhum (Jharkhand) |
| ii. | Town in alignment (Near By) | Durgapur, Bankura, Jamshedpur | Durgapur, Bankura, Jamshedpur | Durgapur, Bankura, Jamshedpur |
| iii. | Forest in Km/Ha. | 3.773 Kms/ 17.36Ha. (including 1.25 Km wild life) | 12.900 Kms/ 59.34Ha. (3.75 Km wild life) | 18.870 Kms/ 86.80Ha. ( 4.00 Km wild life) |
| iv. | Type of Forest |  |  |  |
|  | Reserved/protected/mangrove/ Wild life area/biosphere reserve/any other environment | Protected | Protected | Protected |
| v. | Density of forest | 0.4 | 0.5 | 0.4 |
| vi. | Type of Fauna & Flora | Palash,Khashi, Arjun, Siris etc and fox, jackal, elephant etc. | Palash,Khashi, Arjun, Siris etc and fox, jackal elephant etc. | Palash,Khash Arjun, Siris etc and fox, jackal elephant etc. |
| vii. | Endangered species if any | Nil | Nil | Nil |
| viii. | Historical/Cultural monument | Nil | Nil | Nil |
| ix. | Any other relevant information | - | - | - |
| 3 | Compensation Cost |  |  |  |
| i) | Crop | Rs.62.03 lacs | Rs.61.209 lacs | Rs.59.36 lacs |
| ii) | Forest | Rs.165.48 lacs | Rs.565.63 lacs | Rs.827.38 lacs |
| 4. | No. of Crossing |  |  |  |
| i) | Railway | 6 | 6 | 6 |
| ii) | Power Line | 18 | 18 | 18 |
| iii) | River Xing etc | 1 | 1 | 1 |
| 5. | Construction problem | Approachable, roads are available | Difficult to approach | Difficult to approach |
| 6. | O&M problem | Not anticipated | Due to difficult to approaches, construction problem anticipated | Due to difficult to approaches, construction problem anticipated |
| 7. | Overall Remarks | 1. Easily approachable 2.Involvement of less forest area | Poor communication facilities for construction of EHV transmission line | Poor communication facilities for construction of EHV transmission line |

It may be seen from above that Alternative-I is the most optimum route as the line length is shorter than Alt. II & III. Alt-I involves less forest and less wild life area (1.25 Kms) in comparison than II (3.75 Km Wild life area) & III (4.0 Km Wild life area). Alt. I is approachable. Hence Alternative–I has been found to be most optimum and recommended for detailed survey.

4.3 Evaluation of alternatives Route Alignment of 400 KV D/C Jamshedpur-Baripada Transmission line

- Three different alignments (Map-5) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these four alternatives are as follows:

| SL.NO. | DESCRIPTION | ALIGNMENT-I | ALIGNMENT-II | ALIGNMENT-III |
| --- | --- | --- | --- | --- |
| 01 | Route particulars |  |  |  |
| i. | Length(in Kms) | 133.5 | 137 | 138 |
| ii. | State |  |  |  |
| ii. | Terrain |  |  |  |
| a. | Hilly/Plain in (Kms) | Hilly-NIL | Hilly – 4.35 | Hilly - NIL |
| b. | Agriculture | 118.260 km | 110 | 109 |
| c. | Wet/marshy |  |  |  |
| d. | Estuarine |  |  |  |
| e. | Other type of land |  |  |  |
| 02 | Environmental Details |  |  |  |
| i | Name of district/District details (Through which transmission line pass) | Seraikela-Kharsawan (Jharkahand) East Singhbhum (Jharkahand) Mayurbhanj (Orissa) | Seraikela-Kharswan (Jharkahand) East singhbhum (Jharkahand) Mayurbhanj (Orissa) | Seraikela-Kharswan (Jharkahand) East singhbhum (Jharkahand) Mayurbhanj (Orissa) |
| ii | Town in Alignment (Near By) | Jamshedpur Jadugoda Ghatshilla Baharagora Baripada | Jamshedpur Jadugoda Ghatshilla Baharagora Baripada | Jamshedpur Jadugoda Ghatshilla Baharagora Baripada |
| iii. | House within ROW | NIL | NIL | NIL |
| iv. | Forest in Km./Ha | 1.5/6.9 | 1.5/6.9 | 3.0/13.8 |
| v. | Type of forest Reserve protected/ Mangrove/Wild life area /Biosphere reserve / any other environment sensitive area | PF/RF– NIL) | PF/ | PF/RF |
| vi. | Density of forest | < 0.6 | < 0.6 | < 0.7 |
| vii. | Type of Fauna | Sal, Kondu, Malwa | Sal, Kondu, Malwa | Sal, Kondu, Malwa |
| viii. | Flora | Fox, Wolf, Reptiles, Monkey | Fox, Wolf, Reptiles, Monkey | Fox, Wolf, Reptiles, Monkey |
| ix. | Endangered species if any | NIL | NIL | NIL |
| x. | Historical/Cultural monument | NIL | NIL | NIL |
| f) | Any other relevant information | NIL | NIL | NIL |
| 03 | Compensation cost |  |  |  |
| i. | Crop. | Rs.49.236 lacs | Rs.50.54 lacs | Rs.53.36 lacs |
| ii. | Forest & NPV | Rs.65.70 lacs | Rs.65.70 lacs | Rs.131.54 lacs |
| 04 | No. of Crossing |  |  |  |
| i ii iii | Railway Power line River Xing etc. | 06 09 (132 KV and above 04 | 06 10 | 03 10 |
| 05 | Construction Problem | Very less as most of the part is approachable and near to National Highway | Comparatively more problem as approach is difficult for many parts | Fewe problems as there is no hilly terrain |
| 06 | O&M Problem | Very less as most of the part is approachable and near to National Highway | Comparatively more problem as this consists of hilly terrain and approach is difficult | Less problems as this consists of mainly agricultural land |

It may be seen from above that Alternative-I is the most optimum route as the line length is the shorter than Alternative II&III. Alt-I involves less forest in comparison than Alt. III. Forest density is also less in comparison than Alt. II and Alt. III. It passes near to National Highways and easily approachable in comparison to other alternatives. Hence, Alternative-I has been found to be most optimum and recommended for detailed survey.

4.4    Evaluation of alternatives Route Alignment of 400 KV, D/C

Baripada-Mendhasal Transmission line

- Three different alignments (Map-6) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these four alternatives are as follows:

| SL.NO | DESCRIPTION | ALT - I | ALT - II | ALT – III |
| --- | --- | --- | --- | --- |
| 1 | Route Particulars |  |  |  |
| i. | Length (in Kms) | 267.51 | 313.00 | 265.39 |
| ii. | State |  |  |  |
| ii. | Terrain | Hilly & Plain | Hilly & Plain | Hilly & Plain |
| 2 | Environment Details |  |  |  |
| i. | Name of district/District details (Through which transmission line pass) | Mayurbhanj Balsore Bhadrak Jaipur Cuttack Khurda | Mayurbhanj Balsore Bhadrak Jaipur Cuttack Khurda | Mayurbhanj Balsore Bhadrak Jaipur Cuttack Khurda |
| ii. | Town in alignment (Nearby) | Baripada, Anadapur, Balasore, Bhadrak, Jaipur, Cuttack, Bhubaneswar. | Baripada, Anadapur, Bhadrak, Bhubaneswar | Baripada, Anadapur, Balasore, Soro,Bhadrak, Jaipur, Panikoli, Chandikhol,Cuttack, Bhubaneswar |
| iii. | House within ROW | Nil | 2 | 11 |
| iv. | Forest in Km/Ha | 27.295 Km/125.58 Ha. (Including 120 meter wild life ) | 71.295 km/328.03 Ha. (Including 1.30Km Wild life) | 67.745 km/ 311.69 Ha ( Wild life path) |
| v. | Type of forest | Revenue forest, village forest | Revenue forest, village forest | Wild Life Zone, Protected forest, Revenue forest, village forest, Similipal Biosphere |
| vi. | Density of forest | 0.15 | 0.22 | 0.3 |
| vii. | Type of flora | Sal, Kendu, Mahua etc. | Sal, Kendu, Mahua etc. | Sal, Piasal, sandalwood, Kendu, Herbal plant etc. |
| viii | Type of fauna | Fox, Wolf, Reptiles, Monkeys, Elephant | Fox, Wolf, Reptiles, Monkeys, Elephant | Fox, Wolf, Reptiles, Monkeys, Elephant |
| ix | Endangered species if any | Nil | Wolf | Tiger, Elephants, Peacock etc. |
| x | Historical/Cultural monuments | NIL | NIL | 03 No. of Old Temples |
| xi. | Any other relevant information, if any | - | - | Naxlalite dominated area and may create problem during construction |
| 3 | Compensation Cost ( Rs. In Lakhs) |  |  |  |
|  | i) Crop/Trees | Rs.89.596 lacs | Rs.90.156 lacs | Rs.73.72 lacs |
|  | ii) Forest and NPV | Rs.1197.02 lacs | Rs.3126.78 lacs | Rs.2971.02 lacs |
| 4 | No. of Crossing | 28 | 37 | 20 |
| i) | Railway | 5 | 5 | 5 |
| ii) | Trans. Line | 13 | 13 | 7 |
| iii) | River crossing etc. | 4 | 6 | 6 |
| 5. | Construction Problem | 1. Easily approach to line. 2. Well connectivity through pacca road & moron road | 1. Vehicle movement for transporting the construction material shall be very difficult. 2. House with ROW | 1. Heavily populated patch; may create ROW problem during construction. 2. House within ROW. 3. Wildlife Sanctuary involves. 4. 03 no. of old temples are in ROW. |
| 6. | O&M Problem | 1. Easily approach to TL 2. Well connectivity through pacca road & moron road | 1. Very remote area. 2. Vehicle movement may be restricted during monsoon season. | 1. Modferate approach road available. |

It may be seen from above that Alternative-I is the most optimum route as the line length is the shorter than Alternative II and longer than Alt-III. Alt-I involves less forest (including wild life area 120 meters) in comparison than Alt. II (involve 1.30 Km wild life area) & Alt. III. Forest density is also less in comparison than Alt. II and Alt. III. It passes near to National Highways and easily approachable in comparison to other alternatives. Hence, Alternative-I has been found to be most optimum and recommended for detailed survey.

# SECTION-V:   SCREENING OF POTENTIAL

# ENVIRONMENTAL IMPACT, EVALUATION

# AND ITS MANAGEMENT

- 5.0	Impact Due to Project Location and Design

Environmental impact of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. In order to get latest information and further optimization of route modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS in route selection result in access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. In the instant project also these techniques are used. All possible measures have been taken during the finalization of route alignment for the proposed transmission system but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to specific environment review criteria based on preliminary survey are as follows:

- (i)	Resettlement

- As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be seen from the above description of proposed route alignment and also keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

- The proposed project does not envisage construction of any new substation and only extension of bays at existing substation is involved. Land for such extension is already available for proposed bays in these existing substations. Hence no fresh land acquisition or R&R issues are involved in the proposed projects.

- (ii)	 Land value depreciation

- Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

- (iii) Historical/cultural monuments/value

- As per the POWERGRID’s policy of route selection only that route alignment is finalized this avoids all the historical and cultural monuments. As per the preliminary assessment carried out during finalization of route alignment in consultation with State revenue authorities and Archeological Survey of India (ASI), no such monuments are coming in the proposed route alignment.

- (iv)	 Encroachment into precious ecological areas

- As already explained all precautions have been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries. However, complete avoidance of forest area and wild life area was not possible in Durgapur-Jamshedpur and Baripada-Mendhasal transmission lines and wildlife area is not involved in Jamshedpur-Baripada line though some forests are involved. The routes of proposed transmission line are so finalized that it affects minimum forest area and wildlife area in consultation with forest department.  Out of total transmission lines length of about 571 Kms about 33 Kms ( 5.78%) length shall  pass  through  forest  land  consisting  of  148 Ha. forest area  in  the  states  of   West Bengal, Jharkhand and Orissa.

- Durgapur–Jamshedpur transmission line shall pass (about 1.25 Kms) through Dalma Wild life Sanctuary. In this portion this line would be accommodated on multi ckt. Tower in the existing Right Of Way (ROW) of Maithan –Jamshedpur line. Existing tower of Maithan-Jamshedpur line would be replaced by multi circuit tower and no fresh land would be required in the wild life area.  Baripada –Mendhasal line encounters 120 meters stretch of Chandaka Wild life sanctuary (near Mendhasal) i.e. wildlife corridor. This would be crossed overhead with about 400m span and with increased tower height for more ground clearance due to safe movement of wildlife animal if there is any. As such no tower shall be constructed/ installed in this portion of wildlife sanctuary except crossing overhead.  Prior approval of GOI/MOEF shall be obtained for affected forest under Forest (Conservation) Act, 1980 after detail survey and finalization of route through forest area in consultation with local forest authorities. Most  of  the forests to  be  traversed  by  the  lines  are  already  heavily  degraded  and the  wildlife species  present  are  those  who  have  been  adapted  to  open  or  disturbed habitat. Therefore with provision  of  Compensatory  Afforestation  the  overall  forest status  will  in  many  cases  improve. Nonetheless, to  mitigate  losses  to  existing forests, clearing  of  the  transmission  line  Right-of-way  will  be done  under supervision  of  Forest  Department, and  some  low  canopy  seed  trees  and  shrubs may  be  kept  intact  if  they  do not  interfere  with  tower  erection and  line installation. The wood will be sold by the Forest Department, who will also retain the sale proceeds. Three-meter wide strips of land under each conductor will be cleared and maintained as maintenance rows, but the remaining land will be allowed to regenerate. Lopping of trees to maintain line clearance will be done under the direction of Forest Department. POWERGRID will provide construction crews with fuel wood or alternative fuels as a precaution against collection of fuel wood from nearby forest.

- Transmission lines can serve as new access routes into previously inaccessible or poorly accessible forests, thereby accelerating forest and wildlife loss. In such cases, POWERGRID cannot take action itself, but local Forest Department personnel will normally assess the dangers and take appropriate action, such as establishing guard stations at the entrance to the forest etc. cost of which is borne by POWERGRID and is included in the compensatory afforestation scheme. Given the already easy access and degraded conditions at the proposed projects sites, this problem is not expected to be encountered.  Nonetheless, POWERGRID staff will report to the Forest Department any noticeable encroachment induced by the Projects.

- Budget provision of Rs.1428 lakhs has been kept in the cost estimate to meet the requirement of Compensatory Afforestation and Net Present Value (NPV).

- (v)     Encroachment into other valuable lands

- Impacts on agricultural land will be restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1). It is estimated that for Durgapur-Jamshedpur 400 KV D/C line of 170 Km a total of 510 towers shall be required which may impact land area of 0.0510ha. and for Jamshedpur-Baripada 400 KV line of 133 Km a total of 399 towers are required resulting in loss of 0.0399 ha. In case of Baripada-Mendhasal 400KV transmission line of 268 Km a total of 804 towers shall be required which may impact land area of 0.0804 ha.  The total land loss estimated to be about 0.1713ha. which is negligible and will not adversely affect the land holding.

- In areas where lines will traverse agricultural land, compensation will be paid to owners for any crop damage incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation.  Revenue department, after evaluating the loss due to construction activity and productivity of land, arrives at the compensation cost that is paid to farmer.  Agricultural activities will be allowed to continue following the construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner's satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss of products from the tree over an eight year period (for fruit bearing trees) and for other trees compensation is finalized in consultation with local forest authorities. Agricultural lands under private ownership will be identified, and in accordance with normal POWERGRID procedures compensation will be paid to the affected villagers.  Budgetary provision Rs. 200.36 lakh is made in the cost estimate to meet these expenses.

- (vi)	  Interference with other utilities and traffic

- As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission lines affect nearby telecommunication circuits by causing electrical interference.  A standing committee -- Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like rerouting of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

- The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50000/- per km for POWERGRID. Provision to meet these expenses has been made in the cost estimate for the same.

- Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission lines pass by the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights are placed atop these towers.

- (vii) Interference with drainage pattern

- As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings, which is very small, there is little possibility of affecting drainage pattern.  In the infrequent instances where the drainage is affected, flow will be trained and guided to safe zones.

- 5.1	Environmental Problems Due to Design

- (i)	Escape of polluting materials

- The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials.

- (ii)	Explosion/fire hazards

- During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires.

- Fires due to flashover from lines can be a more serious problem in forest. However, adequate safety measures shall be taken to avoid such incidence besides this forest authorities also incorporate measures like making fire lines to prevent spreading of fire in the affected forest area.

- (iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

- Adequate measures are taken to re-surface the area where excavation works are done.  Topsoil disturbed during the development of sites will be used to restore the surface of the platform. Infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for tower foundations.

- (iv)	 Environmental aesthetics

- Since spacing between the towers in case of 400 KV lines is approx. 330-400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible.   POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted grey or green to merge with the background.

- (v)	 Noise/vibration nuisances

- The equipment installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian standards. The noise levels reported during normal operating conditions are about 60 to 70 dB at 2 m. distance from the equipment.  To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipment are undertaken.  In addition, plantations of sound absorbing species like Casuarinas, Tamarind, and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.  Actual noise levels measured at perimeters of existing Substations are 20 to 30 dB.

- (vi)  Blockage of wildlife passage

- The lines are passing through mostly agricultural, wasteland and forests. Durgapur-Jameshedpur line will be accommodated in existing Right of Way of Maithan-Jameshpur line in Dalma wildlife sanctuary with multi-circuit tall towers. Additional corridor will not be involved. Baripada – Mendhasal line will cross Chandaka wildlife sanctuary (120 Meter) near Mendhasal. No tower will be constructed within Chandaka wild life sanctuary area and this stretch (120 Meter) shall be crossed overhead with a span of 400meter approx. with tall tower. Hence possibilities of disturbance to wildlife are remote.

- 5.2	Environmental Problems during Construction Phase

- (i)	Uncontrolled silt runoff

- The Project involves only small scale excavation for tower foundations at scattered locations that are re-filled with excavated material therefore uncontrolled silt run off is not expected.

- (ii)  Nuisance to nearby properties

- As already described in preceding paras, during site selection due care is taken to keep the transmission line and substations away from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour therefore nuisance to the nearby properties if any, is not expected.

- (iii)  Interference with utilities and traffic and blockage of access way

- Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures.

- As and when a transmission line crosses any road/ railways line, the terminal towers are located at sufficient distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

- (iv)	  Inadequate resurfacing for erosion control

- The proposed lines are to be constructed in plain area where erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained above adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is generally undertaken outside the rainy season.

- (v)	Inadequate disposition of borrow area

- As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area.

- (vi)  Protection of Worker's health/safety

- The Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents will guide provisions for workers’ health and safety. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the Erection Conditions of Contract (ECC), a part of bidding documents.

In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act. POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Deptt. POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. These guidelines/checklist include work permits and safety precautions for work on the transmission lines both during construction and operation (Annexure-I) and is monitored regularly by site in-charge and corporate Operation Services.

- 5.3	Environmental Problems Resulting from Operation

- (i)     O&M Staff/Skills less than acceptable resulting in variety of adverse effects

- The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However in respect of the long distance transmission lines there are monitoring offices that are located at various points en-route. Monitoring measures employed include patrolling and thermo-vision scanning.

- The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

- A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard.  Because of issues relating to need to ensure health and safety relating to the line such as fire safety, safe voltages on metallic parts of buildings, and safety clearances to avoid flashover, the transmission lines will not pass directly over any residential properties and as such the potential for EMF effects to occur will be further diminished. Given that it will be necessary to ensure that there are no properties in the ROW beneath and to the sides of the overhead line, automatic mitigation against EMF will be provided between the source of potentially high strengths (the transmission line) and the residential properties.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India :

“Power Grid was the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid does not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

5.4     CRITICAL ENVIRONMENTAL REVIEW CRITERIA

(i)	Loss of irreplaceable resources

The transmission projects do not involve any large scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As only 5.78 % of the lines in the subject project are passing through forest area and the said forest is degraded, the problem of losing natural resources in this project is not envisaged. Moreover, on the other hand with the provision of raising compensatory afforestation on double the area getting affected will increase the forest cover.

(ii)	 Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area. There is no possibility of endangering/causing extinction of any species.

(iv)	Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

5.6    PUBLIC CONSULTATION

Public consultation/information is an integral part of the project implementation. Public is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted.

Apart from this, public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting  as  per  Environmental Social Policy  & Procedures of POWERGRID  (ESPP) shall also be carried out during different activities of project cycle. During such consultation the public will be informed about the project in general and in particular about the following:

- complete project plan (i.e. its route and terminating point and substations, if any, in between);

- POWERGRID design standards in relation to approved international standards;

- Health impacts in relation to EMF;

- Measures taken to avoid public utilities such as school, hospitals, etc.;

- Other impacts associated with transmission lines and POWERGRID’s approach to minimizing and solving them;

- Land acquisition details, proposed R&R measures and compensation packages in line with POWERGRID’s policy;

- Trees and crop compensation process.

Apart from organising many informal group meetings in different villages (Table-5.1) public meeting were also organised in the routes of transmission lines. To get the maximum participation during the Public consultation Program a notice was served well in advance to the villagers is enclosed as Annexure-II. The details of lines and its importance were explained to the villagers. Photographs of public consultation are enclosed as Plate-I. The program was arranged in interactive way and queries like crop compensation, route alignment etc. were replied. Most of the participants were small farmers and were worried about their land through which the line will pass.  They were informed that POWERGRID will not acquire their land for construction of transmission lines.  Only towers will be spotted in their fields where they can do farming without any fear because the tower height is very high and even tractor can pass below the tower.  Moreover, there is no risk of passing current from the above line as there is foolproof system of earthing for tower. The consultation process was appreciated by the villagers.  They were happy to know about the transparent policy of POWERGRID for execution of the project and promised to extend their cooperation during construction of the line.  The process of such consultation and its documentation shall continue during project implementation and even during O&M stage.

Table 5.1

Informal Group Meeting

| Sl. No. | Name of transmission line | Date of meeting | No. of villagers attended | Name of village | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Durgapur-Jamshedpur 400 KV D/C | 04.09.02 | 30 | Nachan | The people from all sphere of life such as farmers, teachers, servicemen etc. attended the meeting |
|  |  | 05.09.06 |  | Ballavpur |  |
|  |  | 11.09.06 | 25 | Kenjakura |  |
|  |  | 12.09.06 | 35 | Tarapur |  |
|  |  | 20.09.06 | 20 | Puncha |  |
|  |  | 21.09.06 | 27 | Ragma |  |
|  |  | 10.10.06 | 30 | Katjore |  |
|  |  | 11.10.06 | 24 | Pendrabera |  |
| 2. | Jamshedpur-Baripada 400 KV D/C line | 10.01.07 | 07 | Jhargobindpur | Village panchayat representatives, teachers, villagers were contacted and explained regarding this project during these consultations. |
| 02. | -do- | 10.01.07 | 09 | Ramasa |  |
| 03. | -do- | 10.01.07 | 12 | Rajgaun |  |
| 04. | -do- | 11.01.07 | 09 | Brindabanpur |  |
| 05. | -do- | 11.01.07 | 08 | Hitku |  |
| 06. | -do- | 14.01.07 | 10 | Ulda |  |
| 07. | -do- | 14.01.07 | 09 | Jagannath khunta |  |
| 08. | -do- | 14.01.07 | 12 | Kendudiha |  |
| 3. | Baripada-Mendhasal 400 KV D/C | 20.10.06 | 10 | Ambadubhi | Free electricity, road path during construction and O&M stage, Compensation for Crop/tree were main concerned which was clarified during the meeting |
| 2. | -do- | 23.10.06 | 11 | Naripur |  |
| 3 | -do- | 28.10.06 | 12 | Dulapatna |  |
| 4 | -do- | 02.11.06 | 9 | Gedama |  |
| 5 | -do- | 07.11.06 | 12 | Mantir |  |
| 6 | -do- | 08.11.06 | 9 | Alian |  |
| 7 | -do- | 10.11.06 | 7 | Chandaka |  |
| 8 | -do- | 12.11.06 | 8 | Bajabati |  |
| 9 | -do- | 19.11.06 | 6 | Sana Munduli |  |

5.7	Conclusions

From the above discussion, it would seem that the area is rich in physical resources. But careful route selection has minimized involvement of forest area to the extent possible but could not be completely avoided due to terrain and other physiographical reasons.  Thus, routes selected for detailed survey are the most optimum alignment and involved minimum forest and no R&R issues are anticipated as no fresh land acquisition is involved and only extension of existing substations within the existing boundary is involved. The existing tower of Maithan -Jamshedpur transmission line would be upgraded to multi Ckt tower in Dalma Wild Life sanctuary.  The proposed Durgapur - Jamshedpur transmission line would be accommodated on upgraded multi ckt tower in Dalma wild life area. No extra/fresh wild life area/forest area would be required for construction of transmission line in wild life area. In case of Baripada-Mendhasal line no tower would be constructed in wild life area as it is only 120 meter wide strip which would be crossed aerially with tall towers (2 nos.-400 meters apart).   Hence, based on the criteria agreed with The Bank the project is coming under the B category or non sensitive as only 5.78% of total length is passing through forest, which is already degraded in nature and no major impact on wild life is envisaged as the forest is not the habitat of any endangered or endemic sp. of fauna and flora.  The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts while on the one hand underline the need for implementation of the project for overall development of the area and on another hand suggest that a detailed E.I.A. may not be necessary.

# SECTION-VI:  MONITORING AND ORGANISATIONAL

# SUPPORT STRUCTURE

6.0      ENVIRONMENTAL MONITORING PROGRAM IN POWERGRID:

- Monitoring is a continuous process for POWERGRID projects at all the stages be it the site selection, construction or maintenance.

- The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan:

- 6.1    Corporate Level

- An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and Site to carry out environmental and social surveys for new projects.

- Assisting RHQs and site to finalize routes of entire power transmission line considering environmental and social factors that could arise enroute

- Help RHQs and Site to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & site Officials on environment and social issues and their management plan.

- 6.2     Regional Level

- At its Regional Office POWERGRID has a Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the Corporate level and the Construction Area Office (CAO). The key functions envisaged for ESMC are:

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and CAO to finalise routes of entire power transmission lines considering the environmental and social factors that could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and CAOs on various environmental and social aspects.

6.3      Site Office

- At the Site level , POWERGRID has made  the head of the CAOs responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social  Management Team (ESMT) . Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalise the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorised Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

- As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park , it  is generally  done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties. A monitoring system (done by the Forest Department) is also in place for compensatory forests established as part of the Project.  A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn (Table- 6.1) and will be implemented during various stage of project execution.

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID’s CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan.

Table-6.1 Environment Management Plan

| Project activity /stage | Potential impact | Proposed mitigation measure | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-construction |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Tower location and line alignment selection with respect to nearest dwellings | Setback distances to nearest houses - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformers stated in tender specification - once | POWERGRID | Part of tender specifications for the equipment |
|  |  | Processes, equipment and systems not to use chlorofluorocarbons (CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – once | POWERGRID | Part of tender specifications for the equipment |
|  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design |
| Transmission line design | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Electromagnetic field strength for proposed line design | Line design compliance with relevant standards - once | POWERGRID | Part of detailed alignment survey and design |
| Location of transmission towers and transmission line alignment and design | Impact on water bodies and land | Consideration of tower location at where they could be located to avoid water bodies | Tower location and line alignment selection (distance to water bodies. | Consultation with local authorities and avoiding tower foundation in water bodies | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
|  | Social inequities | Careful route selection to avoid existing settlements | Tower location and line alignment selection (distance to nearest dwellings or social institutions) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
|  |  | Minimise need to acquire agricultural land | Tower location and line alignment selection (distance to agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
| Encroachment into precious ecological areas | Loss of precious ecological values/ damage to precious species | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest designated ecological protection area) | Consultation with local forest authorities to avoid/minimize forest involvement - once | POWERGRID | Part of detailed siting and alignment survey /design |
| Transmission line through forestland | Deforestation and loss of biodiversity | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest protected or reserved forest) | Consultation with local authorities - once | POWERGRID | Part of detailed siting and alignment survey/design |
|  |  | Minimise the need by using existing towers, tall towers and RoW, wherever possible |  | Consultation with local authorities and design engineers - once |  |  |
|  |  | Obtain statutory clearances from the Government | Statutory approvals from Government | Compliance with regulations – once for each subproject |  |  |
| Encroachment into farmland | Loss of agricultural productivity | Use existing tower footings/towers wherever possible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
|  |  | Avoid siting new towers on farmland wherever feasible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once |  | Part of detailed siting and alignment survey /design |
|  |  | Farmers compensated for any permanent loss of productive land | Design of Implementation of Crop Compensation (based on affected area) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  | Farmers/landowners compensated for significant trees that need to be trimmed/ removed along RoW. | Design of Implementation of Tree compensation (estimated area to be trimmed/removed) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  |  | Statutory approvals for tree trimming /removal | Compliance with regulations – once for each subproject |  | Part of detailed sitting and alignment survey /design |
| Interference with drainage patterns/Irrigation channels | Flooding hazards/loss of agricultural production | Appropriate siting of towers to avoid channel interference | Tower location and line alignment selection (distance to nearest flood zone) | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
| Construction |  |  |  |  |  |  |
| Equipment layout and installation | Noise and vibrations | Construction techniques and machinery selection seeking to minimize ground disturbance. | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance - once at the start of each construction phase | POWERGRID (Contractor through contract provisions) | Construction period |
| Physical construction | Disturbed farming activity | Construction activities on cropping land timed to avoid disturbance of field crops (within one month of harvest wherever possible). | Timing of start of construction | Crop disturbance –Post harvest as soon as possible but before next crop - once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Mechanized construction | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment – estimated noise emissions | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Noise, vibration, equipment wear and tear | Turning off plant not in use. | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction of roads for accessibility | Increase in airborne dust particles | Existing roads and tracks used for construction and maintenance access to the line wherever possible. | Access roads, routes (length and width of new access roads to be constructed) | Use of established roads wherever possible - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Temporary blockage of utilities | Overflows, reduced discharge | Temporary placement of fill in drains/canals not permitted. | Temporary fill placement (m3) | Absence of fill in sensitive drainage areas - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Marking of vegetation to be removed prior to clearance, and strict control on clearing activities to ensure minimal clearance. | Vegetation marking and clearance control (area in m2) | Clearance strictly limited to target vegetation - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Trimming/cutting of trees within RoW | Fire hazards | Trees allowed growing up to a height within the RoW by maintaining adequate clearance between the top of tree and the conductor as per the regulations. | Species-specific tree retention as approved by statutory authorities (average and maximum tree height at maturity, in meters) | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Loss of vegetation and deforestation | Trees that can survive pruning to comply should be pruned instead of cleared. | Species-specific tree retention as approved by statutory authorities | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Felled trees and other cleared or pruned vegetation to be disposed of as authorized by the statutory bodies. | Disposal of cleared vegetation as approved by the statutory authorities (area cleared in m2) | Use or intended use of vegetation as approved by the statutory authorities – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Wood/vegetation harvesting | Loss of vegetation and deforestation | Construction workers prohibited from harvesting wood in the project area during their employment, (apart from locally employed staff continuing current legal activities). | Illegal wood /vegetation harvesting (area in m2, number of incidents reported) | Complaints by local people or other evidence of illegal harvesting - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Surplus earthwork/soil | Runoff to cause water pollution, solid waste disposal | Soil excavated from tower footings disposed of by placement along roadsides, or at nearby house blocks if requested by landowners. | Soil disposal locations and volume (m3) | Acceptable soil disposal sites - every 2 weeks | POWER GRID (Contractor through contract provisions) | Construction period |
| Tower construction – disposal of surplus earthwork/fill | Waste disposal | Excess fill from tower foundation excavation disposed of next to roads or around houses, in agreement with the local community or landowner. | Location and amount (m3)of fill disposal | Appropriate fill disposal locations - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Storage of chemicals and materials | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports (type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction schedules | Noise nuisance to neighbouring properties | Construction activities only undertaken during the day and local communities informed of the construction schedule. | Timing of construction (noise emissions, [dB(a)]) | Daytime construction only - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Provision of facilities for construction workers | Contamination of receptors (land, water, air) | Construction workforce facilities to include proper sanitation, water supply and waste disposal facilities. | Amenities for Workforce facilities | Presence of proper sanitation, water supply and waste disposal facilities - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period |
| Encroachment into farmland | Loss of agricultural productivity | Use existing access roads wherever possible | Usage of existing utilities | Complaints received by local people /authorities - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Ensure existing irrigation facilities are maintained in working condition | Status of existing facilities |  |  |  |
|  |  | Protect /preserve topsoil and reinstate after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  |  | Repair /reinstate damaged bunds etc after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  | Social inequities | Compensation for temporary loss in agricultural production | Implementation of Crop compensation (amount paid, dates, etc.) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Uncontrolled erosion/silt runoff | Soil loss, downstream siltation; | Need for access tracks minimised, use of existing roads. | Design basis and construction procedures (suspended solids in receiving waters; area re-vegetated in m2; amount of bunds constructed [length in meter, area in m2, or volume in m3]) | Incorporating good design and construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Limit site clearing to work areas |  |  |  |  |
|  |  | Regeneration of vegetation to stabilise works areas on completion (where applicable) |  |  |  |  |
|  |  | Avoidance of excavation in wet season |  |  |  |  |
|  |  | Water courses protected from siltation through use of bunds and sediment ponds |  |  |  |  |
| Nuisance to nearby properties | Losses to neighbouring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | As much as possible existing access ways will be used. | Design basis and layout | Incorporating good design engineering practices – once for each site |  |  |
|  |  | Productive land will be reinstated following completion of construction | Reinstatement of land status (area affected, m2) | Consultation with affected parties – twice – immediately after completion of construction and after the first harvest |  |  |
|  | Social inequities | Compensation will be paid for loss of production, if any. | Implementation of Tree/Crop compensation (amount paid) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Health and safety | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work days caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  |
|  |  | Contractor to arrange for health and safety training sessions |  |  |  |  |
| Inadequate construction stage monitoring | Likely to maximise damages | Training of POWERGRID environmental monitoring personnel | Training schedules | Number of programs attended by each person – once a year | POWERGRID | Routinely throughout construction period |
|  |  | Implementation of effective environmental monitoring and reporting system using checklist of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof. | Submission of duly completed checklists of all contracts for each site - once |  |  |
|  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures. | Compliance report related to environmental aspects for the contract | Submission of duly completed compliance report for each contract - once |  |  |
| Operation and Maintenance |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Compliance with setback distances (“as-built” diagrams) | Setback distances to nearest houses – once in quarter | POWERGRID | During operations |
| Oil spillage | Contamination of land/nearby water bodies | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks. | Substatio bunding (Oil sump) (“as-built” diagrams) | Bunding (Oil sump) capacity and permeability - once | POWERGRID | During operations |
| Inadequate provision of staff/workers health and safety during operations | Injury and sickness of staff /workers | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |
|  |  | Safety awareness raising for staff. | Training/awareness programs and mock drills | Number of programs and percent of staff /workers covered – once each year |  |  |
|  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |
| Electric Shock Hazards | Injury/mortality to staff and public | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |
|  |  | Security fences around substations | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |
|  |  | Barriers to prevent climbing on/dismantling of transmission towers | Maintenance of barriers |  |  |  |
|  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |
|  |  | Electricity safety awareness raising in project areas | Training /awareness programs and mock drills for all concerned parties | Number of programs and percent of total persons covered – once each year |  |  |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | Processes, equipment and systems using cholofluorocarbons (CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government. | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |
| Transmission line maintenance | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Required ground clearance (meters) | Ground clearance - once | POWERGRID | Operations |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels (dB(a)) | Noise levels at boundary nearest to properties and consultation with affected parties if any - once | POWERGRID | Operations |