SECTION I: PROJECT DESCRIPTION

- BACKGROUND:

POWERGRID, the Central Transmission Utility (CTU) of the country, is engaged in power transmission with the mandate for planning, co-ordination, supervision and control over complete inter-State transmission system. As on March’10 POWERGRID has established about 77,000 ckt kms of transmission lines at 765 kV, 400 kV, 220 kV & 132 kV EHVAC & +500 kV HVDC levels and 131 sub-stations with transformation capacity of about 89,000 MVA. This transmission network, spread over length and breadth of the country, is consistently maintained at an availability of over 99.7% through deployment of state-of-the-art Operation & Maintenance techniques at par with global standards. About 45% of total power generated in the country is wheeled through this transmission network.

In Southern Region a number of generation projects under Central Sector like Neyveli TS-II Expn. (500 MW), Simhadri-II (1000 MW), Kalpakkam PFBR (500 MW), Vallur TPS (1500 MW), Kudankulam (2000 MW) etc, are being envisaged for commissioning during 11th plan. Further Krishnapatnam UMPP (4000 MW) is also envisaged to be implemented by early 12th plan. All the regional constituents shall have shares from these new generating projects which shall result in enhancement of shares of beneficiaries including Karnataka. The power from some of these generating stations is available at Gooty through various transmission lines and the power is required to be supplied to the major load centers in the state of Karnataka like Bangalore etc. To enable delivery of enhanced share of power as well as to meet growing load demand, augmentation of transmission system including establishment of new 400/220 kV substation shall be required. Madhugiri (Tumkur), near Bangalore in Karnataka State, is one such location identified for establishing new 400/220 kV substation which is to be connected to Gooty and Yelahanka to feed growing power demand in and around Bangalore area as a regional strengthening scheme.

The above scheme has been discussed and agreed in 28th meeting of Standing Committee of Southern Region Transmission System Planning held on June 15, 2009, at Coorg, Karnataka and 10th & 11th meeting of SRPC held on July 2, 2009 at Tirupati & September 17, 2009 at Bangalore respectively for taking this scheme as a Regional System Strengthening.

  - BENEFITS OF THE PROJECT:

The main benefit of the project will be meeting the long term power load demand of Bangalore city and also to export surplus power of Southern region to Western/Northern regions with reliability and security. Additionally, the project is likely to generate direct and indirect employment opportunities, promote industrial growth and stimulate overall development of the area. The basic infrastructure around the proposed substation will also improve substantially. POWERGRID, as a responsible corporate entity, always undertakes community development works around the location of its substations.

1.2 	PROJECT DESCRIPTION :

The present peak demand of the State of Karnataka is of the order of 5940 MW (July, 2009) which as per the 17th EPS is projected to increase to about 8830 MW by the end of 11th plan (year 2011-12). KPTCL has indicated that area around the Bangalore is likely to experience higher load growth rate. This requires additional feeds to Bangalore meet its growing load requirements. Therefore, establishment of a 400/220kV, 2x500 MVA substation at Madhugiri(Tumkur) has been planned to provide additional feed to Bangalore through proposed Madhugiri (Tumkur) - Yelahanka 400kV D/C quad line and also connected through Gooty - Madhugiri(Tumkur) 400kV D/C line. Further provision has been kept for establishment of a 765/400kV substation in future in the same switchyard which shall facilitate a high capacity corridor from Salem towards Basawan Bagewadi & Western region to export surplus power of Southern region to Western/Northern regions.

1.3 	PROJECT HIGHLIGHTS :

| a) | Project Name | : | System Strengthening XIII of SRTS |
| --- | --- | --- | --- |
| b) | Location | : | Andhra Pradesh & Karnataka |
| c) | Beneficiary States/UT | : | Andhra Pradesh, Karnataka, Kerala, Tamil Nadu and Pondicherry |
| d) | Project Cost | : | Rs. 494.83 Crores (including IDC) |
| e) | Commissioning Schedule | : | 2013 |

1.4 	PROJECT SCOPE :

The scope of the transmission line and Substation to be implemented under the above scheme are as follows:

Transmission lines

## a)  Gooty – Madhugiri (Tumkur) 400 kV D/C line –   210 Kms

## Madhugiri (Tumkur) - Yelahanka 400 kV D/C quad line – 77 Kms

Substations

a. Establishment of new 400/220 kV substation at Madhugiri (Tumkur)

This would be new substation owned by POWERGRID and shall have provision for establishment of 765/400kV substation in future in the same switchyard and also to accommodate following under this project:

- 2 number 400 kV line bays for Gooty – Madhugiri(Tumkur) 400 kV D/C line

- 2 number 400 kV line bays for Madhugiri(Tumkur) – Yelahanka 400 kV D/C quad

line

- 2 number 400 kV transformer bays for 2x500 MVA transformers

- 1 number 400 kV reactor bay for 1x63 MVAR Bus Reactor

- 2 number 220 kV transformer bays for 2x500 MVA transformers

- 4 number 220 kV line bays for 220 kV feeders of Karnataka

b.  Extension of 400/220 kV substation at Gooty

This is an existing substation owned by POWERGRID and shall have provision to accommodate following under this project:

- 2 number 400 kV line bays for Gooty – Madhugiri(Tumkur) 400 kV D/C line

c.  Extension of 400/220 kV GIS substation at Yelahanka

This substation is under implementation by POWERGRID and shall have provision to accommodate following under this project:

i.     2 number of 400 kV line bays (GIS) for Madhugiri (Tumkur) – Yelahanka 400 kV

D/C quad line

A power map showing the transmission grid of Southern Region highlighting the above line and other new projects placed as Exhibit - I.

SECTION II : BASE LINE DATA

2.0  	The project is located in the State of Andhra Pradesh and Karnataka in India. The basic details of the area under project are given below:

2.1 	ANDHRA PRADESH :

Andhra Pradesh has a geographic area of 27.51 million ha, which constitutes 8.37% of the total area of the country.  It is situated in the tropical zone and lines between latitude 120 40’ and 190 50 ‘N and longitude 76 0 45’ and 840 40’ E.

Physiographically the state can be divided into 3 distinct regions viz. (i) the mountainous region having Nallamalai and Erramalai Hills of the Rayalaseema and the Eastern Ghats (ii) the plateau having an altitude of 100 m to 800m and (iii) the deltas of rivers and the sea coast. The proposed project is located in the district of Anantapur of Rayalaseema region. The general land use pattern of the state is given in Table 2.1.

Table-2.1 Land use Pattern

| Land Use | Area in ‘000 ha | Percentage |
| --- | --- | --- |
| Total geographical area | 27,507 |  |
| Reporting area for land utilization | 27,505 | 100.00 |
| Forests | 6,210 | 22.58 |
| Not available for cultivation | 4,781 | 17.38 |
| Permanent pastures and other grazing lands | 602 | 2.19 |
| Land under misc.tree crops & groves | 320 | 1.16 |
| Culturable wasteland | 695 | 2.53 |
| Fallow lands other than current fallows | 1,583 | 5.76 |
| Current Fallows | 3,166 | 11.51 |
| Net area sown | 10,147 | 36.89 |

Source: Land use statistics, Ministry of Agriculture, GOI, 2006

Climate of the state is hot and humid with temperatures ranging from 15 0C to 450 C.  Annual rainfall rages from 1,100 to 1,250 mm in the north, reducing to about 500 mm in the south.  The north-eastern areas along the coastline are periodically lashed by cyclones.

The major rivers of the State are the Godavari, the Krishna and the Pennar, which   pass through the Deccan Plateau and drain into the Bay of Bengal. Nearly 75% of the state area is covered by the river basins of the Godavari, Krishna and Pennar, and their tributaries. There are 17 smaller rivers like the Sarada, Nagavali and Musi, as well as several streams.

Ecological Resources: The recorded forest area is 63,814 Sq Km, which constitutes 23.20% of the geographic area of the state.  Reserved forest constitutes 95.92%, Protected Forest 3.08%, and un-classed Forest 1.0%. However, total forest cover in state is 55,474 Sq. Km comprising of 820 Sq. Km very dense forest, 24,757 Sq. Km moderately dense forest and 19,525 Sq. Km is open forest and 10,372 Sq. Km is scrub (Map-1). The forests are the main source of supply of fodder and fuel and subsistence for the poorest sections of the people and tribal population in the interior under-developed areas of the state. The five forest types occurring in the state are Tropical Dry Deciduous, Tropical Thorn, Tropical Moist Deciduous, Tropical Dry Evergreen, and Littoral and Swamp forests.  Forests are distributed mainly in the form of a wide strip in the north starting from Nizamabad district in the west to Srikakulam in the east.  Besides this, a belt runs from central to the southern part of the state in the Nallamalai hills. There are 4 National Parks spread over an area of 0.33 million ha. and 21 Wildlife Sanctuaries over 1.25 million ha.. Thus a total of 1.58 m ha area constituting 5.76% of the geographical area of the State is under protected area network.

The largest Tiger Reserve of the country, Nagarjunasagar-Srisailam spread over 0.36 million ha and the Kolleru Wildlife sanctuary which is a Ramsar site are located in this State. The proposed project involves one districts of Andhra Pradesh viz. Anantapur. The details of forest resources available in the project area are as follows:

| District | Geographic area | Very Dense forest | Moderately Dense forest | Open forest | Total | % Forest cover |
| --- | --- | --- | --- | --- | --- | --- |
| Anantapur | 19130 | 0 | 73 | 353 | 426 | 2.23 |

The economy of Andhra is based on widely diversified farming base with a rich variety of cash crops. It is surplus in food grains, and produces over 10 m tons of rice therefore called as the granary of South. Agriculture sector accounts for around 50% of state economy and provides livelihood for 70% of the population. The major crops of the state are paddy, jawar, bazra, ragi, maize, groundnut, tobacco, cotton, castor and sugarcane. The district Anantapur is not rich in the Forest Wealth.  The name ' Forest’ in Anantapur District does not indicate any dense tree population with thick foliage of variform of pastures.

2.2  	KARNATAKA :

Karnataka is located in southwestern part of the country and lies between latitude 11°10’ to 18° 25’N and longitude 74°10’ to 78°35’E. It has a geographic area of 19.18 million ha constituting 5.83% of the total area of the country. It has a coastline of approximately 400 km.

Physiographically the State can be divided into two distinct regions viz the ‘Maland’ or hilly region comprising of Western Ghats and the ‘Maidan’ or the plain region comprising he inland plateau of varying height. Karnataka has representatives of all types of variations in topography - high mountains, plateaus, residual hills and coastal plains. The State is enclosed by chains of mountains to its west, east and south. It consists mainly of plateau which has higher elevation of 600 to 900 metres above mean sea level. The entire landscape is undulating, broken up by mountains and deep ravines. General land use pattern of the state is given in Table 2.2.

Table-2.2 Land use Pattern

| Land Use | Area in ‘000 ha | Percentage |
| --- | --- | --- |
| Total geographical area | 19,179 |  |
| Reporting area for land utilization | 19050 | 100.00 |
| Forests | 3,072 | 16.13 |
| Not available for cultivation | 2,151 | 11.29 |
| Permanent pastures and other grazing lands | 934 | 4.90 |
| Land under misc. tree crops & groves | 292 | 1.53 |
| Culturable wasteland | 416 | 2.18 |
| Fallow lands other than current fallows | 515 | 2.70 |
| Current Fallows | 1,565 | 8.22 |
| Net area sown | 10,105 | 53.04 |

Source: Land use statistics, Ministry of Agriculture, GOI, 2006

Climate: The state enjoys three main types of climates. For meteorological purposes, the state has been divided into three sub-divisions namely Coastal Karnataka, North Interior Karnataka and South Interior Karnataka  The Tropical Monsoon climate covers the entire coastal belt and adjoining areas. Southern half of the State experiences hot, seasonally dry tropical savana climate while most of the northern half experiences hot, semi-arid, tropical steppe type of climate. The climate of the State varies with the seasons.

Soil: The Red soil constitutes major soil type, followed by Black soil. In some area Lateritic soils, Alluvio-Colluvial Soils are also found.

Rainfall: The annual rainfall in the State varies from 2,000 3,200 mm in the Western Ghats and between 400-500 mm in the northern and northeastern parts. Average summer and winter temperature varies from 26°C to 35°C and 14°C to 25°C respectively.

Temperature: Both day and night temperatures are more or less uniform over the State, except at the coastal region and high elevated plateau. They generally decrease south-westwards over the State due to higher elevation and attain lower values at high level stations. April and May are the hottest months.

Mineral Resources: Karnataka is endowed with fairly rich mineral wealth distributed more or less evenly over its territory. It has one of the oldest Geological Survey Department in the country, started as far back as 1880.The State contains deposits of asbestos, bauxite, chromite, dolomite, gold, iron ore, kaolin, limestone, magnesite, manganese, ochre, quartz and silica sand. Karnataka is the sole producer of felsite and leading producer of gold (84%), moulding sand (63%) and fuchsite quartzite (57%).

Water Resources: Karnataka accounts for about six per cent of the country's surface water resources of 17 lakh million cubic metres (Mcum). About 40 percent of this is available in the east flowing rivers and the remaining from west flowing rivers. The major rivers of the State are Krishna, Cauvery, Godavari, North Pennar and South Pennar

Ecological Resources: The recorded forest area of State is 38,284 Square Kms which constitutes 19.96% of the geographic area of the State. Reserved Forest constitutes 74.94%, Protected Forest 10.27% and Unclassed Forest 14.79%. However, total forest cover in state is 36,190 Sq. Km comprising of 1,777 Sq. Km very dense forest, 20,181 Sq. Km moderately dense forest, 14,232 Sq. Km is open forest and 3,176 Sq. km is Scrub (Map-2). The forests are the main source of supply of fodder and fuel and subsistence for the poorest sections of the people and tribal population in the interior under-developed areas of the state. The five forest types found in the State are Tropical Wet Evergreen, Tropical Semi Evergreen, Tropical Dry Evergreen, Tropical Dry Deciduous and Tropical Thorn forests. Most of the forests in Karnataka are situated in a belt running from north to south starting from Belgaum to Mysore in the Western Ghats. There are 5 National Parks spread over an area of 0.25 million ha and 21 Wildlife Sanctuaries spread over 0.39 million ha. A total of 0.64 million ha constituting 3.32% of the geographical area of the State is under protected area network. Karnataka has two Tiger Reserves, Bandipur Nagarhole (extension) and Bhadra, covering 0.2 million ha. It shares Nilgiri Biosphere Reserve (area 5,520 km2) with Tamil Nadu and Kerala.

There are three patches of forest- Devarayanadurga forest; Bukkapattana forest and Huliyurudurga, Huttaridurga forest. Since rainfall is low the diversity of flora is not much and the species are restricted to xerophytic type, occasionally we come across with Teak, Mathi, Nandi etc.

The lines of proposed transmission system shall pass through two districts viz. Tumkur & Bangalore Urban District having forest cover 5.21% & 6.80% respectively. It may be noted from the table below that the forest cover in the above said districts are generally open/degraded. With due precaution in route alignment the forest area has been avoided completely.

| District | Geographic area | Very Dense forest | Moderately Dense forest | Open forest | Total | % Forest cover |
| --- | --- | --- | --- | --- | --- | --- |
| Tumkur | 10597 | 0 | 62 | 490 | 552 | 5.21 |
| Bangalore Urban | 2190 | 0 | 39 | 110 | 149 | 6.80 |

Karnataka is one of the more economically progressive states in India.  Nearly 56% of the workforce in Karnataka is engaged in agriculture and related activities Karnataka is the manufacturing hub for some of the largest public sector industries in India, including HAL, NAL, BHEL, ITI, BEML and HMT, which are based in Bangalore.  Since the 1980s, Karnataka has emerged as the pan-Indian leader in the field of Information Technology.   Exports from IT firms exceeded Rs. 50,000 crores ($12.5 billion) in 2006-07, accounting for nearly 38% of all IT exports from India.  Karnataka also leads the nation in biotechnology. It is home to India's largest bio cluster, with 158 of the country's 320 biotechnology firms being based here.  The state also accounts for 75% of India's floriculture.

SECTION III:   POLICY, LEGAL & REGULATORY FRAMEWORK

- 3.0   POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and directives and guidelines with respect to environmental and social considerations of Funding Agencies.

- 3.1      ENVIRONMENTAL :

3.1.1  Constitutional Provisions :

Subsequent to the first United Nations Conference on Human Environment at Stockholm in June, 1972, which emphasized the need to preserve and protect the natural environment, the Constitution of India was amended through the historical 42nd Amendment Act, 1976 by inserting Article 48-A and 51-A (g) for protection and promotion of the environment under the Directive Principles of State Policy and the Fundamental Duties respectively. The amendment, inter alia provide:

"The State shall endeavour to protect and improve the environment and to safeguard the forests and wildlife of the country". (New Article 48A)

"It shall be the duty of every citizen of India to protect and improve the natural environment including forests, lakes, rivers and wildlife and to have compassion for living creatures”. (New Article 51 A(g)

Article 21 of the constitution provides, “no person shall be deprived of his life or personal liberty except according to procedure established by law”.

Article 21 is the heart of the fundamental rights and has received expanded meaning from time to time after the decision of the Supreme Court in 1978. The Article 21 guarantee fundamental right to life – a life of dignity to be lived in a proper environment, free of danger of disease and infection. The right to live in a healthy environment as part of Article 21 of the Constitution. Recently, Supreme Court has broadly and liberally interpreted the Article 21, transgressed into the area of protection of environment, and held that the protection of environment and citizen’s right to live in eco-friendly atmosphere interpreted as the basic right guaranteed under Article 21.

Thus the Indian Constitution has now two fold provision:

(a) On the one hand, it gives directive to the State for the protection and 
        improvement of environment.

  - On the other hand the citizens owe a constitutional duty to protect improve  and

natural environment.

3.1.2 	Mandatory Requirements (National) :

- MOP order/sanction under The Electricity Act, 2003 :

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance under The Forest (Conservation) Act, 1980 :

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986 :

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded, hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001 :

MOEF vide its notification dt. 16th May, 2001 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/reconditioner or at the designated collection centres and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)Amendment Rules, 2003 :

MOEF vide its notification dt. 20th May, 2003 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned/sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000 :

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organization in near future.

- The Biological Diversity Act, 2002 :

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalizing the route alignment.

    - Funding Agencies :

- WB Operational Policies (OP) 4.01/ADB’s Safeguard Policy Statement (June 2009) and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.4   Prescriptive Framework (National) :

- Applicable Legislations

- 3.1.4   Relevant Policies :

- National Conservation Strategy and Policy Statement on Environment and

Development, 1992

- Policy statement for Abatement of pollution, 1992

- National Environment Policy, 2006

- 3.2.0   SOCIAL :

    - Constitutional Provisions :

Constitutional provisions in regard to social safeguards are well enshrined in the preamble such as JUSTICE, social, economic and political; LIBERTY of thought, expression, belief, faith and worship; EQUALITY of status and of opportunity; FRATERNITY assuring the dignity of the individual and the unity and integrity of the Nation. Fundamental Rights and Directive Principles guarantee the right to life and liberty.  Health, safety and livelihood have been interpreted as part of this larger right.  Social safeguards provisions are dealt in detail in different Article such as Article-14, 15 17, 23, 24, 25, 46, 330, 332 etc. POWERGRID, through this document, ESPP, commits itself to implementing the said constitutional provision in true sprit to fulfill its environmental and social obligations and responsibilities.

- 3.2.2   Mandatory Requirements (National) :

- National Rehabilitation and Resettlement Policy, 2007 :

Ministry of Rural Development, Government of India has notified a National policy on R&R for PAFs in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succor to the asset less rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003 :

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984 :

- When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - Funding Agencies :

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Safeguard Policy Statement, June 2009.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement :

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups.

- World Bank OD 4.20: Indigenous People (IP ):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan/Tribal Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Safeguard Policy Statement, June 2009 :

The SPS, June 2009 describes Bank Policy and operational procedures on three key safeguard areas viz Environmental, Involuntary resettlement and Indigenous Peoples as well as a set of specific safeguard requirements that borrowers are expected to meet during operations when addressing social and environment impacts and risks. Its objective is to ensure social and environmental sustainability of projects through avoidance, minimization, mitigation and/or compensate of adverse impacts on environment and affected peoples. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require neither resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.4    Prescriptive Framework (National) :

- National and State-wide Laws and Policies Relating to Land Acquisition and

Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan)

Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.5   Relevant Policies :

- Resettlement and Rehabilitation Policy- Coal India Ltd., May,2008

- Resettlement and Rehabilitation Policy- NHPC Ltd., 2007

- Policy for Rehabilitation and Resettlement of Land Owners – Land Acquisition

- Oustees – Haryana State, December, 2007;

- The Orissa Resettlement and Rehabilitation Policy, Orissa , May, 2006;

- Resettlement and Rehabilitation Policy – NTPC Ltd., June,2005;

# SECTION-IV: POWERGRID APPROACH FOR ROUTE SELECTION

- 4.0     ROUTE SELECTION - (ASSESSMENT & MANAGEMENT PROCESS) :

- At the system planning stage itself one of the factors that govern the evolution of system is the possible infringement with the forest. Wherever such infringements are substantial, different alternative options are considered. The route/ site selection criteria followed by POWERGRID is detailed below:

- While identifying the transmission system for a generation project or as a part of National Power Grid, preliminary route selection is done by POWERGRID based on the Top sheets of Survey of India and Forest Atlas (Govt. of India's Publication).  During route alignment all possible efforts are made to avoid the forest area involvement completely or to keep it to the barest minimum, whenever it becomes unavoidable due to the geography of terrain or heavy cost involved in avoiding it.

- 4.1      STUDY OF ALTERNATIVES :

- Environmental Criteria for Route selection

- For selection of optimum route, the following points are taken into consideration:

- The route of the proposed transmission lines does not involve any human

rehabilitation.

- Any monument of cultural or historical importance is not affected by the route

of the transmission line.

- The proposed route of transmission line does not create any threat to the

survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility

services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

- In addition, care is also taken to avoid National parks, sanctuaries, eco-sensitive zones, tiger reserves and biosphere reserves etc.

- Keeping above in mind the routes of proposed lines under the project has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps etc. to arrive at most optimum route which can be taken up for detailed survey and assessment of environmental & social impacts for their proper management.

    - Evaluation of Alternatives Route Alignment 400 kV D/C Gooty –  Madhugiri

(Tumkur) Transmission Line :

- Three different alignments (Map-3) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three alternatives are as follows:

| S.N | Description | Alternative-I | Alternative-II | Alternative-III |
| --- | --- | --- | --- | --- |
| 1. | Route particulars |  |  |  |
|  | Route Length | 210.45 | 210.35 | 207.40 |
|  | Terrain |  |  |  |
|  | Plain | 98 % | 95 % | 90 % |
|  | Hilly | 02 % | 05 % | 10 % |
| 2. | Environmental impact |  |  |  |
|  | Town in alignment | The line is not passing through any town however major town in vicinity are Gooty, Anantapur & Tumkur. | The line is not passing through any town however major town in vicinity are Gooty, Anantapur & Tumkur. | The line is not passing through any town however major town in vicinity are Gooty, Anantapur & Tumkur. |
|  | House within ROW | Nil | Nil | Nil |
|  | Tree/Crops and its extent of damage | To be ascertained after detailed survey. | To be ascertained after detailed survey. | To be ascertained after detailed survey. |
|  | Forest involvement. | Nil | 5.850 Kms | 1.700 Kms |
|  | Type of fauna & flora | Flora: Coconut, Bamboo, Mango, Neem, eucalyptus etc Fauna: squirrels, monkeys etc. | Flora: Coconut, Bamboo, Mango, Neem, eucalyptus, etc Fauna: squirrels, monkeys etc. | Flora: Coconut, Bamboo, Mango, Neem, eucalyptus, etc Fauna: squirrels, monkeys etc. |
|  | Endangered species, if any | Nil | Nil | Nil |
|  | Historical/cultural monuments | Nil | Nil | Nil |
|  | Details of tribal areas, if any. | Nil | Nil | Nil |
| 3 | Compensation Cost |  |  |  |
|  | Towards forest clearance | Nil | 5.0 Crores (Approx.) | 1.5 Crores (Approx.) |
|  | Tree/crop | 210.45 lakhs @ Rs 1 lakh per km. | 204.5 lakhs @ Rs. 1 lakh per km. | 205.7 lakhs @ Rs. 1 lakh per km. |
| 4. | Major Crossings: |  |  |  |
|  | River / water body | 1 | 1 | 1 |
|  | Power line | 14 | 16 | 14 |
|  | Railway line | 2 | 2 | 2 |
|  | Highway | 3 | 3 | 3 |
| 5. | Construction problems | Easy approaches, Less width of Penna River Xing, Moderate ROW problems. | Easy approaches More width of Penna River Xing, Moderate ROW problems. Reserve Forest involvement. | Easy approaches, More width of Penna River Xing, Moderate ROW problems. Hilly terrains. Reserve Forest involvement. |
| 6. | O&M problems | O&M shall be easier due to non involvement of forest, Better approaches, and plain terrain. | O&M shall be difficult due to involvement of forest | O&M shall be difficult due to involvement of forest & Hilly terrain. |
| 7. | Overall remarks | Moderate route length with no forest involvement, Less width of Penna River Xing therefore recommended | More width of Penna River Xing & Reserve Forest involvement hence not recommended. | More width of Penna River Xing & Reserve Forest & Hilly terrain involvement hence not recommended. |

From the above comparison of the three different alternative, it is evident that the Alternative-I is the most optimum route without involvement of forest, thereby avoiding ecological sensitive areas. Though the line length is slightly higher in comparison to other alternatives as the route has no involvement of forest, minimum hilly terrain, shortest river crossing span, less construction problems, etc., suggests the alternative-I to be the most optimum and recommended for detailed survey.

    - Evaluation of Alternatives Route Alignment Of 400 Kv D/C Madhugiri -Yehalanka Transmission Line :

Three different alignments were studied with the help of published data / Survey of India topographical maps and walkover survey to arrive at most optimum route for detailed survey (Map 4). The comparative details of these three alternatives are as follows:

| S. N. | Description | Alternative-I (Dark Green Line) | Alternative-II (Purple Line) | Alternative-III (Pink Line) |
| --- | --- | --- | --- | --- |
| 1. | Route particulars |  |  |  |
| i | Route Length | 77 KM | 76 KM | 81 KM |
| ii | Terrain | Passing through plain and undulating terrain with gentle slopes | Passing through plain and undulating terrain with gentle slopes | Passing through plain and undulating terrain. |
| 2. | Environmental impact |  |  |  |
| i | Town in alignment | The line is not passing through any major towns. | The line is not passing through any major town however the only major town in the vicinity is tumkur. | The line is not passing through any major towns. |
| ii | House within ROW | Nil | Nil | Nil |
| iii | Tree/Crops and its extent of damage | To be ascertained after detailed survey | To be ascertained after detailed survey | To be ascertained after detailed survey |
| iv | Forest involvement. | Nil | 2kms | Nil |
| v | Type of fauna & flora | Flora: Coconut, Bamboo, Mango, Neem, eucalyptus, etc Fauna: squirrels, monkeys, reptiles etc. | Flora: Coconut, Bamboo, Mango, Neem, eucalyptus. etc Fauna: deer, squirrels, monkeys, reptiles etc. | Flora: Coconut, Bamboo, Mango, Neem, eucalyptus etc Fauna: squirrels, monkeys, reptiles etc. |
| vi | Endangered species, if any | Nil | Nil | Nil |
| vii | Historical/cultural monuments | Nil | Nil | Nil |
| viii | Details of tribal areas, if any. | Nil | Nil | Nil |
| 3 | Compensation Cost |  |  |  |
|  | Towards forest clearance | Nil | To be ascertained after detailed survey | Nil |
|  | Tree/crop | 77 lakhs @ Rs. 1 lakh per km. | 74 lakhs @ Rs. 1 lakh per km. | 81 lakhs @ Rs. 1 lakh per km. |
| 4. | Major Crossings: |  |  |  |
| i | River / water body | Nil | Hesarghatta tank crossing | Nil |
| ii | Power line | 5 lines | 5 lines | 5 lines |
| iii | Railway line / highway | Rly – Nil NH – 1 SH – 4 | Rly – Nil NH – 1 SH – 4 | Rly – Nil NH – 1 SH – 4 |
| 5. | Construction problems | Easy approaches, Relatively less ROW problems. | Forest crossing involved, moderate ROW problems | Relatively more line length moderate ROW problems |
| 6. | O&M problems | O&M shall comparatively be easy due to better approach. | Comparatively easy except in the forest area due to difficult approach. | O&M shall comparatively be easy due to better approach. |
| 7. | Overall remarks | This route is recommended due to no forest area involved, relatively less ROW problems and easy approach | Though the length of route is slightly less, it is not recommended due to forest crossing and Hessargatta tank crossing | Not recommended due to long route length |

From the above comparison of the three different alternative it is evident that the, Alternative-I is the most suitable route as avoidance of forest, habitation and less ROW problems. Hence, Alternative –I has been found to be most optimum and recommended for detailed survey.

# SECTION-V:  SCREENING OF POTENTIAL ENVIRONMENTAL IMPACT,

# EVALUATION AND ITS MANAGEMENT

- 5.0	IMPACT DUE TO PROJECT LOCATION AND DESIGN :

Environmental impact of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. In order to get latest information and further optimization of route, modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS in route selection result in access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. In the instant project also these techniques are used and detail survey using GIS/GPS is under progress.  Although, all possible measures have been taken during the finalization of route alignment for the proposed transmission line but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to specific environment review criteria based on preliminary survey are as follows:

- (i)	Resettlement

- As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be seen from the above description of proposed route alignment and also keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

The proposed project involves construction of one new substation at Madhugiri (Tumkur) and the extension of one existing substation at Gooty. However, fresh land acquisition is only involved for Madhugiri substation. In case of 765/400/220 KV Madhugiri substation,  site selection committee after studying different alternative selected land belonging to Karnataka Industrial Areas Development Board (KIADB), a Government of Karnataka organization. The land measuring 130 acres (52.6 ha.) is located in village Kempanadodderi, Madhugiri of district Tumkur, Karnataka.  Some land is being acquired by KIADB on consent basis shall also be handed over to POWERGRID.

The formal application for allotment of required land in the name of POWERGRID has been submitted to KIADB for further processing. The social assessment of the allotted land shall be carried out to know about the acquisition process adopted by KIADB and existence of any encroachers etc. Based on the assessment, a detailed Rehabilitation Action Plan (RAP) (if required) shall be prepared and disclosed by July, 2011 listing all measures for Rehabilitation of PAPs/encroachers and community development work to be undertaken for up-liftment/improvement of infrastructure etc. of affected village.

- Land value depreciation

- Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

- (iii) Historical/cultural monuments/value

- As per the POWERGRID’s policy of route selection only that route alignment is finalized this avoids all the historical and cultural monuments. As per the preliminary assessment carried out during finalization of route alignment in consultation with State revenue authorities and ASI, no such monuments are coming in the proposed route alignment.

- (iv)	 Encroachment into precious ecological areas

- As already explained all precautions have been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries. In order to minimize forest involvement and subsequent environmental impact Double Circuit tower design for 400 KV line has been developed which has reduced the requirement of ROW by 6 m (from 52 m to 46 m) thereby reduction in tree cutting along ROW.

- (v)  Encroachment into other valuable lands

- Impacts on agricultural land will be restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1). The innovative tower design followed in case of 400 KV D/C line has already reduced the Right of Way (ROW) requirement to 46 m from 52 m a reduction of 6 m will further reduced the impact on cultivable land.

- In areas where lines will traverse agricultural land, compensation will be paid to owners for any crop damage incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation.  Revenue department after evaluating the loss due to construction activity and productivity of land arrives at the compensation cost which is paid to farmer.  Agricultural activities will be allowed to continue following the construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner's satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss of products from the tree over an eight year period (for fruit bearing trees). Agricultural lands under private ownership will be identified, and in accordance with normal POWERGRID procedures compensation will be paid to the affected villagers (Annexure-1).  Budgetary provision Rs. 287 lakhs is made in the cost estimate to meet these expenses.

- (vi)	Interference with other utilities and traffic

- As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission lines affect nearby telecommunication circuits by causing electrical interference.  A standing committee -- Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like rerouting of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

- The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50000/- per km for POWERGRID. Provision to meet these expenses has been made in the cost estimate for the same.

- Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission lines pass by the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights are placed atop these towers.

- (vii) Interference with drainage pattern

- As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings, which is very small, there is little possibility of affecting drainage pattern.  In the infrequent instances where the drainage is affected, flow will be trained and guided to safe zones. The proposed project involves two towers to be placed in river beds of Krishna and Tungabhadra for river crossing. All possible engineering interventions are tried / applied to avoid placing of towers in the river beds. However, if become completely unavoidable, due precautions shall be implied to avoid/minimize any adverse impact on aquatic flora and fauna.

- 5.1	ENVIRONMENTAL PROBLEMS DUE TO DESIGN :

- (i)	Escape of polluting materials

- The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials.

- (ii)	Explosion/fire hazards

- During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires.

- Fires due to flashover from lines can be a more serious problem in forest. However, adequate safety measures shall be taken to avoid such incidence besides this forest authorities also incorporate measures like making fire lines to prevent spreading of fire in the affected forest area.

- (iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

- Adequate measures are taken to re-surface the area where excavation works are done.  Topsoil disturbed during the development of sites will be used to restore the surface of the platform. Infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for tower foundations.

- (iv)	Environmental aesthetics

- Since spacing between the towers in case of 400 KV D/C lines is approx. 400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible.   POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted grey or green to merge with the background.

- (v)	 Noise/vibration nuisances

- The equipment installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian standards. The noise levels reported during normal operating conditions are about 60 to 70 dB at 2 m. distance from the equipment.  To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipment are undertaken.  In addition, plantations of sound absorbing species like Casuarinas, Tamarind, and Neem are raised at the sub-stations that reduce the sound level appreciably. Actual noise levels measured at perimeters of existing Substations are 30 to 40 dB.

- (vi)  Blockage of Wildlife passage

- Since the line is passing through mostly agricultural, wasteland and no  protected area, migration path of wild life etc. are involved, hence possibility of disturbance to wild life are nil/remote.

- 5.2	ENVIRONMENTAL PROBLEMS DURING CONSTRUCTION PHASE :

- (i)	Uncontrolled silt runoff

- The Project involves only small scale excavation for tower foundations at scattered locations in plain area that are re-filled with excavated material therefore uncontrolled silt run off is not expected.

- (ii)   Nuisance to nearby properties

- As already described in preceding paras, during site selection due care is taken to keep the transmission line and substations away from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour therefore nuisance to the nearby properties if any, is not expected.

- (iii)  Interference with utilities and traffic and blockage of access way

- Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures. Even if at some places it is found that access road is not available than existing field/path may be utilized and compensation for any damage to crop or field is paid to the owner.

- As and when a transmission line crosses any road/ railways line, the short span angle (DT) towers are located at a distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

- (iv)	Inadequate resurfacing for erosion control

- The proposed lines are to be constructed in plain area where erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils, internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained above adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is generally undertaken outside the rainy season. As the proposed line is mostly passing through plain areas these problems are not anticipated.

- (v)	Inadequate disposition of borrow area

- As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area. As such acquisition/opening of borrow area is not needed.

- (vi)   Protection of Worker's health/safety

The Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents will guide provisions for workers’ health and safety. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the General Conditions of Contract (GCC), a part of bidding documents. As a deterrent or to minimize accident during construction a provision in the contract has been added that stipulates a fine/penalty of Rs.10 lakhs for each accidental death and Rs1.0 lakh/each for any injury and is deducted from the contractor’s payment and paid to the deceased/affected family, in case of occurrence (Annexure-2).

POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Department POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. These guidelines/checklist include work permits and safety precautions for work on the transmission lines both during construction and operation (Annexure-3) and is monitored regularly by site in-charge and corporate Operation Services. In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act.  Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses will be disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID will conduct awareness building programs on such issues for the construction workers.

- 5.3	ENVIRONMENTAL PROBLEMS RESULTING FROM OPERATION :

- (i)   O&M Staff/Skills less than acceptable resulting in variety of adverse effects

- The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However in respect of the long distance transmission lines, there are monitoring offices that are located at various points en-route. Monitoring measures employed include patrolling and thermo-vision scanning.

- The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

- A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard.  Because of issues relating to need to ensure health and safety relating to the line such as fire safety, safe voltages on metallic parts of buildings, and safety clearances to avoid flashover, the transmission lines will not pass directly over any residential properties and as such the potential for EMF effects to occur will be further diminished. Given that it will be necessary to ensure that there are no properties in the ROW beneath and to the sides of the overhead line, automatic mitigation against EMF will be provided between the source of potentially high strengths (the transmission line) and the residential properties.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has discontinued procurement electrical equipments containing PCB more than 2 mg/kg and specification (as per IEC 61619 or ASTM D4059) is being stated in the tender document. POWERGRID in association with NGC, UK has setup a Regional Testing Laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India:

“Power Grid was the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid do not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

5.4    CRITICAL ENVIRONMENTAL REVIEW CRITERIA :

(i)	Loss of irreplaceable resources

The transmission projects do not involve any large scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As the lines in the subject project are not passing through any forest area, the problem of losing natural resources in this project is not envisages. Moreover, with the provision of water harvesting in the all the three substations, it is estimated that such provision shall help in recharging the ground water resulting in conservation of precious natural resource like water.

(ii)	 Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as no reserve forest is getting  affected thus there is no possibility of endangering/causing extinction of any species.

(iv)	Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

5.6      PUBLIC CONSULTATION :

Public consultation/information is an integral part of the project implementation. Public is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted. Public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting as per Environmental Social Policy & Procedures of POWERGRID (ESPP) shall also be carried out during different activities of project cycle. During such consultation the public will be informed about the project in general and in particular about the following:

- Complete project plan (i.e. its route and terminating point and substations, if any, in between);

- POWERGRID design standards in relation to approved international standards;

- Health impacts in relation to EMF;

- Measures taken to avoid public utilities such as school, hospitals, etc.;

- Other impacts associated with transmission lines and POWERGRID’s approach to minimizing and solving them;

- Land acquisition details, proposed R&R measures and compensation packages in line with POWERGRID’s policy;

- Trees and crop compensation process.

Apart from organising many informal group meetings in different villages (Table-5.1) Public meetings were also organised in the Erraguntla Village of Garaladinne Mandal, and Avanikunta Village, Rolla Mandal Distt. Ananathapur of Andhra Pradesh state and also Golarahatti Village, Distt.- Tumkur of Karnataka state. To get the maximum participation during the Public consultation Programme a notice was served well in advance to the villagers (Annexure-4).  The   details of line and its importance were explained to the villagers by the officials of POWERGRID.  Villagers including Sarpanch and POWERGRID representative participated in the programme (Plate- 1 to 4).The programme was arranged in interactive way and queries like tree/crop compensation etc. were replied.  The programme was appreciated by the villagers including Sarpanch and they assured to extend their cooperation for construction of transmission project. The process of such consultation shall continue during project implementation and even during O&M stage.

Table-5.1: Public Consultation en-route of Transmission Lines

| S No. | Line / Date | Village Name | Person attended |
| --- | --- | --- | --- |
| 1. a. | 400 KV D/C GOOTY – TUMKUR(MADHUGIRI) LINE 3RD th DEC 2010 Distt.- Anantapur, Andhra Pradesh | Erraguntla | Village Sarpanch / VRO and general/interested resident of village |
| b. | 4TH DEC 2010 Distt.- Anantapur, Andhra Pradesh | Avanikunta | -Do- |
| 2. a. | 400 KV D/C MADHUGIRI(TUMKUR)- YELAHANKA LINE 12TH JAN 2011 Distt.- Tumkur, Karnataka | Golarahatti | Members of Gram- panchayat and general/interested resident of village |

5.7	CONCLUSIONS :

From the above discussion, it seems that the area is rich in physical resources. But careful route selection has avoided involvement of forest. Thus, route selected for detailed survey is the most optimum and do not involve any protected area.  Hence, based on the criteria agreed with Bank the project is coming under the P2 category.  The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts while on the one hand underline the need for implementation of the project for overall development of the area and on another hand suggest that a detailed EIA may not be necessary.

# SECTION-VI:  ORGANISATIONAL SUPPORT STRUCTURE AND

# ENVIRONMENT MANAGEMENT PLAN

6.0     ENVIRONMENTAL MONITORING PROGRAM IN POWERGRID :

- Monitoring is a continuous process for POWERGRID projects at all the stages, be it the site selection, construction or maintenance.

- The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan. Flow chart of origination support structure is depicted in Exhibit 2.

- 6.1      CORPORATE LEVEL :

- An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and Site to carry out environmental and

social  surveys for new projects.

- Assisting RHQs and site to finalize routes of entire power transmission line

considering environmental and social factors that could arise enroute

- Help RHQs and Site to follow-up with the state forest offices and other state

departments in expediting forest clearances and the land acquisition process

of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest

clearances  and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & site Officials on environment and

social issues and their management plan.

- 6.2     REGIONAL LEVEL :

- At its Regional Office POWERGRID has a Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the Corporate level and the Divisional Headquarters. The key functions envisaged for ESMC are:

- Advising and coordinating field offices to carry out environmental and social

surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and site to finalize routes of entire power T/L considering

the environmental and social factors that could arise en- route

- To follow-up forest clearances and land acquisition processes with state

forest offices and other state depts. for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and site on various

environmental and social aspects.

- 6.3      SITE OFFICE :

- At the Divisional Headquarters level, POWERGRID has made the head of the division responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social Management Team (ESMT). Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalize the route

for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it

up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with

Authorized Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

- As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park, it is generally done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties.  As no forest area is involved in the instant project such monitoring/provision shall not be applicable to proposed project.  A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn (Table- 6.1) and will be implemented during various stage of project execution. A budget estimate towards tree/crop compensation and EMP implementation is prepared and is placed at Annexure-5. Since many provisions of EMP are to be implemented by contractor hence for proper monitoring EMP has included in the contract document (Annexure-6).

6.4      ENVIRONMENTAL REVIEW :

Periodic review by corporate ESMD and higher management including review by POWERGRID CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan.

Table- 6.1 Environment Management Plan

| Project activity /stage | Potential impact | Proposed mitigation measure | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-construction |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Tower location and line alignment selection with respect to nearest dwellings | Setback distances to nearest houses - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformers stated in tender specification - once | POWERGRID | Part of tender specifications for the equipment |
|  |  | Processes, equipment and systems not to use chlorofluorocarbons (CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – once | POWERGRID | Part of tender specifications for the equipment |
|  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design |
| Transmission line design | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Electromagnetic field strength for proposed line design | Line design compliance with relevant standards - once | POWERGRID | Part of detailed alignment survey and design |
| Substation location and design | Exposure to noise | Design of plant enclosures to comply with noise regulations. | Expected noise emissions based on substation design | Compliance with regulations - once | POWERGRID | Part of detailed siting survey and design |
| Location of transmission towers and transmission line alignment and design | Impact on water bodies and land | Consideration of tower location at where they could be located to avoid water bodies or agricultural land. | Tower location and line alignment selection (distance to water and/or agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
|  | Social inequities | Careful route selection to avoid existing settlements | Tower location and line alignment selection (distance to nearest dwellings or social institutions) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
|  |  | Minimise need to acquire agricultural land | Tower location and line alignment selection (distance to agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
| Involuntary resettlement or land acquisition | Social inequities | Compensation paid for temporary/ permanent loss of productive land as per LAA & its process | RAP implementation | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction phase |
| Encroachment into precious ecological areas | Loss of precious ecological values/ damage to precious species | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest designated ecological protection area) | Consultation with local forest authorities - once | POWERGRID | Part of detailed siting and alignment survey /design |
| Transmission line through forestland | Deforestation and loss of biodiversity | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest protected or reserved forest) | Consultation with local authorities - once | POWERGRID | Part of detailed siting and alignment survey/design |
|  |  | Minimise the need by using existing towers, tall towers and RoW, wherever possible |  | Consultation with local authorities and design engineers - once |  |  |
|  |  | Obtain statutory clearances from the Government | Statutory approvals from Government | Compliance with regulations – once for each subproject |  |  |
| Encroachment into farmland | Loss of agricultural productivity | Use existing tower footings/towers wherever possible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
|  |  | Avoid siting new towers on farmland wherever feasible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once |  | Part of detailed siting and alignment survey /design |
|  |  | Farmers compensated for any permanent loss of productive land | Design of Implementation of Crop Compensation (based on affected area) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  | Farmers/landowners compensated for significant trees that need to be trimmed/ removed along RoW. | Design of Implementation of Tree compensation (estimated area to be trimmed/removed) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  |  | Statutory approvals for tree trimming /removal | Compliance with regulations – once for each subproject |  | Part of detailed siting and alignment survey /design |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels | Noise levels to be specified in tender documents - once | POWERGRID | Part of detailed equipment design |
| Interference with drainage patterns/Irrigation channels | Flooding hazards/loss of agricultural production | Appropriate siting of towers to avoid channel interference | Tower location and line alignment selection (distance to nearest flood zone) | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
| Escape of polluting materials | Environmental pollution | Transformers designed with oil spill containment systems, and purpose-built oil, lubricant and fuel storage system, complete with spill cleanup equipment. | Equipment specifications with respect to potential pollutants | Tender document to mention specifications - once | POWERGRID | Part of detailed equipment design /drawings |
|  |  | Substations to include drainage and sewage disposal systems to avoid offsite land and water pollution. | Substation sewage design | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
| Explosions/Fire | Hazards to life | Design of substations to include modern fire control systems/firewalls. | Substation design compliance with fire prevention and control codes | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
|  |  | Provision of fire fighting equipment to be located close to transformers. |  |  |  |  |
| Construction |  |  |  |  |  |  |
| Equipment layout and installation | Noise and vibrations | Construction techniques and machinery selection seeking to minimize ground disturbance. | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance - once at the start of each construction phase | POWERGRID (Contractor through contract provisions) | Construction period |
| Physical construction | Disturbed farming activity | Construction activities on cropping land timed to avoid disturbance of field crops (within one month of harvest wherever possible). | Timing of start of construction | Crop disturbance –Post harvest as soon as possible but before next crop - once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Mechanized construction | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment – estimated noise emissions | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Noise, vibration, equipment wear and tear | Turning off plant not in use. | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction of roads for accessibility | Increase in airborne dust particles | Existing roads and tracks used for construction and maintenance access to the line wherever possible. | Access roads, routes (length and width of new access roads to be constructed) | Use of established roads wherever possible - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Increased land requirement for temporary accessibility | New access ways restricted to a single carriageway width within the RoW. | Access width (meters) | Access restricted to single carriageway width within RoW - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Temporary blockage of utilities | Overflows, reduced discharge | Temporary placement of fill in drains/canals not permitted. | Temporary fill placement (m3) | Absence of fill in sensitive drainage areas - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Marking of vegetation to be removed prior to clearance, and strict control on clearing activities to ensure minimal clearance. | Vegetation marking and clearance control (area in m2) | Clearance strictly limited to target vegetation - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Trimming/cutting of trees within RoW | Fire hazards | Trees allowed growing up to a height within the RoW by maintaining adequate clearance between the top of tree and the conductor as per the regulations. | Species-specific tree retention as approved by statutory authorities (average and maximum tree height at maturity, in meters) | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Loss of vegetation and deforestation | Trees that can survive pruning to comply should be pruned instead of cleared. | Species-specific tree retention as approved by statutory authorities | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Felled trees and other cleared or pruned vegetation to be disposed of as authorized by the statutory bodies. | Disposal of cleared vegetation as approved by the statutory authorities (area cleared in m2) | Use or intended use of vegetation as approved by the statutory authorities – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Wood/vegetation harvesting | Loss of vegetation and deforestation | Construction workers prohibited from harvesting wood in the project area during their employment, (apart from locally employed staff continuing current legal activities). | Illegal wood /vegetation harvesting (area in m2, number of incidents reported) | Complaints by local people or other evidence of illegal harvesting - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Surplus earthwork/soil | Runoff to cause water pollution, solid waste disposal | Soil excavated from tower footings disposed of by placement along roadsides, or at nearby house blocks if requested by landowners. | Soil disposal locations and volume (m3) | Acceptable soil disposal sites - every 2 weeks | POWER GRID (Contractor through contract provisions) | Construction period |
| Substation construction | Loss of soil | Fill for the substation foundations obtained by creating or improving local water supply ponds or drains, with the agreement of local communities. | Borrow area siting (area of site in m2 and estimated volume in m3) | Acceptable borrow areas that provide a benefit - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Substation construction | Water pollution | Construction activities involving significant ground disturbance (i.e. substation land forming) not undertaken during the monsoon season. | Seasonal start and finish of major earthworks | Timing of major disturbance activities - prior to start of construction activities | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Tree clearances for easement establishment to only involve cutting trees off at ground level or pruning as appropriate, with tree stumps and roots left in place and ground cover left undisturbed. | Ground disturbance during vegetation clearance (area, m2) | Amount of ground disturbance - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  |  | Statutory approvals | Statutory approvals for tree clearances – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Tower construction – disposal of surplus earthwork/fill | Waste disposal | Excess fill from tower foundation excavation disposed of next to roads or around houses, in agreement with the local community or landowner. | Location and amount (m3)of fill disposal | Appropriate fill disposal locations - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Storage of chemicals and materials | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports (type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction schedules | Noise nuisance to neighbouring properties | Construction activities only undertaken during the day and local communities informed of the construction schedule. | Timing of construction (noise emissions, [dB(A)]) | Daytime construction only - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Provision of facilities for construction workers | Contamination of receptors (land, water, air) | Construction workforce facilities to include proper sanitation, water supply and waste disposal facilities. | Amenities for Workforce facilities | Presence of proper sanitation, water supply and waste disposal facilities - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period |
| Encroachment into farmland | Loss of agricultural productivity | Use existing access roads wherever possible | Usage of existing utilities | Complaints received by local people /authorities - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Ensure existing irrigation facilities are maintained in working condition | Status of existing facilities |  |  |  |
|  |  | Protect /preserve topsoil and reinstate after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  |  | Repair /reinstate damaged bunds etc after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  | Social inequities | Compensation for temporary loss in agricultural production | Implementation of Crop compensation (amount paid, dates, etc.) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Uncontrolled erosion/silt runoff | Soil loss, downstream siltation; | Need for access tracks minimised, use of existing roads. | Design basis and construction procedures (suspended solids in receiving waters; area re-vegetated in m2; amount of bunds constructed [length in meter, area in m2, or volume in m3]) | Incorporating good design and construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Limit site clearing to work areas |  |  |  |  |
|  |  | Regeneration of vegetation to stabilise works areas on completion (where applicable) |  |  |  |  |
|  |  | Avoidance of excavation in wet season |  |  |  |  |
|  |  | Water courses protected from siltation through use of bunds and sediment ponds |  |  |  |  |
| Nuisance to nearby properties | Losses to neighbouring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | As much as possible existing access ways will be used. | Design basis and layout | Incorporating good design engineering practices – once for each site |  |  |
|  |  | Productive land will be reinstated following completion of construction | Reinstatement of land status (area affected, m2) | Consultation with affected parties – twice – immediately after completion of construction and after the first harvest |  |  |
|  | Social inequities | Compensation will be paid for loss of production, if any. | Implementation of Tree/Crop compensation (amount paid) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Inadequate siting of borrow areas | Loss of land values | Existing borrow sites will be used to source aggregates, therefore, no need to develop new sources of aggregates | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Health and safety | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work days caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  |
|  |  | Contractor to arrange for health and safety training sessions |  |  |  |  |
| Inadequate construction stage monitoring | Likely to maximise damages | Training of POWERGRID environmental monitoring personnel | Training schedules | Number of programs attended by each person – once a year | POWERGRID | Routinely throughout construction period |
|  |  | Implementation of effective environmental monitoring and reporting system using checklist of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof. | Submission of duly completed checklists of all contracts for each site - once |  |  |
|  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures. | Compliance report related to environmental aspects for the contract | Submission of duly completed compliance report for each contract - once |  |  |
| Operation and Maintenance |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Compliance with setback distances (“as-built” diagrams) | Setback distances to nearest houses – once in quarter | POWERGRID | During operations |
| Equipment submerged under flood | Contamination of receptors (land, water) | Equipment installed above the high flood level (HFL) by raising the foundation pad. | Substation design to account for HFL (“as-built” diagrams) | Base height as per flood design - once | POWERGRID | During operations |
| Oil spillage | Contamination of land/nearby water bodies | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks. | Substatio bunding (Oil sump) (“as-built” diagrams) | Bunding (Oil sump) capacity and permeability - once | POWERGRID | During operations |
| Inadequate provision of staff/workers health and safety during operations | Injury and sickness of staff /workers | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |
|  |  | Safety awareness raising for staff. | Training/awareness programs and mock drills | Number of programs and percent of staff /workers covered – once each year |  |  |
|  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |
|  |  | Provide adequate sanitation and water supply facilities | Provision of facilities | Complaints received from staff /workers every 2 weeks |  |  |
| Electric Shock Hazards | Injury/mortality to staff and public | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |
|  |  | Security fences around substations | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |
|  |  | Barriers to prevent climbing on/dismantling of transmission towers | Maintenance of barriers |  |  |  |
|  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |
|  |  | Electricity safety awareness raising in project areas | Training /awareness programs and mock drills for all concerned parties | Number of programs and percent of total persons covered – once each year |  |  |
| Operations and maintenance staff skills less than acceptable | Unnecessary environmental losses of various types | Adequate training in O&M to all relevant staff of substations and transmission line maintenance crews. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
|  |  | Preparation and training in the use of O&M manuals and standard operating practices. |  |  |  |  |
| Inadequate periodic environmental monitoring. | Diminished ecological and social values. | Power Grid staff to receive training in environmental monitoring of project operations and maintenance activities. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | Processes, equipment and systems using cholofluorocarbons (CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government. | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |
| Transmission line maintenance | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Required ground clearance (meters) | Ground clearance - once | POWERGRID | Operations |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels (dB(A)) | Noise levels at boundary nearest to properties and consultation with affected parties if any – once | POWERGRID | Operations |