SECTION I:                  PROJECT DESCRIPTION

(Biswanath Chariyali-Agra ± 800kV HVDC Transmission line)

- Background

Arunachal Pradesh in North Eastern Region & Sikkim of India and Bhutan are endowed with large hydro potential. Generation addition of about 35,000 MW in Arunachal Pradesh and 15,000 MW in Sikkim & Bhutan is expected in future. Considering the growth of power demand of NER including Sikkim and Bhutan, it is estimated that power to the order of about 42,000-45,000 MW would be surplus in these areas. Generation addition scenarios of Northern, Western and Southern Regions indicate that these regions would remain in deficit situation during 11th Plan and beyond. Therefore, power from the above generation sources would have to be transmitted to the load centers of NR, WR and SR over long distance through the narrow corridor in north of West Bengal. To optimally utilize the transmission corridor of the Chicken neck area and the difficult terrain of NER, it is necessary to plan evacuation system of major projects in NER and Bhutan in a comprehensive manner keeping in view the future generation expansion.

For transmission of power from NER/ Sikkim/ Bhutan to NR/WR, different transmission options i.e. high voltage HVDC and 765kV AC are examined and it is found that hybrid system of ± 800kV HVDC with 400kV AC system is the most optimal one.

Generation addition, out of above potential in NER, as presently planned from Lower Subansiri (8x250MW=2000MW) and Kameng (4x150MW=600MW) Hydro Electric Projects in Arunachal Pradesh/Assam is expected to come up by XI Plan, beneficiaries being mainly NER, NR & WR. Evacuation of power from these Hydro Electric Projects has been envisaged with 400kV D/C Transmission systems at Biswanath Chariyali in Assam where 220/400/765kV HVAC power pooling point & ± 800kV HVDC Terminal is proposed. Transfer of bulk power from this Terminal Stn. (Biswanath Chariyali) has been proposed through ± 800kV HVDC Transmission line to Agra in Uttar Pradesh in Northern Grid for further dispersal to National Grid.

1.1	Project Highlights

| a) | Project Name | : | + 800 kV, 6000 MW HVDC bi-pole Transmission Line from Biswanath Chariyali (Assam) to Agra (Uttar Pradesh) including Earth Electrode Lines at both end, Length-1825 KM. |
| --- | --- | --- | --- |
| b) | Location | : | Assam, West Bengal, Bihar and Uttar Pradesh |
| c) | Beneficiary | : | Constituent States of North East, Northern & Western Regions. |

1.2      Benefits of the project

Bulk power would be transferred uninterruptedly over a long distance with much more transmission reliability and stability. It will also facilitate inter-state trading of exportable power of State sector generation apart from evacuation of power from the central sector Generations. Moreover, it will increase the capacity of National Grid. The per capita consumption of electricity (presently about 370 units) is likely to increase by 2 to 3 % on an average, main beneficiaries being consumers of rural India including agricultural sector, industry, commercial sectors with improved electricity supply with stability.

1.3     Scope of presentation in this report

The length of Biswanath Chariyali to Agra + 800 kV, 6000 MW HVDC bi-pole line is too long. Preliminary investigation/survey has been carried out in 4 (four packages) to estimate/arrive at for selection of one best feasible alignment route out of at least 3 alternative alignments  studied, for detailed survey to be undertaken during execution of main contracts. Therefore, to facilitate preparation of initial Environment Examination (IEE), entire project (Line) has been sectioned (in 4 parts) matching Preliminary investigation/survey as under:

| Sl. No. | Initial Env. Examination Section |  | Sub-project/Contract Package |  |
| --- | --- | --- | --- | --- |
|  | Name of section as Per IEE | Length in Kms | Package No | Contract Description |
| 1. | Biswanath Chairiyali to Bongaigaon | *320.566 + 50 | A1 A2 | Part-I B’chariyali-B’gaon HVDC line-160 Km with Earth Electrode line-50 Km. Part-II B’chariyali-B’gaon HVDC line-165 Km |
| 2. | Bongaigaon to Saharsa | *463.0 | A3 A4 A5 A6 | Part-I B’gaon-Purnea HVDC line -194 Km Part-II B’gaon-Purnea HVDC line-194 Km Purnea-Muzaffarpur HVDC line - 241Km Muzaffarpur-Gorakhpur HVDC line-261 Km |
| 3. | Saharsa to Gorakhpur | *363.528 |  |  |
| 4. | Gorakhpur to Agra via Lucknow | *580.0 + 40 | A7 A8 A9 | Part-I Gorakhpur-Agra HVDC line-200 Km Part-II Gorakhpur-Agra HVDC line-200 Km Part-III Gorakhpur-Agra HVDC line-200 Km with Earth Electrode line-40 Km. |

* Length as per preliminary/GIS Survey, may not match with the contract packages as above.

A power map showing the Biswanath Chariyali to Agra HVDC transmission line is placed     as Map-1.

SECTION II: 	             BASE LINE DATA

2.0  	The project is an inter-state one and is spread/located in the States of Assam, West Bengal, Bihar and Uttar Pradesh.

2.1	Assam

Assam has a geographic area of 7.84 million ha, which constitutes 2.39% of the country’s total area. It is situated between lat. 24°57' to 28°00' N and long. 89° 42' to 96° 02’E.

Physiography:

Topographically, the state can be divided into three parts, viz. the Brahmaputra valley, the Surma valley and the Assam range. The first two parts are plain areas, while the Assam range is a mountainous region.The general land use pattern of the state is given below:

Land use Pattern

| Land use | Area in ‘000 ha | Percentage |
| --- | --- | --- |
| Geographical Area | 7,844 |  |
| Total Reporting area | 7,850 | 100.00 |
| Forests | 1,930 | 24.58 |
| Not available for cultivation | 2,493 | 31.76 |
| Permanent Pasture & Grazing Land | 170 | 2.17 |
| Land under misc. tree crops & groves | 243 | 3.09 |
| Culturable waste land | 87 | 1.11 |
| Fallow land other than current fallows | 69 | 0.88 |
| Current fallows | 114 | 1.45 |
| Net area sown | 2,744 | 34.96 |

Source: Land use statistics At a Glance 1996-97, Ministry of Agriculture, GOI, 2000

Population:

| Details of population of Assam Population | 26,638,407 |
| --- | --- |
| Female | 12,850,608 |
| Male | 13,787,799 |
| Density (Persons per Sq. KM) | 340 |
| Urban Population | 12.72% |
| Sex Ratio (Females per thousand males) | 932 |
| Literacy | 64.28% |
| Males | 71.93% |
| Females | 56.03% |

as per 2001 census are as follows:

Census survey 2001

Climate:

Assam has four well defined seasons in a year. Summer, monsoon, winter and spring these four seasons presents Assam in different moods. The best time to visit Assam is during the winters and spring seasons. October to April offer a mild and moderate climate. Assam is never extremely cold or hot. Climate of Assam is sub tropical.

Rainfall:

Rainfall, one of the highest in the world (between 178 and 305 cm), is concentrated in 4 months, June to September. The state experiences floods and droughts, Annual rainfall varying from 1,500 mm to 3,750 mm.

Temperature:

The average temperature in January ranges from 10°C to 23° C  and in July it ranges from 26° C to 32° C.

Minerals:

Coal, petroleum and natural gas, limestone and minor minerals are produced in Assam. Tertiary coal occurs in North Cachar Hills, Sivasagar and Lakhimpur districts. Assam coal is friable in nature and has high sulphur content. It is mainly utilised by local railways, steamers, and hydro power stations. Low moisture, low volatile cooking coal has been discovered in Hallidayganj Singrimari area. Deposits of banded magnetic quartzite occurs in Kamrup and Goalpara districts, Limestone occurs in Lakhimpur, North Cachar Hills, Karbi Anglong ,Nagaon and Sivasagar districts. Kaolin is found in Karbi Anglong and Lakhimpur district. The Digboi oil fields in Lakhimpur district and Moran and Rudrasagar oil fields in Sivasagar district are the major source of oil and gas. Hydrocarbons are struck in Borsilla, Changmaigaon, Kurgaon and Rajgarh in the past. Sillimanite bearing rocks occur in Karbi Anglong district. Assam continued to be the 3rd largest producer of Petroleum (crude) and natural gas in the country accounting for 16 % and 8% respectively of the total production of this mineral in the country.

Soils:

The major tea growing state of India, occupies a geographical area of 7.8 million hectares of which total cropped area is 3.9 million hectares. Only 14.4% of the gross cultivated area are irrigated and the average cropping intensity of the state 145%. The major soils of Assam belong to Inceptisols (49.3%), Entisols (32.3%), Alfisols (12.3%) and Ultisols (6.1%). The most typical characteristics of Assam soil is acidity, where pH of the soils generally ranges between 4.2 to 5.8.

Water Resources:

Brahmaputra Basin comprises of sub-basin of Subansiri, Jia Bharali, Badeng-Pubnoi, Dhansiri, anas, Champamati, DholaiBuridihing, Disang, Kopili, Kalang and Meghna Basin comprises of sub- Basin of Barak River.

Assam is dominated by the Brahmaputra river (length: 2900 km). Its drainage area is roughly 935,500 sq. km

Ecological resources:

Forests

The recorded forest area is 27,018 km (sq.) which constitutes 34.45% of the geographic area of the state. According to legal classification, Reserved Forest constitutes 18,060 km (sq.) and Un-classed Forest 8958 km (sq.). Forest Map of Assam is enclosed as Map-2.

Forest types occurring in the state are Tropical Wet Evergreen, Tropical Semi-Evergreen, Tropical Moist Deciduous, Sub Tropical Broad Leaved Hill, Sub Tropical Pine and Littoral and Swamp Forests.

Protected Areas

There are 3 National Parks and 13 Wildlife Sanctuaries in the state. Total protected area is 0.21 million ha which constitutes 2.69% of the total geographic area of the state. Manas Tiger Reserve is located in the state. Manas has also been declared as a Biosphere Reserve. Kaziranga National Park and Manas Wildlife Sanctuary are also included in the World Heritage sites.

Human and Economic Development:

Assam is a state rich in natural resources like natural oil, natural gas, coal, rubber, tea and some minerals like granite, limestone and kaolin. The present state is much smaller than what it was forty years ago. It is still the largest economy in the North East. It is like the gateway to the other North Eastern states with an infrastructure index (according to CMIE in 1992-93) of 93, quite close to the average of 100. Although it is more industrially developed than the other North Eastern states, it is primarily an agrian economy with 74% of its population engaged in agriculture and allied activities.

In 1997-98, the contribution of the tertiary sector to the SDP is the highest. It comprised almost half of the State’s SDP. Over the years, there has been a shift in the sectoral contribution from primary to the tertiary sector while the secondary sector indicates a fluctuating contribution to the SDP.

The annual compound growth rate of NSDP during the period from 1980-81 to 1997-98 has been worked out at 12.41% at current prices and 3.54% at constant (1980-81) prices. Agriculture demonstrated a growth rate of 11.7% at current prices and 2.14% at constant prices over the same period. Similarly for manufacturing sector it was 12.05% at current prices and 1.99% at constant prices during the same period. Per capita income of Assam continues to lag behind the national average.

Tea is a major industry. There are nearly 750 tea plantations in the state. Assam contributes 15.6% of world’s tea production and 55% of the country’s tea ouput. Assam is first state in the country where oil was struck in 1889 at Digboy. The state has two oil refineries and the third with a petrochemical complex is under way. There is also a public sector fertilizer factory at Namrup. The Numligarh refinery (3m tones capacity, cost Rs.2350 Crore) was commissioned on July 9, 1999. Other industries are sugar, jute, silk, paper, plywood, rice and oil drilling. Important cottage industries are handloom, sericulture, manufacture of cane and bamboo articles, carpentry, smithy and manufacture of brass utensils. Assam is he largest producer in the world of the golden colored muga silk.

2.2   West Bengal

Physiography:

West Bengal is strategically placed with three international frontiers of 1600-km long - Bangladesh, Nepal and Bhutan and a hinge between the bulk of Indian Territory and the north-east of the country. West Bengal is located at 21°31' and 27°14' North Latitude at the head of the Bay of Bengal and 86°35' and 89°53' East Longitude, with the Tropic of Cancer running through it. West Bengal stretches from the Himalayas to the Bay of Bengal.

Northern part comprises the sub-Himalayan region called the Terai, while the rest of the state forms part of the Indo-Gangetic plain. The course of the river Ganges, much of which lies now in Bangladesh, has for centuries been shifting eastward. Very little of its water now goes to the sea via its western distributaries, the principal one being the Hooghly that flows by Calcutta. Ganga meets the ocean forming well-known mangrove forests in Sundarbans, mainly embraces the fertile alluvial Gangetic plains.

The deep alluvial soil of the Gangetic plain is very suitable for agriculture.

The Terai region has some of the finest tea plantations in India. From the Terai, the Himalayan mountain ranges rise abruptly along the state's northern boundary. Mount Kanchenjunga overlooks Darjeeling in north-west Bengal. 

 The general land use pattern of the state is given below:

| Land use | Area in ‘ 000 ha | Percentage |
| --- | --- | --- |
| Total Geographical area | 8,875 |  |
| Reporting Area for land utilisation | 8,696 | 100.00 |
| Forests | 1,195 | 13.74 |
| Not available for cultivation | 1,659 | 19.08 |
| Permanent Pasture & Grazing land | 8 | 0.09 |
| Land under misc. tree crops & groves | 80 | 0.92 |
| Culturable waste land | 50 | 0.58 |
| Fallow land & other than current fallows | 29 | 0.33 |
| Current fallows | 212 | 2.44 |
| Net area Sown | 5,463 | 62.82 |

Source: Land use statistics At a Glance 1996-97, Ministry of Agriculture, GOI, 2000

Population:

| Population | 80,221,171 |
| --- | --- |
| Female | 38,733,477 |
| Male | 4,14,87,694 |
| Density (Persons per Sq. Km) | 904 |
| Urban Population | 28.03% |
| Sex Ratio(Females per thousand males) | 934 |
| Literacy | 69.22% |
| Males | 77.58% |
| Females | 60.22% |

Details of population of West Bengal

as per 2001 census are as follows:

Census survey 2001

Climate:

West Bengal has a tropical climate. The plains are hot except during the short winter season. The mountainous region in the north is cold on account of its altitude but there humidity is high. The classical tradition speaks of six seasons-springs, summer, the Rainy, autumn, mild winter and severe winter. Only four clearly marked seasons with a brief interregnum of spring are observed, namely the hot season, the rainy season, the post monsoon season corresponding to autumn and the cold season. At nights, cool southerly breeze carrying moisture from the Bay of Bengal is usually present.

Rainfall:

The average rainfall in the State is 1750 mm. In the Himalayan Region i.e. in northern part the average rainfall ranges from 2500 - 6000 mm. In the southern part average rainfall ranges from 1125 - 1900 mm.

Temperature:

The hot season lasts from mid-March to mid-June, with the day temperature ranging from 38o C to 45oC in different parts of the state. Winter, which lasts about three months, is mild over the plains, the average minimum temperature not falling 15o C.

Minerals:

Minerals, like rock phosphate, fire clay, iron, limestone, copper, silica, quartz, manganese and sandstone are found in a considerable amount in the state.

Soil:

The soil patterns in the state are Tarai Soil and Gangetic Alluvium - found in northern part of the state along the border of Nepal.

Water resources:

The State of West Bengal falls under 3 major River Basins: - The Brahmaputra Basin - Part of Darjeeling, Jalpaiguri and Coochbihar districts.

- The Ganga Basin - Part of Darjeeling, Jalpaiguri and all the remaining districts.

- The Subarnarekha Basin-Parts of Purulia, E&W Medinipur District.

Ecological resources:

Forests

The recorded forest area is 11,879 km2 which constitutes 13.4% of the geographic area of the state. By legal status, the Reserved Forest constitutes 7,054 km2, Protected Forest 3,772 km2 and Un-classed Forest 1,053 km2. Forest Map of West Bengal is enclosed as Map-3.

There are eight forest types in the West Bengal. These are eight forest types in the state.  They are Tropical Semi Evergreen, Tropical Moist Deciduous.  Tropical Dry Deciduous, Subtropical Broadleaved Hill, Subtropical Pine, Himalayan Moist Temperate, Montane Wet Temperate, Littoral and Swamp forests.  Forests are mainly distributed in the north, south west and south-east.

Protected Area:

There are 5 National Parks and 16 Wildlife sanctuaries covering an area of 0.28 million ha. It constitutes 3.15% of the geographic area of the state. The state has 2 Tiger Reserves namely Sundarbans and Buxa. Sunderbans Biosphere Reserve is one of the 11 Biosphere Reserves of the country. Sunderbans National Park is also a World Heritage site.

Human and Economic Development:

The geographic area of the state is 88,752 km2 which is 2.7% of the country’s geographic area. The human population of the state is 80.22 million, constituting 7.8% of the country’s population, of which 72.0% is rural and 28.0% urban. The average population density is 904 persons per km. The tribal population is 5.6% of the state’s population. The livestock population is 47.09 million, which is 7.5% of the country’s livestock population. The State ranks 6th amongst States/UTs in terms of percentage of area under tree cover. Minerals, like rock phosphate, fire clay, iron, limestone, copper, silica, quartz, manganese and sandstone are found in a considerable amount in the state. As a result, a few industries related to these are being established in the state. Steel and alloy steel plants in Durgapur and another steel plant in Burnpur are some of the important industries of the state. Other industries include jute, tea, cotton textiles, automobiles, bicycle, footwear, leather, paper, pharmaceuticals, chemicals, aluminum, sugar, timber processing, ceramic, glass, bone metal, and dairy.

Agriculture dominates both the landscape and the economy. Fifty-five percent of the population of West Bengal depends upon agriculture. In fact, agriculture makes a substantial contribution to the state’s income. West Bengal accounts for 57.3% of the total jute and 24% of the total tea produced in the country. Pulses, oilseeds, barley, maize, betel leaf, tobacco and sugarcane are some of the other agricultural products of West Bengal. Moreover, there are several multipurpose irrigation schemes in West Bengal, namely, the Damodar Valley, Mayurakshi, Kangsbati and Subarnarekha Barrage. The irrigation schemes are Teesta-Mahananda link canal in Jalpaiguri district, Terageria and Turga irrigation scheme in Purulia, Hinlow irrigation scheme in Birbhum district and Sohajare scheme in Bankura district.

2.3    Bihar

Physiography:

Bihar is located in the eastern part of the country (between 83°-30' to 88°-00' longitude). It is an entirely land–locked state, although the outlet to the sea through the port of Kolkata is not far away. Bihar lies mid-way between the humid West Bengal in the east and the sub humid Uttar Pradesh in the west which provides it with a transitional position in respect of climate, economy and culture. It is bounded by Nepal in the north and by Jharkhand in the south. The Bihar plain is divided into two unequal halves by the river Ganga which flows through the middle from west to east.

The general land use pattern of the state is given below:

| Land use | Area in ‘000 ha | Percentage |
| --- | --- | --- |
| Total geographical area | 17,388 |  |
| Reporting area for land utilisation | 17,330 | 100 |
| Forests | 2,949 | 17.02 |
| Not available for cultivation | 3,390 | 19.56 |
| Permanent pasture and other grazing lands | 107 | 0.62 |
| Land under misc. tree crops & groves | 337 | 1.94 |
| Culturable wasteland | 353 | 2.04 |
| Fallow land other than current fallows | 962 | 5.55 |
| Current fallows | 1,895 | 10.93 |
| Net area sown | 7,337 | 42.34 |

Source: Land use statistics at a Glance 1996-97, Ministry of Agriculture, GOI, 2000

Population:

| Details of population of Bihar as Population | 82,878,796 |
| --- | --- |
| Female | 39,724,832 |
| Male | 43,153.964 |
| Density(Persons per Sq.KM) | 880 |
| Urban Population | 10.47% |
| Sex Ratio(Females per thousand males) | 921 |
| Literacy | 47.53% |
| Males | 60.32% |
| Females | 33.57% |

per 2001 census are as follows:

Census survey 2001

Climate:

Bihar has varied climate. Only four clearly marked seasons with a brief interregnum of spring are observed, namely the hot season, the rainy season, the post monsoon season corresponding to autumn and the cold season. The hot winds (loo) of Bihar plains blow during April and May. Like the rest of the northern India, Bihar also experiences dust-storms, thunder-storms and dust raising winds during the hot season. This hot wind greatly affects human comfort during this season. The cold weather commences early in November and comes to an end in the middle of March. The climate in the cold weather is pleasant. The days are bright and warm.

Rainfall:

The rainy season begins in June and ends in September. It also experience rainfall during retreating monsoon season. The average rainfall is about 900 mm and maximum being 1800 in some areas.

Temperature:

The highest temperature is often registered in May which is the hottest month in the state. Temperature varies in the state from 38 o to 45 o C during summer and 7 o to 22 o C in winter.

Minerals:

Due to bifurcation Jharkhand as a separate state from Bihar, it has now less mineral resources. However, minerals like Mica, Basal, dolomite, Cement Mortar are generally found still now.

Soils:

Soil is one of the most important resources of a nation. It is the gift of nature of immense value. The most common use of the word soil is in the sense of a medium in which plants grow, although it has a different connotation at different time and place, and for persons engaged in different professions. Almost all the economic activities are directly or indirectly dependent on soil. Thus soil is the backbone of agricultural and industrial development.

Soil has a number of characteristics, which may be regarded as the aggregate of the physical, chemical and biological properties. The Bihar plane consists of a thick alluvial mantle of drift origin overlying in most part. The siwalik and older tertiary rocks. The soil is mainly young loam rejuvenated every year by constant deposition of silt, clay and sand brought by different streams. This soil is deficient in phosphoric acid, nitrogen and humus, but potash and lime are usually present in sufficient quantity.

There are three major types of soil in Bihar:

- Piedmont Swamp Soil-found in northwestern part pf West Champaran district.

- Terai Soil-found in northern part of the state along the border of Nepal.

- The Gangetic Alluvium - the plain of Bihar is covered by gangetic alluvium (both new as well as old).

The soil of the project corridor, in general, mostly belongs to the third category i.e., The Gangetic alluvium.

Water resources:

Water like ground and mineral resources is of great significance as it provides means of drinking water for man and animals, irrigation for agriculture, industrial uses, production of hydro-electricity, transportation and recreation etc. The importance of water is so immense that the people in ancient times worshipped it.

Bihar is richly endowed with water resources, both the ground water resource and the surface water resource. Not only by rainfall but it has considerable water supply from the rivers which flow within the territory of the State. Ganga is the main river which is joined by tributaries with their sources in the Himalayas. Some of them are Saryu (Ghaghra), Gandak, Budhi Gandak, Bagmati, Kamla-Balan and Mahananda.

There are some other rivers that start from the platue area and meet in Ganges or its associate rivers after flowing towards north. Some of them are Sone, Uttari Koyal, Punpun, Panchane and Karmnasha.

There are several rivers in Bihar which contribute a lot to the peoples of Bihar. These rivers make the water available for irrigation purpose and also help in generating the hydro-thermal energy for the state. Apart from this they provide a medium for water transport, provide fishes for fishery industry and enrich the natural resources of state in many other ways.

All the above rivers have their impact on the Bihar plain. State also has non-exhaustible source of ground water which is in use for drinking purposes, irrigation and industries.

Ecological resources:

Forest

The recorded forest area is 6,473km (sq.) which constitutes 6.87% of the geographic area of the state. By legal status, the Reserved Forest constitutes 693 km2, Protected Forest 5,779 km2.  and Un-classed Forest  1 km2. Forest Map of Bihar is enclosed as Map-4.Main forest type in Bihar is dry deciduous. Forests are found only in southern part of Bihar adjoining Jharkhand except west Champaran forest, which is in Northern part of Bihar.

Protected Areas

There is 1 National Park and 11 Wildlife Sanctuaries in the state.  They cover an area of 0.24 million has, constituting 2.55% of the total geographic area of the state.  Valmiki is the only tiger reserve in the state.  Kabar, situation in Begusarai district with an area of 6,738 ha. is an important wetland of national importance.

Human and Economic Development:

In 2000, the most industrial and mineral rich section of the State, the southern half, was taken away from Bihar to form the new State of Jharkhand. This was devastating to the economy base of Bihar, and resulted in a considerable drop in the tax base. It also left agriculture as the main economic activity. The new Bihar State’s agriculture production is still just slightly over half of what it had been before the division, so much of the agricultural base was lost as well.

There are many reasons for this. The land is ideal for agriculture. There are wide areas of very fertile soil, and a number of rivers. The climate is not harsh. The living conditions of the people should be higher, and the existences of large and populous empires here in the past testify that the land and the climate is not to blame. Most place the blame on neglect and the isolation of the area from the centers of Indian power, and before that British power. The local leaders have lacked visions and there has been little or no attention to those areas that spur economic growth like the basic infrastructure and education; the area suffered from the effects of the caste system and wide spread political corruption

India is a sleeping giant that is slowly waking. Although far from the seat of the new post Independent National Government, Bihar is waking also. There have been some attempts to attract and begin industry in the State. Some are agricultural based industries, but an oil refinery, a motor scooter plant, and a power plant have recently been built. It is a little too early to judge the outcome of these early steps, and many problems still remain. Bihar was once a rich and lush corner of the World and it is the dream of its people that it will be so again.

Even in the area of agriculture which should be providing the economic boost to raise the level of living in the State, very inadequate investments have been made. Despite the fertile land, almost nothing has been done in the area of irrigation. The people are as dependent on nature, and at its mercy, as were the inhabitants of those ancient empires

2.4   Uttar Pradesh (U.P)

Physiography:

Uttar Pradesh is bounded by Uttaranchal and Nepal in North, Madhya Pradesh and Chhattisgarh in south, Rajasthan, Haryana and Delhi in West and Bihar and Jharkhand in east.  It lies between lat.23º52’ and 31º28’ N and long.77º5’ and 84º38’E.  Physiographically Uttar Pradesh can be divided in to two distinct regions i) Indo-gangetic plain ii) Southern hills.

The general land use pattern of the state is given below:

| Land use | Area in ‘ 000 ha | Percentage |
| --- | --- | --- |
| Total Geographical area | 29,441 |  |
| Reporting Area for land utilisation | 29,794 | 100.00 |
| Forests | 5,150 | 17.29 |
| Not available for cultivation | 3,516 | 11.80 |
| Permanent Pasture & Grazing land | 296 | 0.99 |
| Land under misc. tree crops & groves | 513 | 1.72 |
| Culturable waste land | 945 | 3.17 |
| Fallow land other than current fallows | 832 | 2.79 |
| Current fallows | 1,067 | 3.58 |
| Net area Sown | 17,475 | 58.65 |

Source: Land use statistics At a Glance 1996-97, Ministry of Agriculture, GOI, 2000

Population:

| Details of population of Uttar Pradesh Population | 166,052,859 |
| --- | --- |
| Female | 78,586,558 |
| Male | 87,466,301 |
| Density(Persons per Sq.KM) | 689 |
| Urban Population | 20.78% |
| Sex Ratio(Females per thousand males) | 898 |
| Literacy | 57.36% |
| Males | 70.23% |
| Females | 42.98% |

as per 2001 census are as follows:

Census survey 2001

Climate:

The climate in Uttar Pradesh varies substantially. The Gangetic plain, which covers three-quarters of the state, is dry and dusty in summer. But during the monsoons between June and September, it is transformed into carpets of lush green fields. The monsoons also spell disaster for some regions, when the Ganga and its tributaries overflow their banks and flood large tracts of land. Winter is severe; the Gangetic plains are fairly cold during winter. Summers are extremely hot in the plains with maximum temperatures reaching as high as. The intensity of the summer months is magnified by the hot winds called ‘loo’ that blows across the plains in May and June, the two hottest months of the year.

Rainfall:

The average annual rainfall ranges between 1,000 to 1,200 mm.

Temperature:

The temperature varies maximum 45° C to 35° C during summer and 25 ° C to 3 ° C during winter.

Mineral resources:

The state of Uttar Pradesh is poor in mineral resources. Only considerable deposits are of limestone, andalusite and red sandstone in Mirzapur, Dolomite occurs in small quantities in Bandal and Varanasi. Pyrophyllite and diaspore in Jhansi and Hamirpur districts. Coal is available in Singrauli (Mirzapur district).

Soil:

Transmission line is passing through Indo-Gangetic plain area.  Predominant soils in the area are sandy soil.

Water resources:

The main rivers of the state from west to east are the Yamuna, the Ganga, the Ramganga, the Gomati and the Ghaghara.  All the rivers, except the Gomati, emerge from the Himalaya. The Yamuna and the Ganga flow from north-east to south-west in their upper mountainous courses, from north to the south in western parts of the state and thereafter from north-west to south-east joining at Allahabad.

Ecological resources:

Forests

In Uttar Pradesh the recorded total forest is 168.26 million ha. which is 7% of the state’s area.  Reserved Forest constitutes 65.9%, Protected Forest 14.4% and Un-classed Forest 19.7%.   Forest Map of Uttar Pradesh is enclosed as Map-5.

The 3 types of forests namely Tropical Moist Deciduous Tropical Dry Deciduous and Tropical Thorny Forests are found in Uttar Pradesh.

Protected Areas

There is one National park and 23 wild life sanctuaries, covering an area of 0.57 million Ha. which constitutes 2.36 % of the geographic area. Dudhwa Tiger reserve is located in the state.

Human and Economic Development:

ttar Pradesh is rich in human and natural assets. Most of State’s farm land is well watered and naturally fertile. U.P is the largest producer of food grains and oilseeds in the country. It leads all the states in India in the production of wheat, maize, barley, gram, sugarcane and potatoes.  The state (India’s sugar bowl) produces about one half of the total sugarcane output in the country. The western region of the state is more advanced in terms of agriculture. Majority of the population depends upon farming as its main occupation. Wheat, rice, sugar cane, pulses, oil seeds and potatoes are its main products. Sugar cane is an important cash crop almost through out the state and sugar mills and other cane crushers who produce Gur and Khandsari are common throughout the state. Uttar Pradesh is an important state in so far as horticulture is concerned.

SECTION III:   POLICY, LEGAL & REGULATORY FRAMEWORK

- 3.0   POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and directives and guidelines with respect to environmental and social considerations of Funding Agencies.

- 3.1     ENVIRONMENTAL

3.1.1 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under The Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under The Forest (Conservation) Act, 1980

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded; hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/re-conditioner or at the designated collection centers – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)Amendment Rules, 2003:

MOEF vide its notification dt. 20th May, 2003 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned/sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organization in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalizing the route alignment.

    - FUNDING AGENCIES:

- WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM)- F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.3   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- Applicable Legislations

- 3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement

on Environment and Development, 1992

- Policy statement for Abatement of pollution, 1992

- 3.2.0   SOCIAL

- 3.2.1    MANDATORY REQUIREMENTS (NATIONAL)

- National Policy on Resettlement and Rehabilitation for Project Affected Families: Ministry of Rural Development, Government of India has notified a National policy on R&R for  PAFs in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the assetless rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups.

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan/Tribal Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require neither resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.3    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- National and State-wide Laws and Policies Relating

to Land Acquisition and Issues of R&R

- Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

- Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.4   RELEVANT POLICIES

- The Orissa Resettlement and Rehabilitation of Project Affected Persons Policy, 1994 (water resources projects)

- Resettlement & Rehabilitation Policy – NTPC

# SECTION-IV: POWERGRID APPROACH FOR ROUTE SLECTION

- 4.0     ROUTE SELECTION - (ASSESSMENT & MANAGEMENT PROCESS)

- At the system planning stage itself one of the factors that govern the evolution of system is the possible infringement with the forest. Wherever such infringements are substantial, different alternative options are considered. The route/ site selection criteria followed by POWERGRID is detailed below:

- While identifying the transmission system for a generation project or as a part of National Power Grid, preliminary route selection is done by POWERGRID based on the topo sheets of Survey of India and Forest Atlas (Govt. of India's Publication). Further optimization is done during detailed survey using GIS/GPS. During route alignment all possible efforts are made to avoid the forest area involvement completely or to keep it to the barest minimum, whenever it becomes unavoidable due to the geography of terrain or heavy cost involved in avoiding it.

- 4.1   STUDY OF ALTERNATIVES

- Environmental Criteria for Route selection

- For selection of optimum route, the following points are taken into    consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

(ii) 	Any monument of cultural or historical importance is not affected by the route of the

transmission line.

(iii)	The proposed route of transmission line does not create any threat to the survival of any

community with special reference to Tribal Community.

(iv)		The proposed route of transmission line does not affect any public utility services like

playgrounds, schools, other establishments etc.

(v)	The line route does not pass through any sanctuaries, National Park etc.

(vi)		The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

- In addition, care is also taken to avoid National parks and sanctuaries and any other forest area rich in wild life.

- Keeping above in mind the routes of proposed line under this transmission System has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps etc. to arrive at most optimum route which can be taken up for detailed survey using GIS/GPS and assessment of environmental & social impacts for their proper management.

  - Evaluation of alternatives Route Alignment of Biswanath Chariyali-Bogaigaon

± 800kV HVDC Transmission line (Sub-project A1 & A2)

- Three different alignments (Route Map-1) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three (3) alternatives are as follows:

| Sl. No. | Description | R1 (Route 1) | R 2(Route 2) | R3 (Route 3) |
| --- | --- | --- | --- | --- |
| 1. | Route Particulars |  |  |  |
| i) | Length | 320.566 Km | 330.033 Km | 330.347 Km |
| 2. | Terrain |  |  |  |
| a) | Plain | 320.566 Km | 330.033 Km | 330.347 Km |
| b) | Hill | Nil | Nil | Nil |
| iii) | Agriculture | The major portion of the line will pass over agriculture land (75 % of the ROW area). etc. | The major portion of the line will pass over agriculture land (70 % of the ROW area etc. | The major portion of the line will pass over agriculture land (75 % of the ROW area). |
| iv) | Wet/ Marsh | 10 % | 10 % | 10 % |
| iii) | Other Type of Land | 15 % Tea Garden and Sub Urban area | 20 % Tea Garden and Sub Urban area | 15 % Tea Garden and Sub Urban area |
| 3. | Environmental Details |  |  |  |
|  | Name of District | Sonitpur, Udalguri, Baska, Barpeta, Chirang, Kokrajhar | Sonitpur, Udalguri, Baska, Barpeta, Chirang, Kokrajhar | Sonitpur, Udalguri, Baska, Barpeta, Chirang, Kokrajhar |
|  | Town Near Alignment | Biswanath Chariali, Sotia, Jamuguri, Balipara, Dhekiajuli, Rowta, Udalguri, Tangla, Barama, Tamulpur, Dhamdhama, Pathsala, Barpeta Road, Sarbhog, Bijni,Dhaligaon,Basugaon, Kokrajhar | Biswanath Chariali, Sotia, Jamuguri, Balipara, Dhekiajuli, Rowta, Udalguri, Tangla, Barama, Tamulpur, Dhamdhama, Pathsala, Barpeta Road, Sarbhog, Bijni,Dhaligaon,Basugaon, Kokrajhar | Biswanath Chariali, Jamuguri, Balipara, Barama, Dhekiajuli, Rowta, Udalguri, Tangla, Tamulpur, Bijni, Dhamdhama, Pathsala, Barpeta Road, Sarbhog, Dhaligaon, Sotia, Basugaon, Kokrajhar |
| iv) | Huts/Houses/Other structure/APs within ROW | 353 | 388 | 377 |
| v) | Forest (In Kms) | 0.25 Km road side plantation | 0.50 Km road side plantation | 0.60Km road side plantation |
|  | (In Ha.) | 1.73 | 3.45 | 4.14 |
| c) | Type of Forest- reserve/Protected / Mangrove/ Protected area/ Wild Life area/ Biosphere reserve/any other Sensitive area | Road side Plantation (Social Forestry) | Road side Plantation (Social Forestry) | Road side Plantation (Social Forestry) |
| b) | Density of Forest | 0.3 | 0.4 | 0.5 |
| c) | Type of Fauna & Flora | Crow, Sparrow, Pigeon, Fox, Squirrel Beetle-Nut, Banana, Mango, Jackfruit, Jamun, Tea Plantation etc. | Crow, Sparrow, Pigeon, Fox, Squirrel Beetle -Nut, Banana, Mango, Jackfruit, Jamun, Tea Plantation etc | Crow, Sparrow, Pigeon, Fox, squirrel Beetle-Nut, Banana, Mango, Jackfruit, Jamun, Tea Plantation etc |
| d) | Endangered | Nil | Nil | Nil |
| e) | Historical/ Cultural Monuments | Nil | Nil | Nil |
| f) | Any other relevant Information | - | Some brick kilns are near by the alignment. | Brick kilns in the alignment |
| 4. | Compensation Cost | ( Rs. in Lacs.) | ( Rs. in Lacs.) | ( Rs. in Lacs.) |
| i) | Forest |  |  |  |
|  | Compensatory Afforestation cost | 1.56 | 3.11 | 3.73 |
|  | NPV (Net Present Value) | 15.92 | 31.05 | 38.09 |
| ii) | Crop Compensation | 320.31 | 329.5 | 330 |
| iii) | Huts/Houses/APs/ Other Structure in ROW | 300.00 | 380.00 | 340.00 |
|  | Total | 637.79 | 743.66 | 711.82 |
| 5. | No of Crossing |  |  |  |
| i) | Railway | 4 Nos. | 4 Nos. | 4 Nos. |
| ii) | Transmission line crossings | 6 Nos | 7 Nos | 8 Nos. |
| iii) | Major River Crossing | 15 Nos | 17 Nos | 16 Nos |
| 6. | Construction Problem | i) Better approach roads available almost along the alignment. ii) Less ROW problem. | i) For major portion of the line approach road is difficult and at a few places construction of new approach road required. ii) A section of the line is passing through thick populated area for which severe ROW problem will arise during construction period. iii) More forest area involvement iv) Brick kilns along some portion of Trans. Line. v) More river crossings and huts are involved. | i) For portion of the line approach road is difficult and at a few places construction of new approach road required. ii) A section of the line is passing through thick populated area for which severe ROW problem will arise during construction period. iii) More forest area involvement iv) Brick kilns along some portion of Trans Line. v) More river crossings and huts are involved. |
| 7. | O&M problem | Due to better approach roads, maintenance of the line will become easier. | Due to difficult approach road in some of the portion of the line the maintenance problem of the line will come. In some portion of the line all season roads are not available | In this route approach is difficult and at some places approach is not possible without construction of path. In some portion of the line all season roads are not available |
| 8. | Overall Remarks. | Better approach roads are available through out the alignment making it is easy for construction and O&M works in future. The line traverses mostly over agricultural land and avoiding thick populated area. However, some unavoidable small huts will be encountered within the ROW corridor. At few places tribal area shall be encountered. | A portion of the line approach road is difficult Also a section of the line is passing through thick populated area for which not only arise severe ROW problem during construction period but also involve maximum Environmental & social impact. A major portion of the line will be routed through tribal area and which is unavoidable. | Approach road are not available in major portion of the line and this affect the progress of the works and also some portion of the line is passing through thick populated area also. A major portion of the line will be routed through tribal area and which is unavoidable. More involvement of. Rich plantation (Tea) along with trees in this alignment |

It may be seen from above that Alternative-I is the most optimum route as the alignment is shorter, involved less forest area and density in comparison Alternative II & III.   Alternative-I is also easily approachable in all seasons and hence, has been recommended for detailed survey.

  - Evaluation of alternatives Route Alignment of Bogaigaon-Saharasa ± 800kV HVDC Transmission line (Sub-project A3 & A4)

- Three different alignments (Route Map-2) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three (3) alternatives are as follows:

| S.N | Description | R1 (Route 1) | R 2(Route 2) | R3 (Route 3) |
| --- | --- | --- | --- | --- |
| 1 | Route Particulars |  |  |  |
| i) | Length | 463 Km | 486 Km | 495Km |
| 2) | Terrain in (Kms) |  |  |  |
| i) | Plain in (Km) | 463.171 | 485.9 | 494.6 |
|  | Hilly | NIL | NIL | NIL |
| ii) | Agriculture | Paddy, wheat jute, potato, Green Vegetable | Paddy, wheat jute, potato, Green Vegetable | Paddy, wheat jute, potato, Green Vegetable |
| v) | Wet/marshy | Wet | Wet | Wet |
| vi) | Estuarine | - | - | - |
| vii) | Other type of land | Agricultural land | Agricultural land | Agricultural land |
| 3 | Environmental Details |  |  |  |
| i) | Name of States /District details (Through which transmission line pass) | Kokrajhar in Assam, Jalpaiguri, Darjeeling, Uttar Dinajpur in West Bengal & Kishenganj, Araria, Madhepura and Saharsa in Bihar Darjeeling Darjeeling | Kokrajhar in Assam, Jalpaiguri, Darjeeling, Uttar Dinajpur in West Bengal & Kishenganj,Araria,Madhepura and Saharsa in Bihar | Kokrajhar in Assam, Jalpaiguri, Darjeeling, Uttar Dinajpur in West Bengal & Kishenganj, Araria,Madhepura, and Saharsa in Bihar |
| ii) | Town in Alignment (Nearby) | Kokrajhar,Barobisa, Alipurduar,Falakata, Dupguri, Jalpaiguri, Siliguri, Islampur, Kishenganj, Araria, Raniganj,Murliganj, Madhepura, Saharsa | Kokrajhar, Barobisa, Alipurduar, Falakata, Dupguri, Jalpaiguri, Siliguri, Islampur, Kishenganj, Araria, Raniganj, Murliganj, Madhepura, Saharsa | Kokrajhar, Barobisa, Alipurduar, Falakata, Dupguri, Jalpaiguri, Siliguri, Islampur, Kishenganj,Araria, Raniganj, Murliganj, Madhepura, Saharsa |
| iv) | Huts/House/other structure in ROW | 11 | 23 | 22 |
| iii) | Forest (in Km) | 1.00 | 3.5 | 7 |
|  | Forest (in Ha.) | 6.9 | 24.15 | 48.3 |
| a) | Type of Forest : Reserved protected/ Mangrove/Wild Life area/ Biosphere reserve/ any other sensitive area | NIL | Reserved Forest, Village Forest and Wild Life Sanctuary. | Reserved Forest, Village Forest and Wild Life Sanctuary. |
| b) | Type of Fauna & Flora | Elephant, Monkeys, Fox, Jackals, Reptiles etc. Eucalyptus, Shessam, Mango,Neem etc. | Elephant, Fox, Monkeys, Jackals, Reptiles etc. Eucalyptus, Shessam, Mango, Neem etc. | Elephant, Fox, Monkeys, Reptile, Jackals, etc. Eucalyptus, Shessam, Mango, Neem etc. |
| c) | Endangered species, if any | Nil | Single Horn Rhino | Single Horn Rhino, |
| d) | Historical/Cultural monument | Nil | Nil | Nil |
| e) | Any other relevant information | Less densely Populated Area. | Densely Populated Area. | Densely Populated Area. |
| 4 | Compensation Cost | Rs. In lakhs | Rs. In lakhs | Rs. In lakhs |
| i) | Corp. | 462.00 | 482.50 | 488.00 |
| ii) | Forest |  |  |  |
| a. | Compensatory Afforestation | 6.21 | 21.74 | 43.47 |
| b. | Net present value | 62.10 | 217.35 | 434.70 |
|  | Total | 530.31 | 721.59 | 966.17 |
| 5 | No. of Crossing |  |  |  |
| i) | Railway | 12 | 16 | 15 |
| ii) | N.H. Crossing | 6 | 6 | 6 |
|  | S.H. Crossing | 5 | 5 | 5 |
| ii) | Power Line |  |  |  |
|  | a)400 KV Trans. Line | 4 | 6 | 6 |
|  | b)220 KV Trans. Line | 2 | 5 | 6 |
|  | c)132 KV Trans. Line | 9 | 9 | 11 |
|  | d)66 KV Trans. Line | 1 | 1 | 1 |
| iii) | River Crossing. |  |  |  |
|  | a)Major | 7 | 7 | 7 |
|  | b)Minor | 26 | 25 | 31 |
| 6 | Construction Problem | Most of portion is connected with metal roads and easily accessible | i). Difficult approach road ii) More ROW Problem during construction period due to dense population. iii) Reserve forest, more forest area. | i) Difficult approach road ii) More ROW Problem during construction period due to dense population. iii) Reserve forest, more forest area. |
| 7 | O&M Problem | Not anticipated | Remote Area Passing through Dense forest, Vehicle movement may be restricted in during monsoon season | Remote Area Passing through Dense forest, Vehicle movement may be restricted in during monsoon season |
| 8 | Overall Remarks | Optimum Route is suitable based on economic viabilities and other important features. | Not suitable for construction of transmission line. | Not suitable for construction of transmission line. |

It may be seen from above that Alternative-I is the most optimum route as the line length is shorter than Alternative II&III and involves less forest in comparison to Alternative-II & Alternative-III. Forest density is also less in comparison to Alternative-II & Alternative-III. Locations are approachable in comparison to other alternatives. Hence, Alternative-I has been found to be most optimum and recommended for detailed survey.

  - Evaluation of alternatives Route Alignment of  Saharasa-Gorakhpur ± 800kV HVDC Transmission Line

- Three different alignments (Route Map-3) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three (3) alternatives are as follows:

| S.No | Description | R1 (Route 1) | R 2(Route 2) | R3 (Route 3) |
| --- | --- | --- | --- | --- |
| 1 | Route Particulars |  |  |  |
| i | Length(Km) | 363.528 | 385 | 390 |
| ii | State | Bihar , Uttar Pradesh | Bihar , Uttar Pradesh | Bihar , Uttar Pradesh |
| 2. | Terrain | Almost Plain | Almost Plain | Almost Plain |
|  |  | 90% Agriculture | 91% Agriculture | 92% Agriculture |
| 3. | Environment Details |  |  |  |
| i | District details (Through which transmission line pass) | Saharsa, Samastipur, Muzaffarpur, Darbhanga, Siwan, Chhapra, Gopalganj, Vaishali, Deoria, Gorakhpur | Saharsa, Samastipur, Muzaffarpur, Darbhanga, Siwan, Chhapra, Gopalganji, Vaishali, Deoria, Gorakhpur | Saharsa, Samastipur, Muzaffarpur, Darbhanga, Siwan, Chhapra, Gopalganji, Vaishali, Deoria, Gorakhpur |
| ii | Town in alignment (Nearby) | Saharsa, Rusera, Pusa, Samastipur, Muzaffarpur, Vaishali, Sariya, Musrak, Siwan, Gopalganji, Mirganji, Hathuva, Devoriya, Gorakhpur | Saharsa, Rusera, Pusa, Samastipur, Muzaffarpur, Vaishali, Sariya, Musrak, Siwan, Gopalganji, Mirganji, Hathuva, Devoriya, Gorakhpur | Saharsa, Rusera, Pusa, Samastipur, Muzaffarpur, Vaishali, Sariya,Musrak, Siwan, Gopalganji, Mirganji, Hathuva, Devoriya, Gorakhpur |
| iii | House within ROW | Nil | 4 nos. (Kamlabari, Mahri, Kothia, Hathauri) | 9 nos. (Bishunia, Lodhipur, Madhulia, Dharampur, Gopalpur, Nurachak, Dadhua, Begampatti, Chak Suleman) |
| iv | Forest |  |  |  |
|  | (in Km) | 6.08 | 8 | 9 |
|  | ( in Ha) | 41.95 | 55.2 | 62.1 |
| v | Type of forest | Social Forestry located along NH, Railway , River Crossing ROW) | Protected and Social Forestry located along NH, Railway, River Crossing | Forestry located along NH, Railway, River Crossing. |
| vi | Density of forest | 0.1 | 0.2 | 0.30 |
| vii | Type of flora | Mango, Lichi, Sheesam , Khajoor, Eucalyptus | Mango , Lichi, Sheesam , Khajoor, Bamboo | Mango, Lichi, Sheesam, Khajoor, Bamboo |
| viii | Type of fauna | Jackal, Fox, Snake, Crow | Jackal, Fox, Snake, Crow | Jackal, Fox, Snake, Crow |
| ix | Endangered specious, if any | Nil | Nil | Nil |
| x | Historical /Cultural monuments | Nil | Nil | Nil |
| xi | Any other relevant information, if any | Nil | This route is passing through some low land and marshy area near village Benduli where construction activity will be difficult. | This route is also passing through some low land and marshy area near village Bishunia where construction activity will be difficult. |
| 4. | Compensation Cost | Rs. In Lakhs | Rs. In Lakhs | Rs. In Lakhs |
| i) | Crop/Trees | 358 | 377 | 381 |
| ii) | Forest |  |  |  |
| a. | CA cost | 37.76 | 49.68 | 55.89 |
| b. | NPV | 377.55 | 496.8 | 558.9 |
|  | Total | 773.31 | 923.48 | 995.79 |
| 5. | No. of Crossing |  |  |  |
|  | i) Railway | 7 | 8 | 7 |
|  | ii)Trans.Line | 9 | 12 | 11 |
|  | iii) River Crossing | 5 | 5 | 7 |
| 6. | Const. Problems | i) Approach road is not available at most of locations. ii) Proposed line is passing parallel to existing 400 KV DC Purnia-Muzaffarpur-Gorakhpur TL which is recently constructed. iii) Less forest area involvement. iv) Avoidance of houses within ROW. v) Construction of line is easy as compare to other alternative. | i) Approach road is not available at most of locations. ii) Difficult at some locations especially in Monsoon iii) More forest lying along this route. iv) Difficult approaches at some places near village Benduli, Basantpur, Pakri, Murli etc. v) New approach road is required during construction at some places. v) More angle points are in this line. vi.) Involvement of houses within ROW. | i)Approach road is not available ii) Difficult at some locations especially in Monsoon . iii.) More forest lying along this route. iv) Difficult approaches at some places near village Bhama, Beldawar, Bishnia, Madhulia, Hasopur etc. v.) New approach road is required during construction at some places. vi.) More angle points are in this line. vi.) Involvement of houses within ROW. |
| 7. | O&M Problems | Not anticipated due to availability of approaches. This line section is parallel to existing 400 KV D/C Purnia-Muzaffarpur-Gorakhpur TL. So, O&M activity will be easy. | Anticipated due to approach road, low lying area etc. | Anticipated due to approach road, low lying area etc. |
| 8. | Overall remarks | -Approach paths available in all weather conditions. -Almost Parallel to existing line, No maintenance problem -No house/structure in ROW -Less forest area -Less Row problem during construction Optimum Route | -Approach paths not available during monsoon -Avoidance of house / structure in ROW not possible -more forest area -Row problem during construction | -Approach paths available during monsoon -avoidance of house/ structure in ROW not possible -more forest area -Row problem during construction |

It may seen from above Alternative-I is the most optimum route as it involves minimum   forest  in comparison to Alternative-II and III. Due to availability of approach road, avoidance of houses within ROW and alignment almost parallel to existing 400 KV Purnea-Muzaffarpur-Gorakhpur Transmission Line for O&M purpose   Alternative-I has been selected for detailed survey.

  - Evaluation of alternatives Route Alignment of  Gorakhpur-Agra via Lucknow  ±

800kV HVDC Transmission Line

Three different alignments (Map-4) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three (3) alternatives are as follows

| Sl.No | Description | R1 (Route 1) | R 2 (Route 2) | R3 (Route 3) |
| --- | --- | --- | --- | --- |
| 1. | Route Particulars |  |  |  |
| i. | Route Length (Kms) | 580 | 585 | 588 |
| 2. | Terrain | Plain area | Plain area | Plain area |
| 3. | Environment Details |  |  |  |
| i) | District Detail in alignment. | Gorakhpur, Santkabirnager, Basti, Faizabad, Barabanki,Lucknow, Unnao, Kanpur Nagar & Dehat, Etawa, Mainpuri, Firozabad, Agra | Gorakhpur, Santkabirnager, Basti, Faizabad, Barabanki, Lucknow, Unnao, Kanpur Nagar & Dehat, Etawa, Mainpuri, Firozabad, Agra | Gorakhpur, Santkabirnager, Basti, Faizabad, Barabanki. Lucknow, Unnao, Kanpur Nagar & Dehat, Etawa, Mainpuri, Firozabad, Agra |
|  | Major town in alignment | Rudrapur,Hariharpur, Mahuli,Jagannathpur, Tanda, Mayabazar, Sahganj, Vindwa, Mohanlalganj, Arjun Mau, Navaganj, Unnao, Etawah, Fatehabad, Samasabad | Rudrapur, Hariharpur, Mahuli, Jagannathpur, Tanda, Mayabazar, ahganj, Vindwa, Mohanlalganj, Arjun Mau, Navaganj, Unnao, Etawah, Fatehabad, Samasabad | Rudrapur, Hariharpur, Mahuli, Jagannathpur, Tanda, Mayabazar, Sahganj, Vindwa, Mohanlalganj, Arjun Mau, Navaganj, Unnao, Etawah, Fatehabad, Samasabad |
| ii) | House within ROW | Nil | Due to fast development few buildings are encountered within the ROW. Due care has been taken to provide sufficient clearance to all such buildings without affecting any displacement | Due to fast development few buildings are encountered within the ROW. Due care has been taken to provide sufficient clearance to all such buildings without affecting any displacement |
| iii) | Forest (in Km) | 4.5 | 6.55 | 9.30 |
|  | (in Ha.) | 31.05 | 58.95 | 64.17 |
| a | Type of forest | Reserved Forest (RF) | Reserve (RF) and Protected Forest (PF) | Reserve (RF) and Protected Forest (PF) |
| b | Density of forest | 0.3 | 0.4 | 0.4 |
| c | Type of fauna & flora | Mango, Jamun, Acacia, Neem, Eucalyptus etc. Crow, Squirrel, Rabbit, Fox etc. | Mango, Jamun, Acacia, Neem, Eucalyptus etc. Crow, Squirrel, Rabbit, Fox etc | Mango, Jamun, Acacia, Neem, Eucalyptus etc. Crow, Squirrel, Rabbit, Fox etc |
| d | Endangered species, if any | Nil | Nil | Nil |
| iv) | Historical/cultural monument | None | None | None |
| v) | Any other relevant information | Nil | Nil | Nil |
| 4. | Compensation | Rs.in Lakhs | Rs.in Lakhs | Rs.in Lakhs |
| i) | Crop | 575.5 | 548.45 | 548.7 |
| ii) | Forest |  |  |  |
|  | CA cost | 27.95 | 53.06 | 57.75 |
|  | NPV Cost | 279.45 | 530.55 | 577.53 |
|  | Total Cost | 882.90 | 1132.06 | 1183.99 |
| 5. | Major Crossing |  |  |  |
| i | Road (NH) | 7 | 8 | 8 |
| ii | Railway | 5 | 5 | 5 |
| iii | Trans. Line | 13 | 13 | 15 |
| iv | River Crossing | 4 | 5 | 5 |
| 6. | Construction problem | i. No Water log areas and terrain is plain ii. Maximum route is passing near by Cart Track. | i. About 10 Km. water log area near Bela (5 Km.) and Mainpuri (5 Km.) and terrain is plain. Water logging are also near Dubaliya, Kalwari bazar ii. Maximum route is passing away from Road. iii. Due to fast developing near Bela / Bithur, few buildings may encounter ROW. iv. Line will pass through Mango/ Guavava near Safipur/Ajgain | i. About 15 Km. water lodge area near Bela (10 Km.) and Mainpuri (5 Km.) and terrain is plain. Water logging are also near Dubaliya, Kalwari bazar ii. Maximum route is passing away from Road iii. Due to fast developing near Bela/ Bithur, few buildings may encounter ROW. iv. Line will pass through Mango/ Guavava near Safipur/Ajgain |
| 7. | O&M | Nil | Due to non availability of approach road &water logging area O&M will be difficult. | Due to non availability of approach road &water logging area O&M will be difficult |
| 8. | Overall Remarks | This route is most suitable | Not suitable | Not suitable |

It may seen from above Alternative-I is the most optimum route as it involves minimum   forest  in comparison to Alternative-II and III.   Available of approach road, alignment passing almost parallel to existing 400 KV Purnea-Muzaffarpur-Gorakhpur Transmission Line-easy to carry out O&M activity, Avoidance of houses /water logging area with in ROW and less forest involvement are considered to select Alternative-I for detailed survey.

  - Earth Electrode Line at Biswanath Chariyali end ( Associated with  ± 800kV HVDC

Terminal in Assam) – 50 Km

Earth Electrode Line being of small length only one alignment is done. Out of 50 Km in almost plain area, 13.08 km is to cross forest land devoid of forests (tree) in Balipara R.F. of Sonitpur West division in Sonitpur Distric of Assam involving about 30 Ha. of forestland. Cost of Rs. 24.4 Lakhs for raising compensatory afforestion and Rs.199.36 lakhs towards Net Present value (NPV) for use of forest land has been estimated to be paid to the forest department, Assam. A provision for Rs.50.0 lakhs has been kept for the crop/tree compensation. (Ref. CPTD).

  - Earth Electrode Line at Agra end (Associated with   ± 800kV HVDC Terminal in

Uttar Pradesh) – 40 Km.

Earth Electrode Line being of small length only one alignment is done. Out of 40 Km in almost plain area, 2.0 km is to cross forest land in Agra District of Uttar Pradesh involving about 4.4 Ha. of forestland. Cost of Rs. 4.4 Lakhs for raising compensatory afforestion and Rs.36.5 lakhs towards Net Present value (NPV) for use of forest land has been estimated to be paid to the forest department, Uttar Pradesh. A provision for Rs.24.45.0 lakhs has been kept towards crops/tree compensation. (Ref. CPTD).

4.8   Impact on Ecological sensitive Area (Forest Land)

Details of forest land: ± 800kV HVDC Transmission Line from Biswanath Chariyali (Assam) to Agra (Uttar Pradesh).

- Biswanath Chairiyali to Bongaigaon (Assam):Sub-project A1 & A2:- 320.566 Km with

Earth Electrode line-50 Km

This will pass through following districts of Assam.  Details of forest cover of these districts are as follows:

(Area in Km2)

| State/District | Geographic Area | Forest Cover |  |  |  | % of Forests |
| --- | --- | --- | --- | --- | --- | --- |
| Assam |  | Very Dense | Moderately Dense | Open Forest | Total Forest |  |
| Barpeta | 3,245 | 35 | 183 | 183 | 401 | 12.36 |
| Bongaigaon (Chirang) | 2,150 | 33 | 267 | 212 | 512 | 20.40 |
| Darrang (Udalguri) | 3,481 | 17 | 121 | 375 | 513 | 14.74 |
| Kokrajhar | 3,169 | 207 | 709 | 267 | 1183 | 37.33 |
| Nalbari ( Baska) | 2257 | 4 | 66 | 210 | 280 | 12.41 |
| Sonitpur | 5324 | 71 | 334 | 636 | 1041 | 19.55 |
|  | Total |  |  |  | 3890 |  |

Forest survey report 2003

The above Section of the line including Earth electrode line will involve 31.73Ha. forest land in Assam state, which is about 0.0082% of the total forest area of the abovementioned districts of the state.

2.   Bongaigaon (Assam) to Saharsa (Bihar): Sub-project A3 & A4:-463.0 Km

This section of the line will pass through following districts of Assam, West Bengal and Bihar.  Details of forest cover of these districts are as follows:

(Area in Km2)

| State/District | Geographic Area | Very Dense | Moderately Dense | Open Forest | Total Forest | % of Forests |
| --- | --- | --- | --- | --- | --- | --- |
| Assam |  |  |  |  |  |  |
| Kokrajhar | 3,169 | 207 | 709 | 267 | 1183 | 37.33 |
| Total | 3,169 | 207 | 709 | 267 | 1183 |  |
| West Bengal |  |  |  |  |  |  |
| Jalpaiguri | 6,227 | 607 | 566 | 1,220 | 2,393 | 38.43 |
| Darjeeling | 3,149 | 472 | 893 | 856 | 2,221 | 70.53 |
| Total | 9376 | 1079 | 1459 | 2085 | 4614 |  |
| Bihar |  |  |  |  |  |  |
| Kishanganj | 1,884 | 0 | 1 | 8 | 9 | 0.48 |
| Araria | 2,830 | 0 | 14 | 14 | 28 | 0.99 |
| Madhepura | 1,788 | 0 | 6 | 4 | 10 | 0.56 |
| Saharsa | 1,680 | 0 | 5 | 1 | 6 | 0.36 |
| Total | 8182 | 0 | 26 | 27 | 53 |  |

Forest survey report 2003

The above Section of the line will involve 6.9 Ha. forest land in three state (Assam-Nil, West Bengal-3 Ha.& Bihar-3.9Ha.) which is about 0.00 %, 0.0007 %  & 0.074 % respectively of the total forest area of the districts of the respective States.

3.   From Saharsa (Bihar) to Gorakhpur (Uttar Pradesh) Sub-project A5 & A6:- 363.528 Km

This section of transmission line will pass through following districts of Bihar and Uttar Pradesh.  Details of forest cover of these districts are as follows:

(Area in Km2  )

| S. No. | State/District | Geographic Area | Very Dense Forest | Moderately dense | Open dense Forest | Total Forest | Percent |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A. | Bihar |  |  |  |  |  |  |
| 1. | Sharasa | 1,680 | 0 | 5 | 1 | 6 |  |
| 2. | Samstipur | 2,904 | 0 | 9 | 3 | 12 |  |
| 3. | Muzzafarpur | 3172 | 0 | 2 | 2 | 4 |  |
| 3. | Darbhanga | 2,279 | 0 | 5 | 6 | 11 |  |
| 4. | Siwan | 2,219 | 0 | 1 | 3 | 4 |  |
| 5. | Chappra (Saran) | 2,641 | 0 | 7 | 4 | 11 |  |
| 6. | Gopalgang | 2,033 | 0 | 0 | 0 | 0 |  |
| 7. | Vaishali | 2,036 | 0 | 12 | 0 | 12 |  |
|  | Total | 18,964 | 0 | 41 | 19 | 60 |  |
| B. | Uttar Pradesh |  |  |  |  |  |  |
| 1. | Deoria | 2,538 | 0 | 1 | 16 | 17 |  |
| 2. | Gorakhpur | 3,321 | 0 | 40 | 25 | 65 |  |
|  | Total | 5,859 | 0 | 41 | 41 | 142 |  |

Forest survey report 2003

The above Section of the line will involve 41.95 Ha. forest land in two state (Bihar-30.05 Ha.& U.P.-11.90 Ha..), which is about 0.5008 % & 0.0838% respectively of the total forest area of the districts.

4. From Gorakhpur-Agra via Lucknow (Uttar Pradesh): Sub-project A7, A8 & A9:- 580.0 Km

with Earth Electrode line-40 Km

This section of transmission line will pass through following districts of Uttar Pradesh.  Details of forest cover of these districts are as follows:

(Area in Km2)

| District | Geographic Area | Forest Cover |  |  |  | Percent |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Very Dense | Moderately Dense | Open Forest | Total Forest |  |
| Gorakhpur | 3,321 | - | 40 | 25 | 65 | 1.96 |
| Sant Kabir Nagar | 1,646 | - | - | 2 | 2 | 0.12 |
| Basti | 2,688 | - | 6 | 12 | 18 | 0.67 |
| Faizabad | 2,174 | - | 5 | 51 | 56 | 2.58 |
| Barabanki | 4,402 | - | 4 | 82 | 86 | 1.95 |
| Lucknow | 2,528 | - | 115 | 183 | 298 | 11.79 |
| Unnao | 4,558 | - | 34 | 197 | 231 | 5.07 |
| Kanpur Nagar & Dehat | 6,176 | - | 16 | 97 | 113 | 1.83 |
| Etawa | 2,311 | - | 46 | 139 | 185 | 8.01 |
| Mainpuri | 2,760 | - | 1 | 15 | 16 | 0.58 |
| Firozabad | 2,361 | - | 5 | 39 | 44 | 1.86 |
| Agra | 4,027 | - | 74 | 199 | 273 | 6.78 |
| Total | 60,202 | 0 | 346 | 1041 | 1387 |  |

Forest survey report 2003

The above Section of the line will involve 33.05 Ha. forest land in Uttar Pradesh, which is about 0.0238% of the total forest area of the districts.

Note: State wise break up of Line Length:

- Assam            -  370.496 Km + 50 Km E/E line (Subproject-A1 & A2, Part of A3)

- West Bengal  -  207.91 Km (Subproject-A3& Part of A4)

- Bihar              -  495.97 Km (Subproject- Part of A4, A5 & Part of A6)

- Uttar Pradesh -  652.718 Km+ 40Km E/E line (Subproject-Part of A6, A7, A8 & A9)

i.e. Total Length of ± 800kV HVDC Line= 1727.094 Km & E/E Line- 90 Km

The entire Line will involve forest area of 112.82 Ha. (Assam-31.73 Ha., West Bengal-3 Ha., Bihar-33.95 and Uttar Pradesh-44.95 Ha.) i.e. forest stretch of about 11.83 Km (considering ROW-69 Meter) of HVDC Transmission Line and 14.55 Km of Earth Electrode Line (Considering ROW-22 Meter.)

It may	be  seen from the above that only 11.83 Km forest stretch is involved in 1727.094 Km of HVDC Line i.e. forest land/area involvement is about 0.68 % and 14.55 Km of forest stretch is involved in 90 Km of earth Electrode Line i.e. forest land/area involvement is about 16.16%.

Forests are mainly Social forestry raised as strip plantations along road side for aesthetic look. Affected nos. of Trees/Plants would be around 2000 in the forestland of entire route. These affected trees/plants may also be avoided by raising tower height.

State wise forest involvement:

| State | Total forest Area | Total forest area in HVDC & E/E Line | % involvement of forest | Remarks |
| --- | --- | --- | --- | --- |
| Assam | 1,930,000 Ha. | 31.73 Ha. | 1.64x10-3 | Negligible |
| West Bengal | 1,195,000 Ha. | 3.00 Ha. | 2.5x10-4 | Negligible |
| Bihar | 2,949,000 Ha. | 33.14 Ha. | 1.15x10-3 | Negligible |
| Uttar Pradesh | 5,150,000 Ha. | 44.95 Ha. | 8.7x10-4 | Negligible |
| Total | 11224000 Ha. | 112.82 Ha. | 1.0123x10-3 | Negligible |

Above table analyses that impact on ecological sensitive area is negligible.

SECTION V: INFORMATION FOR ASSOCIATED FACILITIES (LOWER

SUBANSIRI-I AND KAMENG HYDRO-GENERATION PROJECT)

5.0     Introduction

As per MOE&F guidelines Hydro-projects are covered under Schedule-1 of Environmental (Protection) Act 1986 (Notification dt. 14.09.06-attached at Annexure-I in supersession of earlier notification dt.27.01.1994) which stipulates that all such project require prior approval of GOI (MOE&F) for taking up project activities. This act is a comprehensive law, which supplements existing law on pollution control, lays down standard for the quality of environment and restricts areas in which industrial activity can be carried out. A study on Environment Impact Assessment (EIA) of all such project is required for the processing of proposal of Environmental Clearance. The major environmental discipline covered under EIA studies includes impact on Air, Water, Noise, Soil, Land use Socio-economic, terrestrial ecology, aquatic ecology and health etc. It also requires a comprehensive Environment Management Plan (EMP) including safeguards and control measure proposed to prevent/ mitigate environmental impact, rehabilitation plan, afforestation plan and Environmental monitoring plan during operation of the project, occupational safety and health, Risk Assessment and Disaster Management Plan.

The Schedule-1 i.e. Environmental (Protection) Act 1986, however, excludes UHV/EHV Transmission project/corridor. Therefore, EIA study is not required to be undertaken by POWERGRID and no Environmental Clearance is required but  Forest Clearance under Forest (Conservation) Act, 1980 is applicable, if forest land is involved, in case project / line is located or traversing through forestland.

National Hydro Electric Power Corporation-NHPC, a Government of India Undertaking has undertaken construction of Lower Subansiri-I Hydro-Electric Project (8x250 MW) in Arunachal Pradesh and Assam and North Eastern Electric Power Corporation, NEEPCO, a Government of India Undertaking has undertaken construction of Kameng Hydro-Electric Project (4x150 MW) in Arunachal Pradesh for generation of  electricity.

Environmental Clearance obtained by NHPC and NEEPCO from MOEF/GOI is discussed below in brief:

5.1   Environmental Clearance:

Lower Subanshri-I Hydro-Electric Project

The Environmental proposal of Lower Subansiri was discussed before Appraisal Committee at Ministry of Environment and Forests (MOE&F)/GOI, New Delhi. On the recommendation of the committee, MOEF has issued Environmental clearance on 16.07.2003 to NHPC. It involves 3999 Ha. of Forest area. Ministry of Environment and Forests (MOEF) has issued in-principle (i.e.1st Stage) forest clearance to NHPC on 10.6.2003 and final clearance (i.e. 2nd Stage) on 12.10.2004 after compliance of condition stipulated in 1st Stage clearance for diversion of 3999 Ha. forest area (Arunachal Pradesh-3183.0 ha. & Assam-816.3). Out of 3183.0 ha.of forest area in Arunachal Pradesh, Tale Valley wild life sanctuary area-42 Ha. is also involved for which MEOF has accorded wild life clearance on 27.05.2003 after having IBWL (Indian Board for Wild Life) and Hon’ble Supreme Court (SC) concurrences as prior permission from SC is required for any activity on Wildlife Sanctuary/National park.

Environment clearance and forest Clearance letters are enclosed at Annexure-II.

Kameng Hydro Electric Project

The Environmental proposal of Kameng Hydro Electric Project was discussed before Appraisal Committee at Ministry of Environment and Forests (MOE&F) New Delhi. On the recommendation of the committee, MOEF has issued Environmental clearance on 29.03.2001 to NEEPCO.  It involves 710 Ha Forest areas. NEEPCO has taken Forest clearance from MoE&F on 03.08.2000. There is no Wildlife Sanctuary/National Park involved in this project.

Environment clearance and Forest Clearance letters are enclosed as Annexure-III

Public Hearing

Public hearing is mandatory for all projects, as per Govt. of India. Notification no. 244 dtd 10 April 1997 covered under Schedule-1 of Environmental Protection Act, 1986 and still continue to be there under notification dt.14.09.06. In case of Lower Subansiri Hydro-Electric Project, NHPC undertook EIA study and EIA report was submitted to MOEF, New Delhi. Public hearings were also conducted in Arunanchal Pradesh and Assam.

In case of Kameng Hydro Electric project, NEEPCO undertook EIA study and EIA report was submitted to MoE&F, New Delhi and public hearing was conducted in Arunachal Pradesh.

The details of public consultation of the both projects are given below.

| S.No. | Place/State | Date |
| --- | --- | --- |
| A. Lower Subansiri Hydro Electric Project (NHPC) |  |  |
|  | Arunachal Pradesh |  |
| 1. | Zerhopoli | 17.08.2001 |
| 2. | Daponjo | 20.08.2001 |
| 3. | Along | 20.08.2001 |
| 4. | Papumpure | 24.08.2001 |
|  | Assam |  |
| 5. | Gerukhamuk NHPC complex/Assam | 04.09.2001 |
| B. Kameng Hydro Electric project (NEEPCO) |  |  |
|  | Arunanchal Pradesh |  |
| 1. | Bomdila | 17.11.1999 |

# SECTION-VI: SCREENING OF POTENTIAL ENVIRONMENT IMPACT

# AND ITS MANAGEMENT

6.0	Impact Due to Project Location and Design

Environmental impact of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. In order to get latest information and further optimization of route modern survey techniques/tools like GIS/ GPS & aerial photography are also applied.  Introduction of GIS / GPS in route selection has resulted access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. In the instant project also, these techniques are used and detailed survey using GIS/GPS is in progress to assess possible impact and engineering /technical estimation. Physical detail survey shall be taken up during actual construction and actual impact would be known, implemented applying mitigation measure as planned and monitored/audited. Although, all possible measures have been taken during the finalization of route alignment for the proposed transmission line but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to specific environment review criteria based on preliminary survey are as follows:

(i) Resettlement

As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be seen from the above description of proposed route alignment and also keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

The proposed project does not involve construction of any new substation and no fresh land acquisition is involved hence, R&R issue shall not be involved.

(ii) Land value depreciation

Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

(iii) Historical/cultural monuments/value

As per the POWERGRID’s policy of route selection process, route alignment is finalized avoiding all the historical and cultural monuments. As per the preliminary assessment carried out during finalization of route alignment in consultation with State revenue authorities and ASI, no such monuments are coming in the proposed route alignment.

(iv) Encroachment into precious ecological areas

As already explained all precautions have been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries. In spite of this it was not possible to avoid forest completely in this project though National park and Sanctuaries have been avoided completely. However, the route of proposed HVDC transmission line/Earth Electrode Line is so finalised that it affects minimum forest area. It may be  seen  from  the  tables at 4.8 of section-4  that  out of entire HVDC transmission line length of about 1727.094 Km about 11.83 Km ( 0.68% only) length shall  pass  through  forest  land  consisting  of  81.63 Ha. forest area in  the  states  of Assam, West-Bengal, Bihar and  Uttar Pradesh and out of 90 Km of Earth Electrode Line about 14.55 Km length shall pass through forestland consisting 32 Ha. forest area in the state of Assam and Uttar Pradesh. About 90% of the total forest area is strip plantations raised along road and canal side only. The forests to be traversed  by  the  lines  are  already degraded and the  wildlife species may be present are those who have been adapted  to  open or disturbed habitat. Therefore with provision of Compensatory Afforestation the overall forest status will improve.  In  forest area, tree felling, if necessary for  Right-of-way (ROW) of  transmission line  will  be done  under the supervision  of  Forest  Department and  some  low  canopy  seed  trees  and  shrubs may  be  kept  intact,  if  they  do not  interfere  with  tower  erection and  line installation. The wood, if any, will be sold by the Forest Department, who will also retain the sale proceeds. Seven (7) meter wide strips of forestland under each conductor will be cleared and maintained as maintenance rows, but the remaining land will be allowed to regenerate to a permissible height (2 to 3 meter maximum). Lopping of trees to maintain required electrical clearance will be done under the direction of Forest Department. POWERGRID will provide to construction crews with fuel wood or alternative fuels as a precaution against collection of fuel wood from nearby forest.

Transmission lines can serve as new access routes into previously inaccessible or poorly accessible forests, thereby accelerating forest and wildlife loss. In such cases, POWERGRID cannot take action itself, but local Forest Department personnel will normally assess the dangers and take appropriate action, such as establishing guard stations at the entrance to the forest.  Given the already easy access and degraded conditions at the proposed projects sites, this problem is not expected to be encountered.  Nonetheless, POWERGRID staff will report to the Forest Department any noticeable encroachment induced by the Projects.

(v)  Encroachment into other valuable lands

Impacts on agricultural land will be restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1). The proposed project envisages constructing 1727 Km of HVDC Line which involves approximately 4320 towers. Construction of these towers will result in loss of approx. 4320 sq. m. or  0.4320 ha. of land. For Earth Electrode Line of 90 Km, about 240 towers of 220kV class shall be used and construction of these towers will result in loss of approx. 240sq. m. or  0.024 ha. of land. Thus, total Land loss would be 0.456 Ha. only including forest land which is insignificant.

In areas where line will traverse agricultural land, compensation will be paid to owners for any crop damage or any other temporary damages incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation. Revenue department, after evaluating the loss due to construction activity and productivity of land arrives at the compensation cost which is paid to farmers. Based on preliminary survey, Rs. 2125.62 lakhs has been estimated towards cost compensation of crops/tree or any other temporary damages during construction in the state of Assam, West Bengal, Bihar and Uttar Pradesh. Agricultural activities will be allowed to continue following the construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner's satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss from the tree/plant to be assessed by Rev./Forest authority. For fruit bearing trees, estimation of loss of products would be done over an eight year period. Agricultural lands under private ownership will be identified, and in accordance with normal POWERGRID procedures compensation will be paid to the affected villagers as per compensation plan for temporary damages (CPTD) prepared separately (Ref. CPTD).

(vi) Interference with other utilities and traffic

As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission line affects nearby telecommunication circuits by causing electrical interference. A standing committee -- Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like re-routing of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50000/- per km for POWERGRID.

Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission line passes near the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights (aviation) are placed atop these towers.

(vii) Interference with drainage pattern

As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings which is very small, there is little possibility of affecting drainage pattern.  In the infrequent instances where the drainage is affected, flow will be trained and guided to safe zones.

6.1	Environmental Problems Due to Design

(i) Escape of polluting materials

The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials.

(ii) Explosion/fire hazards

During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires.Fires due to flashover from lines can be a more serious problem in deciduous forest. Since forest involved is mostly degraded and scattered in the proposed project it is not going to be a problem. Apart from this state of art safety instruments are installed in the substations on both the ends so that line gets tripped within milliseconds in case of any fault.

(iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

Adequate measures are taken to re-surface the area where excavation works are done.  Topsoil disturbed during the development of sites will be used to restore the surface of the platform. Infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for tower foundations.

(iv) Environmental aesthetics

Since normal spacing between the towers in case of ± 800 kV HVDC lines is approx. 400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible. POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted grey or green to merge with the background.

(v) Noise/vibration nuisances

The equipments installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian and International standard-7194.  POWERGRID had monitored noise/sound level by measuring it at different places in and around reactor and transformer (at 1 m distance). The noise level was found to be ranging from 75 to 83 well within permissible limit of 85 dB. To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipments are undertaken. For Transmission line, noise/sound level measured in some specific case study has been found to be less than 50 dB.

In addition, plantations of sound absorbing species like Casuarinas, Tamarind, Banyan and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.

(vi)  Blockage of wildlife passage

Since the line is passing through mostly agricultural, wasteland and no forest area or migration path of wild life is getting affected possibility of disturbance to wild life is nil/remote.

6.2	Environmental Problems during Construction Phase

(i)	Uncontrolled silt runoff

The Project involves only small scale excavation for tower foundations at scattered locations that are re-filled with excavated material therefore uncontrolled silt run off is not expected.

(ii)  Nuisance to nearby properties

As already described in preceding Para, during site selection due care is taken to keep the transmission line from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour, therefore, nuisance to the nearby properties if any, is not expected.

(iii)  Interference with utilities and traffic and blockage of access way

Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures.

As and when a transmission line crosses any road/ railways line, the terminal towers are located at sufficient distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

(iv)  Inadequate resurfacing for erosion control

Since proposed line is to be constructed mostly in plain area, erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils, internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained in above Para, adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is avoided during the rainy season.

(v) Inadequate disposition of borrow area

As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area.

(vi)  Protection of Worker's health/safety

Provisions for workers' health and safety will be guided by the Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the Erection Conditions of Contract (ECC), a part of bidding documents.

In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act.

POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Services. POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. This guidelines/checklist includes work permits and safety precautions for work on the transmission lines both during construction and operation (Annexure-IV) and is monitored regularly by site in-charge and corporate Operation Services. In addition, training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act.  Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses will be disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID will conduct awareness building programs on such issues for the construction workers.

6.3  Environmental Problems Resulting from Operation

(i)     O&M Staff/Skills less than acceptable resulting in variety of adverse effects

The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However, in respect of the long distance transmission lines there are monitoring/maintenance offices which are located at various points en-route. Monitoring measures employed include patrolling and thermo-vision scanning. The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, and air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

Exposure to Electro Magnetic Fields (EMF): There have been some concerns about possible increased risk of cancer from exposure to electromagnetic radiation from overhead transmission lines and research has been undertaken into this matter throughout the world. A World Health Organization (WHO) review was held in 1996 as part of the International EMF Project concluded that:

“From the current scientific literature there is no convincing evidence that exposure to radiation field shortens the life span of humans or induces or promotes cancer”.

No EMF exposure guidelines have been drawn in India though exposure guidelines have been drawn up outside India including:

- State Transmission Lines Standards and Guidelines in the USA;

- International Commission on Non-Ionizing  Radiation Protection (ICNIRP);

- US National Council on Radiation; andAmerican Conference on Government and Industrial

Hygienist (ACGIH).

The magnetic field below 400 kV overhead power transmission lines is estimated at 40 µT.  The ICNIRP guidelines present limiting exposure to EMFs, although it adds that the levels quoted should not be interpreted as distinguishing ‘safe’ from ‘unsafe’ EMF levels. The ICNIRP guideline for the general public (up to 24 hours a day) is maximum exposure levels of 1,000 mG or µ100 T. A study carried out by Central Power Research Institute (CPRI) on POWERGRID lines reveals that the EMF about 1 m above ground near a 400 kV single circuit transmission line range from 3-7.2 µT in the ROW. The impact of EMF is also dependent on the duration of exposure and therefore no significant adverse impact is envisaged.  POWERGRID complies with international norms for field strength limits which are certified by Power Technologies Inc, USA. POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard. (Synopsis of the Power Technologies Inc. report enclosed as Annexure-V.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India:

“POWERGRID were the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid does not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

6.4  Environmental Monitoring Program in POWERGRID

Monitoring is a continuous process for POWERGRID projects at all the stages are it the site selection, construction or maintenance. The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan:

6.4.1    Corporate Level

An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and DHQs to carry out environmental and social surveys for new projects.

- Assisting RHQs and DHQs to finalise routes of entire power transmission line considering environmental and social factors that could arise enroute

- Help RHQs and DHQs to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & DHQs on environment and social issues and their management plan.

6.4.2     Regional Level

At its Regional Office POWERGRID has an Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the corporate level and the Divisional Headquarters. The key functions envisaged for ESMC are:

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and DHQs to finalise routes of entire power transmission lines considering the environmental and social factors that

could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and DHQs on various environmental and social aspects.

6.4.3      Site Office

At the Divisional Headquarters level, POWERGRID has made the head of the division responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social Management Team (ESMT). Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalise the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorised Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park, it is generally done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties. A monitoring system (done by the Forest Department) is also in place for compensatory forests established as part of the Project. A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn (Table- 6.1) and will be implemented during various stage of project execution.

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan.

6.5     Critical Environmental Review Criteria

(i)	Loss of irreplaceable resources

The transmission projects do not involve any large scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As the portions of lines in the subject project are passing through only degraded forest (for HVDC Line-11.83 Km and earth electrode Line-14.55 Km) forest land loss would be around 68 m2 or 0.0068 Ha. which is negligible/insignificant. However, compensatory afforestation raised over double the area of degraded forest not only mitigates the impact but also increases the forest cover, thus overall improvement of Environment.

(ii)    Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as no reserve forest is getting  affected thus there is no possibility of endangering/causing extinction of any species.

(iv)  Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

6.6 Public consultation

Public consultation/information is an integral part of the project implementation. Public is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted.

Apart from this, public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting  as  per  Environmental Social Policy  & Procedures of POWERGRID  (ESPP) shall also be carried out during different activities of project cycle. During such consultation the public will be informed about the project in general and in particular about the following:

- Complete project plan (i.e. its route and terminating point and substations, if any, in

between);

- POWERGRID design standards in relation to approved international standards;

- Health impacts in relation to EMF;

- Measures taken to avoid public utilities such as school, hospitals, etc.;

- Other impacts associated with transmission lines and POWERGRID’s approach to

minimizing and solving them;

- Land acquisition details, proposed R&R measures and compensation packages in line with

POWERGRID’s policy;

- Trees and crop compensation process.

The process of such consultation is to be continued during project implementation and even during O&M stage. The progress and proposed plan for Public consultation is as follows:

| S.No. | Activity | Technique | Schedule |
| --- | --- | --- | --- |
| 1. | Walk over Survey | Informal group discussion | Completed |
| 2. | Preliminary / GIS Survey | Group discussion, one to one meeting | Under Progress for details see Annexure-VI. |
| 3. | Detailed/Check survey | Public Meeting at different places (50-100 Km) en-route final route alignment of line | 1-2 such meeting during Jauary’08 to April’08 |
| 4. | Construction Phase | Localized group meeting, Pamphlet/Information brochures, Public display etc. | During entire construction period March-08-09 |
| 5. | O&M Phase | Information brochures, Operating field offices, Response to public enquiries, Press release etc. | Continuous process as and when required. |

6.7    Budget estimate for EMP Implementation

The detailed item wise budget estimate is annexed as Annexure-VII.

Summerised estimated cost of EMP implementation is given below:

| Item | Sub –Item | Total Cost ( Rs. in million) |
| --- | --- | --- |
| Mitigation measures | As prescribed under EMP and IEE As prescribed in CPTD | 196.93 212.56 |
| Implementation & Monitoring activities | As detailed under EMP | 18.17 |
| Independent Audit |  | 1.82 |
| Contingency | 3 % | 12.88 |
| Total Rs. 452.34 Million |  |  |

* Estimated costs are only indicative

6.8       Conclusions

From the above discussion, it would seem that the area is not rich in physical resources. Careful route selection has minimized impact on ecologically/environmentally sensitive areas to the barest minimum as analysed in tables at 4.8 in section 4 at previous chapter and the project may be categorized as category B as per environmental safeguard norms/framework agreed upon, even it may also be considered as category C as impact is insignificant.

Thus, route selected for detailed survey is the most optimum route alignment. The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts while on the one hand underline the need for implementation of the project for overall development of the area and on another hand suggest that a detailed E.I.A. may not be necessary.

Table-6.1 Environment Management Plan

| Project activity /stage | Potential impact | Proposed mitigation measure | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-construction |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Tower location and line alignment selection with respect to nearest dwellings | Setback distances to nearest houses - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformers stated in tender specification - once | POWERGRID | Part of tender specifications for the equipment |
|  |  | Processes, equipment and systems not to use chlorofluorocarbons (CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – once | POWERGRID | Part of tender specifications for the equipment |
|  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design |
| Transmission line design | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Electromagnetic field strength for proposed line design | Line design compliance with relevant standards - once | POWERGRID | Part of detailed alignment survey and design |
| Location of transmission towers and transmission line alignment and design | Impact on water bodies and land | Consideration of tower location at where they could be located to avoid water bodies | Tower location and line alignment selection (distance to water bodies. | Consultation with local authorities and avoiding tower foundation in water bodies | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
|  | Social inequities | Careful route selection to avoid existing settlements | Tower location and line alignment selection (distance to nearest dwellings or social institutions) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
|  |  | Minimise need to acquire agricultural land | Tower location and line alignment selection (distance to agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
| Encroachment into precious ecological areas | Loss of precious ecological values/ damage to precious species | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest designated ecological protection area) | Consultation with local forest authorities to avoid/minimize forest involvement - once | POWERGRID | Part of detailed siting and alignment survey /design |
| Transmission line through forestland | Deforestation and loss of biodiversity | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest protected or reserved forest) | Consultation with local authorities - once | POWERGRID | Part of detailed siting and alignment survey/design |
|  |  | Minimise the need by using existing towers, tall towers and RoW, wherever possible |  | Consultation with local authorities and design engineers - once |  |  |
|  |  | Obtain statutory clearances from the Government | Statutory approvals from Government | Compliance with regulations – once for each subproject |  |  |
| Encroachment into farmland | Loss of agricultural productivity | Use existing tower footings/towers wherever possible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
|  |  | Avoid siting new towers on farmland wherever feasible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once |  | Part of detailed siting and alignment survey /design |
|  |  | Farmers compensated for any permanent loss of productive land | Design of Implementation of Crop Compensation (based on affected area) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  | Farmers/landowners compensated for significant trees that need to be trimmed/ removed along RoW. | Design of Implementation of Tree compensation (estimated area to be trimmed/removed) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  |  | Statutory approvals for tree trimming /removal | Compliance with regulations – once for each subproject |  | Part of detailed siting and alignment survey /design |
| Interference with drainage patterns/Irrigation channels | Flooding hazards/loss of agricultural production | Appropriate siting of towers to avoid channel interference | Tower location and line alignment selection (distance to nearest flood zone) | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
| Construction |  |  |  |  |  |  |
| Equipment layout and installation | Noise and vibrations | Construction techniques and machinery selection seeking to minimize ground disturbance. | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance - once at the start of each construction phase | POWERGRID (Contractor through contract provisions) | Construction period |
| Physical construction | Disturbed farming activity | Construction activities on cropping land timed to avoid disturbance of field crops (within one month of harvest wherever possible). | Timing of start of construction | Crop disturbance –Post harvest as soon as possible but before next crop - once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Mechanized construction | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment – estimated noise emissions | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Noise, vibration, equipment wear and tear | Turning off plant not in use. | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction of roads for accessibility | Increase in airborne dust particles | Existing roads and tracks used for construction and maintenance access to the line wherever possible. | Access roads, routes (length and width of new access roads to be constructed) | Use of established roads wherever possible - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Temporary blockage of utilities | Overflows, reduced discharge | Temporary placement of fill in drains/canals not permitted. | Temporary fill placement (m3) | Absence of fill in sensitive drainage areas - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Marking of vegetation to be removed prior to clearance, and strict control on clearing activities to ensure minimal clearance. | Vegetation marking and clearance control (area in m2) | Clearance strictly limited to target vegetation - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Trimming/cutting of trees within RoW | Fire hazards | Trees allowed growing up to a height within the RoW by maintaining adequate clearance between the top of tree and the conductor as per the regulations. | Species-specific tree retention as approved by statutory authorities (average and maximum tree height at maturity, in meters) | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Loss of vegetation and deforestation | Trees that can survive pruning to comply should be pruned instead of cleared. | Species-specific tree retention as approved by statutory authorities | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Felled trees and other cleared or pruned vegetation to be disposed of as authorized by the statutory bodies. | Disposal of cleared vegetation as approved by the statutory authorities (area cleared in m2) | Use or intended use of vegetation as approved by the statutory authorities – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Wood/vegetation harvesting | Loss of vegetation and deforestation | Construction workers prohibited from harvesting wood in the project area during their employment, (apart from locally employed staff continuing current legal activities). | Illegal wood /vegetation harvesting (area in m2, number of incidents reported) | Complaints by local people or other evidence of illegal harvesting - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Surplus earthwork/soil | Runoff to cause water pollution, solid waste disposal | Soil excavated from tower footings disposed of by placement along roadsides, or at nearby house blocks if requested by landowners. | Soil disposal locations and volume (m3) | Acceptable soil disposal sites - every 2 weeks | POWER GRID (Contractor through contract provisions) | Construction period |
| Tower construction – disposal of surplus earthwork/fill | Waste disposal | Excess fill from tower foundation excavation disposed of next to roads or around houses, in agreement with the local community or landowner. | Location and amount (m3)of fill disposal | Appropriate fill disposal locations - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Storage of chemicals and materials | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports (type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction schedules | Noise nuisance to neighbouring properties | Construction activities only undertaken during the day and local communities informed of the construction schedule. | Timing of construction (noise emissions, [dB(a)]) | Daytime construction only - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Provision of facilities for construction workers | Contamination of receptors (land, water, air) | Construction workforce facilities to include proper sanitation, water supply and waste disposal facilities. | Amenities for Workforce facilities | Presence of proper sanitation, water supply and waste disposal facilities - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period |
| Encroachment into farmland | Loss of agricultural productivity | Use existing access roads wherever possible | Usage of existing utilities | Complaints received by local people /authorities - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Ensure existing irrigation facilities are maintained in working condition | Status of existing facilities |  |  |  |
|  |  | Protect /preserve topsoil and reinstate after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  |  | Repair /reinstate damaged bunds etc after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  | Social inequities | Compensation for temporary loss in agricultural production | Implementation of Crop compensation (amount paid, dates, etc.) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Uncontrolled erosion/silt runoff | Soil loss, downstream siltation; | Need for access tracks minimised, use of existing roads. | Design basis and construction procedures (suspended solids in receiving waters; area re-vegetated in m2; amount of bunds constructed [length in meter, area in m2, or volume in m3]) | Incorporating good design and construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Limit site clearing to work areas |  |  |  |  |
|  |  | Regeneration of vegetation to stabilise works areas on completion (where applicable) |  |  |  |  |
|  |  | Avoidance of excavation in wet season |  |  |  |  |
|  |  | Water courses protected from siltation through use of bunds and sediment ponds |  |  |  |  |
| Nuisance to nearby properties | Losses to neighbouring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | As much as possible existing access ways will be used. | Design basis and layout | Incorporating good design engineering practices – once for each site |  |  |
|  |  | Productive land will be reinstated following completion of construction | Reinstatement of land status (area affected, m2) | Consultation with affected parties – twice – immediately after completion of construction and after the first harvest |  |  |
|  | Social inequities | Compensation will be paid for loss of production, if any. | Implementation of Tree/Crop compensation (amount paid) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Health and safety | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work days caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  |
|  |  | Contractor to arrange for health and safety training sessions |  |  |  |  |
| Inadequate construction stage monitoring | Likely to maximise damages | Training of POWERGRID environmental monitoring personnel | Training schedules | Number of programs attended by each person – once a year | POWERGRID | Routinely throughout construction period |
|  |  | Implementation of effective environmental monitoring and reporting system using checklist of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof. | Submission of duly completed checklists of all contracts for each site - once |  |  |
|  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures. | Compliance report related to environmental aspects for the contract | Submission of duly completed compliance report for each contract - once |  |  |
| Operation and Maintenance |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Compliance with setback distances (“as-built” diagrams) | Setback distances to nearest houses – once in quarter | POWERGRID | During operations |
| Oil spillage | Contamination of land/nearby water bodies | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks. | Substatio bunding (Oil sump) (“as-built” diagrams) | Bunding (Oil sump) capacity and permeability - once | POWERGRID | During operations |
| Inadequate provision of staff/workers health and safety during operations | Injury and sickness of staff /workers | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |
|  |  | Safety awareness raising for staff. | Training/awareness programs and mock drills | Number of programs and percent of staff /workers covered – once each year |  |  |
|  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |
| Electric Shock Hazards | Injury/mortality to staff and public | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |
|  |  | Security fences around substations | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |
|  |  | Barriers to prevent climbing on/dismantling of transmission towers | Maintenance of barriers |  |  |  |
|  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |
|  |  | Electricity safety awareness raising in project areas | Training /awareness programs and mock drills for all concerned parties | Number of programs and percent of total persons covered – once each year |  |  |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | Processes, equipment and systems using cholofluorocarbons (CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government. | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |
| Transmission line maintenance | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Required ground clearance (meters) | Ground clearance - once | POWERGRID | Operations |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels (dB(a)) | Noise levels at boundary nearest to properties and consultation with affected parties if any - once | POWERGRID | Operations |