SECTION I:PROJECT DESCRIPTION (Grid Strengthening for Kerala–II )

- BACKGROUND:

Southern regional grid covers the Regional as well as State grids of Andhra Pradesh, Karnataka, Tamil Nadu, Kerala and UT of Pondicherry. In the past few years the Southern regional grid had witnessed remarkable growth as compared to other regional grids of the country. For example, in the last two years the region has experienced 15% growth in installed capacity i.e. installed capacity has grown from 24734 MW in April 2001 to 28501MW in April 2003.

Further, with the growth in installed capacity and power demand, the existing transmission arrangement around the major load centers are required to be strengthened by establishment of new 400/200 kV substations, some of which are already taken up by State sectors. These substations are required to be connected to the main grid by providing the missing links in the vicinity of new substation. POWERGRID is responsible for construction, operation and maintenance of transmission lines required for (i) evacuation of power from central sector and mega IPPs, (ii) intra-regional grid strengthening projects and (iii) inter-regional transmission system, for the benefit of beneficiary States. POWERGRID is also responsible for implementation of Unified Load Despatch and Communication (ULDC) facilities.

1.1	BENEFITS OF THE PROJECT:

Kerala is one of the constituent State of Southern Region located along the western coast close to peninsular portion of the Indian geography. The present peak demand of Kerala is of the order of 2,450 MW which, as per the 16th EPS periodically published by CEA, is likely to grow to about 4,300 MW by the end of 10th Plan (year 2006-07) and about 6,400 MW by the end of 11th Plan (year 2011-2012). The demand of Kerala is primarily met through its share of about  1250 MW from Central Sector generations (like Ramagundam STPP, Neyveli TS, Kaiga and MAPS) and its own State Sector generations with installed capacity of about 2,619 MW. Here, it is pertinent to mention that State generations mainly (about 70%) comprise of hydro generations that are subjected to vagaries of Monsoon. Therefore, adequate transmission system is required to be established to enable additional power import from the grid during the poor monsoon years. Kerala presently draws its central share from a lone 400 kV double circuit line from Udumalpet to North Thrissur. Further, additional 400 kV in-feed is being created for Kerala through Madurai – Trivandrum 400 kV D/C line. Thus, Kerala is being provided power through radial points at its different parts and as such there is no inter-connection amongst these feed points.

The proposed grid strengthening scheme is aimed at creating transmission highway to meet the long-term transmission requirement of Kerala in the Southern Region.

1.2  PROJECT DESCRIPTION

Objective

The objective of the project is to improve reliability through strengthening of grid for smooth operation of the southern region grid particularly of Kerala.

Project Highlights

| a) | Project Name | : | Grid Strengthening for Kerala–II of Southern Region Grid |
| --- | --- | --- | --- |
| b) | Location | : | Kerala |
| c) | Beneficiary States | : | Andhra Pradesh, Karnataka, Kerala, Tamil Nadu and Pondicherry |
| d) | Project Cost | : | Rs. 248 Crores |

Scope of work

- Transmission Line:

    - Muvattupuzha (East Cochin)  - North Trichur 400kV D/C , 80 km

- Substations:

i.     Setting up of 400/220 kV Substation at Muvattupuzha(East Cochin)

ii.    Bay extension at existing North-Trichur substation

A power map showing the transmission grid of Southern Region highlighting the above scope of works is placed as Annexure-1.

SECTION II: BASE LINE DATA

2.0  	The project is to be located in the southern State of Kerala in India. The basic details of the area under project are given below:

2.1	Kerala:

Kerala has a geographic area of 3.89 million ha.  It lies on the west coast between lat.8 17’ and 12 47’N  and long. 74 52’ and 77 24’E.  Physiographically, the state can be divided into the coastal, the midland and the highland zones.

Coastal Belt

This strip near the coastline, is comparatively plain. Extensive paddy fields, thick groves of coconut trees and picturesque backwaters, interconnected with canals and rivers, are the features of this region. In the southern and northern parts of the state, the coastal belt also has some hills and valleys.

Midland Plains

In this central region, the hills are not very steep and the valleys are wide. The valleys have been developed as paddy fields and the elevated lands and hill slopes, converted into estates of rubber, fruit trees and other cash crops like pepper, tapioca, etc.

Hills & Valleys

This strip of land on the eastern edge, close to the Ghats, comprises of steep mountains and deep valleys, covered with dense forests. Almost all the rivers of the state originate here. Tea and coffee estates have cropped up in the high ranges during the last two centuries.

The Western Ghats, bordering the eastern boundary of the State, form an almost continuous mountain wall, except near Palakkad where there is a natural mountain pass known as the Palakkad Gap. The average elevation of the Ghats is about 1500 meters above sea level, occasionally soaring to peaks of 2000 to 2500 m. From the Ghats, the land slopes to west on to the plains, into an unbroken coastline.  Following is the detailed land use pattern of the Kerala:

Land use Pattern

| Land use | Area in ‘000 ha | Percentage |
| --- | --- | --- |
| Total geographical area | 3,886 |  |
| Reporting area for land utilization | 3,886 | 100.00 |
| Forests | 1,122 | 28.9 |
| Not available for cultivation | 359 | 9.24 |
| Permanent pasture & other grazing lands | 1 | 0.02 |
| Land under misc.tree crops & groves | 23 | 0.59 |
| Cultruable wasteland | 68 | 1.75 |
| Fallow land other than current fallows | 29 | 0.75 |
| Current fallows | 56 | 1.44 |
| Net area sown | 2,269 | 58.39 |

Sashtam kotta and Ashtamudi, located in Quilon district, with an area of 375 ha and 3200 ha, respectively, are important wetlands of national importance.  

Population Data :

Kerala has 31,838,619 people according to the 2001 census, which is nearly 3.44 per cent of the country's population. Her population density is 819 persons per sq.km., the third in India and clocks at thrice the national average. The sex-ratio recorded in this census is 1058 females per 1000 males. The state has a unique place in the literacy map of India, with a literacy rate of 90.92%.According to 2001, the birth rate is 18.3 and death rate is 6.4. The coastal belt is thickly populated, and it is the hilly tracts where the density of population is the lowest.

| Population & General Data |  |
| --- | --- |
| Area (in Sq.km) | 38,863 |
| Population | 3,18,41,374 |
| Males | 15,468,614 |
| Females | 16,372,760 |
| Sex ratio : Females/1000 | 1058 |
| Literacy rate | 90.86%; Male 94.24%; Female 87.72% |
| Density of Population | 819 |
| Per Capita Income 2002-03 | Rs. 25,764 |

Climate of the state is warm and humid with temperatures ranging from 19.80 C to 36.70 C. Generally March and April are the hottest and December and January are the coldest months.  The diversity of rainfall makes it very cold high up in the mountains, while lower down at an elevation of 1000 to 1500 meters, a bracing climate is experienced in the plains and lowlands.

The South  West Monsoon contributes the major part of rainfall as compared to the North East Monsoon.  It is reported that the normal annual rainfall recorded at the hill ranges is the highest followed by the uplands and the coastal plains. Kerala receives a fairly good annual rainfall varying from 1250 to 5000 mm.The normal annual rainfall of Kerala is 3107 mm.(national average is 1197mm).  Although, quantum wise the rainfall received is fairly high, its distribution shows wide temporal and spatial variations. On an average the number of rainy days is in the range of 120-140 in a year. The highest rainfall occurs in the high ranges of Idukki district where it exceeds 500 cm. The most prevalent wind speeds fall in the range 12 -19 Km/h during evening hours, where as those in the morning hours are in the range of 6 - 11 Km/h. higher wind speed (i.e. 19 Km/h) usually occurs in the evening hours particularly in the winter and also early part of summer.

Forest Resources

The recorded forest area of the state is 1.12 million ha, which constitutes 28.9% of the land area (Map-1).  Out of this, Reserved Forest constitutes 84.22% and the remaining 15.78% has been declared as Vested Forest under the Kerala Private Froests (Vesting & Assignment) Act, 1971. There are seven forest types viz. Tropical Wet Evergreen, Tropical Semi-Evergreen, Tropical Moist Deciduous, Tropical Dry Deciduous, Subtropical Broad leaved Hill, Montane Wet Temperate and Littoral and Swamp forests.

There are 3 National Parks and 12 Wildlife Sanctuaries spread over 0.27 million ha, which constitute 6.90% of the geographic area of the state. Periyar tiger reserve is located in the state, A part of the Nilgiri Biosphere Reserve falls in Kerala.

The proposed lines are passing through two districts of Kerala details of forest resources available in the project area are as follows:

Area in Km2

| District | Geographical Area | Dense Forest | Open forest | Total | % |
| --- | --- | --- | --- | --- | --- |
| Ernakulam | 2407 | 433 | 128 | 561 | 23.31 |
| Trichur | 3032 | 549 | 344 | 893 | 29.45 |

Mineral Resources

Several valuable minerals occur in many parts of the state. The beach sands in Kollam are rich in heavy minerals like Monozite, Ilmenite, Rutile, Zircon and Silimanite. China clay or Kaolin, suitable for the manufacture of porcelain is found in many parts of the state. An inferior type of clay in many places and is being used for the manufacture of tiles and bricks. Deposits of graphite also occur in many parts of the state.

It has the finest China clay in the country, about 80 million tonnes, which hold out possibilities for the manufacture of high tension insulators, sanitary wares, crockery and glazed tiles. Deposits of silica and quarts sad are estimated at around 75 million tonnes. Fire clay amount to about 12 million tonnes. There are 79 million tonnes of iron ores, 25 million tonnes of limestones, 11 million tonnes of bauxite, 35 million tonnes of ilmenite, three million tonnes of rutile, one million tonnes of monozite and 0.7 million tonnes of borophite. The tremendous industrial possibilities of most of these rare minerals have not yet been fully harnessed. The atomic era has put the state into prominence by virtue of its large deposits of Thorium-bearing mineral, Monozite.

Backwaters: The backwaters are a peculiar feature of the state. Canals link the lakes and backwaters to facilitate an uninterrupted inland water navigation system from Thiruvananthapuram to Vadakara, a distance of 450 kms. The Vembanad lake stretching from Alappuzha to Kochi is the biggest water basin and is over 200 sq.kms. in area. Water-logged Kuttanad alone forms more than 20 per cent of India's total length of waterways.

Rivers: There are 44 rivers in the state, of which 41 originate from the Western Ghats and flow towards west into the Arabian sea. Three tributaries of the river Cauvery originate in Kerala and flow east into the neighbouring States. These rivers and streams flowing down from the Western Ghats either empty themselves in to the backwaters in the coastal area or directly into the Arabian Sea. As the Western Ghats are nowhere more than 120 kms from the sea, all these rivers are comparatively short. The presence of a large number of rivers has made Kerala rich in water resources which are being harnessed for power generation and irrigation.

The pressure on land in Kerala is the highest in India and the state's economy continues to be dominantly agrarian. The annual per capita income in state at current prices has risen to Rs. 19,461 in the year 1999-2000 from Rs 5,065 in 1992-93. The area has a high density of population and has been experiencing a high growth rate.  The economy being predominantly agricultural is also reflected in the urban area, which exhibit semi-urban characteristics with substantial dependency on primary activities.  Although about 84% of the area is under cultivation, workers in agricultural activities comprise about 49% of the work force.  The scope of bringing more land under agriculture is limited. A characteristic of the occupational pattern in the region is the prevalence of seasonal unemployment especially in agriculture and fishing due to dependence on seasonality in cropping and fish breeding. Also, the high level of migration from the region owing to economic reasons is a sharp pointer to the paucity of employment opportunities.

Agriculture is the main avenue of employment, the major crops being paddy, coconut, vegetables, tapioca, bananas and sugarcane.  Single cropping is the main practice though double cropping is also observed in the some area.  Agricultural lands are mostly observed adjoining the houses.  30% of the agriculturists own the land they cultivate.  The average size of the holding is 0.30ha.  During the rainy season, water-logging is a common problem and water has to be pumped out from the low-lying areas.

Fishing is another important occupation in Allappuzha, Ambalapuzha, Purrakkad and Thrikunnapuzha Panchyat of the coastal belt.  Country boats and engine-powered rowing boats are used by the fishermen.  The trawlers in the region are owned by large industrial units, who in turn are able to process and market the output to the larger market outside.  Although, fishermen's co-operatives exist but due to their unorganised nature and the benefits accruing from big catch of fish are drained out of the region by trawlers, income from fishing is very low.  The catch consists mostly of fish, which fetch little remuneration as compared to prawns.

There is no major industry in this area.  However, certain small scale and household industries, generally non-polluting in nature are in operation. The total number of registered SSI units in the state as on 31-3-1998 was 1.8 lakh. This sector provides employment to more than 909,859 people in Kerala. Considering the importance of this sector, Government of Kerala have announced a new Industrial Policy in 1998, which provides very attractive package of incentives to the entrepreneurs. There are 511 manufacturing industrial enterprises in the State. Out of these19in the central sector, 62 in the State sector, 16 in the co-operative sector, 29 in the joint sector and 385 in the private sector.

Power Scenario:

The presence of a large number of rivers has made Kerala rich in water resources which are being harnessed for power generation and irrigation. There are 23 Hydel Projects owned by KSEB and 2 by private agencies. The Wind Farm located at Kanjikkode with an installed capacity of 2.025 MW is the only one utilising the non-conventional energy source. The Brahmapuram and Kozhikode Diesel Power Plants with an installed capacity of 106.6 MW and 128 MW uses LSHS as fuel and are owned by KSEB.

The Kerala State Electricity Board has an installed capacity of 2065.70 MW on its own with another 570.016 MW contributed by NTPC and Private Sector producers taking the total installed capacity of State to 2637.74 MW. The other plants that are catering to the energy demands of the state are the Kayamkulam plant of NTPC with an installed capacity of 359.58 MW and the BSES, Kochi Plant with an installed capacity of 157.00 MW. The 20.44 MW plant of KPCL, Kasaragod is the latest entrant in the IPP segment of generation. The balance is met through import of power from the Central Sector.

The grid is connected to the Southern Region Transmission system through one 400kV double circuit. Another 400kV double circuit at Thiruvananthapuram is expected to be commissioned soon. There are 5 major inter-state transmission lines. The major substations include one 400 KV sub-station, and twelve 220 KV substations and four 220kV substations under construction. The main grid comprises of the 220 KV system. KSE Board is the sole distributor of the Electrical Energy for the state of Kerala except Thrissur Corporation and Munnar where the distribution is managed by licensees. In Thrissur, City Corporation and in Munnar M/s Tata Tea Ltd are the licensees.

SECTION III:   POLICY , LEGAL & REGULATORY  FRAMEWORK

3.0     POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  In addition, Funding Agencies’ directives and guidelines with respect to environmental and social considerations.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and guidelines of Funding Agencies.

- 3.1     ENVIRONMENTAL

3.1.1 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under The Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under The Forest (Conservation) Act, 1980

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded, hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF.    Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/reconditioner or at the designated collection centers – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)  Amendment Rules, 2003:

MOEF vide its notification dt. 20th May, 2003 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned/sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organisation in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalising the route alignment.

    - FUNDING AGENCIES:

- WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM)- F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.3   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- Applicable Legislations

- 3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement

- on Environment and Development, 1992

- Policy statement for Abatement of pollution, 1992

- 3.2.0   SOCIAL

- 3.2.1    MANDATORY REQUIREMENTS (NATIONAL)

- National Policy on Resettlement and Rehabilitation for Project Affected Families: Ministry of Rural Development, Government of India has notified a National policy on R&R for  PAFs in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the assetless rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

- When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.3    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- National and State-wide Laws and Policies Relating

to Land Acquisition and Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.4   RELEVANT POLICIES

- The Orissa Resettlement and Rehabilitation of Project Affected Persons Policy, 1994 (water resources projects)

- Resettlement & Rehabilitation Policy – NTPC

- Definition of Projects affected persons (PAPs) Nature and extent of entitlements.

# SECTION-IV: POWERGRID APPROACH FOR ROUTE SELSCTION

- 4.0     ROUTE SELECTION - (ASSESSMENT & MANAGEMENT PROCESS)

- At the system planning stage itself one of the factors that govern the evolution of system is the possible infringement with the forest. Wherever such infringements are substantial, different alternative options are considered. The route/ site selection criteria followed by POWERGRID is detailed below:

- While identifying the transmission system for a generation project or as a part of National Power Grid, preliminary route selection is done by POWERGRID based on the Top sheets of Survey of India and Forest Atlas (Govt. of India's Publication).  During route alignment all possible efforts are made to avoid the forest area involvement completely or to keep it to the barest minimum, whenever it becomes unavoidable due to the geography of terrain or heavy cost involved in avoiding it.

- 4.1.1  STUDY OF ALTERNATIVES

- POWERGRID approach towards Route selection

- For selection of optimum route, the following points are taken into consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

- Any monument of cultural or historical importance is not affected by the route of the transmission line.

- The proposed route of transmission line does not create any threat to the survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

- In addition, care is also taken to avoid National parks and sanctuaries and any other forest area rich in wild life.

- Keeping above in mind the routes of proposed two lines under Grid Strengthening for Kerala–II of Southern Region Transmission system has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps, satellite imageries etc. to arrive at most optimum route which can be taken up for detailed survey and assessment of environmental & social impacts for their proper management.

    - EVALUATION OF ROUTE ALIGNMENT ALTERNATIVES OF PROPOSED     TRANSMISSION SYSTEM:

a)  400 KV  D/C  Muvattupuzha (East Cochin – North Trichur  Line:

- Three different alignments (Map-2) were studied with the help of published data/maps and walkover/GIS survey to arrive at most optimum route for detailed survey. The comparative details of these three alternatives are as follows:

| S. No. | Description | Alignment I | Alignment II | Alignment III |
| --- | --- | --- | --- | --- |
| 1. | Route Particulars |  |  |  |
| i. | Route length | 78 Kms | 80 Kms | 86 Kms |
| ii. | Terrain | Passing through plain and undulated terrain with small gentle slopes | Passing through plain and undulated terrain with small gentle slopes | Passing through plain and undulated terrain with small gentle slopes |
| 2. | Env. Details |  |  |  |
| i) | Major town in alignment | Ernakulam, Trichur | Ernakulam, Trichur | Ernakulam, Trichur |
| ii. | House with Row | Due to fast development, few permanent buildings are encountered within the ROW. Due care has been taken to provide sufficient clearances to all such permanent buildings without affecting any displacement | Due to fast development, few permanent buildings are encountered within the ROW. Due care has been taken to provide sufficient clearances to all such permanent buildings without affecting any displacement | Due to fast development, few permanent buildings are encountered within the ROW. Due care has been taken to provide sufficient clearances to all such permanent buildings without affecting any displacement |
| iii. | Forest in Km./Ha. | 0. 250 Km. 1.15 ha. (If complete ROW of 46 m is taken), 0.275 ha taking 11 m ROW (35 m existing) | NIL | NIL |
| a. | Type of forest | RF | NA | NA |
| b. | Density of forest | 0.4 | NA | NA |
| c. | Type of fauna & flora | Fauna: Deers, fox, wolf, squirrels, reptiles, monkeys etc. Flora: Mango, Coconut, Aracanut Bamboo, Teak , neem, jack fruit, rubber etc. | NA | NA |
| d. | Endangered species if any | NIL | NIL | NIL |
| e. | Historical/cultural measurement | NIL | NIL | NIL |
| f. | Any other relevant information | As the line traverses in Kerala. There will be severe ROW problems during construction. Around 10 Km of line will pass through rubber/coconut /.banana plantation. To avoid / minimize ROW problem it is proposed to utilize KSEB’s Idukki-Trichur 220 KV existing line corridor for 58 Km. | As the line is traversing in Kerala. There will be severe ROW problems during construction. Around 15-20 Km of line will pass through rubber / coconut plantation. | As the line is traversing in Kerala. There will be severe ROW problems during construction. Around 5-10 Km of line will pass through rubber / coconut plantation. Apart from this some portion of line is passing through densly populated area and required corridor may not be available. |
| 3 i). | Compensation Crop | Rs. 1558 lakh | Rs.2810 lakhs | Rs.3020 lakhs |
| ii) | Forest | 14 lakh including NPV @ Rs. 9.2 lakh / ha. | NA | NA |
| 4 | No. of crossing |  |  |  |
| i. | Railway crossing | NIL | NIL | NIL |
| ii. | Trans. .Lines 36 kV and above | 8 | 8 | 9 |
| iii. | River crossing etc. | 2 (Periyar & Chalakudy) | 2 (Periyar & Chalakudy) | 4 (Periyar , Chalakudy & 2 other streams) |
| 5 | Construction problem | Only ROW problem may arise and rainy season during May to Aug. and from Oct. to Dec. Since the total alignment is in Kerala state, there may be resistance during construction from the local public. Problem in plantation area is also anticipated due to resistance and fruiting season. | Only ROW problem may arise and rainy season during may to Aug. and from Oct. to De. Since the total alignment is in Kerala state, there may be resistance during construction from the local public. Problem in plantation area is also anticipated due to resistance and fruiting season. | Only ROW problem may arise and rainy season during may to Aug. and from Oct. to De. Since the total alignment is in Kerala state, there may be resistance during construction from the local public. Problem in plantation area is also anticipated due to resistance and fruiting season. |
| 6 | O&M problem | Minimum since existing corridor of 220 KV line is used for major portion of line. | Moderate | Moderate |
| 7 | Overall Remarks | This alignment is most preferable | This alignment has more ROW problem as complete line is passing through private land and may delay the line completion. Hence not suitable. | This alignment has more ROW problem as complete line is passing through private land and may delay the line completion. Line length is also more hence rejected. |

It may be seen from above that Alternative-I is the most optimum route though it  involve small portion of ecological sensitive areas like forest that due to following of existing corridor. However, that too has been further minimized by using existing corridor whereas other alternatives have severe ROW problem and  may pose as limiting factor for timely completion of line construction as well as O&M may also be problematic. Hence alternative –I has been recommended for detailed survey.

# SECTION-V: SCREENING OF POTENTIAL ENVIRONMENT IMPACT AND ITS

# MANAGEMENT

- 5.0	Impact Due to Project Location and Design

Environmental impact of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. In order to get latest information and further optimization of route  modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS in route selection result in access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. In the instant project also these techniques are used and detail survey using GIS/GPS is under progress.  Although, all possible measures have been taken during the finalization of route alignment for the proposed transmission line but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to specific environment review criteria based on preliminary survey are as follows:

- (i)	Resettlement

- As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be seen from the above description of proposed route alignment and also keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

- The proposed project envisages construction of  one new and extension of one existing substation. Construction of new substation involves acquisition of 34.27 acres (13.877 ha.) of land in Kunnathunad village of Ernakulam district which shall affect around 94 people. A detailed social assessment of these affected population shall be carried out as per POWERGRID policy and based on this a detailed Rehabilitation Action Plan (RAP/RP) shall be prepared in line with agreed entitlement matrix for rehabilitation/restoration of livelihood of affected people. As far as extension of Trichur substation is concerned, land for extension of bays is already available and no fresh land acquisition is involved hence, R&R issues shall not be involved in case of Trichur substation. However, social issues shall be dealt separately.

- (ii)	 Land value depreciation

- Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

- (iii) Historical/cultural monuments/value

- As per the POWERGRID’s policy of route selection only that route alignment is finalized this avoids all the historical and cultural monuments. As per the preliminary assessment carried out during finalization of route alignment in consultation with State revenue authorities and ASI, no such monuments are coming in the proposed route alignment.

- (iv)	Encroachment into precious ecological areas

- As already explained all precautions has been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries. In spite of this it was not possible to avoid forest completely in this project though we were able to avoid National park and Sanctuaries completely. However, the routes of proposed transmission line is so finalised that it affect minimum forest area.   It may be  seen  from  the  above  referred  tables  that  out of total transmission line length of about 78 Km about 0.250 Km (0.32% only) length shall  pass  through  forest  land  consisting  of 1.15 Ha. forest area  in  the  states  of  Kerala. In order to minimize environmental impact and to reduce the tree felling even in the forest area, existing and cleared corridor of 220 KV Idukki -Trichur  KSEB line is utilized by dismantling this line and making a new 400 KV D/C line the existing corridor thereby the actual requirement of forest presently is restricted to only 11m (0.275 ha.) instead of 46 m. Though it involve huge capital expenditure on account of dismantling and compensation to KSEB but will minimize tree felling and clearing of natural forest to the barest minimum.   Most  of  the forests to  be  traversed  by  the  lines  are  already   degraded  and the  wildlife species  present  are  those  who  have  been  adapted  to  open  or  disturbed habitat. Therefore with provision  of  Compensatory  Afforestation  the  overall  forest status  will  in  many  cases  improve. Nonetheless, to  mitigate  losses  to  existing forests, clearing  of  the  transmission  line  Right-of-way  will  be done  under supervision  of  Forest  Department, and  some  low  canopy  seed  trees  and  shrubs may  be  kept  intact  if  they  do not  interfere  with  tower  erection and  line installation. The wood will be sold by the Forest Department, who will also retain the sale proceeds. Three-meter wide strips of land under each conductor will be cleared and maintained as maintenance rows, but the remaining land will be allowed to regenerate. Lopping of trees to maintain line clearance will be done under the direction of Forest Department. POWERGRID will provide construction crews with fuel wood or alternative fuels as a precaution against collection of fuel wood from nearby forest.

- Transmission lines can serve as new access routes into previously inaccessible or poorly accessible forests, thereby accelerating forest and wildlife loss. In such cases, POWERGRID cannot take action itself, but local Forest Department personnel will normally assess the dangers and take appropriate action, such as establishing guard  stations  at  the  entrance  to  the  forest.  Given the already easy access and degraded conditions at the proposed projects sites, this problem is not expected to be encountered.  Nonetheless, POWERGRID staff will report to the Forest Department any noticeable encroachment induced by the Projects.

- (v)	Encroachment into other valuable lands

- Impacts on agricultural land will be restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1). The proposed project envisages to construct 78 Km of line which involve approximately 216 towers.  Construction of 216 towers will result in loss of approx. 216 sq. m. or 0.021 ha. of  land  which is quite negligible and may not result in any adverse impact on farm land.

- In areas where lines will traverse agricultural land, compensation will be paid to owners for any crop damage incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation.  Revenue department after evaluating the loss due to construction activity and productivity of land arrives at the compensation cost which is paid to farmer. Based on past experience crop compensation cost is around Rs. 35 lakh per Km in Kerala . Accordingly provision of Rs. 2056 lakh towards crop compensation is made in the cost estimate of the project to meet the requirement during construction. Agricultural activities will be allowed to continue following the construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner's satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss of products from the tree over an eight year period (for fruit bearing trees). Agricultural lands under private ownership will be identified, and in accordance with normal POWERGRID procedures compensation will be paid to the affected villagers.

- (vi)	 Interference with other utilities and traffic

- As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission lines affect nearby telecommunication circuits by causing electrical interference.  A standing committee -- Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like rerouting of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

- The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50000/- per km for POWERGRID.

- Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission lines pass by the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights are placed atop these towers.

- (vii) Interference with drainage pattern

- As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings which is very small, there is little possibility of affecting drainage pattern.  In the infrequent instances where the drainage is affected, flow will be trained and guided to safe zones.

- 5.1	Environmental Problems Due to Design

- (i)	Escape of polluting materials

- The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials.

- (ii)	Explosion/fire hazards

- During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires.

- Fires due to flashover from lines can be a more serious problem in deciduous forest. Since forest involved is mostly semi-evergreen in the proposed project it is not going to be a problem. Apart from this state of art safety instruments are installed in the substations on both the ends so that line gets tripped within milliseconds in case of fault.

- (iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

- Adequate measures are taken to re-surface the area where excavation works are done.  Topsoil disturbed during the development of sites will be used to restore the surface of the platform. Infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for tower foundations.

- (iv)	 Environmental aesthetics

- Since spacing between the towers in case of 400 KV lines is approx. 400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible.   POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted grey or green to merge with the background.

- (v)	 Noise/vibration nuisances

- The equipments installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian and International standard- 7194.  POWERGRID had monitored noise/sound level by measuring it at different places in and around reactor and transformer (at 1 m distance). The noise level was found to be ranging from 75 to 83 well within permissible limit of 85 dB. To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipments are undertaken.

- In addition, plantations of sound absorbing species like Casuarinas, Tamarind, Banyan and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.

- (vi)  Blockage of wildlife passage

- Since the line is passing through mostly agricultural, wasteland and no forest area or migration path of wild life is getting affected possibility of disturbance to wild life are nil/remote.

- 5.2	Environmental Problems During Construction Phase

- (i)	Uncontrolled silt runoff

- The Project involves only small scale excavation for tower foundations at scattered locations that are re-filled with excavated material therefore uncontrolled silt run off is not expected.

- (ii)  Nuisance to nearby properties

- As already described in preceding paras, during site selection due care is taken to keep the transmission line and substations away from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour therefore nuisance to the nearby properties if any, is not expected.

- (iii)  Interference with utilities and traffic and blockage of access way

- Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures.

- As and when a transmission line crosses any road/ railways line, the terminal towers are located at sufficient distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

- (iv)	Inadequate resurfacing for erosion control

- Since proposed line is to be constructed mostly in plain area, erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils, internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained in above paras, adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is generally undertaken outside the rainy season.

- (v)	Inadequate disposition of borrow area

- As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area.

- (vi)  Protection of Worker's health/safety

- Provisions for workers' health and safety will be guided by the Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the Erection Conditions of Contract (ECC), a part of bidding documents.

- In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act.

POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Deptt. POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. These guidelines/checklist include work permits and safety precautions for work on the transmission lines both during construction and operation (Annexure-2) and is monitored regularly by site in-charge and corporate Operation Services. In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act.  Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses will be disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID will conduct awareness building programs on such issues for the construction workers.

- 5.3	Environmental Problems Resulting from Operation

- (i)     O&M Staff/Skills less than acceptable resulting in variety of adverse effects

- The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However in respect of the long distance transmission lines, there are monitoring offices which  are located at various  points en-route. Monitoring measures employed include patrolling and thermo-vision scanning. The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

- A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

Exposure to Electro Magnetic Fields (EMF)There have been some concerns about possible increased risk of cancer from exposure to electromagnetic radiation from overhead transmission lines and research has been undertaken into this matter throughout the world. A World Health Organization (WHO) review also in 1996 held as part of the International EMF Project concluded that:

“From the current scientific literature there is no convincing evidence that exposure to radiation field shortens the life span of humans or induces or promotes cancer”.

No EMF exposure guidelines have been drawn in India though exposure guidelines have been drawn up outside India including:

- State Transmission Lines Standards and Guidelines in the USA;

- International Commission on Non-Ionizing  Radiation Protection (ICNIRP);

- US National Council on Radiation; and

- American Conference on Government and Industrial Hygienist (ACGIH).

The magnetic field below 400 kV overhead power transmission lines is estimated at 40 T.  The ICNIRP guidelines present limiting exposure to EMFs, although it adds that the levels quoted should not be interpreted as distinguishing ‘safe’ from ‘unsafe’ EMF levels. The ICNIRP guideline for the general public (up to 24 hours a day) is a maximum exposure levels of 1,000 mG or 100 T. A study carried out by Central Power Research Institute (CPRI) on POWERGRID lines reveals that the EMF about 1 m above ground near a 400 kV single circuit transmission line range from 3 – 7.2 T in the RoW. The impact of EMF is also dependent on the duration of exposure and therefore no significant adverse impact is envisaged.  POWERGRID complies with international norms for field strength limits which are certified by Power Technologies Inc, USA. POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard. (synopsis of the Power Technologies Inc. report  enclosed as Annexure-3)

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India :

“ Power Grid were the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid do not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

5.4      ENVIRONMENTAL MONITORING PROGRAM IN POWERGRID:

- Monitoring is a continuous process for POWERGRID projects at all the stages be it the site selection, construction or maintenance.

- The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan:

- 5.4.1    Corporate Level

- An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and DHQs to carry out environmental and social surveys for new projects.

- Assisting RHQs and DHQs to finalise routes of entire power transmission line considering environmental and social factors that could arise enroute

- Help RHQs and DHQs to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & DHQs on environment and social issues and their management plan.

- 5.4.2     Regional Level

- At its Regional Office POWERGRID has a Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the Corporate level and the Divisional Headquarters. The key functions envisaged for ESMC are :

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and DHQs to finalise routes of entire power transmission lines considering the environmental and social factors that could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and DHQs on various environmental and social aspects.

- 5.4.3      Site Office

- At the Divisional Headquarters level, POWERGRID has made  the head of the division responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social  Management Team (ESMT) . Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalise the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorised Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

- As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park , it  is generally  done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties. A monitoring system (done by the Forest Department) is also in place for compensatory forests established as part of the Project. A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn (Table- 5.1) and will be implemented during various stage of project execution.

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan.

5.5     CRITICAL ENVIRONMENTAL REVIEW CRITERIA

(i)	Loss of irreplaceable resources

The transmission projects do not involve any large scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As the portion of lines in the subject project are passing through forest area but due to efforts to utilize existing KSEB line corridor  of 220 KV Idukki-Trichur line to further minimize the tree felling or loss of natural resources in this project. Hence no such problem is anticipated.

(ii)	 Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as no reserve forest is getting  affected thus there is no possibility of endangering/causing extinction of any species.

(iv)	Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

5.6    PUBLIC CONSULTATION:

Public consultation/information is an integral part of the project implementation. Public  is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted.

Apart from this, public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting  as  per  Environmental Social Policy  & Procedures of POWERGRID  (ESPP) shall also be carried out during different activities of project cycle. During such consultation the public is informed about the project in general and in particular about the following:

- complete project plan (i.e. its route and terminating point and substations, if any, in between);

- POWERGRID design standards in relation to approved international standards;

- health impacts in relation to EMF;

- measures taken to avoid public utilities such as school, hospitals, etc.;

- other impacts associated with transmission lines and POWERGRID’s approach to minimizing and solving them;

- land acquisition details, proposed R&R measures and compensation packages in line with POWERGRID’s policy;

- Trees and crop compensation process.

The process of such consultation be continued during project implementation and even during O&M stage. The progress and proposed plan for Public consultation is as follows:

| S.No. | Activity | Technique | Schedule |
| --- | --- | --- | --- |
| 1. | Walk over Survey | Informal group discussion | completed |
| 2. | Preliminary / GIS Survey | Group discussion, one to one meeting | Under Progress for details see Annexure-4 |
| 3. | Detailed/Check survey | Public Meeting at different places (50 Km) enroute final route alignment of line | 1-2 such meeting during Jauary’06 to April’06 |
| 4. | Construction Phase | Localized group meeting, Pamphlet/Information brochures , Public display etc. | During entire construction period |
| 5. | O&M Phase | Information brochures, Operating field offices, Response to public enquiries,Press release etc. | Continuous process as and when required. |

5.7	CONCLUSIONS :

From the above discussion, it would seem that the area is rich in physical resources. But careful route selection and use of existing 220 KV line corridor in major portion of line and in forest  has minimized impact on  ecologically/environmentally sensitive areas to the barest minimum. Thus route selected for detailed survey is the most optimum route alignment.   The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts while on the one hand underline the need for implementation of the project for overall development of the area and on another hand suggest that a detailed E.I.A. may not be necessary.

Table-5.1 Environment Management Plan

| Project activity /stage | Potential impact | Proposed mitigation measure | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-construction |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Tower location and line alignment selection with respect to nearest dwellings | Setback distances to nearest houses - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformers stated in tender specification - once | POWERGRID | Part of tender specifications for the equipment |
|  |  | Processes, equipment and systems not to use chlorofluorocarbons (CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – once | POWERGRID | Part of tender specifications for the equipment |
|  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design |
| Transmission line design | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Electromagnetic field strength for proposed line design | Line design compliance with relevant standards - once | POWERGRID | Part of detailed alignment survey and design |
| Substation location and design | Exposure to noise | Design of plant enclosures to comply with noise regulations. | Expected noise emissions based on substation design | Compliance with regulations - once | POWERGRID | Part of detailed siting survey and design |
| Location of transmission towers and transmission line alignment and design | Impact on water bodies and land | Consideration of tower location at where they could be located to avoid water bodies or agricultural land. | Tower location and line alignment selection (distance to water and/or agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
|  | Social inequities | Careful route selection to avoid existing settlements | Tower location and line alignment selection (distance to nearest dwellings or social institutions) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
|  |  | Minimise need to acquire agricultural land | Tower location and line alignment selection (distance to agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
| Involuntary resettlement or land acquisition | Social inequities | Compensation paid for temporary/ permanent loss of productive land as per LAA & its process | RAP implementation | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction phase |
| Encroachment into precious ecological areas | Loss of precious ecological values/ damage to precious species | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest designated ecological protection area) | Consultation with local forest authorities - once | POWERGRID | Part of detailed siting and alignment survey /design |
| Transmission line through forestland | Deforestation and loss of biodiversity | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest protected or reserved forest) | Consultation with local authorities - once | POWERGRID | Part of detailed siting and alignment survey/design |
|  |  | Minimise the need by using existing towers, tall towers and RoW, wherever possible |  | Consultation with local authorities and design engineers - once |  |  |
|  |  | Obtain statutory clearances from the Government | Statutory approvals from Government | Compliance with regulations – once for each subproject |  |  |
| Encroachment into farmland | Loss of agricultural productivity | Use existing tower footings/towers wherever possible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
|  |  | Avoid siting new towers on farmland wherever feasible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once |  | Part of detailed siting and alignment survey /design |
|  |  | Farmers compensated for any permanent loss of productive land | Design of Implementation of Crop Compensation (based on affected area) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  | Farmers/landowners compensated for significant trees that need to be trimmed/ removed along RoW. | Design of Implementation of Tree compensation (estimated area to be trimmed/removed) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  |  | Statutory approvals for tree trimming /removal | Compliance with regulations – once for each subproject |  | Part of detailed siting and alignment survey /design |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels | Noise levels to be specified in tender documents - once | POWERGRID | Part of detailed equipment design |
| Interference with drainage patterns/Irrigation channels | Flooding hazards/loss of agricultural production | Appropriate siting of towers to avoid channel interference | Tower location and line alignment selection (distance to nearest flood zone) | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
| Escape of polluting materials | Environmental pollution | Transformers designed with oil spill containment systems, and purpose-built oil, lubricant and fuel storage system, complete with spill cleanup equipment. | Equipment specifications with respect to potential pollutants | Tender document to mention specifications - once | POWERGRID | Part of detailed equipment design /drawings |
|  |  | Substations to include drainage and sewage disposal systems to avoid offsite land and water pollution. | Substation sewage design | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
| Explosions/Fire | Hazards to life | Design of substations to include modern fire control systems/firewalls. | Substation design compliance with fire prevention and control codes | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
|  |  | Provision of fire fighting equipment to be located close to transformers. |  |  |  |  |
| Construction |  |  |  |  |  |  |
| Equipment layout and installation | Noise and vibrations | Construction techniques and machinery selection seeking to minimize ground disturbance. | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance - once at the start of each construction phase | POWERGRID (Contractor through contract provisions) | Construction period |
| Physical construction | Disturbed farming activity | Construction activities on cropping land timed to avoid disturbance of field crops (within one month of harvest wherever possible). | Timing of start of construction | Crop disturbance –Post harvest as soon as possible but before next crop - once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Mechanized construction | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment – estimated noise emissions | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Noise, vibration, equipment wear and tear | Turning off plant not in use. | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction of roads for accessibility | Increase in airborne dust particles | Existing roads and tracks used for construction and maintenance access to the line wherever possible. | Access roads, routes (length and width of new access roads to be constructed) | Use of established roads wherever possible - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Increased land requirement for temporary accessibility | New access ways restricted to a single carriageway width within the RoW. | Access width (meters) | Access restricted to single carriageway width within RoW - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Temporary blockage of utilities | Overflows, reduced discharge | Temporary placement of fill in drains/canals not permitted. | Temporary fill placement (m3) | Absence of fill in sensitive drainage areas - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Marking of vegetation to be removed prior to clearance, and strict control on clearing activities to ensure minimal clearance. | Vegetation marking and clearance control (area in m2) | Clearance strictly limited to target vegetation - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Trimming/cutting of trees within RoW | Fire hazards | Trees allowed growing up to a height within the RoW by maintaining adequate clearance between the top of tree and the conductor as per the regulations. | Species-specific tree retention as approved by statutory authorities (average and maximum tree height at maturity, in meters) | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Loss of vegetation and deforestation | Trees that can survive pruning to comply should be pruned instead of cleared. | Species-specific tree retention as approved by statutory authorities | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Felled trees and other cleared or pruned vegetation to be disposed of as authorized by the statutory bodies. | Disposal of cleared vegetation as approved by the statutory authorities (area cleared in m2) | Use or intended use of vegetation as approved by the statutory authorities – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Wood/vegetation harvesting | Loss of vegetation and deforestation | Construction workers prohibited from harvesting wood in the project area during their employment, (apart from locally employed staff continuing current legal activities). | Illegal wood /vegetation harvesting (area in m2, number of incidents reported) | Complaints by local people or other evidence of illegal harvesting - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Surplus earthwork/soil | Runoff to cause water pollution, solid waste disposal | Soil excavated from tower footings disposed of by placement along roadsides, or at nearby house blocks if requested by landowners. | Soil disposal locations and volume (m3) | Acceptable soil disposal sites - every 2 weeks | POWER GRID (Contractor through contract provisions) | Construction period |
| Substation construction | - Loss of soil | Fill for the substation foundations obtained by creating or improving local water supply ponds or drains, with the agreement of local communities. | Borrow area siting (area of site in m2 and estimated volume in m3) | Acceptable borrow areas that provide a benefit - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Substation construction | Water pollution | Construction activities involving significant ground disturbance (i.e. substation land forming) not undertaken during the monsoon season. | Seasonal start and finish of major earthworks | Timing of major disturbance activities - prior to start of construction activities | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Tree clearances for easement establishment to only involve cutting trees off at ground level or pruning as appropriate, with tree stumps and roots left in place and ground cover left undisturbed. | Ground disturbance during vegetation clearance (area, m2) | Amount of ground disturbance - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  |  | Statutory approvals | Statutory approvals for tree clearances – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Tower construction – disposal of surplus earthwork/fill | Waste disposal | Excess fill from tower foundation excavation disposed of next to roads or around houses, in agreement with the local community or landowner. | Location and amount (m3)of fill disposal | Appropriate fill disposal locations - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Storage of chemicals and materials | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports (type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction schedules | Noise nuisance to neighbouring properties | Construction activities only undertaken during the day and local communities informed of the construction schedule. | Timing of construction (noise emissions, [dB(a)]) | Daytime construction only - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Provision of facilities for construction workers | Contamination of receptors (land, water, air) | Construction workforce facilities to include proper sanitation, water supply and waste disposal facilities. | Amenities for Workforce facilities | Presence of proper sanitation, water supply and waste disposal facilities - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period |
| Encroachment into farmland | Loss of agricultural productivity | Use existing access roads wherever possible | Usage of existing utilities | Complaints received by local people /authorities - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Ensure existing irrigation facilities are maintained in working condition | Status of existing facilities |  |  |  |
|  |  | Protect /preserve topsoil and reinstate after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  |  | Repair /reinstate damaged bunds etc after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  | Social inequities | Compensation for temporary loss in agricultural production | Implementation of Crop compensation (amount paid, dates, etc.) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Uncontrolled erosion/silt runoff | Soil loss, downstream siltation; | Need for access tracks minimised, use of existing roads. | Design basis and construction procedures (suspended solids in receiving waters; area re-vegetated in m2; amount of bunds constructed [length in meter, area in m2, or volume in m3]) | Incorporating good design and construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Limit site clearing to work areas |  |  |  |  |
|  |  | Regeneration of vegetation to stabilise works areas on completion (where applicable) |  |  |  |  |
|  |  | Avoidance of excavation in wet season |  |  |  |  |
|  |  | Water courses protected from siltation through use of bunds and sediment ponds |  |  |  |  |
| Nuisance to nearby properties | Losses to neighbouring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | As much as possible existing access ways will be used. | Design basis and layout | Incorporating good design engineering practices – once for each site |  |  |
|  |  | Productive land will be reinstated following completion of construction | Reinstatement of land status (area affected, m2) | Consultation with affected parties – twice – immediately after completion of construction and after the first harvest |  |  |
|  | Social inequities | Compensation will be paid for loss of production, if any. | Implementation of Tree/Crop compensation (amount paid) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Inadequate siting of borrow areas | Loss of land values | Existing borrow sites will be used to source aggregates, therefore, no need to develop new sources of aggregates | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Health and safety | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work days caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  |
|  |  | Contractor to arrange for health and safety training sessions |  |  |  |  |
| Inadequate construction stage monitoring | Likely to maximise damages | Training of POWERGRID environmental monitoring personnel | Training schedules | Number of programs attended by each person – once a year | POWERGRID | Routinely throughout construction period |
|  |  | Implementation of effective environmental monitoring and reporting system using checklist of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof. | Submission of duly completed checklists of all contracts for each site - once |  |  |
|  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures. | Compliance report related to environmental aspects for the contract | Submission of duly completed compliance report for each contract - once |  |  |
| Operation and Maintenance |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Compliance with setback distances (“as-built” diagrams) | Setback distances to nearest houses – once in quarter | POWERGRID | During operations |
| Equipment submerged under flood | Contamination of receptors (land, water) | Equipment installed above the high flood level (HFL) by raising the foundation pad. | Substation design to account for HFL (“as-built” diagrams) | Base height as per flood design - once | POWERGRID | During operations |
| Oil spillage | Contamination of land/nearby water bodies | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks. | Substatio bunding (Oil sump) (“as-built” diagrams) | Bunding (Oil sump) capacity and permeability - once | POWERGRID | During operations |
| Inadequate provision of staff/workers health and safety during operations | Injury and sickness of staff /workers | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |
|  |  | Safety awareness raising for staff. | Training/awareness programs and mock drills | Number of programs and percent of staff /workers covered – once each year |  |  |
|  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |
|  |  | Provide adequate sanitation and water supply facilities | Provision of facilities | Complaints received from staff /workers every 2 weeks |  |  |
| Electric Shock Hazards | Injury/mortality to staff and public | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |
|  |  | Security fences around substations | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |
|  |  | Barriers to prevent climbing on/dismantling of transmission towers | Maintenance of barriers |  |  |  |
|  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |
|  |  | Electricity safety awareness raising in project areas | Training /awareness programs and mock drills for all concerned parties | Number of programs and percent of total persons covered – once each year |  |  |
| Operations and maintenance staff skills less than acceptable | Unnecessary environmental losses of various types | Adequate training in O&M to all relevant staff of substations and transmission line maintenance crews. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
|  |  | Preparation and training in the use of O&M manuals and standard operating practices. |  |  |  |  |
| Inadequate periodic environmental monitoring. | Diminished ecological and social values. | Power Grid staff to receive training in environmental monitoring of project operations and maintenance activities. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | Processes, equipment and systems using cholofluorocarbons (CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government. | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |
| Transmission line maintenance | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Required ground clearance (meters) | Ground clearance - once | POWERGRID | Operations |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels (dB(a)) | Noise levels at boundary nearest to properties and consultation with affected parties if any - once | POWERGRID | Operations |

PART – II

INITIAL SOCIAL ASSESSMENT CHECKLIST

INITIAL SOCIAL ASSESSMENT CHECKLIST

1.	Clientele Group

- Identify the target client population that is expected to consume the power   which will be produced or distributed.

The proposed scheme is for strengthening of existing grid of extra high voltage lines to improve the reliability and availability of power at different load centre. It is estimated that proposed project would improve the power situation of entire Southern region due to transfer of power from different generating units to all its constituents in general but entire Kerala State having population of more than 3,18,41,374 shall be directly benefited from the proposed project.

- Identify and quantify the subgroups, within the client population (e.g. industrial consumers, residential consumers, electric co-operatives etc.), which are likely to have different needs and demands.

Not applicable, since POWERGRID role is restricted only up to transmission of bulk power from generating station to different load centre and further distribution of available power is taken up by concerned State Electricity Boards( SEBs.)

2.	Clientele Demands

- Assess the ability and willingness of the major consumer groups to pay tariffs for electricity which are needed to sustain the operations of the facilities.

The  transmission system is evolved after extensive discussions with all the beneficiaries individually and collectively at the SREB forum.  The views of the states are taken into account while developing the project scope and is then finalised with CEA. Before submitting the Feasibility Report to CEA, it is ensured that the beneficiary SEB accepts the project configuration and the project cost along with the modality of sharing the transmission tariff associated with the scheme.

3.	Potential Adverse Impacts

- Determine if some groups of people will have to be relocated according to the power generation, transmission or distribution plan (e.g. from dam sites, thermal power sites, rights-of-way).

- As per the existing Indian laws, land for ROW of transmission line is not acquired. As such there is no resettlement and rehabilitation involved in transmission line construction works.  But for the proposed new S/Stn. at Muvattupuzha (East Cohin) 34.27 acres (13.877 ha.) of land is to be acquired that shall affect around 94 people. However, it is absolutely clear from the initial assessment that no resettlement or physical displacement of people will be involved in the present case. As regard extension of Trichur substation is concerned, land for extension of bays is already available within the boundry of existing one. As such no fresh land acquisition is involved hence, R&R issues shall not be involved in case of Trichur substation.

- Determine if the project will cause the loss of farmland, grazing land, forests, streams or groundwater which supply local livelihoods, provide food or water sources.

Land selected is privately owned  agricultural land and is situated on the Aluva-Tripunithura road in the village Kunnathunad of Kunnathunad  Taluk of Ernakulam district of Kerala which is around 19 Km from proposed site.. Acquisition process under Land Acquisition Act, section-4 notification for acquisition of 13.8777 ha. land has been issued in Feb’05 (Annexure-5). Acquisition of 13.8777 ha. land will only affect farm land and no other land including forest land will be affected.

- Determine the possible exposure of certain groups to health hazards (e.g. from downstream river pollution, vector-borne diseases through creation of breeding sites, from vibrations, fly ash from thermal power sites, hazardous wastes, explosions).

Not applicable

- Determine  possibility of conflict over water rights, e.g., water used for downstream irrigation diverted to cool boilers etc.

Not applicable

- Determine if traditional lands, heritage and cultural property of indigenous people will be damaged or lost (e.g. ancestral burial grounds, ritual sites, links with plants/substances essential for traditional medicine etc.).Determine any significant changes in affected groups `life-styles'.

Land is agricultural land and does not come under the category of ancestral burial ground or ritual sites. Since land is only used for farming it will impact around 94 people who will lose portion of their farm land thereby adversely affecting their income. As per the prevailing market rate it is estimated that compensation to the tune of Rs. 10 crores @ of Rs. 25-30 lakh per acre shall be paid to affected people. However,  detailed social assessment of these affected population shall be carried out by an independent agency as per POWERGRID policy and based on this a detailed Rehabilitation Action Plan (RAP/RP) shall be prepared in line with agreed entitlement matrix (Annexure-6) for rehabilitation/restoration of livelihood of affected people.  The entitlement matrix has also been translated in local language for better understanding of local people and is uploaded on POWERGRID’s web site. (Annexure-7).

- Determine how pricing policies would affect the distribution of and access to project benefits by poor clients.  Assess the ability of client groups who are defined as poor to afford a basic level of service; identify financing measures which are affordable for the poor groups on a sustainable basis.

Not applicable

.

- Prepare a socio-economic profile for each group which would be adversely affected to describe and quantity the impact(s) on the affected group.

A brief profile based on preliminary assessment and revenue record is placed at Annexure-8 . However, as explained above detailed profile of each affected people / family shall be prepared after the assessment by independent agency and based on that rehabilitation measures shall be taken to minimize the impact of loss of land and for improvement of their living standard.