SECTION I: PROJECT DESCRIPTION

(Up-gradation of Transmission Corridor from Uttaranchal)

BACKGROUND

Primary energy resources for power generation are unevenly distributed in the country and in most of the cases far away from the load centers.

Tehri Hydro Development Corporation has proposed augmentation of generation at Tehri by another 1000MW under Tehri-II. Transmission system to be associated with Tehri-II was discussed in the 18th Standing committee Meeting of Northern region Transmission Planning. Hydropower potential in Tehri Valley is being tapped by development of 3 integrated power stations: the 1000 MW Tehri-I plant, the 400 MW Koteshwar plant, and the 1000 MW Tehri-II pumped storage facility. In May 2006 POWERGRID commissioned the first phase of the transmission system associated with the power generation plants. The Tehri-I and Koteshwar plants will be operating at full capacity by year end 2007, and the Tehri-II pumped storage facility is planned to be commissioned by May 2010. The transmission capacity must be expanded in order to deliver power to Delhi ring and transfer beyond to various beneficiaries of the Northern Region through existing transmission lines.

Objective

The objective of the project is to provide adequate transmission arrangement for evacuation and reliable transfer of power from Tehri Valley to constituents of Northern Region.

Project Highlights

| a) | Project | : | Up-gradation of Transmission Corridor from Uttaranchal |
| --- | --- | --- | --- |
| b) | Location of the Project | : | Uttaranchal and Uttar Pradesh |
| c) | Beneficiary States | : | Constituent States of Northern Region |

Substations

- Establishment of 765/400KV, 10x500 MVA transmission capacity each at Tehri Pooling Station and Meerut within the existing site and

- Modification of series capacitors for operation at 765kV level

A Power map showing the transmission grid of Northern Region highlighting the above scope of works is placed as Annexure-I

Beneficiaries

The target beneficiaries of this project are the constituent states of Northern Region.   It involves state of Uttar Pradesh, Uttaranchal, Punjab, Haryana, Chandigarh and Himachal Pradesh etc.

Benefits of Project

The main benefits of the project is that the transmission capacity will be expanded   to deliver power to Delhi ring and transfer beyond to various beneficiaries of the Northern Region through existing transmission lines. This will improve system stability in the Northern Region.

SECTION II: BASE LINE DATA

The proposed project i.e. existing 2 sub-stations to be upgraded/constructed at 765 kV levels is situated in two states i.e. Tehri pooling S/s in Uttaranchal and Meerut S/s in Uttar Pradesh. The general descriptions of the States are as follows:

2.1	 UTTARANCHAL

2.2	 PHYSIOGRAPHY

Uttaranchal is bounded by Nepal & China in North, UP in south and east and Himachal Pradesh in west.  It lies between lat.23º52’ and 31º28’ N and long.77º5’ and 84º38’E. The state comprises five litho-tectonically and physiographically distinct subdivisions namely, the Outer Himalaya comprising the Tarai and Bhabhar, Sub-Himalayan belt of the Siwalik, the Lesser Himalaya, the Great Himalaya and the Trans-Himalaya or Tethys. The state is embodying the Kumaon and Garhwal Himalayas with a geographical area of about 53,485 sq. km. The land use pattern of the state (Including Uttar Pradesh l) is given in following table:

| Land use | Area in ‘ 000 ha | Percentage |
| --- | --- | --- |
| Total Geographical area | 29,441 |  |
| Reporting Area for land utilisation | 29,794 | 100.00 |
| Forests | 5,150 | 17.29 |
| Not available for cultivation | 3,516 | 11.80 |
| Permanent Pasture & Grazing land | 296 | 0.99 |
| Land under misc. tree crops & groves | 513 | 1.72 |
| Culturable waste land | 945 | 3.17 |
| Fallow land other than current fallows | 832 | 2.79 |
| Current fallows | 1,067 | 3.58 |
| Net area Sown | 17,475 | 58.65 |

Source: Land use statistics At a Glance 1996-97, Ministry of Agriculture, GOI, 2000

2.3	POPULATION

As per census 2001 the state supports a human population of 84.89 lakhs  (Male-43.26 & Female-41.63). About 51.06 lakhs are literate (Literacy rate-72.28 %). Sex ratio (Female per thousand males)-964.

2.4	CLIMATE

With the exception of the remote valleys to the North of the Great Himalayan Range, the Uttaranchal region is subject to the Indian monsoon. The monsoon begins by mid-June, with the first heavy rainfall generally occurring by the first heavy rainfall generally occurring by the first week of July. Rain continues through August until the first week of September.
The first of the winter snow falls in mid to late October. Heavy snowfall on the main Himalayan range during the winter months, and it is not until the beginning of May that the snow starts melting. To the south of the Himalayan range, some of the valleys, including the Har Ki Dun, enjoy milder climates. At lower altitudes the snowfall is not so severe; still the best time to visit Uttaranchal is from April to June, and after the Monsoon, from September to mid-November.

2.5	RAINFALL & WORKING SEASON

The average annual rainfall ranges between 1,000 to 1,200 mm.

Working Season: From June to September (4months) the work progress is usually hampered due to dominant rainy season.

2.6	SOIL

The project is located in hilly terrain of outer Himalayas and in Indo-Gangatic area.  Predominant soils in the area are sandy soil with slope wash material along the hill slopes, riverine types on the river terraces and alluvial in plains.

2.7	MINERAL RESOURCES

The state of Uttaranchal is poor in mineral resources. Only considerable deposits of limestone, sand, gravels and boulders are available.

2.8	WATER RESOURCES

The main rivers of the state from west to east are the Yamuna, the Ganga, the Ramganga and the Ghaghara.  All the rivers emerge from the Himalaya. The Yamuna and the Ganga flow from north-east to south-west in their upper mountainous courses, from north to the south in western parts of the state

2.9	 ECOLOGICAL RESOURCES

The recorded forest area of Uttaranchal state is 346.62 million ha. which constitutes 64.8% of its geographic area. By legal status, Reserved Forest constitutes 68.7%, Protected Forest 30.8% and Un-classed Forest 0.5%.. Forest Map of Uttaranchal is enclosed as Map-1.The flora of a region includes all the varieties of plants, which grow there. Vegetation is the general effect produced by the growth of some or all these plants.

Tropical Moist Deciduous Forests: These forests are found in the moist region of Tarai having an annual rainfall of over 1200 mm and are found in the Northern and Eastern part of the state in all districts. The forests form the main source of commercial timber in the state. Teak, Sal, Haldu, Sisam, Khair, Katas, Manvel, Devdar, Chir, and Pine etc. are the main species of the forest.

Tropical Dry Deciduous Forests: These forests are found in the region having the rainfall between 600 mm to 1200 mm and found in the northern part of the state.

These forests are mostly disturbed in northern and eastern part of the state. The main forests formations in the state are of teak, deodar and sal. The trees are mostly deciduous. Since sun-light reaches the ground in abundance, shrubs and grasses also grow here. Large tracts of these forests have been cleared for cultivation. Important trees are Sal, Palas, Amaltas, Bel, Anjeer etc. Neem, Peepal, Sheesham, Mango, Jamun, Babool, Imli (Tamarind) etc. grows along riverbanks and in other moist regions.

Tropical Thorny Forests: These are mostly found in south-western parts of the State. Such forests are confined to the areas that have low annual rainfall (50-70 cms), mean annual temperature between 250C to 270C and low humidity (less than 47%). Widely scattered thorny trees mainly, Babool, Thorny, legumes and Euphorbia are extensively found here. During rains, short grasses also grow here. The trees are generally small here forming open dry forests. Important trees of the region are Phulai, Khair, Kokke, Dhaman, Danjha, Neem, etc. Various types of resin and gum are also obtained from these trees.

Fauna

Animals depend on forest not only for food but also for habitat. The diversity of fauna living in water and land in the air are found in the State. Important species mainly found in the state:

Fish

Mahaser, Saul, Rohu, Mirgal, Kata, Labi, Mangur, Cuchia, Eel, Einghi, Mirror Carp, Trout.

Amphibia

Frog and Toad.

Reptiles

Bamania, Pit-viper, Lizard, Goh, Cobra, Tortoise, Krait, Dhaman and Crocodile.

Aves

Cheel, Vulture, Peacock, Nightingale, Pigeon, Parrot, Owl, Nilkanth and Sparrow.

Mammals

Shrew, Porcupine, Squirrel, Hare, Mongoose, Cow, Buffalo and Mouse.

Other common species found here are Tiger, Panther, Snow Leopard, Sambhar, Cheetal, Kastura, Chinkara, Black Deer, Nilgai, Back-brown Bear, Mountain Goat, Hyena, Hill Dog, and Elephant etc. Among the birds Fowl, Pheasant, Partridge, Florican, Duck, Goose and Wader are common.

2.10	HUMAN AND ECONOMIC DEVELOPMENT

Uttaranchal has a very ancient history as it find mention in the Hindu scriptures as Kedarkhand, Manaskhand and Himvant. It is often called the Land of Gods (Dev Bhoomi) because of its various holy places and abundant shrines.Uttaranchal became the 27th State of Indian Union on November 09, 2000. One of the most backward states of northern India, Uttaranchal, despite being rich in natural resources, has one of the country's lowest per capita incomes- nearly 70% of the population lives below the poverty line. Uttaranchal depends largely upon agriculture for its income; manufacturing industries are virtually non-existent in the state. The bulk of the state's population is rural and survives by cultivating food grains - Haridwar, Udham Singh Nagar, the Doon Valley and Nainital district being the important agricultural areas. Horticulture may be a more lucrative option in areas where growing grain is not economically viable. There's a lot of scope for growing apple, pear, plum, apricot, litchi, grape, and more- provided agricultural research, storage and distribution systems are improved. Another potential contributor to the economy could be the cultivation of medicinal plants and herbs for the pharmaceutical industry- an activity fairly small at the moment.

The other important revenue earner for Uttaranchal is tourism. Always a crowd-puller as far as Hindu pilgrims are concerned, Garhwal and Kumaon are now also becoming an attractive destination for adventure seekers who want to go wildlife-watching, trekking, white water rafting, angling, mountaineering and skiing. The tourist infrastructure in many places, however, is still inadequate and a lot of development is essential before the moolah-laden, high-spending tourists come rolling in.

  - POWER SCENARIO

The existing installed generating capacity is about 1286 MW (2003 census) and is contributed by hydro generation only. There is no thermal power generation in this state. The major hydro power stations under construction are (i) Maneri Bhali Stage-II (304 MW) (ii) Lakhawar Vyasi Stage-I (300MW) (iii) Lakhawar Vyasi Stage-II (120

MW) (iv) Shrinagar HE Project (330 MW) (v) Vishnuprayag (400MW) (vi) Tehri dam

Stage-I (1000 MW) (vii) Tehri Dam Stage-II (1000MW) (viii) Koteswar Dam Project (400 MW).

  - Uttar Pradesh

Uttar Pradesh is bounded by Uttaranchal and Nepal in North, Madhya Pradesh and Chhattisgarh in south, Rajasthan, Haryana and Delhi in West and Bihar and Jharkhand in east. It lies between lat. 23º 52' and 31º 28' N and long. 77º 5' and 84º 38' E. Physiographically Uttar Pradesh can be divided in to two distinct regions i) Indogangetic plain ii) Southern hills. The land use pattern of the state (Including Uttaranchal) is already given in Para no.2.2

2.13	Population

Details of population as per 2001 census are as follows:

| Description | Nos./Percentage |
| --- | --- |
| Total population | 166,052,859 |
| Male population | 87,466,301 |
| Female population | 78,586,558 |
| Rural population | 131,658,339 |
| Urban population | 34,539,582 |
| Sex ratio (female/1000male) | 916 |
| Literacy % | 57.36 |
| Male | 70.23 |
| Female | 42.98 |

Climate

The climate in Uttar Pradesh varies substantially. The Gangetic plain, which covers three-quarters of the state, is dry and dusty in summer. But during the monsoons between June and September, it is transformed into carpets of lush green fields. The monsoons also spell disaster for some regions, when the Ganga and its tributaries overflow their banks and flood large tracts of land.

Winter is severe, the Gangetic plains are fairly cold with temperatures coming down as low as 3°C though average temperatures remain around a pleasant 18°C. Summers are extremely hot in the plains with maximum temperatures reaching as high as 45° C. The intensity of the summer months is magnified by the hot winds called ‘loo’ that blow across the plains in May and June, the two hottest months of the year.

Rainfall

The average annual rainfall and mean temperature ranges between 1,000 to 1,200 mm.

Soil

The project is located in Indo-Gangetic plain area.  Predominant  soils in the area are alluvial soil.

Water resources

The main rivers of the state from west to each are the Yamuna, the Ganga, the Ramganga, the Gomati and the Ghaghara.  All the rivers, except the Gomati, emerge from the Himalaya.  The Yamuna and the Ganga flow from north-east to south-west in their upper mountainous courses, from north to the south in western parts of the state and thereafter from north-west to south-east joining at Allahabad.

Mineral Resources

The state is poor in mineral resources. Only considerable deposits are of limestone, andalusite and red sandstone in Mirzapur, Dolomite occurs in small quantities in Bandal and Varanasi. Pyrophyllite and diaspore in Jhansi and Hamirpur districts.  Coal is available in Singrauli (Mirzapur district).

Ecological resources

The recorded forest area of the state is 1.70 million ha. which constitutes 7.06% of its geographic area. By legal status, Reserved Forest constitutes 74.10%, Protected Forest 8.08% and Un-classed Forest 17.82%. Forest map of Uttar Pradesh is enclosed as Map-2. The flora of a region includes all the varieties of plants that grow there. Vegetation is the general effect produced by the growth of some or all these plants. The plains of Uttar Pradesh have been very rich in natural vegetation that has, however, diminished due to wide-ranging needs of the people.

Only a few patches of natural forest are now found scattered here and there in the plains, while such forests are extensively found on a very large scale in sub-mountain and mountain regions, which is in Uttranchal now. The following 3 types of forest are found in Uttar Pradesh.

- Tropical Moist Deciduous Forests: These forests are found in the moist region of Terai. They grow in regions that record 100 to 150 cm. of rainfall annually, have an average temperature between 260 -270 C. and have considerable degree of humidity. A special feature of the forests is that deciduous trees of uneven size grow on higher altitude regions. Lower regions have several species interspersed with Bamboo, Climbers, Cand and ever green shrubs. Main trees are Sal, Ber, Gular, Jhingal, Palas, Mahua Semal, Dhak, Amla, Jamun, etc.

- Tropical Dry Deciduous Forests: These forests are found in all parts of the plains, and usually in central eastern and western regions. The trees are mostly deciduous. Since sunlight reaches the ground in abundance, shrubs and grasses also grow here. Large tracts of these forests have been cleared for cultivation. Important trees are Sal, Palas, Amaltas, Bel, Anjeer etc. Neem, Peepal, Sheesham. Mango, Jamun, Babool, Imli (Tamarind) etc. grows along riverbanks and in other moist regions.

- Tropical Thorny Forests: These are mostly found in south-western parts of the State. Such forests are confined to the areas that have low annual rainfall (50-70 cms), mean annual temperature between 250C to 270C and low humidity (less than 47%). Widely scattered thorny trees mainly, Babool, Thorny, legumes and

Euphorbias are extensively found here. During rains, short grasses also grow here. The trees are generally small here forming open dry forests. Important trees of the

Region is Phulai, Khair, Kokke, Dhaman, Danjha, Neem, etc. Various types of resin and gum are also obtained from these trees.

Protected area

There are 1 National Parks and 23 wildlife sanctuaries covering an area of 0.57 million Ha. Which constitute 2.36% of geographic area. The proposed project does not involve all these mentioned area.

Fauna

Animals depend on forest not only of food but also for habitat. The diversity of fauna living in water and land in the air are found in the State. Important species mainly found in the state: 

Fish

Mahaser, Hilsa, Saul, Rohu, Mirgal,Kata, Labi,  Mangur, Cuchia, Eel, Einghi, Mirror Carp, Trout. 
                                                                                                                             Amphibia

Frog and Toad.

Reptiles

Bamania, Pit-viper, Lizard, Goh, Cobra,Tortoise, Krait, Dhaman and Crocodile.

Aves

Cheel, Vulture, Peacock, Nightingale, Pigeon, Parrot, Owl, Nilkanth and       Sparrow.

Mammals

Shrew, Porcupine, Sqirrel, Hare, Mongoose, Cow, Buffalo and Mouse.

Other common species found here are Tiger, Panther, Snow Leopard, Sambhar, Cheetal, Kastura, Chinkara, Black Deer, Nilgai, Back-brown Bear, Mountain Goat, Hyena, Hill Dog, Elephant etc. Among the birds Fowl, Pheasant, Partridge, Florican, Duck, Goose and Wader are common.

Human and Economic development

Uttar Pradesh:  UP is the most populous state in the country. It is the fourth largest state in geographical area (238,566 Sq. Km.)

Economy:

Uttar Pradesh is rich in human and natural assets. Most of State’s farmland is in the well watered and naturally fertile U.P is the largest producer of food grains and oilseeds in the country.  It leads all the states in India in the production of wheat, maize, barley, gram, sugarcane and potatoes.  The state (India’s sugar bowl) produces about one half of the total sugarcane output in the country.

The western region of the state is more advanced in terms of agriculture. Majority of the population depends upon farming as its main occupation. Wheat, rice, sugar cane, pulses, oil seeds and potatoes are its main products. Sugar cane is an important cash crop almost through out the state and sugar mills and other cane crushers who produce Gur and Khandsari are common throughout the state. Uttar Pradesh is an important state in so far as horticulture is concerned.

Power Scenario

U.P. generates 5613 Mw and receives 3454 Mw from central sector projects. Per capita electricity consumption in the state is 176 Kw.  16.96% rural house holds in the state are electrified. At present, about 40,000 villages in the state remain to be electrified. Moreover, only about 80% of the urban and 20% of the rural households in the state has access to electricity. The per capita energy consumption in the state is also extremely low at about 188 kW as against the national average of about 363 kWh. Even in the electrified villages, the quality of supply and the supply hours need to improve significantly to help the consumer, existing and potential, to benefit from the multipliers that electrify access afford. This necessitates specific and sustained actions for expanding access to electricity.

SECTION III: POLICY , LEGAL & REGULATORY  FRAMEWORK

3.0    POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes. Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  In addition, Funding Agencies’ directives and guidelines with respect to environmental and social considerations.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and guidelines of Funding Agencies.

3.1     ENVIRONMENTAL

3.1.1 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorizes POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under Forest (Conservation) Act, 1980

When transmission projects pass through forestland, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forestland or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded; hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF.    Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk

consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/re-conditioner or at the designated collection centers – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling) Amendment

Rules, 2003:

MOEF vide its notification dt. 20th May 2003 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste that requires proper handling and disposal. As per the notification, all used oil is to be auctioned/sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules,

2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment, which uses these substances and planning to achieve CFC free organisation in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalising the route alignment.

3.1.2  FUNDING AGENCIES:

WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM) - F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and

would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the

natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

3.1.3   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- Applicable Legislations

3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement on Environment and Development, 1992

- Policy statement for Abatement of pollution, 1992

3.2.0   SOCIAL

3.2.1    MANDATORY REQUIREMENTS (NATIONAL)

- National Policy on Resettlement and Rehabilitation for Project Affected Families: Ministry of Rural Development, Government of India has notified a National policy on R&R for PAFs in Feb’04 applicable to all developmental projects where 500 or more families en-mass in plain areas or 250 or more families en-mass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the asset less rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act, 2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and

construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large-scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IP is consulted. Thus, the Indigenous People Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as

possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where

Resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlements of peoples are foreseen and require neither resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

3.2.3    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Constitutional Guarantees

- National and State-wide Laws and Policies Relating

to Land Acquisition and Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

3.2.4   RELEVANT POLICIES

- The Orissa Resettlement and Rehabilitation of Project Affected Persons Policy, 1994 (water resources projects)

- Resettlement & Rehabilitation Policy – NTPC

- Definition of Projects affected persons (PAPs) Nature and extent of entitlements.

# SECTION-IV: POWERGRID APPROACH FOR ROUTE SELECTION

4.0     ROUTE SELECTION - (ASSESSMENT & MANAGEMENT PROCESS)

At the system planning stage itself one of the factors that govern the evolution of system is the possible infringement with the forest. Wherever such infringements are substantial, different alternative options are considered. The route/ site selection criteria followed by POWERGRID is detailed below:

While identifying the transmission system for a generation project or as a part of National Power Grid, preliminary route selection is done by POWERGRID based on the Top sheets of Survey of India and Forest Atlas (Govt. of India's Publication).  During route alignment all possible efforts are made to avoid the forest area involvement completely or to keep it to the barest minimum, whenever it becomes unavoidable due to the geography of terrain or heavy cost involved in avoiding it.

4.1.1 STUDY OF ALTERNATIVES

POWERGRID approach towards Route selection

For selection of optimum route, the following points are taken into consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

- Any monument of cultural or historical importance is not affected by the route of the transmission line.

- The proposed route of transmission line does not create any threat to the survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during

site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

In addition, care is also taken to avoid National parks and sanctuaries and any other forest area rich in wild life.

The above-mentioned criteria are being followed by POWERGRID during routing of transmission line but the proposed project does not involve construction of transmission line.

- SECTION V: ENVIRONMENT IMPACT ASSESSMENT OF TEHRI HYDRO-GENERATION PROJECT (TEHRI-I, TEHRI-II (PSP) & KOTESWAR)

5.0     INTRODUCTION

As per MOE&F guidelines Hydro projects are covered under Schedule-1 of Environmental (Protection) Act 1986 which stipulates that all such project require prior approval of GOI (MOE&F) for taking up project activities. This act is a comprehensive law, which supplement existing law on pollution control, lays down standard for the quality of environment and restricts areas in which industrial activity can be carried out. A study on Environment Impact Assessment (EIA) of all such project is required for the processing of proposal of Environmental Clearance. The major environmental discipline covered under EIA studies includes impact on Air, Water, Noise, Soil, Land use Socio-economic, terrestrial ecology, aquatic ecology and health etc. It also requires a comprehensive Environment Management Plan (EMP) including safeguards and control measure propose to prevent/ mitigate environmental impact, rehabilitation plan, afforestation plan and Environmental monitoring plan during operation of the project, occupational safety and health, Risk Assessment and Disaster Management Plan.

The Schedule-1 i.e. Environmental (Protection) Act 1986, however, excludes UHV/EHV Transmission project/corridor. Therefore, EIA study is not required to be undertaken by POWERGRID and no Environmental Clearance is required but  Forest Clearance under Forest (Conservation) Act, 1980 is applicable if forest land is involved in case project / line is located or traversing through forestland.

5.1   ENVIRONMENTAL CLEARANCE:

The Environmental proposal of Tehri complex 2400Mw (Tehri-I, 1000MW, Tehri-II, 1000 MW and Koteswar 400 MW) was discussed before Appraisal Committee at Ministry of Environment and Forests (MOE&F) New Delhi. On the recommendation of committee, MOEF has issued Environmental clearance on 19.07.1990 subject to approval of Environment Management Plan (EMP). Environment clearance letter is enclosed as Annexure-2   Tehri Hydro Development Corporation (THDC) prepared EMP and submitted it to MoE&F in 1993-1994 and got approval from MoE&F New Delhi.

  - PUBLIC HEARING/CONSULTATION:

Public hearing is an integral part of all developmental projects under Schedule-1 of EPA. In the instant case THDC has already completed EIA studies and its report was submitted to MoE&F, New Delhi.  Public hearing has also been made   mandatory for the project covered under schedule-1 as per Govt. of India. Notification no. 244 dtd 10 April 1997.  In the present case approval was accorded before this notification, public hearing was not applicable.

As regard transmission project which is not covered under the said notification, public hearing is not mandatory. However, POWERGRID ESPP had made public consultation as an integral and important part of its project cycle. Public is consulted/informed at every stage of project execution, may it be transmission line routing or construction of sub-station.

POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted.

The proposed project does not involve construction of transmission line.  It involves only up-gradation of Sub-stations within existing boundaries.  However, Public information centers (PIC) shall be available for public enquiry. The proposed project only involves up-gradation of existing 400kV systems to establish 765/400KV, 10x500 MVA transmission capacities each at Tehri Pooling Station and at Meerut Sub-station within the existing substations.

# SECTION-VI: SCREENING OF POTENTIAL ENVIRONMENT IMPACT AND ITS

# MANAGEMENT

6.0	Impact Due to Project Location and Design

Environmental impact of transmission system projects is not far reaching and is mostly localized. These impacts can be minimized by careful selection of site/land for sub-station. In order to get latest information and further optimization modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS also results in access to updated/latest information, through satellite images and further optimisation having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location.

The proposed project involves only up-gradation of substations at higher voltage level at already existing facilities/substations. Therefore, impact due to project location and design is almost negative. However, during construction of Sub-stations adequate precautions would be taken to avoid interference with drainage pattern.

(i)	Resettlement

The proposed project envisages only up-gradation of two sub-stations (Tehri pooling and Meerut S/s).  Land is already available in the existing Sub-stations at Koteswar and at Meerut S/s and no fresh land acquisition is involved hence, R&R issues shall not be involved.

6.1	Environmental Problems Due to Design

(i)	Escape of polluting materials

The equipments to be installed in substations are static in nature and do not generate any fumes or waste materials.

(ii)	Explosion/fire hazards

During the site selection for sub-stations, it was ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires.

(iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

This is not called for in any case in substation project. However, infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for leveling the area.

(iv)    Environmental aesthetics

In the proposed project environmental aesthetics is not a problem. POWERGRID will take up plantation of trees to buffer the visual effect around its sub-stations and to provide better living conditions. Wherever POWERGRID feels it appropriate,

discussions will be held with local Forest/Horticulture Department officials to determine feasibility of planting trees along roads and available spaces in colony for visual effect in these areas.

(v)	 Noise/vibration nuisances

The equipments installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian and International standard -7194.  POWERGRID had monitored noise/sound level by measuring it at different places in and around reactor and transformer (at 1 m distance). The noise level was found to be ranging from 75 to 83 well within permissible limit of 85 dB. To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipments are undertaken.

In addition, plantations of sound absorbing species like Casuarinas, Tamarind, Banyan and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.

(vi)   Blockage of wildlife passage

The proposed project does not involve any wild life area or Migration path of wild   life.  Hence, the possibility of disturbance to wild life is nil or remote.

6.2	Environmental Problems during Construction Phase

(i)	Uncontrolled silt runoff

This is not called for in any case in substation project; therefore uncontrolled silt run off is not expected.

(ii)  Nuisance to nearby properties

As already described in preceding paras, during site selection due care was taken to keep substations away from settlements.  Further, all the construction activities will be undertaken through the use of mechanical devices. In the proposed project construction activities is restricted only in the existing sub-stations. Hence, nuisance to nearby property is not expected.

(iii)  Interference with utilities and traffic and blockage of access way

The proposed project construction activities is restricted only in the existing sub-stations, therefore, interference with utilities and traffic and blockage of access way is not expected.

- Inadequate resurfacing for erosion control

Since proposed Meerut S/s is to be constructed in plain area, erosion problem is ruled out. However, in case of Tehri pooling station, which is situated in hilly area best engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and

downhill slopes will be treated with revetments, if required. As explained in above paras, adequate steps shall be taken to ensure to prevent soil erosion.

(v)	Inadequate disposition of borrow area

Borrowed earth is not required for this project. However, in case of surplus excavated soil, it shall be utilized in the project area for filling and leveling purpose etc.

(vi)   Protection of Worker's health/safety

Provisions for workers' health and safety will be guided by the Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the Erection Conditions of Contract (ECC), a part of bidding documents.

In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.   Workers are also covered by the statutory Workmen (Compensation) Act.

POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Deptt. POWERGRID has framed guidelines/checklist for workers’ safety. This guidelines/checklist includes work permits and safety precautions for work during construction of Sub-stations.  (Annexure-3) and is monitored regularly by site in-charge and corporate Operation Services. In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses will be disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID will conduct awareness-building programs on such issues for the construction workers.

6.3	Environmental Problems Resulting from Operation

(i)     O&M Staff/Skills less than acceptable resulting in variety of adverse effects

The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, and air handling units, electrical panels and compressors.

Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

Exposure to Electro Magnetic Fields (EMF):  The proposed project requires only up-gradation of existing Substations. Hence, there is remote possibility of EMF exposure. In any case required electrical clearance is maintained throughout i.e. from design till O & M phase.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India:

“ POWERGRID were the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid does not appear to have any significant issues regarding PCB management and have initiated a

testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

  - Environmental Monitoring Program in POWERGRID

Monitoring is a continuous process for POWERGRID projects at all the stages be it

the site selection, construction or maintenance.

The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis, which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed, and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan:

6.4.1    Corporate Level

An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment

& Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and DHQs to carry out environmental and social surveys for new projects.

- Assisting RHQs and DHQs to finalise routes of entire power transmission line considering environmental and social factors that could arise enroute

- Help RHQs and DHQs to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & DHQs on environment and social issues and their management plan.

6.4.2     Regional Level

At its Regional Office POWERGRID has an Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the corporate level and the Divisional Headquarters. The key functions envisaged for ESMC are:

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and DHQs to finalise routes of entire power transmission lines considering the environmental and social factors that

could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and DHQs on various environmental and social aspects.

6.4.3      Site Office

At the Divisional Headquarters level, POWERGRID has made the head of the division responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social Management Team (ESMT). Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalise the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorised Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn (Table- 6.1) and will be implemented during various stage of project execution.

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan.

6.5     Critical Environmental Review Criteria

(i)	Loss of irreplaceable resources

Trees felling or loss of natural resources in this project is not involved, rather after upgrading the sub-stations plantation of trees in existing boundary is practiced in all POWERGRID projects which not only gives different aesthetic look but also enhance environmental parameter.

(ii)	 Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as cement, steel and equipment etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus, the project shall not cause any accelerated use of resources for short-term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as no forest is getting affected thus there is no possibility of endangering/causing extinction of any species.

(iv)	 Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

  - Budget estimate for EMP Implementation

The detailed item wise budget estimate is annexed as Annexure-4.

Summerised estimated cost of EMP implementation is given below:

| Item | Sub –Item | Total Cost ( Rs. in Lacs) |
| --- | --- | --- |
| Mitigation measures | As prescribed under EMP and IEE | 10.00 |
| Implementation Monitoring & Audit | As detailed under EMP | 5.00 |
| Contingency | 3 % | 0.45 |
| Total= 15.45 |  |  |

* Estimated costs are only indicative

6.7    Conclusions

From the above discussion, it may be seen that the area is not rich in physical resources and ecologically/environmentally sensitive areas are not involved. The proposed project only involves up-gradation of existing 400kV systems to establish 765/400KV, 10x500 MVA transmission capacity each at Tehri Pooling Station and at Meerut Sub-station within the existing substations; therefore, no fresh land acquisition is involved. Hence, Rehabilitation and Resettlement is not involved and environmental impact is almost negligible.  However, EMP so prepared and attached identifying potential mitigation measures to be undertaken shall be implemented during project development.

Table-6.1 Environment Management Plan

| Project activity /stage | Potential impact | Proposed mitigation measure | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-construction |  |  |  |  |  |  |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformers stated in tender specification - once | POWERGRID | Part of tender specifications for the equipment |
|  |  | Processes, equipment and systems not to use chlorofluorocarbons (CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – once | POWERGRID | Part of tender specifications for the equipment |
|  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design |
| Substation location and design | Exposure to noise | Design of plant enclosures to comply with noise regulations. | Expected noise emissions based on substation design | Compliance with regulations - once | POWERGRID | Part of equipment and process design |
| Involuntary resettlement or land acquisition | Fresh Land acquisition not involved | Not Required | Not Required | Not Required | Not Required | Not Required |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels | Noise levels to be specified in tender documents - once | POWERGRID | Part of detailed equipment design |
| Escape of polluting materials | Environmental pollution | Transformers designed with oil spill containment systems, and purpose-built oil, lubricant and fuel storage system, complete with spill cleanup equipment. | Equipment specifications with respect to potential pollutants | Tender document to mention specifications - once | POWERGRID | Part of detailed equipment design /drawings |
|  |  | Substations to include drainage and sewage disposal systems to avoid offsite land and water pollution. | Substation sewage design | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
| Explosions/Fire | Hazards to life | Design of substations to include modern fire control systems/firewalls. | Substation design compliance with fire prevention and control codes | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
|  |  | Provision of fire fighting equipment to be located close to transformers. |  |  |  |  |
| Construction |  |  |  |  |  |  |
| Equipment layout and installation | Noise and vibrations | Construction techniques and machinery selection seeking to minimize ground disturbance. | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance - once at the start of each construction phase | POWERGRID (Contractor through contract provisions) | Construction period |
| Mechanized construction | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment – estimated noise emissions | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Noise, vibration, equipment wear and tear | Turning off plant not in use. | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Temporary blockage of utilities | N/A |  |  |  |  |  |
| Site clearance | N/A |  |  |  |  |  |
| Surplus earthwork/soil | N/A |  |  |  |  |  |
| Storage of chemicals and materials | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports (type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction schedules | Noise nuisance to neighbouring properties | Local communities informed of the construction schedule. | Timing of construction (noise emissions, [dB(a)]) | Construction only - every 2 weeks | POWERGRID | Construction period |
| Provision of facilities for construction workers | Contamination of receptors (land, water, air) | Construction workforce facilities to include proper sanitation, water supply and waste disposal facilities. | Amenities for Workforce facilities | Presence of proper sanitation, water supply and waste disposal facilities - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period |
| Encroachment into farmland | Loss of agricultural productivity | N/A | N/A |  |  |  |
|  | Social inequities | N/A | N/A |  |  |  |
| Uncontrolled erosion/silt runoff | Soil loss, downstream siltation; | N/A | N/A |  |  |  |
| Nuisance to nearby properties | Losses to neighbouring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Inadequate siting of borrow areas | Loss of land values | Existing borrow sites will be used to source aggregates, therefore, no need to develop new sources of aggregates | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Health and safety | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work days caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  |
|  |  | Contractor to arrange for health and safety training sessions |  |  |  |  |
| Inadequate construction stage monitoring | Likely to maximise damages | Training of POWERGRID environmental monitoring personnel | Training schedules | Number of programs attended by each person – once a year | POWERGRID | Routinely throughout construction period |
|  |  | Implementation of effective environmental monitoring and reporting system using checklist of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof. | Submission of duly completed checklists of all contracts for each site - once |  |  |
|  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures. | Compliance report related to environmental aspects for the contract | Submission of duly completed compliance report for each contract - once |  |  |
| Operation and Maintenance |  |  |  |  |  |  |
| Oil spillage | Contamination of land/nearby water bodies | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks. | Substation bunding (Oil sump) (“as-built” diagrams) | Bunding (Oil sump) capacity and permeability - once | POWERGRID | During operations |
| Inadequate provision of staff/workers health and safety during operations | Injury and sickness of staff /workers | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |
|  |  | Safety awareness raising for staff. | Training/awareness programs and mock drills | Number of programs and percent of staff /workers covered – once each year |  |  |
|  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |
|  |  | Provide adequate sanitation and water supply facilities | Provision of facilities | Complaints received from staff /workers every 2 weeks |  |  |
| Electric Shock Hazards | Injury/mortality to staff and public | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |
|  |  | Security fences around substations | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |
|  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |
|  |  | Electricity safety awareness raising in project areas | Training /awareness programs and mock drills for all concerned parties | Number of programs and percent of total persons covered – once each year |  |  |
| Operations and maintenance staff skills less than acceptable | Unnecessary environmental losses of various types | Adequate training in O&M to all relevant staff of substations maintenance crews. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
|  |  | Preparation and training in the use of O&M manuals and standard operating practices. |  |  |  |  |
| Inadequate periodic environmental monitoring. | Diminished ecological and social values. | Power Grid staff to receive training in environmental monitoring of project operations and maintenance activities. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | Processes, equipment and systems using chlorofluorocarbons (CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government. | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels (dB (a)) | Noise levels at boundary nearest to properties and consultation with affected parties if any - once | POWERGRID | Operations |