SECTION I: PROJECT DESCRIPTION

- BACKGROUND:

In Western Region (WR), a number of generation projects having beneficiaries in WR and outside WR are envisaged to come up in next 3-4 years. In addition, Sasan and Mundra Ultra Mega Power Project (UMPP) of 4000 MW capacity each is proposed to be set up in Madhya Pradesh & Gujarat respectively in the Region. Power from these projects is proposed to be transferred to various beneficiaries in Western /Northern Region. For evacuation of power from these projects, separate transmission systems have been identified. However, for dispersal of power to major load centers in WR with reliability and security, strengthening of regional transmission system in WR in the time frame of above UMPPs has been identified and agreed in the Standing committee on Power System Planning of Western & Northern Regions. Further, it was decided to implement the above strengthening scheme as regional system strengthening scheme for WR.

1.1	OBJECTIVE OF THE PROJECT:

The objective of this system is to :

- Provide adequate transmission system for reliable transfer of power to constituents of Western Region,

- Maintain system stability, security of the combined grid (comprising Western, Eastern, North-Eastern & Northern Regions) under all operating conditions, and

- Increase the power transmission capacity of National Grid.

1.2     PROJECT DESCRIPTION

For transfer of power from Sasan UMPP, transmission corridor right from Sasan to Bina (PG) via Satna has been planned. However, system strengthening is required beyond Bina in Madhya Pradesh for transfer of power to the load centres of WR for which a separate high capacity 765 kV transmission corridor from Bina to Indore, a major load center in Madhya Pradesh along with establishment of new 765/400 kV substation at Indore, has been proposed. Further, establishment of 765/400 kV substations at Bina and Gwalior is also proposed as part of this scheme so as to facilitate charging of 765kV lines interconnecting Bina and Gwalior substations. As per load flow studies carried out for “System Strengthening in Western Region for Sasan UMPP” scheme, there would be reduction of losses of  about 285 MW in the grid.

Further, substantial power transfers are envisaged from eastern part to western part of Western Region in view of addition of a number of proposed generation projects in Chhattisgarh, Jharkhand and Orissa. A long term perspective plan towards this has been formulated. Bina-Indore 765kV corridor dovetails into this long-term transmission plan which envisages high capacity 765 kV corridor from Bina to Vadodara via Indore.

1.3        Project Highlights

| a) | Project Name | : | System Strengthening in Western Region for Sasan UMPP |
| --- | --- | --- | --- |
| b) | Location | : | Madhya Pradesh |
| c) | Beneficiary States | : | Constituents of Western Region |
| d) | Project Cost | : | Rs. 8922.68 million (USD 190.89 million) excluding IDC |

1.4         Project Scope

The scope of the transmission lines and Substations to be implemented under the above scheme is as follows:

Transmission line

- Bina (PG)-Indore(PG) 765kV S/C line, 320 Km;

- Indore(PG)-Indore(MPPTCL) 400kV D/C line (Quad), 50 Km.

Substations

- Establishment of 765/400kV, 2x1500 MVA substation at Indore (PG),

- Upgrading Gwalior Substation to 765kV by 765/400kV, 2x1500 MVA transformers,

- Upgrading Bina Substation to 765kV by 765/400kV, 2x1000 MVA   transformers.

A power map showing the transmission grid of Western Region highlighting the above scope of works is placed as Exhibit-1.

SECTION II: BASE LINE DATA

2.0  	The project is located in the State of Madhya Pradesh in India. The basic details of the area under project are given below:

2.1      MADHYA PRADESH:

PHYSIOGRAPHY: Madhya Pradesh as its name implies, lies in the heart of India.It consists of a geographical area of 3,08,250 sq. km which constitutes 9.38% of the land area of the country & bordering the states - Uttar Pradesh, Chhattisgarh, Maharashtra, Gujarat and Rajasthan. It lies between lat.210 04’&260 52’N and long.740 02’ & 820 49’ E. The land use pattern is shown in Table 2.1.

Table-2.1 Land Use Pattern

| Land use | Area in ‘000 ha | Percentage |
| --- | --- | --- |
| Total Geographic Area | 30,825 | 9.38 |
| Reporting area for land utilization | 30,756 | 99.78 |
| Forests | 8,683 | 28.17 |
| Not available for land cultivation | 3,350 | 10.87 |
| Permanent pastures and other grazing lands | 1,360 | 4.41 |
| Land under misc.tree crops & groves | 19 | 0.06 |
| Culturable Wastelands | 1,177 | 3.82 |
| Fallow land other than current fallows | 621 | 2.01 |
| Current fallows | 599 | 1.94 |
| Net area sown | 14,949 | 48.48 |

Source: land Use Statistics, Ministry of Agriculture, 2005

Climate:  The climatic condition is generally Sub-Tropical wet and dry. MP state has three main seasons:

- Winter (November through February);

- Summer (March through May); and

- Monsoon season (June through September).

Temperature:  During the winter average temperatures range from 10° to 27° C. Summers are hot, with an average temperature of 29° C and a high temperature that at times reaches 48° C. During the monsoon season temperatures average 19° to 30° C.

Rainfall:  Annual rainfall tends to decrease from south to north and from east to west.   The average rainfall in the different regions of the state ranges from 450 to 900 mm.  The heaviest rainfall in 24 hours has been recorded as 239 mm at Ujjain.

Wind: The mean wind speed at Ujjain is 10 km/hr though the wind speeds are normally recorded to exist between 4 km/hr to 20 km/hr.

Soil: Red, Yellow and black soils are generally available in the state.

Mineral Resources: Madhya Pradesh is one of the major mineral producing states of the country.  It has large deposits of a variety of minerals.  Important among them are bauxite, copper, manganese, coal, dolomite and limestone.

Water Resources: The important rivers of the state are Chambal, the Betwa, the Sone and the Narmada.

ECOLOGICAL   RESOURCES: The forest cover of the state based on the satellite data of October-December, 2004, is 76,013 Sq. km. which constitutes 24.66% of the geographic area (Map-1).  Out of these dense forest accounts for 41,082 sq.km. (Very dense-4239 sq. km. and moderately dense- 36,843 sq. km.) having  crown  density  of  more  than  40% and  open  or  degraded  forest  of  34,931 sq. km. having crown  density  ranging  between  10-40%. By legal status Reserve Forest constitutes 65.36%, Protected Forest 32.84% and unclassified Forest 1.80%. The forests are the main source of supply of fodder and fuel and subsistence for the poorest sections of the people and tribal population in the interior under-developed areas of the state. There are four forest types:

- Tropical Moist Deciduous

- Tropical Dry Deciduous

- Tropical Thorn and

- Sub Tropical Broadleaved Hill Forests.

Forests are largely distributed in central, southern and eastern parts of the undivided state of Madhya Pradesh. Northern and western parts of the state are deficient in forest vegetation. Teak and Sal are the two most important forest formations of the state, covering 18.0% and 16.7% forest area, respectively while miscellaneous forests cover 65.3%. Madhya Pradesh forest reserves are logged for Teak, Sal, Bamboo and Salai. These forests catered to the needs of the people and cattle for generations, largely because they contained good cropping species. However, population explosion and developmental needs have exerted a steadily increasing demand on the ever-diminishing extent of forests. Over-exploitation resulted in reduction of area under forests. The maximum forest cover is recorded in the district Sheopur having 55.01% of forest cover and minimum in the Ujjain district having only 0.34%.    There are 9 National Parks (NP) and 25 Wild Life Sanctuaries spread over an area of 1.1 million ha. Constituting 11.40% of the total forest area and 3.52% of the geographical area of the state. The Kanha tiger reserve in Jabalpur district is the most important National Park of the state. The State has one Biosphere Reserve called Pancmarhi Biosphere Reserve.

The lines of proposed transmission system shall pass through mainly five district of this state having forest cover ranging from 25 % to 38 %. It may be noted from the table below that in all five districts the forest cover is a good mixture of both dense and open/degraded forest nature this warrant extra precaution in routing of line through forest area. Details of forest cover of these districts are as follows:

In sq. Km.

| District | Geographic area | Dense forest | Open forest | Total | % forest Cover |
| --- | --- | --- | --- | --- | --- |
| Dewas | 7,020 | 1003 | 754 | 1,757 | 25.03 |
| Indore | 3,898 | 319 | 268 | 587 | 15.06 |
| Sehore | 6,578 | 756 | 686 | 1,442 | 21.92 |
| Shajapur | 6,195 | 1 | 120 | 121 | 1.95 |
| Sagar | 10,252 | 1,708 | 1,207 | 2,915 | 28.43 |
| Vidisha | 7,371 | 517 | 365 | 882 | 11.97 |

HUMAN  AND  ECONOMIC  DEVELOPMENT: The total population  of  Madhya  Pradesh  based on 2001  census  is 6,03,85,118  out  of  which  73.3%  is  rural  and  26.7 % is  urban. The state supports 5.9% of  country’s  total  population. The population density is 196 persons per sq.km. Sex ratio is females per thousand males are 920. Literacy in males=76.80% & in females 50.28%. Madhya Pradesh in its present form came into existence on November 1, 2000 following its bifurcation to create a new state of Chhattisgarh.

Agriculture is the mainstay of the state’s economy and 74.73% of the people are rural. Almost 49% of the land area is cultivable.   Madhya Pradesh is one of the leading states in the country in mineral production 26 percent of total Minerals of India are found in M.P. The share of M.P, in India’s production of such vital minerals as diamond (100%), dolomite (39%), bauxite (28%), iron ore (24%), coal (23%), and limestone (23%). In short about 30 different types of minerals are known to occur in the state. Recently deposits of tin and uranium have been located in this state. Rich and varied mineral deposits have been surveyed and identified in this state. Nearly 4 lac sq. Kms. area has been surveyed and mapped geologically.   A large number of automobile industries have been established at Pithambore near Indore. M.P is rich in low-grade coal suitable for power generation and has also got immense potential of hydro-energy. The total installed power generation capacity in year 2000-2001 was 2900 MW. There are eight hydro-electric power stations with installed capacity 747.5 MW.  A total of 50,271 out of 51,806 villages have been electrified by 2000-2001. Power generation is 14023.7 m Kwh. The Govt.  of  M.P  has  formed  a joint  venture (Narmada  Hydro  Electric  Development  Corporation)  with  National  Hydro-Electric Power  Corporation, a Govt.  of  India  undertaking  for  execution  of  1000MW Power from Indira  Sagar  Hydro-Electric  Project  and  520 MW Omkareshwar  Hydro-Electric  Power  Project.   The total length of roads in the state is 1,00,600 km. The length of National Highways in the state is 4,882 km while State Highways extend to 9,882 km. The main rail route linking Northern India with Southern India passes through   Madhya Pradesh. The main junctions in the state are Bhopal, Bina, Gwalior, Indore, Itarsi, Jabalpur, Katni, Ratlam, and Ujjain etc. Panchmarhi, the hill station; Dhuandhar Fall at Bhedaghat, Kanha National Park near Jabalpur are some of the major tourist attractions of the state. Maheshwar, Omkareshwar, Ujjain, Chitrakoot and Amarkantak are major centers of pilgrimage. The unique temple of Khajuraho are famous all over the world.

SECTION III:   POLICY , LEGAL & REGULATORY  FRAMEWORK

- 3.0    POWERGRID’s activities by their inherent nature and flexibility have negligible impacts on environmental and social attributes.  Indian laws relating to environmental and social issues have strengthened in the last decade both due to local needs and international commitments.  POWERGRID undertakes its activities within the purview of Indian laws keeping in mind appropriate international obligations and directives and guidelines with respect to environmental and social considerations of Funding Agencies.

- 3.1      ENVIRONMENTAL

- 3.1.1 Constitutional Provisions

Subsequent to the first United Nations Conference on Human Environment at Stockholm in June, 1972, which emphasized the need to preserve and protect the natural environment, the Constitution of India was amended through the historical 42nd Amendment Act, 1976 by inserting Article 48-A and 51-A (g) for protection and promotion of the environment under the Directive Principles of State Policy and the Fundamental Duties respectively. The amendment, inter alia provide:

"The State shall endeavour to protect and improve the environment and to safeguard the forests and wildlife of the country". (New Article 48A)

"It shall be the duty of every citizen of India to protect and improve the natural environment including forests, lakes, rivers and wildlife and to have compassion for living creatures”. (New Article 51 A(g)

Article 21 of the constitution provides, “no person shall be deprived of his life or personal liberty except according to procedure established by law”.

Article 21 is the heart of the fundamental rights and has received expanded meaning from time to time after the decision of the Supreme Court in 1978. The Article 21 guarantee fundamental right to life – a life of dignity to be lived in a proper environment, free of danger of disease and infection. The right to live in a healthy environment as part of Article 21 of the Constitution. Recently, Supreme Court has broadly and liberally interpreted the Article 21, transgressed into the area of protection of environment, and held that the protection of environment and citizen’s right to live in eco-friendly atmosphere interpreted as the basic right guaranteed under Article 21.

Thus the Indian Constitution has now two fold provision:

(a) On the one hand, it gives directive to the State for the protection and 
        improvement of environment.

  - On the other hand the citizens owe a constitutional duty to protect and

improve natural environment.

3.1.2 	MANDATORY REQUIREMENTS (NATIONAL)

- MOP order/sanction under The Electricity Act, 2003:

Sanction of MOP, GOI is a mandatory requirement for taking up any new transmission project under the section 68(1) of The Electricity Act, 2003. The sanction authorize POWERGRID to plan and coordinate activities to commission the new project. Electricity act does not explicitly deal with environmental implications of activities related to power transmission. However, POWERGRID always integrates environmental protection within its project activities.

- Forest Clearance Under The Forest (Conservation) Act, 1980:

When transmission projects pass through forest land, clearance has to be obtained from relevant authorities under the Forest (Conservation) Act, 1980.  This Act was enacted to prevent rapid deforestation and environmental degradation.  State governments cannot de-reserve any forest land or authorize its use for any non-forest purposes without approval from the Central government.  POWERGRID projects, when involving forest areas, undergo detailed review and approval procedures to obtain a Forest Clearance certificate from MOEF, Government of India before starting any construction activity in designated forest area.

- Environmental Clearances under Environment (Protection) Act,1986:

Since transmission line projects are environmentally clean and do not involve any disposal of solid waste, effluents and hazardous substances in land, air and water they are kept out of the purview of Environment (Protection) Act, 1986.  However, the recent amendment in the Environment (Protection) Act, 1986 made it necessary to obtain clearance from MoEF for power transmission projects in two districts in the Aravalis (viz., Alwar in Rajasthan and Gurgaon in Haryana). The Aravali range, in these two areas, is heavily degraded, hence, any industrial activity there becomes critical. Environment Impact Notification, 1994 lays down specific project categories that require clearance from MoEF Power transmission projects are not included in this list.

- Batteries (Management and Handling) Rules, 2001:

MOEF vide its notification dt. 16th May, 2001 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put certain restriction on disposal of used batteries and its handling. As per the notification it is the responsibility of bulk consumer (POWERGRID) to ensure that used batteries are not disposed of, in any manner, other than by depositing with the dealer/manufacturer/registered recycler/importer/reconditioner or at the designated collection centres – and to file half yearly return in prescribed form to the concerned State Pollution Control Board.

- Hazardous Wastes (Management and Handling)Amendment Rules, 2003:

MOEF vide its notification dt. 20th May, 2003 under the section of 6,8 and 25 of the Environment (Protection) Act, 1986 has put used mineral oil under the category of hazardous waste which require proper handling and disposal. As per the notification, all used oil is to be auctioned/sold to registered recyclers only and file annual return on prescribed form to the concerned State Pollution Control Board.

- Ozone Depleting Substances (Regulation and Control) Rules, 2000:

MOEF vide its notification dt. 17th July, 2000 under the section of 6, 8 and 25 of the Environment (Protection) Act, 1986 has notified rules for regulation /control of Ozone Depleting Substances under Montreal Protocol adopted on 16th September 1987. As per the notification certain control and regulation has been imposed on manufacturing, import, export and use of these compound. POWERGRID is following provisions of notification and is phasing out all equipment which uses these substances and planning to achieve CFC free organisation in near future.

- The Biological Diversity Act, 2002:

Under the United Nations Convention on Biological Diversity signed at Rio de Janeiro on the 5th day of June, 1992 of which India is also a party, MoEF has enacted the Biological Diversity Act, 2002 to provide for conservation of biological diversity, sustainable use of its components and fair and equitable sharing of the benefits arising out of the use of biological resources, knowledge and for matters connected therewith. As per the provision of act certain area which are rich in biodiversity and encompasses unique and representative ecosystems are identified and designated as Biosphere Reserve to facilitate its conservation. All restrictions applicable to protected areas like National Park & Sanctuaries are also applicable to these reserves. POWERGRID will abide by the provision of act wherever applicable and try to totally avoid these biosphere reserves while finalizing the route alignment.

    - FUNDING AGENCIES:

- WB Operational Policies (OP) 4.01/ADB’s Operations Manuals (OM)- F1/BP and JBIC Environmental Guidelines: These outlines funding agencies policy and procedures for environmental assessment (EA) of different developmental projects. All these guidelines classified developmental projects into three categories (A-C) based on its possible environmental and social impacts though WB & ADB has another category F1 applicable only to projects involving a credit line through a financial intermediary.

Transmission line projects are categorized as category-B project having limited impact that can be further minimized through mitigative/management measures and would normally require only an environmental review.  POWERGRID takes remedial measures to prevent, minimize, mitigate, or compensate for adverse impact and improve environmental performance. Environment Assessment will take account the natural environment, human health and safety, and social aspects and trans- boundary and global environmental aspects. During EA process public is also informed at every stage of project execution and their views are considered during decision-making process.

- 3.1.4   PRESCRIPTIVE FRAMEWORK (NATIONAL)

- Applicable Legislations

- 3.1.4   RELEVANT POLICIES

- National Conservation Strategy and Policy Statement on Environment and

Development, 1992

- Policy statement for Abatement of pollution, 1992

- National Environment Policy, 2006

- 3.2.0   SOCIAL

- 3.2.1 Constitutional Provisions

Constitutional provisions in regard to social safeguards are well enshrined in the preamble such as JUSTICE, social, economic and political; LIBERTY of thought, expression, belief, faith and worship; EQUALITY of status and of opportunity; FRATERNITY assuring the dignity of the individual and the unity and integrity of the Nation. Fundamental Rights and Directive Principles guarantee the right to life and liberty.  Health, safety and livelihood have been interpreted as part of this larger right.  Social safeguards provisions are dealt in detail in different Article such as Article-14, 15 17, 23, 24, 25, 46, 330, 332 etc. POWERGRID, through this document, ESPP, commits itself to implementing the said constitutional provision in true sprit to fulfil its environmental and social obligations and responsibilities.

- 3.2.2    MANDATORY REQUIREMENTS (NATIONAL)

- National Rehabilitation and Resettlement Policy, 2007:

Ministry of Rural Development, Government of India has notified a National policy on R&R for  PAFs in Feb’04 applicable to all developmental projects where 500 or more families enmass in plain areas or 250 or more families enmass in hilly areas are displaced due to project activity. It essentially addresses the need to provide succour to the assetless rural poor, support the rehabilitation efforts of the resources and provide a broad canvas for an effective consultation between PAFs and authorities responsible for their R&R. It has also listed R&R measures and entitlements for different category of PAFs. Though the National policy as such is not applicable to POWERGRID because transmission projects do not involve displacement of such a large numbers of families since land required for substations is quite small. However, the entitlement benefits listed in the National policy for PAFs have been adopted by POWERGRID in its “Social Entitlement Framework” that is being implemented wherever land acquisition for substations is undertaken.

- Rights of Way And Compensation Under Electricity Act,2003:

The act has a provision for notifying transmission company under section 164 (B) to avail benefits of eminent domain provided under the Indian Telegraph Act, 1885. MOP, GOI vide gazette notification dt 23rd Dec’03 had already notified POWERGRID under this section of said act. Therefore, for the purpose of placing of any wires, poles, etc., POWERGRID has all the powers that the telegraph authority possesses.  Thus, POWERGRID can erect and construct towers without actually acquiring the land.  However, all damages due to POWERGRID activity are compensated at market rate. Power transmission schemes are always planned in such a way that the power of eminent domain is exercised responsibly.

- Provisions Under Land Acquisition Act, 1894, as amended in 1984:

- When land is acquired for sub-stations, POWERGRID will follow procedures laid down under the Land Acquisition Act (LA Act), 1894. POWERGRID sub-stations have never resulted in large scale displacement or loss of livelihoods. There have been only marginal impacts due to flexibility exercised by POWERGRID in selecting sites. The LA Act specifies that in all cases of land acquisition, no award of land can be made by the government authorities unless all compensation has been paid.

    - FUNDING AGENCIES

For POWERGRID, mandatory requirements vis-à-vis Funding Agencies are  comprehensive Resettlement and Rehabilitation (R&R) guidelines and an entitlement framework as per World Bank Operational Directives 4.30 (OP-4.12) and 4.20 and ADB’s Operations Manual OM-F2/BP.

- World Bank OD 4.30 (OP-4.12): Involuntary Resettlement:

This directive describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement of affected groups. It requires a entitlement framework aimed at restoration, replacement and participation of affected groups. A detailed social assessment and development of an action plan having list of measures for betterment/restoration of lost assets/income is required to be submitted to bank before start of project work. However where only a few people (e.g. about 100-200 individuals) are to be relocated at a particular location, appropriate compensation for assets, logistical support for moving and a relocation grant may be the only requirements but the principle on which compensation is to be based will remain same as for larger groups.

- World Bank OD 4.20: Indigenous People (IP):

This directive describes World Bank policies and procedures for projects that affect indigenous people. The objective is to ensure that development benefits are socially and culturally compatible and that the IPs are consulted. Thus, the Indigenous People Development Plan/Tribal Development Plan is to be prepared as a prerequisite.  POWERGRID will not only incorporate the IP component whenever necessary, but will also pay attention to marginalized groups such as women, children, etc.

- ADB Operations Manual –F2/BP: Involuntary Resettlement:

The OP describes Bank Policy and procedures on involuntary resettlement as well as conditions that borrowers are expected to meet during operations involving resettlement. Its objective is to avoid such resettlement as far as possible if unavoidable measures like assistance to affected persons for restoration of their assets/livelihood as would have been in the absence of project. It also classified project into three categories like category-A where resettlement is significant and involve physical displacement of more than 200 persons, which require a detailed resettlement plan. Category-B where resettlement is not that significant and requires a short resettlement plan. Category-C where no resettlement of peoples are foreseen and neither require resettlement plan nor a resettlement framework.

POWERGRID emphasizes that displacement is not an issue with transmission projects because land below tower/line is not acquired and only a small piece of land is required for substations. However, all affected persons/families shall be provided compensation and rehabilitation assistance along with other measures as per POWERGRID's social entitlement framework which is based on these directives/manuals and National R&R Policy to restore income/livelihood of all affected persons.

- 3.2.4    PRESCRIPTIVE FRAMEWORK (NATIONAL)

- National and State-wide Laws and Policies Relating to Land Acquisition and Issues of R&R

    - Madhya Pradesh Pariyojana Ke Karan Visthapit Vyakti (Punsthapan) Adhiniyam, 1985

    - Maharashtra Project Affected persons Rehabilitation Act, 1986

- 3.2.5   RELEVANT POLICIES

- Resettlement and Rehabilitation Policy- Coal India Ltd., May,2008

- Resettlement and Rehabilitation Policy- NHPC Ltd., 2007

- Policy for Rehabilitation and Resettlement of Land Owners – Land Acquisition Oustees – Haryana State, December,2007;

- The Orissa Resettlement and Rehabilitation Policy, Orissa , May, 2006;

- Resettlement and Rehabilitation Policy – NTPC Ltd., June,2005;

# SECTION-IV: POWERGRID APPROACH FOR ROUTE SELSCTION

- 4.0     ROUTE SELECTION - (ASSESSMENT & MANAGEMENT PROCESS)

- At the system planning stage itself one of the factors that govern the evolution of system is the possible infringement with the forest. Wherever such infringements are substantial, different alternative options are considered. The route/ site selection criteria followed by POWERGRID is detailed below:

- While identifying the transmission system for a generation project or as a part of National Power Grid, preliminary route selection is done by POWERGRID based on the Top sheets of Survey of India and Forest Atlas (Govt. of India's Publication).  During route alignment all possible efforts are made to avoid the forest area involvement completely or to keep it to the barest minimum, whenever it becomes unavoidable due to the geography of terrain or heavy cost involved in avoiding it.

- 4.1.1   STUDY OF ALTERNATIVES

- Environmental Criteria for Route selection

- For selection of optimum route, the following points are taken into consideration:

- The route of the proposed transmission lines does not involve any human rehabilitation.

- Any monument of cultural or historical importance is not affected by the route of the transmission line.

- The proposed route of transmission line does not create any threat to the survival of any community with special reference to Tribal Community.

- The proposed route of transmission line does not affect any public utility services like playgrounds, schools, other establishments etc.

- The line route does not pass through any sanctuaries, National Park etc.

- The line route does not infringe with area of natural resources.

- In order to achieve this, POWERGRID undertakes route selection for individual transmission lines in close consultation with representatives from the Ministry of Environment and Forests and the Department of Revenue. Although under National law POWERGRID has right of eminent domain yet alternative alignments are considered keeping in mind the above-mentioned factors during site selection, with minor alterations often added to avoid environmentally sensitive areas and settlements at execution stage.

- As a rule, alignments are generally cited 10-15 km away from major towns, whenever possible, to account for future urban expansion.

- Similarly, forests are avoided to the extent possible, and when it is not possible, a route is selected in consultation with the local Divisional Forest Officer, that causes minimum damage to existing forest resources.

- Alignments are selected to avoid wetlands and unstable areas for both financial and environmental reasons.

- In addition, care is also taken to avoid National parks, sanctuaries, eco-sensitive zones, tiger reserves and biosphere reserves etc.

- Keeping above in mind the routes of proposed lines under the project has been so aligned that it takes care of above factors. As such different alternatives were studied with the help of Govt. published data like Forest atlas, Survey of India topo maps etc. to arrive at most optimum route which can be taken up for detailed survey and assessment of environmental & social impacts for their proper management.

    - Evaluation of alternatives Route Alignment of 765 KV S/C  Bina-Indore Line:

- Four different alignments (Map-2) were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey. The comparative details of these three alternatives are as follows:

| Sl.NO. | Description | Alternative-I | Alternative-II | Alternative-III |
| --- | --- | --- | --- | --- |
| 1. | Route particulars i) Length in Kms ii) Terrain – (a) Plain (b) Hilly | 320 Kms 95% 05% | 323 Kms 95% 05% | 328 Kms 90% 10% |
| 2. | Environmental impact - Town in alignment - House within ROW - Tree/Crops and its extent of damage - Forest involvement - Type of forest - Reserve Forest (RF) - Protected Forest (PF) - Density of forest - Type of founa & floura - Endangered species, if any - Historical/cultural monuments vi)Details of Tribal areas, if any | The line is not passing through any town however major town in vicinity are Bina, Vidisha,Sehor,Shujalpur,Dewas,Indore. Nil To be ascertained after detailed survey. Nil Nil Nil Nil | The line is not passing through any town however major town in vicinity are Bina, Vidisha,Sehor, Sonkutchh,Dewas, Indore. Nil To be ascertained after detailed survey. Nil Nil Nil | The line is not passing through any town however major town in vicinity are Bina, Sironj, Sehor, Shujalpur,Dewas,Indore. Nil To be ascertained after detailed survey. 12.57Km / 81.6 ha. RF-7.00 Km/45ha 0.1 Flora:Teak, Babool, Banyan, Neem, Palash,Tamarind & other Misc trees. Fauna : Fox, Rabitt,Dear, Jackal, etc. Nil Nil Nil |
| 3 | Compensation Cost - Towards forest clearance - Tree/crop | Nil Can be assessed after tower spotting | nil Can be assessed after tower spotting | Rs. 460 lakh (Including NPV @ 8 lakh/ha.) Can be assessed after tower spotting |
| 4. | Major Crossings: River Power line Railway line | 10 12 6 | 10 10 5 | 10 04 4 |
| 4. | Construction problems | Easy approaches, Less ROW problems, as the route is passing through plain terrain. | Right of way problems are more due to more damage of trees/crops. Comparatively more difficult approaches.. | Due to involvement of Reserve forest area approaches are difficult , More ROW Problems. |
| 5 | O&M problems | O&M shall be more easy due to non involvement of forest, Better approaches, plain terrain. | Comparatively difficult for O&M. | O&M shall be difficult due to involvement of Reserve forest area, difficult approaches, . |
| 6 | Overall remarks | Least route length with no forest involvement, Less ROW problems, Easy approaches , therefore recommended | More route length, more ROW problems, difficult approaches, hence not recommended. | Route length is maximum, as forest is involved in this route, more project cost, more ROW problems, More difficult approaches hence not recommended. |

From the above comparison of the three different alternative it is evident that the  Alternative-I is the most optimum route without involvement of forest, thereby avoiding ecological sensitive areas. Moreover the route length is minimum hence project cost will be less.  The other alternatives are more in route length and involve forest area in alternate -III, having more difficult approaches, more ROW problems.  Hence Alternative-I has been found to be most optimum and recommended for detailed survey.

    - Evaluation of alternatives Route Alignment of 400 KV D/C Indore (PG) – Indore (MPPTCL) Line

Three different alignments were studied with the help of published data/maps and walkover survey to arrive at most optimum route for detailed survey (Map-3). The comparative details of these three alternatives are as follows:

| Sl.NO | Description | Alternative-I | Alternative-II | Alternative-III |
| --- | --- | --- | --- | --- |
| 1. | Route particulars i) Length in Kms ii) Terrain – (a) Plain (b) Hilly | 54 Kms 95% 05% | 51 Kms 95% 05% | 48 Kms 90% 10% |
| 2. | Environmental impact - Town in alignment - House within ROW - Tree/Crops and its extent of damage - Forest involvement - Type of forest - Reserve Forest (RF) - Protected Forest (PF) - Density of forest - Type of fauna & flora - Endangered species, if any - Historical/cultural monuments vi)Details of Tribal areas, if any | The line is not passing through any town however route is around Indore city. Nil To be ascertained after detailed survey. NIL NA NA Nil Nil Nil | The line is not passing through any town however big housing projects are coming up in this route. Nil To be ascertained after detailed survey. NIL NA NA Nil Nil Nil | The line is not passing through any town however big housing projects are coming up in this route. Nil To be ascertained after detailed survey. NIL NA NA Nil Nil Nil |
| 3 | Compensation Cost - Towards forest clearance - Tree/crop | Nil Shall be assessed after tower spotting | Nil shall be assessed after tower spotting | Nil Shall be assessed after tower spotting |
| 4. | Major Crossings: River Power line Railway line | nil 09 01 | nil 09 01 | nil 09 01 |
| 5. | Construction problems | Easy approaches, Less/negligible ROW problems. | Right of way problems are more due to upcoming housing projects enroute. Comparatively more difficult approaches.. | Right of way problems are more due to upcoming housing projects enroute. Comparatively more difficult approaches. |
| 6. | O&M problems | O&M shall comparatively be easy due to better approaches. | Comparatively difficult | Difficult for O&M due to some difficult approaches, . |
| 7. | Overall remarks | Though route length is more however due to less ROW problems easy approaches, this route is recommended | More ROW problems, hence not recommended. | More ROW problems, difficult approaches hence not recommended. |

From the above comparison of the three different alternative it is evident that the, Alternative-I is the most suitable route, though the route length is more however avoidance of forest,  habitation and developing area of the city is possible and achieved.  Hence Alternative–I has been found to be most optimum and recommended for detailed survey.

# SECTION-V: SCREENING OF POTENTIAL ENVIRONMENTAL IMPACT, EVALUATION AND ITS MANAGEMENT

- 5.0	Impact Due to Project Location and Design

Environmental impact of transmission line projects are not far reaching and are mostly localized to ROW.  However, transmission line project has some affects on natural and socio-culture resources.  These impacts can be minimized by careful route selection. In order to get latest information and further optimization of route  modern survey techniques/tools like GIS, GPS aerial photography are also applied.  Introduction of GIS and GPS in route selection result in access to updated/latest information, through satellite images and further optimization of route having minimal environmental impact. Moreover, availability of various details, constraints like topographical and geotechnical details, forest and environmental details etc. help in planning the effective mitigative measures including engineering variations depending upon the site situation/location. In the instant project also these techniques are used and detail survey using GIS/GPS is under progress.  Although, all possible measures have been taken during the finalization of route alignment for the proposed transmission line but due to peculiarity of terrain and demography of the area where project is being implemented, some environmental impacts may be there. The explanations in brief with regard to specific environment review criteria based on preliminary survey are as follows:

- (i)	Resettlement

- As described earlier all measures are undertaken by POWERGRID at line routing stage itself to avoid settlements such as cities, villages etc. It may be seen from the above description of proposed route alignment and also keeping in mind that no land is acquired for tower foundation as per existing law, the project does not require any resettlement of villagers.

The proposed project involve construction of one new substation at Indore where approx. 39 ha. Land is being acquired for proposed substation and extension of two existing substation Bina and Gwalior where sufficient land is already available for proposed bays. Hence fresh land acquisition and R&R issues are only involved in the Indore substation.  As per the provisions of ESPP, M/s Center for Management & Social Research (CMSR), Hyderabad has been entrusted with the work of detailed social assessment including socio-economic survey and preparation of Rehabilitation Action Plan( RAP)   Based on the outcome of such assessment a detailed RAP shall be prepared listing all measures for rehabilitation of PAPs and community development work to be undertaken for up-liftment / improvement of infrastructure etc. of affected village (the issue of land acquisition and R&R shall be dealt  in detail separately under RAP.

- (ii)	 Land value depreciation

- Based on past experience land prices are generally expected to rise in the areas receiving power.  Further, transmission lines generally pass through uninhabited area, agriculture fields and forests, where the land-use is not going to change in foreseeable future. Therefore, the value of land will not be adversely affected to a significant degree.

- (iii) Historical/cultural monuments/value

- As per the POWERGRID’s policy of route selection only that route alignment is finalized this avoids all the historical and cultural monuments. As per the preliminary assessment carried out during finalization of route alignment in consultation with State revenue authorities and ASI, no such monuments are coming in the proposed route alignment.

- (iv)	 Encroachment into precious ecological areas

- As already explained all precautions have been taken to avoid routing of line through forest and ecological sensitive areas and National park/Sanctuaries.  It may be noted   from  the  above description that with due care during the route alignment  forest and protected areas have been completely avoided in the transmission lines covered under the scheme.

- (v)  Encroachment into other valuable lands

- Impacts on agricultural land will be restricted to the construction phase and when large-scale maintenance measures are required. Some stretch of the line will pass through Agricultural fields. Agricultural land will be lost at the base of the tower, which is estimated to be 0.2-1 sq. m per average farm holding (Fig-1).  The innovative tower design followed in case of 765 KV line has already reduced the Right of Way (ROW) requirement to 64 m from 85 m a reduction of 21 m will further reduced the impact on cultivable land.

- In areas where lines will traverse agricultural land, compensation will be paid to owners for any crop damage incurred as a result of construction activities. POWERGRID field staff will consult affected villagers and local revenue department and apprise him about the project and tower location, which shall be erected in the agricultural land, for compensation.  Revenue department after evaluating the loss due to construction activity and productivity of land arrives at the compensation cost which is paid to farmer.  Agricultural activities will be allowed to continue following the construction period. If bunds or other on-farm works are disturbed during construction or maintenance, they will be restored to the owner's satisfaction following cessation of construction or maintenance activities. In the event that private trees are felled during construction or maintenance operations, compensation will be paid to the owner in an amount determined by the estimated loss of products from the tree over an eight year period (for fruit bearing trees). Agricultural lands under private ownership will be identified, and in accordance with normal POWERGRID procedures compensation will be paid to the affected villagers (Annexure-1).  Budgetary provision Rs. 1179.23 lakh @ Rs. 3.50 lakh per Km for 765 KV line and Rs. 1.00 lakh for 400 KV line is made in the cost estimate to meet these expenses.

- (vi)	 Interference with other utilities and traffic

- As per regulations enacted by Government of India, it is mandatory for POWERGRID to seek clearance prior to construction from department of Railways, Telecommunications and wherever necessary from aviation authorities that are likely to be affected by the construction of transmission lines. The transmission lines affect nearby telecommunication circuits by causing electrical interference.  A standing committee -- Power Telecom Co-ordination Committee (P.T.C.C.) has been constituted by Government of India to plan and implement the mitigating measures for the induced voltage which may occur to nearby telecom circuit and suggest necessary protection measures to be adopted. The committee suggests measures like rerouting of the telecom circuits, conversion of overhead telecom circuits into cables etc. to minimize the interference.

- The cost of such measures is determined by the Committee and is shared by POWERGRID and Telecom Department on the basis of prevailing norms and guidelines.  Though the exact cost to mitigate the impacts of induction in neighboring telecom circuits would vary from case to case, the cost on an average works out to be Rs.50000/- per km for POWERGRID. Provision to meet these expenses has been made in the cost estimate for the same.

- Wherever transmission line crosses the railways, clearance is taken from that department.  In general, the system is planned and executed in such a way that adequate clearance is maintained between transmission lines on the one hand, and railways, civil aviation and defense installations on the other.  Wherever the transmission lines pass by the airports the towers beyond specified height are painted in alternate orange and white stripes for easy visibility and warning lights are placed atop these towers.

- (vii) Interference with drainage pattern

- As the transmission lines are constructed aerially and the blockage of ground surface is limited to area of tower footings, which is very small, there is little possibility of affecting drainage pattern.  In the infrequent instances where the drainage is affected, flow will be trained and guided to safe zones.

- 5.1	Environmental Problems Due to Design

- (i)	Escape of polluting materials

- The equipments installed on lines and substations are static in nature and do not generate any fumes or waste materials.

- (ii)	Explosion/fire hazards

- During the survey and site selection for transmission lines and sub-stations, it has been ensured that these are kept away from oil/gas pipelines and other sites with potential for creating explosions or fires.

- Fires due to flashover from lines can be a more serious problem in forest. However, adequate safety measures shall be taken to avoid such incidence besides this forest authorities also incorporate measures like making fire lines to prevent spreading of fire in the affected forest area.

- (iii)   Erosion hazards due to inadequate provision for resurfacing of exposed area

- Adequate measures are taken to re-surface the area where excavation works are done.  Topsoil disturbed during the development of sites will be used to restore the surface of the platform. Infertile and rocky material will be dumped at carefully selected dumping areas and used as fill for tower foundations.

- (iv)	 Environmental aesthetics

- Since spacing between the towers in case of 765 KV lines is approx. 400 meters these will not affect the visual aesthetics of the localities particularly when it is ensured to route the lines as far away from the localities as possible.   POWERGRID takes up plantation of trees to buffer the visual effect around its substations and to provide better living conditions.  Wherever POWERGRID feels it appropriate, discussions will be held with local Forest Department officials to determine feasibility of planting trees along roads running parallel to transmission lines to buffer visual effect in these areas.  In addition, towers may be painted grey or green to merge with the background.

- (v)	 Noise/vibration nuisances

- The equipment installed at sub-station are mostly static and are so designed that the noise level always remains within permissible limits i.e. 85 dB as per Indian standards. The noise levels reported during normal operating conditions are about 60 to 70 dB at 2 m. distance from the equipment.  To contain the noise level within the permissible limits whenever noise level increases beyond permissible limits, measures like providing sound and vibration dampers and rectification of equipment are undertaken.  In addition, plantations of sound absorbing species like Casuarinas, Tamarind, and Neem are raised at the sub-stations that reduce the sound level appreciably. It is reported that 93 m3 of woodland can reduce the noise level by 8 dB.  Actual noise levels measured at perimeters of existing Substations are 20 to 30 dB.

- (vi)  Blockage of wildlife passage

- Since the line is passing through mostly agricultural, wasteland and no  forest area, migration path of wild life etc. are involved, hence possibility of disturbance to wild life are nil/remote.

- 5.2	Environmental Problems During Construction Phase

- (i)	Uncontrolled silt runoff

- The Project involves only small scale excavation for tower foundations at scattered locations in plain area that are re-filled with excavated material therefore uncontrolled silt run off is not expected.

- (ii)   Nuisance to nearby properties

- As already described in preceding paras, during site selection due care is taken to keep the transmission line and substations away from settlements.  Further, all the construction activities will be undertaken through the use of small mechanical devices e.g. tractors and manual labour therefore nuisance to the nearby properties if any, is not expected.

- (iii)  Interference with utilities and traffic and blockage of access way

- Access to the site will be along existing roads or village paths; minor improvements to paths may be made where necessary, but no major construction of roads will be necessary either during construction or as a part of maintenance procedures. Even if at some places it is found that access road is not available than existing field/path may be utilized and compensation for any damage to crop or field is paid to the owner.

- As and when a transmission line crosses any road/ railways line, the short span angle (DT) towers are located at a distance so as not to cause any hindrance to the movement of traffic.  Stringing at the construction stage is carried out during lean traffic period in consultation with the concerned authorities and angle towers are planted to facilitate execution of work in different stages.

- (iv)	Inadequate resurfacing for erosion control

- The proposed lines are to be constructed in plain area where erosion problem is not anticipated. However, if due to terrain at some points transmission towers may be placed on slopes and erosion prone soils, internationally accepted engineering practices will be undertaken to prevent soil erosion.  This will include cutting and filling slopes wherever necessary.  The back cut slopes and downhill slopes will be treated with revetments. As explained above adequate steps shall be taken to resurface the area after construction. Wherever sites are affected by active erosion or landslides, both biological and engineering treatment will be carried out, e.g. provision of breast walls and retaining walls, and sowing soil binding grasses around the site. Furthermore, construction is generally undertaken outside the rainy season. As the proposed line is mostly passing through plain areas these problems are not anticipated.

- (v)	Inadequate disposition of borrow area

- As mentioned earlier the transmission tower foundations involve excavations on small scale basis and the excavated soil is utilized for back filling.  In case of substations generally the sites are selected in such a manner that the volume of cutting is equal to volume of filling avoiding borrowing of the area. As such acquisition/opening  of  borrow area is not needed.

- (vi)    Protection of Worker's health/safety

The Safety Regulations/Safety Manual published by POWERGRID, and included in tender documents will guide provisions for workers’ health and safety. Various aspects such as, work and safety regulations, workmen's compensation, insurance are adequately covered under the General Conditions of Contract (GCC), a part of bidding documents. As a deterrent or to minimize accident during construction a provision in the contract has been added that stipulates a fine/penalty of Rs.10 lakhs for each accidental death and Rs1.0 lakh/each for any injury and is deducted from the contractor’s payment and paid to the deceased/affected family, in case of occurrence (Annexure-2).

POWERGRID has a dedicated unit to oversee all health and safety aspects of its project under the Operation Service Department POWERGRID has framed guidelines/checklist for workers’ safety as its personnel are exposed to live EHV apparatus and transmission lines. These guidelines/checklist include work permits and safety precautions for work on the transmission lines both during construction and operation (Annexure-3) and is monitored regularly by site in-charge and corporate Operation Services. In addition training is imparted to the workers in fire fighting and safety measures.  Safety tools like helmet, safety belt, gloves etc. are provided to them in accordance to the provisions of Safety Manual.  First aid facilities will be made available with the labour gangs, and doctors called in from nearby towns when necessary.  The number of outside (skilled) labourers will be quite small, of the order of 25-30 people per group. The remaining workforce of unskilled labourers will be comprised of local people.  Workers are also covered by the statutory Workmen (Compensation) Act.  Regular health checkups are conducted for construction workers. The construction sites and construction workers’ houses will be disinfected regularly if required. In order to minimize/checking of spread of socially transmitted diseases e.g. HIV/AIDS etc. POWERGRID will conduct awareness building programs on such issues for the construction workers.

- 5.3	Environmental Problems Resulting from Operation

- (i)     O&M Staff/Skills less than acceptable resulting in variety of adverse effects

- The O&M program in POWERGRID is normally implemented by sub-station personnel for both, the lines as well as sub-stations.  However in respect of the long distance transmission lines, there are monitoring offices that are located at various points en-route. Monitoring measures employed include patrolling and thermo-vision scanning.

- The supervisors and managers entrusted with O&M responsibilities are intensively trained for necessary skills and expertise for handling these aspects.

- A monthly preventive maintenance program will be carried out to disclose problems related to cooling oil, gaskets, circuit breakers, vibration measurements, contact resistance, condensers, air handling units, electrical panels and compressors. Any sign of soil erosion is also reported and rectified. Monitoring results are published monthly, including a report of corrective action taken and a schedule for future action.

POWERGRID is following the approved international standards and design, which are absolutely safe. Based on the studies carried out by different countries on the safety of EHV lines in reference to EMF affect POWERGRID have also carried out such studies with the help of PTI, USA and CPRI, Bangalore on their design. The studies inferred that the POWERGRID design are safe and follow the required international standard.  Because of issues relating to need to ensure health and safety relating to the line such as fire safety, safe voltages on metallic parts of buildings, and safety clearances to avoid flashover, the transmission lines will not pass directly over any residential properties and as such the potential for EMF effects to occur will be further diminished. Given that it will be necessary to ensure that there are no properties in the ROW beneath and to the sides of the overhead line, automatic mitigation against EMF will be provided between the source of potentially high strengths (the transmission line) and the residential properties.

Poly Chlorinated Biphenyls (PCBs) due to its high heat capacity, low flammability and low electrical conductivity was extensively used as insulating material in capacitors and transformers. But after the finding that these PCBs are non-biodegradable and has carcinogenic tendency, its use in electrical equipments as insulating medium has been banned all over the world long back. However, it has been reported in some studies that chances of contamination of oil with PCB is possible. Keeping that in mind, POWERGRID has taken all possible steps in association with NGC, UK and setup Regional testing laboratories for testing of existing oil for PCB traces and results of this suggests that PCB contamination is not an issue with POWERGRID. The World Bank has also made following comments after a detailed study on Management of PCBs in India:

“ Power Grid was the most advanced in testing for PCBs of the organizations visited for this project. They have established a procedure for identification of the presence of PCBs in transformer oil and more detailed analysis for positive identification sample. To date no significant concentrations of PCBs have been detected. Power Grid do not appear to have any significant issues regarding PCB management and have initiated a testing program. The experience & laboratories of Power Grid could be used to provide a national PCB auditing service”.

5.4    CRITICAL ENVIRONMENTAL REVIEW CRITERIA

(i)	Loss of irreplaceable resources

The transmission projects do not involve any large scale excavation and land is lost to the extent of 0.2-1 sq m only for each foundation. As the lines in the subject project are not passing through any forest area, the problem of losing natural resources in this project is not envisages. Moreover, with the provision of water harvesting in the all the three substations, it is estimated that such provision shall help in recharging the ground water resulting in conservation of precious natural resource like water.

(ii)	 Accelerated use of resources for short-term gains

The project will not be making use of any natural resources occurring in the area during construction as well as maintenance phases.  The construction material such as tower members, cement etc shall come from factories while the excavated soil shall be used for backfilling to restore the surface.  Thus the project shall not cause any accelerated use of resources for short term gains.

(iii)   Endangering of species

No endangered species of flora and fauna exist in the project area as well as no reserve forest is getting  affected thus there is no possibility of endangering/causing extinction of any species.

(iv)	Promoting undesirable rural-to urban migration

The project will not cause any submergence or loss of land holdings that normally trigger migration. It also does not involve acquisition of any private land holdings.  Hence, there is no possibility of any migration.

5.6    PUBLIC CONSULTATION:

Public consultation/information is an integral part of the project implementation. Public is informed about the project at every stage of execution. During survey also POWERGRID’s site officials meet people and inform them about the routing of transmission lines.  During the construction, every individual, on whose land tower is erected and people affected by ROW, are consulted. Public consultation using different technique like Public Meeting, Small Group Meeting, informal Meeting as per Environmental Social Policy & Procedures of POWERGRID (ESPP) shall also be carried out during different activities of project cycle. During such consultation the public will be informed about the project in general and in particular about the following:

- complete project plan (i.e. its route and terminating point and substations, if any, in between);

- POWERGRID design standards in relation to approved international standards;

- Health impacts in relation to EMF;

- Measures taken to avoid public utilities such as school, hospitals, etc.;

- Other impacts associated with transmission lines and POWERGRID’s approach to minimizing and solving them;

- Land acquisition details, proposed R&R measures and compensation packages in line with POWERGRID’s policy;

- Trees and crop compensation process.

Apart from organising many informal group meetings in different villages (Table-5.1) a public meeting was also organised in the village Hoshiyari and Siya of Dewas district of Madhya Pradesh on 06.05.09 and 14.5.09. To get the maximum participation during the Public consultation Programme a notice was served well in advance to the villagers (Annexure-4 ).  The   details of line and its importance were explained to the villagers by the Chief Manager/Ujjain.  About fifty villagers including Sarpanch,   and POWERGRID representative participated in the programme (Plate-1&2).  Pamphlets in local language were circulated during the programme amongst the participants (Annexure-5). The programme was arranged in interactive way and queries like crop compensation etc. were replied.  The programme was appreciated by the villagers including Sarpanch and they assured to extend their cooperation for construction of transmission project. The process of such consultation shall continue during project implementation and even during O&M stage.

Table-5.1: PUBLIC CONSULTATION ENROUTE OF TRANSMISSION LINES

| S No. | Line / Date | Village Name | Person attended |
| --- | --- | --- | --- |
| 1. a. | 400KV D/C INDORE – INDORE LINE 15th MAY 2009 Distt.-INDORE | 1. Asrawad Khurd 2. Jamuniya Khurd 3. Umaria Khurd 4. Amba moliya | Village Sarpanch and general/interested resident of village |
| b. | 18th May 2009 Distt.-INDORE | 1. Begam Kheri 2. Sulakhera 3. Dakachiya | -Do- |

5.7	CONCLUSIONS:

From the above discussion, it would seem that the area is rich in physical resources. But careful route selection has avoided involvement of forest area any National Park/Sanctuary or protected area. Thus route selected for detailed survey is the most optimum route alignment and do not involved or any other protected area. The fresh land acquisition of 39 ha. Is only involved affecting around 64 persons. The R&R issues of such PAPs are being dealt as per the provisions of ESPP.  Hence, based on the criteria agreed with Bank the project is coming under the P2 category.  The infrastructural constraints are very real and pose a limiting factor on the development of the area.  The above facts while on the one hand underline the need for implementation of the project for overall development of the area and on another hand suggest that a detailed E.I.A. may not be necessary.

# SECTION-VI:  MONITORING AND ORGANISATIONAL SUPPORT STRUCTURE

6.0      ENVIRONMENTAL MONITORING PROGRAM IN POWERGRID:

- Monitoring is a continuous process for POWERGRID projects at all the stages be it the site selection, construction or maintenance.

- The success of POWERGRID lies in its strong monitoring systems.  Apart from the site managers reviewing the progress on daily basis regular project review meetings are held at least on monthly basis which is chaired by Executive Director of the region wherein apart from construction issues the environmental aspects of the projects are discussed and remedial measures taken wherever required.  The exceptions of these meetings are submitted to the Directors and Chairman and Managing Director of the Corporation.  The progress of various on-going projects is also informed to the Board of Directors. Following is the organization support system for proper implementation and monitoring of Environmental & Social Management Plan :

- 6.1    Corporate Level

- An Environmental Management Cell at corporate level was created within POWERGRID in 1992 and subsequently upgraded to an Environment Management Department (EMD) in 1993 and in 1997 it has been further upgraded to Environment & Social Management Deptt. (ESMD) by incorporating social aspect of project. Briefly, the ESMD's responsibilities are as follows:

- Advising and coordinating RHQs and Site to carry out environmental and social surveys for new projects.

- Assisting RHQs and site to finalize routes of entire power transmission line considering environmental and social factors that could arise enroute

- Help RHQs and Site to follow-up with the state forest offices and other state departments in expediting forest clearances and the land acquisition process of various ongoing and new projects

- Act as a focal point for interaction with the M0EF for expediting forest clearances and follow-ups with the Ministry of Power.

- Imparts training to POWERGRID’s RHQs & site Officials on environment and social issues and their management plan.

- 6.2     Regional Level

- At its Regional Office POWERGRID has a Environmental and Social Management cell (ESMC) to manage Environmental and Social issues and to coordinate between ESMD at the Corporate level and the Divisional Headquarters. The key functions envisaged for ESMC are:

- Advising and coordinating field offices to carry out environmental and social surveys for new projects envisaged in the Corporate Investment Plan

- Assisting the ESMD and site to finalize routes of entire power transmission lines considering the environmental and social factors that could arise en-route

- To follow-up forest clearances and land acquisition processes with state forest offices and other state departments for various ongoing and new projects

- Acting as a focal point for interaction with the ESMD and site on various environmental and social aspects.

- 6.3      Site Office

- At the Divisional Headquarters level, POWERGRID has made the head of the division responsible for implementing the Environmental and Social aspect of project and are termed as Environmental and Social Management Team (ESMT) . Key functions of the ESMT are:

- Conduct surveys on environmental and social aspects to finalize the route for the power transmission projects

- Conduct surveys for the sites to being considered for land acquisition

- Interact with the Forest Departments to make the forest proposal and follow it up for MOEF clearance.

- Interact with Revenue Authorities for land acquisition and follow it up with Authorized Agencies for implementation of Social Management Plan (SMP).

- Implementation of Environment  Management Plan (EMP) and SMP

- Monitoring of EMP and SMP and producing periodic reports on the same.

It may be noted that POWERGRID is well equipped to implement and monitor its environment and Social Management plans.

- As regards monitoring of impacts on ecological resources particularly in Forest, Sanctuary or National Park, it is generally done by the concerned Divisional Forest Officer, Chief Wildlife Warden and their staff as a part of their normal duties.  As no forest area is involved in the instant project such monitoring/provision shall not be applicable to proposed project.  A detailed Environment Management Plan (EMP) including monitoring plan for all possible environmental and social impact and its proper management has been drawn (Table- 6.1) and will be implemented during various stage of project execution. A budget estimate towards tree/crop compensation and EMP implementation is prepared and is placed at Annexure-6. Since many provisions of EMP are to be implemented by contractor hence for proper monitoring EMP has included in the contract document (Annexure-7).

Environmental Review: Periodic review by corporate ESMD and higher management including review by POWERGRID CMD of all environmental and social issues is under taken to ensure that EMP and other measures are implemented at site. Besides it annual review by independent Auditor under ISO: 14001 shall also be undertaken for compliance of agreed policy and management plan.

Table- 6.1 Environment Management Plan

| Project activity /stage | Potential impact | Proposed mitigation measure | Parameter to be monitored | Measurement and frequency | Institutional responsibility | Implementation schedule |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-construction |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Tower location and line alignment selection with respect to nearest dwellings | Setback distances to nearest houses - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | PCBs not used in substation transformers or other project facilities or equipment. | Transformer design | Exclusion of PCBs in transformers stated in tender specification - once | POWERGRID | Part of tender specifications for the equipment |
|  |  | Processes, equipment and systems not to use chlorofluorocarbons (CFCs), including halon, and their use, if any, in existing processes and systems should be phased out and to be disposed of in a manner consistent with the requirements of the Government | Process, equipment and system design | Exclusion of CFCs stated in tender specification – once | POWERGRID | Part of tender specifications for the equipment |
|  |  |  |  | Phase out schedule to be prepared in case still in use – once |  | Part of equipment and process design |
| Transmission line design | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Electromagnetic field strength for proposed line design | Line design compliance with relevant standards - once | POWERGRID | Part of detailed alignment survey and design |
| Substation location and design | Exposure to noise | Design of plant enclosures to comply with noise regulations. | Expected noise emissions based on substation design | Compliance with regulations - once | POWERGRID | Part of detailed siting survey and design |
| Location of transmission towers and transmission line alignment and design | Impact on water bodies and land | Consideration of tower location at where they could be located to avoid water bodies or agricultural land. | Tower location and line alignment selection (distance to water and/or agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of tower siting survey and detailed alignment survey and design |
|  | Social inequities | Careful route selection to avoid existing settlements | Tower location and line alignment selection (distance to nearest dwellings or social institutions) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
|  |  | Minimise need to acquire agricultural land | Tower location and line alignment selection (distance to agricultural land) | Consultation with local authorities and land owners - once | POWERGRID | Part of detailed tower siting and alignment survey and design |
| Involuntary resettlement or land acquisition | Social inequities | Compensation paid for temporary/ permanent loss of productive land as per LAA & its process | RAP implementation | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction phase |
| Encroachment into precious ecological areas | Loss of precious ecological values/ damage to precious species | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest designated ecological protection area) | Consultation with local forest authorities - once | POWERGRID | Part of detailed siting and alignment survey /design |
| Transmission line through forestland | Deforestation and loss of biodiversity | Avoid encroachment by careful site and alignment selection | Tower location and line alignment selection (distance to nearest protected or reserved forest) | Consultation with local authorities - once | POWERGRID | Part of detailed siting and alignment survey/design |
|  |  | Minimise the need by using existing towers, tall towers and RoW, wherever possible |  | Consultation with local authorities and design engineers - once |  |  |
|  |  | Obtain statutory clearances from the Government | Statutory approvals from Government | Compliance with regulations – once for each subproject |  |  |
| Encroachment into farmland | Loss of agricultural productivity | Use existing tower footings/towers wherever possible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
|  |  | Avoid siting new towers on farmland wherever feasible | Tower location and line alignment selection | Consultation with local authorities and design engineers - once |  | Part of detailed siting and alignment survey /design |
|  |  | Farmers compensated for any permanent loss of productive land | Design of Implementation of Crop Compensation (based on affected area) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  | Farmers/landowners compensated for significant trees that need to be trimmed/ removed along RoW. | Design of Implementation of Tree compensation (estimated area to be trimmed/removed) | Consultation with affected parties – once in a quarter |  | Prior to construction phase |
|  |  |  | Statutory approvals for tree trimming /removal | Compliance with regulations – once for each subproject |  | Part of detailed siting and alignment survey /design |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels | Noise levels to be specified in tender documents - once | POWERGRID | Part of detailed equipment design |
| Interference with drainage patterns/Irrigation channels | Flooding hazards/loss of agricultural production | Appropriate siting of towers to avoid channel interference | Tower location and line alignment selection (distance to nearest flood zone) | Consultation with local authorities and design engineers - once | POWERGRID | Part of detailed alignment survey and design |
| Escape of polluting materials | Environmental pollution | Transformers designed with oil spill containment systems, and purpose-built oil, lubricant and fuel storage system, complete with spill cleanup equipment. | Equipment specifications with respect to potential pollutants | Tender document to mention specifications - once | POWERGRID | Part of detailed equipment design /drawings |
|  |  | Substations to include drainage and sewage disposal systems to avoid offsite land and water pollution. | Substation sewage design | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
| Explosions/Fire | Hazards to life | Design of substations to include modern fire control systems/firewalls. | Substation design compliance with fire prevention and control codes | Tender document to mention detailed specifications - once | POWERGRID | Part of detailed substation layout and design /drawings |
|  |  | Provision of fire fighting equipment to be located close to transformers. |  |  |  |  |
| Construction |  |  |  |  |  |  |
| Equipment layout and installation | Noise and vibrations | Construction techniques and machinery selection seeking to minimize ground disturbance. | Construction techniques and machinery | Construction techniques and machinery creating minimal ground disturbance - once at the start of each construction phase | POWERGRID (Contractor through contract provisions) | Construction period |
| Physical construction | Disturbed farming activity | Construction activities on cropping land timed to avoid disturbance of field crops (within one month of harvest wherever possible). | Timing of start of construction | Crop disturbance –Post harvest as soon as possible but before next crop - once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Mechanized construction | Noise, vibration and operator safety, efficient operation | Construction equipment to be well maintained. | Construction equipment – estimated noise emissions | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Noise, vibration, equipment wear and tear | Turning off plant not in use. | Construction equipment – estimated noise emissions and operating schedules | Complaints received by local authorities - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction of roads for accessibility | Increase in airborne dust particles | Existing roads and tracks used for construction and maintenance access to the line wherever possible. | Access roads, routes (length and width of new access roads to be constructed) | Use of established roads wherever possible - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Increased land requirement for temporary accessibility | New access ways restricted to a single carriageway width within the RoW. | Access width (meters) | Access restricted to single carriageway width within RoW - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Temporary blockage of utilities | Overflows, reduced discharge | Temporary placement of fill in drains/canals not permitted. | Temporary fill placement (m3) | Absence of fill in sensitive drainage areas - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Marking of vegetation to be removed prior to clearance, and strict control on clearing activities to ensure minimal clearance. | Vegetation marking and clearance control (area in m2) | Clearance strictly limited to target vegetation - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Trimming/cutting of trees within RoW | Fire hazards | Trees allowed growing up to a height within the RoW by maintaining adequate clearance between the top of tree and the conductor as per the regulations. | Species-specific tree retention as approved by statutory authorities (average and maximum tree height at maturity, in meters) | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  | Loss of vegetation and deforestation | Trees that can survive pruning to comply should be pruned instead of cleared. | Species-specific tree retention as approved by statutory authorities | Presence of target species in RoW following vegetation clearance – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Felled trees and other cleared or pruned vegetation to be disposed of as authorized by the statutory bodies. | Disposal of cleared vegetation as approved by the statutory authorities (area cleared in m2) | Use or intended use of vegetation as approved by the statutory authorities – once per site | POWERGRID (Contractor through contract provisions) | Construction period |
| Wood/vegetation harvesting | Loss of vegetation and deforestation | Construction workers prohibited from harvesting wood in the project area during their employment, (apart from locally employed staff continuing current legal activities). | Illegal wood /vegetation harvesting (area in m2, number of incidents reported) | Complaints by local people or other evidence of illegal harvesting - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Surplus earthwork/soil | Runoff to cause water pollution, solid waste disposal | Soil excavated from tower footings disposed of by placement along roadsides, or at nearby house blocks if requested by landowners. | Soil disposal locations and volume (m3) | Acceptable soil disposal sites - every 2 weeks | POWER GRID (Contractor through contract provisions) | Construction period |
| Substation construction | - Loss of soil | Fill for the substation foundations obtained by creating or improving local water supply ponds or drains, with the agreement of local communities. | Borrow area siting (area of site in m2 and estimated volume in m3) | Acceptable borrow areas that provide a benefit - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Substation construction | Water pollution | Construction activities involving significant ground disturbance (i.e. substation land forming) not undertaken during the monsoon season. | Seasonal start and finish of major earthworks | Timing of major disturbance activities - prior to start of construction activities | POWERGRID (Contractor through contract provisions) | Construction period |
| Site clearance | Vegetation | Tree clearances for easement establishment to only involve cutting trees off at ground level or pruning as appropriate, with tree stumps and roots left in place and ground cover left undisturbed. | Ground disturbance during vegetation clearance (area, m2) | Amount of ground disturbance - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  |  | Statutory approvals | Statutory approvals for tree clearances – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Tower construction – disposal of surplus earthwork/fill | Waste disposal | Excess fill from tower foundation excavation disposed of next to roads or around houses, in agreement with the local community or landowner. | Location and amount (m3)of fill disposal | Appropriate fill disposal locations - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Storage of chemicals and materials | Contamination of receptors (land, water, air) | Fuel and other hazardous materials securely stored above high flood level. | Location of hazardous material storage; spill reports (type of material spilled, amount (kg or m3) and action taken to control and clean up spill) | Fuel storage in appropriate locations and receptacles - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Construction schedules | Noise nuisance to neighbouring properties | Construction activities only undertaken during the day and local communities informed of the construction schedule. | Timing of construction (noise emissions, [dB(a)]) | Daytime construction only - every 2 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
| Provision of facilities for construction workers | Contamination of receptors (land, water, air) | Construction workforce facilities to include proper sanitation, water supply and waste disposal facilities. | Amenities for Workforce facilities | Presence of proper sanitation, water supply and waste disposal facilities - once each new facility | POWERGRID (Contractor through contract provisions) | Construction period |
| Encroachment into farmland | Loss of agricultural productivity | Use existing access roads wherever possible | Usage of existing utilities | Complaints received by local people /authorities - every 4 weeks | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Ensure existing irrigation facilities are maintained in working condition | Status of existing facilities |  |  |  |
|  |  | Protect /preserve topsoil and reinstate after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  |  | Repair /reinstate damaged bunds etc after construction completed | Status of facilities (earthwork in m3) |  |  |  |
|  | Social inequities | Compensation for temporary loss in agricultural production | Implementation of Crop compensation (amount paid, dates, etc.) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Uncontrolled erosion/silt runoff | Soil loss, downstream siltation; | Need for access tracks minimised, use of existing roads. | Design basis and construction procedures (suspended solids in receiving waters; area re-vegetated in m2; amount of bunds constructed [length in meter, area in m2, or volume in m3]) | Incorporating good design and construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Limit site clearing to work areas |  |  |  |  |
|  |  | Regeneration of vegetation to stabilise works areas on completion (where applicable) |  |  |  |  |
|  |  | Avoidance of excavation in wet season |  |  |  |  |
|  |  | Water courses protected from siltation through use of bunds and sediment ponds |  |  |  |  |
| Nuisance to nearby properties | Losses to neighbouring land uses/ values | Contract clauses specifying careful construction practices. | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | As much as possible existing access ways will be used. | Design basis and layout | Incorporating good design engineering practices – once for each site |  |  |
|  |  | Productive land will be reinstated following completion of construction | Reinstatement of land status (area affected, m2) | Consultation with affected parties – twice – immediately after completion of construction and after the first harvest |  |  |
|  | Social inequities | Compensation will be paid for loss of production, if any. | Implementation of Tree/Crop compensation (amount paid) | Consultation with affected parties – once in a quarter | POWERGRID | Prior to construction |
| Inadequate siting of borrow areas | Loss of land values | Existing borrow sites will be used to source aggregates, therefore, no need to develop new sources of aggregates | Contract clauses | Incorporating good construction management practices – once for each site | POWERGRID (Contractor through contract provisions) | Construction period |
| Health and safety | Injury and sickness of workers and members of the public | Contract provisions specifying minimum requirements for construction camps | Contract clauses (number of incidents and total lost-work days caused by injuries and sickness) | Contract clauses compliance – once every quarter | POWERGRID (Contractor through contract provisions) | Construction period |
|  |  | Contractor to prepare and implement a health and safety plan. |  |  |  |  |
|  |  | Contractor to arrange for health and safety training sessions |  |  |  |  |
| Inadequate construction stage monitoring | Likely to maximise damages | Training of POWERGRID environmental monitoring personnel | Training schedules | Number of programs attended by each person – once a year | POWERGRID | Routinely throughout construction period |
|  |  | Implementation of effective environmental monitoring and reporting system using checklist of all contractual environmental requirements | Respective contract checklists and remedial actions taken thereof. | Submission of duly completed checklists of all contracts for each site - once |  |  |
|  |  | Appropriate contact clauses to ensure satisfactory implementation of contractual environmental mitigation measures. | Compliance report related to environmental aspects for the contract | Submission of duly completed compliance report for each contract - once |  |  |
| Operation and Maintenance |  |  |  |  |  |  |
| Location of transmission towers and transmission line alignment and design | Exposure to safety related risks | Setback of dwellings to overhead line route designed in accordance with permitted level of power frequency and the regulation of supervision at sites. | Compliance with setback distances (“as-built” diagrams) | Setback distances to nearest houses – once in quarter | POWERGRID | During operations |
| Equipment submerged under flood | Contamination of receptors (land, water) | Equipment installed above the high flood level (HFL) by raising the foundation pad. | Substation design to account for HFL (“as-built” diagrams) | Base height as per flood design - once | POWERGRID | During operations |
| Oil spillage | Contamination of land/nearby water bodies | Substation transformers located within secure and impervious sump areas with a storage capacity of at least 100% of the capacity of oil in transformers and associated reserve tanks. | Substatio bunding (Oil sump) (“as-built” diagrams) | Bunding (Oil sump) capacity and permeability - once | POWERGRID | During operations |
| Inadequate provision of staff/workers health and safety during operations | Injury and sickness of staff /workers | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (lost work days due to illness and injuries) | Preparedness level for using these technologies in crisis – once each year | POWERGRID | Design and operation |
|  |  | Safety awareness raising for staff. | Training/awareness programs and mock drills | Number of programs and percent of staff /workers covered – once each year |  |  |
|  |  | Preparation of fire emergency action plan and training given to staff on implementing emergency action plan |  |  |  |  |
|  |  | Provide adequate sanitation and water supply facilities | Provision of facilities | Complaints received from staff /workers every 2 weeks |  |  |
| Electric Shock Hazards | Injury/mortality to staff and public | Careful design using appropriate technologies to minimise hazards | Usage of appropriate technologies (number of injury incidents, lost work days) | Preparedness level for using these technologies in crisis – once a month | POWERGRID | Design and Operation |
|  |  | Security fences around substations | Maintenance of fences | Report on maintenance – every 2 weeks |  |  |
|  |  | Barriers to prevent climbing on/dismantling of transmission towers | Maintenance of barriers |  |  |  |
|  |  | Appropriate warning signs on facilities | Maintenance of warning signs |  |  |  |
|  |  | Electricity safety awareness raising in project areas | Training /awareness programs and mock drills for all concerned parties | Number of programs and percent of total persons covered – once each year |  |  |
| Operations and maintenance staff skills less than acceptable | Unnecessary environmental losses of various types | Adequate training in O&M to all relevant staff of substations and transmission line maintenance crews. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
|  |  | Preparation and training in the use of O&M manuals and standard operating practices. |  |  |  |  |
| Inadequate periodic environmental monitoring. | Diminished ecological and social values. | Power Grid staff to receive training in environmental monitoring of project operations and maintenance activities. | Training/awareness programs and mock drills for all relevant staff | Number of programs and percent of staff covered – once each year | POWERGRID | Operation |
| Equipment specifications and design parameters | Release of chemicals and gases in receptors (air, water, land) | Processes, equipment and systems using cholofluorocarbons (CFCs), including halon, should be phased out and to be disposed of in a manner consistent with the requirements of the Government. | Process, equipment and system design | Phase out schedule to be prepared in case still in use – once in a quarter | POWERGRID | Operations |
| Transmission line maintenance | Exposure to electromagnetic interference | Transmission line design to comply with the limits of electromagnetic interference from overhead power lines | Required ground clearance (meters) | Ground clearance - once | POWERGRID | Operations |
| Noise related | Nuisance to neighbouring properties | Substations sited and designed to ensure noise will not be a nuisance. | Noise levels (dB(a)) | Noise levels at boundary nearest to properties and consultation with affected parties if any - once | POWERGRID | Operations |

The substations covered under this project are proposed for funding under the World Bank proposed PSDP-V loan. The transmission lines are constructed under domestic funding.