Construction of Crate/gabion wall: Technical specification

Stone bound in galvanizing wire netting including excavation : Stone bound of more than 150 mm size bounded and  packed in galvanized  wire net/mesh of 8 SWG wire & 152mm square (maxm.) mesh confirming to IS:280 & IS:4826 waved diagonally) including 10% extra for laps & joints laid with stone boulders complete as per drawing including supply of all materials, cost of commercial transportation of goods, loading & unloading, necessary earthwork for placement of boulder works on the river bank and as directed by Engineer-in-charge.

Measurement shall be taken in cubic meters and 15% deduction will be made for void from cage measurements.