Section 1

INTRODUCTION and GENERAL INFORMATION

  - Introduction

This Volume II of the Bid Document describes the technical specifications for Telecom Equipment being procured by the Employer (Power Grid Corporation of India Limited) to build a dedicated IP Network for NTAMC. This Specification describes the functional and performance requirements of the New Generation Equipment Package.

The purpose of this section of the specification is to provide general information and define the scope of this project.

  - Project Overview

POWERGRID has a country-wide OPGW Fiber Network connecting the Sub-stations. The details of the Telecom network have been given in Appendix-B.  A pan Indian DWDM Network is in operation for commercial service.

POWERGRID has planned to build a dedicated Telecom Network for the NTAMC using a separate pair of fiber. MPLS-TP equipment along with suitable Amplifiers shall be used to build a Transport Network over this separate pair of fiber and IP/MPLS Routers & associated solution shall be used as per plan for building an IP/MPLS Network over this Transport Network. Equipment under package shall be installed & commissioned at all Sub-stations and other offices of POWERGRID spread across India.  The IP Network shall be used for data communication, Video conferencing, Security Surveillance of all sites, Remote management of Transmission assets at sub-stations etc. So the equipment/ router to be supplied shall conform with all latest L2 & L3 standards including Multicast.

  - Scope and General Requirements

This specification defines the design, engineering, performance, testing and implementation requirements under subject Package. Sections 1 through 7 define the general requirements of the installation, testing & commissioning and maintenance of all equipment and associated subsystems under the package.

The equipment shall be deployed at POWERGRID’s Sub-station, NTAMC, RTAMC and other offices spread across India. The requirement/ quantity of equipment laid out in the BoQ (Annexure-C) is tentative, and individual quantity of any of equipment/ module & or any other sub-system listed there in the BoQ, can be increase/ decrease to any value as per requirement, keeping the increase/ decrease in total value of contract as per applicable limits defined in SCC.

The key functional requirements from this dedicated IP network for NTAMC are as under:

- The network should be able to provide different L2 & L3 VPN topologies viz. Hub & Spoke, Any-to-Any, Transparent Layer-2 Pipe (without MAC learning).

- Hub & Spoke VPNs should be able to configure with Single or Multiple Hubs within same VPN.

- Should Support Multiple VPNs per physical Interface.

- Should Support Data Categorisation and Different Class of Service for assigning priority to desired type to Data.

- Should Support all kind of traffic including Video, Voice & Data.

- Should support Multiple tiered network (Core, Edge, Access) and should further be scalable.

- Multicast VPN should be configured for Video Data (CCTV Feeds).

- The Video Multicast Data should have capabilities to sub-divide the VPN into multiple regions based on different RT values.

- The network should support ‘Route-Leaking’ between multiple VPNs (including for Multicast) to support access of traffic originated from a site from selective multiple locations participating in different VPNs.

- The network should be able to support multiple Router Reflectors (RR) in the network.

- The RRs should have options for deployment in Primary & Back-up mode or in distributed mode on Regional basis (Active-Active).

- The network should have support for Sub-Station Tele protection System and should be able to comply with latency requirements for proper Tele-Protection functionality.

- The network should have seamless failover to Back NOC (DR) in case of communication or System failure at Main NOC.

All types of Equipment (MPLS-TP, Routers, Storage, Firewall, Amplifiers etc) required under the project shall be offered by the bidder from the same/ different manufacturer along with their NMS.

    - Scope of Work

The scope of work under the subject Package includes survey, planning, design, engineering, supply, installation, termination, testing, commissioning, training and maintenance of:

- All type of IP-MPLS Routers, Firewalls, Switches, NOC Hardware etc.

- All type of MPLS-TP Equipment

- Optical Amplifier solution supporting long distance solution with MPLS-TP equipment.

- All the equipment (IP-MPLS Routers, IP-MPLS, Amplifiers etc) shall work on 48V DC.

- Equipment Cabinets/ Racks for all Equipment & Routers

- All associated hardware, cables, accessories and fittings

- Telecommunication Management Network (TMN) for both type of equipment (MPLS-TP & IP-MPLS). Contractor can supply maximum two (02) nos of TMN system for management of two (02) type of equipment.

- Services to be provided through the proposed equipment:

  - Ethernet Service (P2P, P2M)

  - MPLS VPN (L2/L3 VPN services and Multicast)

- Support (Documentation as well as manpower/ licenses) for integration of TMN(s) system with GIS Mapping System, OSS, Network Orchestrator etc. in future without any financial implication to POWERGRID.

- Remote Access Server Solution meeting DoT requirements.

- Security Testing of all Equipment/ elements as per Common Criteria Certification.

- Vulnerability Assessment of Network from a certified third party on completion of project.

- All other associated works/items described in the technical specifications for a viable and fully functional System.

- The scope of work under this subject package shall cover supply, installation and other services anywhere in India, and subsequently AMC of 5 years for all equipment, TMN hardware, any other system/ hardware/ software/ licenses etc.

    - General Requirements

The contractor has to supply ‘Trusted Products’ from ‘Trusted Sources’ in line with the ‘National Security Directive on Telecommunication Sector (NSDTS)’ under subject package. Trusted Products are products whose critical components and the products themselves are sourced from Trusted Sources. The National Cyber Security Coordinator (NCSC) is the Designated Authority (DA) for the determination of inclusion of a vendor as a Trusted Source, of a Telecom product as a Trusted Product and the methodology for the said inclusion. Contractors are advised to do their due diligence while selecting an OEM and offering the model/ type equipment.

The Contractor is encouraged to offer standard products and designs. However, the Contractor must conform to the specification requirements and provide any additional equipment necessary to meet the requirements stated herein.

It should be noted that bill of quantity are indicative only, and the equipment shall be ordered based on requirement and the solution/ design finalized during detail engineering.

The Bidder's proposal shall address all functional and performance requirements within this specification and shall include sufficient information and supporting documentation in order to determine compliance with this specification without further necessity for inquiries. Brochure & Data sheets of all equipment shall be submitted, in this regard.

The Bidder's proposal shall clearly identify all features described in the specifications or in any supporting reference material that will not be implemented; otherwise, those features shall become binding as part of the final contract.

An analysis of the functional and performance requirements of this specification and/or site surveys, design, and engineering may lead the Contractor to conclude that additional items are required that are not specifically mentioned in this specification. All such additional items necessary for a viable and fully functional Network to be implemented, that meets or exceeds the capacity and performance requirements specified, shall be provided/ supplied by Contractor without any additional cost to Employer. Such materials shall be considered to be within the scope of the contract. The Bidders shall identify and include all such additional items in their proposal, to the extent possible.

The offered items shall be designed to operate in varying environments across India. Adequate measures shall be taken during manufacturing of Equipment to provide protection against contaminants, pollutants, water & moisture, lightning & short circuit, vibration and electro-magnetic interference etc. The Contractor shall demonstrate a specified level of performance of the offered items during well-structured factory and field tests, and necessary certification (if any).

The bidders are advised to visit sites (at their own expense), prior to the submission of a proposal, and make surveys and assessments as deemed necessary for proposal submission. The successful bidder (Contractor) is required to visit all sites, if needed. The site visits shall include all necessary surveys to allow the contractor to perform the design and implementation functions. Contractors are encouraged to utilize the available rack space in existing racks at existing locations for intended Equipment under this package, as far as possible. The Contractor shall submit the site survey format for Employer’s approval and inform their site survey schedule to the Employer well in advance. The site survey schedule shall be finalized in consultation with the Employer. The Employer or its representative may be associated with the Contractor during their site survey activities.

After the site/ route survey the Contractor shall submit to the Employer a survey report as per the approved survey format. This report shall include at least the following items:

- Proposed layout of Equipment in the existing rooms and buildings/ shelters.

- Proposed routing of power, earthing, signal cables and patch cords etc.

- Requirement of armoured patch-cords at locations prone to rat cuts.

- Confirmation of adequacy of Space, air conditioning and AC/DC Power supply requirements.

- Utilization of existing rack space

- Measurement of fibre attenuation by power meter.

- Measurement of PMD coefficient of fibre, if required

  - Technical Experience Requirement

    - Technical Requirement for MPLS-TP Equipment

- The manufacturer, whose ‘MPLS-TP Equipment’ are offered, must have manufactured & supplied at least Ten (10) no of ‘MPLS-TP Equipment with 10G or above interfaces’ within last seven (7) years, which should have been in satisfactory operation# for at least one (01) year, as on the date of NOA

- Alternatively, the manufacturer does not meet the requirement as specified in clause 1.4.1(a) above can also be considered provided that:

- The manufacturer must have manufactured & supplied atleast Ten (10) numbers of ‘SDH/ IP-MPLS/ MPLS-TP/ DWDM/ OTN Equipment’ within last seven (7) years, which should have been in satisfactory operation#, as on the date of NOA.

- The manufacturer has necessary agreement for manufacturing of MPLS-TP Equipment based on technological support of Parent company/Collaborator meeting the qualifying requirement specified at para 1.4.1(a) above, and

- A legally enforceable undertaking jointly by manufacturer and the Parent company / Collaborator shall be furnished to guarantee quality, timely supply, performance and warranty obligations for Five (5) years for entire quantity to be manufactured from the works in India.

- The Parent Company/Collaborator shall furnish performance guarantee for an amount of 3% of the ex-works cost of such equipment(s) and this performance guarantee shall be in addition to Contract Performance Guarantee to be submitted by the bidder and this shall be valid till the end of warranty period.

    - Technical Requirement for IP-MPLS router

- The manufacturer, whose ‘IP-MPLS Router’ are offered must have manufactured & supplied at least Ten (10) nos of ‘IP-MPLS Router’ within last seven (7) years, which should have been in satisfactory operation# for at least one (01) year, as on the date of NOA

- Alternatively, the manufacturer does not meet the requirement as specified in clause 1.4.2(a) above shall also be considered provided that:

    - The manufacturer must have manufactured & supplied atleast Ten (10) numbers of ‘SDH/ IP-MPLS/ MPLS-TP/ DWDM/ OTN Equipment’ within last seven (7) years, which should have been in satisfactory operation#, as on the date of bid opening mentioned above.

    - The manufacturer must have necessary agreement for manufacturing of IP-MPLS Router based on technological support of Parent company/Collaborator meeting the qualifying requirement specified at para 1.4.2(a) above, and

    - A legally enforceable undertaking jointly by manufacturer and the Parent company / Collaborator shall be furnished to guarantee quality, timely supply, performance and warranty obligations for Five (5) years for entire quantity to be manufactured from the works in India.

    - The Parent Company/Collaborator shall furnish performance guarantee for an amount of 3% of the ex-works cost of such equipment(s) and this performance guarantee shall be in addition to Contract Performance Guarantee to be submitted by the bidder and this shall be valid till the end of warranty period.

    - Technical Requirement of Sub-contractors:

Installation & Commissioning of the MPLS-TP Equipment & IP-MPLS router shall be executed either by the bidder himself or by the Subcontractor meeting the following technical requirement:

- The bidder/sub-contractor must have installed and commissioned at least Ten (10) numbers of ‘SDH/ IP-MPLS/ MPLS-TP/ DWDM/ OTN Equipment within last seven (7) years, which should have been in satisfactory operation# for at least one (01) year, as on the date of NOA

  - General Responsibilities and Obligations

This section describes the general responsibilities and obligations of the Contractor and the Employer.

    - Responsibilities for the Implementation Plan

The Bidder's technical proposal shall include a project implementation plan and schedule that is consistent with the implementation plan detailed in Section 6 and Appendix-A of this specification. The implementation plan shall be modelled such that it provides the phase wise supply, installation & activation of this Project. The Implementation plan shall include the activities of both the Contractor and the Employer, showing all key milestones and clearly identifying the nature of all information and project support expected from the Employer. The Employer and Contractor shall finalise the detailed Implementation plan following award of the contract.

    - Contractor's Responsibilities and Obligations

The Contractor shall be responsible for the implementation of the Fibre Optic Transmission System.

Contractor's obligations include, but are not limited to, the following:

          - Provide a working system that meets the functional and performance requirements of this specification.

          - Survey of the locations wherever required, and submission of survey report as per agreed formats.

          - Site visits and studies necessary to identify and provide all equipment needed to implement the Package.

          - Engineering and design specific to each location including review of, and conformance with local environmental and earthing requirements.

          - Define source power requirement for each cabinet/ rack of equipment provided at each location.

          - Development of installation and safety guidelines and procedures for the mechanical installation including testing and documentation.

          - Appointment of Project Manager, Design Engineer & other resources for successful & timely completion of project.

          - Project management, project scheduling, including periodic project reports documenting progress during the contract period.

          - Planning and implementation for a smooth transition from existing operations to new upgraded network operations including new equipment, wherever applicable.

          - Submit earlier carried out type test reports or conduct type tests & functional tests as per requirement.

          - Factory and site acceptance testing of all hardware, software and firmware provided.

          - Shipment of all equipment and documentation to the Employer designated locations and/or staging areas.

          - Staging, maintenance and security of the staging area including the full responsibility for protection from fire and theft.

          - Implement all associated minor civil works as per Section 4.4.2 and identify any major civil works such as expansion  or construction of rooms, trenches necessary for installation of proposed equipment and provide details of such works to the Employer.

          - Supply, installation, testing and commissioning of all hardware, software and firmware required to satisfy this specification.

          - All cabling wiring including supply, laying and termination of cables (signal, power supply & earthing etc), DC/AC distribution boards with MCBs required for powering equipment.

          - Installation and integration of Telecommunications Management Network (TMN) system software, hardware and firmware.

          - Configure all circuits (end to end) at Gbe, 10 Gbe, etc. levels in the network as indicated by the Employer during detail Engineering and implementation.

          - System integration & commissioning of the overall telecommunications network and also coordination with other Project Contractors for phased implementation.

          - All documentation and drawings as specified.

          - All necessary information, documents, manpower support, licenses for enabling interfaces etc required for integration of TMN system with OSS, GIS System, Network Orchestrator etc.

          - Training of the Employer personnel.

          - Maintenance, debugging and support of the items through final acceptance, and maintenance (including deployment of resources) throughout the defect liability period and after defect liability period as defined in Section 7.

          - All required spare parts and maintenance aids

          - Ensuring availability of service and spares of the equipment to meet availability requirement specified at Section 6 & 7.

          - Provide spare modules/ cards/ base equipment for Routers/MPLS-TP/ Amplifiers and TMN system.

          - Compliance of all security related requirements as per latest DoT guidelines with subsequent amendments including security certification of equipment.

          - Handing Over of Network after Acceptance testing and completion of Vulnerability Assessment testing & its mitigation plan.

          - Defect liability period (DLP) of 01 year after TOC.

          - AMC of the Equipment/ system for at minimum 5 years.

          - All equipment/material/items, as per Table-A of DoT notification No. 18-10/2017-IP dated 29th August 2018, as applicable under the present scope of works, shall be procured and supplied from domestic manufacturers only meeting the local content (LC) as indicated.

          - Provision to be complied as per latest guidelines of GOI/ MOP/DPIIT/DOT.

          - The Contractor shall ensure following things and at their own cost:

- Any imported equipment/material/item/parts/component to be supplied under the contract shall be tested in the certified laboratories to check for any kind of embedded malware/trojans/cyber threats and for adherence to Indian Standards as per the directions issued by Ministry of Power/Govt. of India from time to time. In case of such import from specified “prior reference” countries, the requirement of prior permission from the Govt. of India including protocol for testing in certified and designated laboratories by Ministry of Power/Govt. of India shall also be complied with by the contractor

- The equipment offered by the contractor shall at least conform to the requirements specified under relevant IS standard. In case of discrepancy between IS and other international standard, provisions of IS shall prevail. The Contractor shall also note that the list of standards presented in this specification is not complete. Whenever necessary, the list of standards shall be considered in conjunction with specific IS. If the IS standard is not available for an equipment/material, then other applicable International standard (IEC/Equivalent), as per the specification, shall be accepted.”

- The bidder/contractor shall list out the products and components producing Toxic e-waste under the contract and shall furnish to the Employer the procedure of safe disposal at the time of closing of the contract.

- The Bidder shall have to furnish a certificate regarding cyber security/safety of the equipment/ process to be supplied/services to be rendered as safe to connect

- The detailed descriptions of the Contractor's obligations, in relation to individual items and services offered, are delineated in other sections of this specification.

    - The Employer Responsibilities and Obligations

The Employer will provide the following items and services as part of this Project:

          - Overall project management of the project.

          - Source power at (nominal) -48 volts DC and/or AC power at (nominal) 230 volts, 50 Hz as required by the equipment/ Hardware to be supplied.

          - Providing support and access to facilities at the sites.

          - Implement the major civil works such as expansion or construction of rooms, trenches necessary for installation of proposed equipment.

          - Provide to the extent possible drawings for existing sites and facilities for which equipment installations are planned.

          - Obtaining statutory clearances from regulatory bodies, if required.

          - Approval of key personnel for the project.

  - Systems To Be Supported

The fibre optic cables consist of DWSM (G.652) & DSSM (G.653) fibres for backbone sections and DWSM (G.652) fibres for Access network.

  - General Bidding Requirements

The Bidder shall be responsive to the Employer's technical requirements as set forth in this specification. The Bidder's proposal shall include the following:

- The Technical Proposal including the documents listed in the Table 1-1: Bid Documents Checklist, shall be provided in the bid.

- A detailed project implementation plan and schedule that is consistent with the scope of the project and Employer's specified objectives.  The plan shall include the activities of both the Contractor and Employer, show all key milestones, and clearly identify the nature of all information and project support to be provided by Employer. Manpower resources proposed to be deployed by the Contractor during the execution phase shall be clearly indicated.

| Table 1-1: Bid Documents Checklist |  |  |  |
| --- | --- | --- | --- |
| Sl. No. | Description: | Enclosure Reference |  |
| 1 | Type Test Certificates/Reports for offered Equipment/Sub-systems as per specification requirement. | Page no. Ref no. |  |
| 2 | Completed Data Requirement Sheets (As per Appendix- D of Technical Specs Volume II ) | Page no. Ref no. |  |
| 3 | Network design document including link budgeting, detailed BoQ and break-up of quantities offered under ‘Lot/ Set’ in BoQ as per specification requirements. | Page no. Ref no. |  |
| 4 | Detailed Project Implementation Plan (As per Section 1.7 General Bidding requirements of Technical Specs Volume II) | Page no. Ref no. |  |

  - Table of Compliance

Bidder shall use one copy of Volume I, "Conditions of Contract," and Volume II, "Technical Specifications" to indicate compliance status with those volumes. Within the right-hand margin, Bidder shall indicate compliance status to each paragraph along with a cross-reference to its proposal and an index key for any explanation or comment.

In addition, the Bidder shall annotate the Table of Contents of each of the above stated volumes to provide a high-level summary of compliance status. In both cases, the following symbols, and no others, shall be used:

| C | Bid complies with all requirements in the adjacent paragraph. |
| --- | --- |
| A | Bid is not compliant with the requirements in the adjacent paragraph, but a functional alternative is proposed. |
| X | Bid takes exception to the requirements of the adjacent paragraph and no functional alternative is proposed. |

Only one symbol shall be assigned to a paragraph and shall indicate the worst-case level of compliance for that paragraph. This annotation may be hand written.

Bidder shall also underline, on the compliance copy, all requirements to which exceptions have been taken (X) or to which alternatives have been proposed (A).

Each alternative shall be clearly and explicitly described. Such descriptions shall use the same paragraph numbering as the bid document sections addressed by the alternatives. All alternative descriptions shall be in one contiguous section of the Bidder's proposal, preferably in the same volume, and titled "Alternatives." A separate section titled "Exceptions" should be provided containing any discussion or explanation Bidder chooses to provide concerning exceptions taken. Alternatives which do not substantially comply with the intent of the bid documents will be considered exceptions.

The Employer will assess the merits of each alternative and exception and will be the sole judge as to their acceptance.

  - Organization of the Technical Specification Document

Sections 2 through 7 provide the project requirements of the Fibre Optic Transmission System to be provided.

Section 2:	contains the hardware description and specifications for the Equipment to be supplied under subject package. It has two parts 2-A & 2-B

Section 3:	contains Telecommunications Management Network (TMN) System requirements. It has two parts 3-A & 3-B.

Section 4:	contains environment, Power Supply, Cabling and Earthing Requirements

Section 5:	contains the inspection and testing requirements.

Section 6:	details requirements for project management, Quality Assurance, Training and Documentation

Section 7:	Maintenance services requirements (During DLP & AMC)

Section 8:	Security requirements

The following is a list of the Volume II Appendices:

Appendix A:	Implementation Schedule

Appendix B: 	Existing Telecom Network,

Appendix C:	Bill of Quantity

Appendix D:	Data Requirement Sheets

Appendix E:	Template of the Agreement between TSP and Vendor as per DoT guidelines

  - Applicable Standards

The following standards (or their equivalent Indian Standards) shall be generally applicable to the equipment and works supplied under this Contract:

ITU-T/CCITT Recommandations G.652, G.653, G.655, G.664, , G.692,  G.694.1, G.697,  G.703, G.709, G.784, G.798, G.806, G.8032, G.8110.1, G.8110v2, G.8112, G.8113, G.8121, G.8131, G.8132, G.8151, G.823, G.826, G.8264,  G.872,  G.959.1, G.975, G.7713, G.7714, G.7715, G.7718, G.8201, G.8251, G.8080/Y.1304, Y.1564, Y.1711, Y1731

ITU-T/CCITT Recommendations M3010, M3100, M.2101.

ITU-T Rec. G.8131.1, G.8131.2, G.8032v2, G.826, G.828

SyncE as per ITU-T G.8261, G.6282 and G.8264 or IEEE 1588v2

Internet Activities Board, RFC-1157 (SNMP)

International Electrotechnical Commission standards,  IEC870-2-1, IEC 60825-1, IS 13252 : Part 22:2019

ETS 300 019-1-1, ETS 300 019-1-2 and ETS 300 019-1-3.

International Electrotechnical Commission standards, IEC 61000-4-xx series

IS 8437(1993), 13252(1993)

IEEE 802.1P/Q, 802.1p, 802.1s, 802.1w, 802.1ad, 802.1ag, 801.2d, 802.2, 802.3, 802.3ah, 802.3ae, 802.3az, 802.3ba

RFC 1918, RFC 2080, RFC 2362, RFC 2460, RFC 2461, RFC 2462, RFC 2463, RFC 2464, RFC 2544, RFC 2545, RFC 2547, RFC 2740, RFC 3097, RFC 3107, RFC 3209, RFC 3270, RFC 3630, RFC 3985, RFC 4385, RFC 4448, RFC 4664, RFC 4761, RFC 4762, RFC 5950, RFC 5951

ANSI X3.296, X3.230, X3.303

International CISPR standards 22 (2006)

Tele Management Forum – TMF 513, 608, 814

Metro Ethernet Forum (MEF)-9, 14

Telecommunication Engineering Centre (TEC) GR No: TEC/GR/IT/CEN-004/03/MAR-17 and other latest applicable TEC GRs including amendments

Specifications and codes shall be the latest version, inclusive of revisions, which are in force at the date of the contract award. Where new specifications, codes, and revisions are issued during the period of the contract, the Contractor shall attempt to comply with such, provided that no additional expenses are charged to the Employer without Employer’s written consent.

In the event the Contractor offers to supply material and/or equipment in compliance to any standard other than Standards listed herein, the Contractor shall include with their proposal, full salient characteristics of the new standard for comparison.

In case the requirements stipulated in the specifications are stringent than those specified by the standards, the specification shall override the standards.

--- End of Vol II Section 1 ---