Section 2-A

IP/MPLS Network for NTAMC

# Introduction:

Employer’s primary objectives in setting up the Pan-India MPLS based network are:

    - To build a common IP infrastructure that shall cater to all network requirements of a power sub-station including SDACA, PMU, Tele-Protection and Video Surveillance data. This network shall also ready to offer Multicast, MPLS L2VPN and L3VPN as per the varied requirements in future.

    - Proposed Network shall be a MPLS based IP transport network providing all types of LAN connectivity (logical as well physical). The function of transport network shall also include transfer of media information as well as transfer of control and management information. It will interact on standard interface with control layer, service layer and access layer.

    - Provide core network for carriage of a diversified range of Sub-Station data including support for VPN (Layer-2 and Layer-3) to meet the requirements of the entire spectrum of current and future services.

    - The network shall have simplified provisioning and dynamic QoS control functionality to meet the customers’ SLAs.

    - The network shall be scalable and flexible to facilitate large- scale deployment and seamless integration with existing & all types of future access technologies including but not limited to Metro Ethernet, STM transport etc.

    - The network shall be capable of on-line measurement and monitoring of network parameters such as latency, packet loss, jitter and availability for end-to-end link basis so as to monitor Service Level Agreements.

    - The solution shall also provide firewall/filtering features in separate firewall hardware that support a number of match conditions that can be applied to single interfaces or to logically grouped interfaces without adversely affecting the packet forwarding performance.

    - The architecture should be ipv6 compliant. All the products offered must support dual stack (ipv4/ipv6) on every interface.

    - The Routers (Core, Edge and Access, NOC Router, RR) should be offered from single OEM & should be managed through a single NMS.

    - Network OS for all proposed family of products must be Evaluation Assurance Level (EAL level-2) / Network Device Protection Profile (NDPP) certified EAL2/NDPP certificate is required only for Routers, Switches & security Equipment (Firewall, IDPS etc.).

# Network Architecture:

The proposed IP/MPLS network is a multi-tier network, it comprises of below layers:

- Core layer – MPLS –P (Provider) Layer

- Edge Layer – MPLS- PE (Provider Edge) Layer

- Access Layer – MPLS- PE (Provider Edge) Layer

The architecture of the proposed IP/MPLS network is as described below:

    - Core-To-Core & Core routers to the Edge routers connectivity shall be done with 10G interfaces as depicted in Figure-2-1 below. However, the same may be subjected to change during detailed Engineering.

    - Edge-to-Access Shall be on 10G/1G based on the site requirements and shall be finalised during detailed engineering

    - The network shall be centrally managed from Network Operation Centre (NOC) located at Delhi/NCR and a Disaster Recovery (DR)–NOC at Bangalore. All Network databases shall reside centrally at the NOC. DR Capacity should be of same as of DC.

# Core layer:

    - The Core network layer shall constitute the high speed Backbone 10G that will comprise of fully or partially meshed Core routers with built-in redundancies supporting both TCP-IP and Advanced MPLS protocol (MPLS TE, AToM, MPLS QOS modes) & segment routing with link capacities in accordance with the traffic justification. The Core nodes shall consist of a dual routing processor/switch fabric, Control card, Power supply modules & fully redundant line cards. MPLS Core routers and shall be interfaced to Edge routers through 10G initially.

    - The network shall have Eight (8) core locations as detailed in Table-2-1 below. The location in table below are tentative and shall be finalised during detailed engineering based on actual requirement at the time of deployment

Table-2-1

| Core Locations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Manesar | Delhi | Chennai | Durgapur | Hyderabad | Bangalore | Allahabad | Mumbai |

    - The Figure 2-1 below gives details of the Core network topology. The below figure is only a pictorial representation and actual link connectivity will be advised during the implementation phase.

Figure-2-1

NETWORK CONNECTIVITY

# Edge Layer:

    - The Edge network shall be the second layer (below the Core layer) of the IP backbone network depicted in Figure-2-1 and shall primarily provide MPLS Edge functionality. The Edge routers shall perform the task of aggregating the traffic and forwarding MPLS traffic to P routers for switching. The layer shall be ready to enforce QoS and other administrative policies.

    - The Edge layer will be used to offer services and shall work as PE (Provider Edge). The interfaces used for this purpose shall be Gigabit Ethernet (electrical or optical). The edge layer will also have layer 2 Ethernet switches connected to it, to provide aggregation of multiple links coming from multiple locations.

    - Edge router shall be dual homed and to be directly connected with Core router using 10G interface and to nearest other available Core router using 10G link.

    - The proposed Edge cities are indicated at Appendix-C.

    - At times the Edge router may also be connected to the nearest Edge router in case of specific requirements and the same maybe decided by the Employer.

# Access Routers:

    - The Access layer shall be used to extend the reach of POWERGRID further to each sub-Stations and to provide services for NTAMC network.

    - The Access layer will be directly connected to the Edge Router in dual-homed fashion based on physical media/Bandwidth availability.

    - This layer will be connected to the Edge Routers using one or multiple 1G links (LAG)/ 10G link.

    - It should support the port density as depicted in Figure 2-1 above & BoQ (Appendix-B). However, the same may be subjected to change during detailed Engineering.

# Miscellaneous Routers:

In addition to Core, Edge & Access routers, some miscellaneous routers are also envisioned to support successful implementation & operation of the proposed IP/MPLS network. The miscellaneous routers shall consist of the following routers:

- NOC Router - At Delhi/NCR & Bangalore, two nos. at each location.

- Route Reflector - (IPv4/IPv6- 4nos) At Manesar/Delhi, Bangalore, Raipur & Durgapur one at each location. The Route Reflector (RR), a network routing component which offers an alternative to the logical full-mesh requirement of Internal Border Gateway Protocol (IBGP).

# Bill of Quantities (BoQ) for Network Equipments:

BoQ for network equipment including CPE is attached at Appendix-B. However, this can be changed (Quantity of any item can vary from Nill (0) up to quantity (as per requirement) from mentioned BoQ) during detailed engineering. Detailed BoQ shall be released after engineering.

# General Router Specifications (Core, Edge):

    - The specifications mentioned in this section are in addition to the specifications mentioned in the specific Router requirements section and specific requirements shall supersede this, in case of any conflict.

    - These specifications are applicable to Core & Edge routers.

    - Should be chassis based & modular architecture for scalability with Redundant Route Processor, Power supply & switching fabric.

    - Router should be provided with 1+1 route processor, 1+1 switch fabric and N+1 power supply redundancy.

    - Should have minimum 30% vacant usable service slots for future expansion. Refer slot definition below.

    - Slot Definition: Means a main slot (full slot) on the router chassis. Only a main slot on the router chassis shall be counted to fulfil any tender requirements for number of free slots (no sub-slot or daughter slot shall be considered as “Slot”).

    - All interfaces on the routers shall provide wire-rate throughput.

    - The router 10 Gig interfaces for SR, LR & ZR should be support LANPHY/WANPHY mode. All optical ports (10G/1G) should be loaded with Short Range SFPs.

    - The operating system of the router shall have CPU based architecture. All Routers are CPU based. However, Modular Architecture is also acceptable.

    - The modular operating system shall run all critical functions like various routing protocol, forwarding plane and management functions in separate memory protected modules. Failure of one module shall not impact operations of rest of the OS. In service bug patching / software upgrade (including software patch released time to time) should be available.

    - Should be supplied with necessary power cards, data cables, connectors, CD’s, manuals, bracket accessories, wire managers and other appropriate accessories. There is no separate BoQ for Passive elements. However, the contractor shall supply appropriate length of POWER, Copper/CAT-6 & Optical Fiber Patch cords with patch panel, conduit etc. as required to install a network element.

    - The router shall have minimum of 20 Gbps (in line with BoQ at Appendix-B) Full Duplex capacity per slot with redundancy. Failure of any switch fabric should not degrade the per slot bandwidth.

    - The router should have capability of handling minimum 256k IPv4 routes in RIB & 128K IPv4 routes in FIB.

    - The router should have capability of minimum 128k IPv6 routes in RIB & 64k IPv6 routes in FIB.

    - The proposed router should be equipped with optimal memory (DRAM & FLASH) to meet all functional and performance requirements stated in Technical Specifications. The route should support wire speed interfaces, routing and MAC tables as asked in TS.

    - The router should support minimum 128k MAC address, minimum 16k Pseudo wires.

    - Router should have 8k multicast routes.

    - The router should support 32 way BGP load balancing and 64 Path ECMP.

    - Shall support online insertion and removal (OIR) that is non-disruptive in nature. Online insertion and removal of one-line card shall not lead to ANY packet loss for traffic flowing through other line cards for both control, unicast and multicast traffic.

    - In case of a line card or Route Processor failure on the router; the multicast and Unicast routing, multicast and Unicast distribution and multicast replication architecture of the router shall ensure no impact & zero packet loss of multicast video, audio & data traffic running on rest of the line cards in the system

    - If the any of the feature and functionality asked in the RFP is achieved using any service module that should be quoted in 1+1 redundancy. 1+1 service card level redundancy still required on nodes that are redundant.

    - Should have IPv4 Routing, Border Gateway Protocol, and Open Shortest Path First [OSPF]), BGP for IPv4 and Ipv6, MPBGP, Policy Based Routing (PBR), Virtual Router Redundancy Protocol (VRRP), IPv6 Routing, and BGP Prefix Independent Convergence, GRE (Generic Routing Encapsulation) Tunnelling, Multicast routing protocols IGMPv1, v2 ,v3, PIM-SM (RFC2362) and PIM-SSM, ,IGMP v2 snooping,DHCPv6 and OSPFv3 for IPv6.

    - Should support 6PE & 6VPE, MPLS Provider /Provider Edge functionality. MPLS VPN, MPLS mVPN (Multicast VPN), Carrier Supporting Carrier (CsC), DiffServ Tunnel Modes, MPLS TE (Fast re-route), DiffServ-Aware TE, Inter-AS VPN, Resource Reservation Protocol (RSVP), VPLS, VPWS, Ethernet over MPLS.

    - Router shall support MPLS OAM, Ethernet OAM protocols - CFM (IEEE 802.1ag), Link OAM (IEEE 802.3ah) and ITU Y.1731 for:

      - MAC pinging- continuity check

      - Trace route

      - Loopback connectivity check

      - 802.3ah to CFM ( 802.1ag) inter-working

      - Maintenance End Points (MEP)

      - SNMP traps

      - Maintenance intermediate points

      - Connectivity fault management over 802.1ad links

    - The routers shall support both L2 and L3 services on all interfaces

    - The Router should support Point to Point and Point to Multipoint LSP for Unicast and Multicast traffic.

    - The router should support RFC 3107 of Carrying Label Information in BGP-4.

    - Configuration Roll Back to recover the mis-configured router to last good configuration and any locally saved configuration on the router.

    - Shall support traffic Classification using various parameters like source physical interfaces, source/destination IP subnet, protocol types (IP/TCP/UDP), source/destination ports, IP Precedence, 802.1p, MPLS EXP, DSCP and by some well-known application types through Application Recognition techniques.

    - Shall support Strict Priority Queuing or Low Latency Queuing to support real time application like Voice and Video with minimum delay and jitter, Congestion Management: WRED, Priority queuing, Class based weighted fair queuing

    - Shall support standards based RSVP for voice & video call admission control.

    - Ability to configure hierarchical queues in hardware for IP QoS at the egress to the edge. Minimum 8K hardware queues per line card.

    - Platform must support nested hierarchical QOS policies. Router should have at least 3 level of scheduling for HQOS.

    - Support Access Control List to filter traffic based on Source & Destination IP Subnet, Source & Destination Port, Protocol Type (IP, UDP, TCP, ICMP etc) and Port Range etc., Time based ACL, AAA using radius or TACACS

    - The routers shall provide hardware accelerated inline IETF Netflow-v9/J-Flow/equivalent feature. This feature shall be available for all interfaces provisioned on the router.

    - Should Support MD-5 route authentication for RIP, OSPF and BGP, URPF, DHCP snooping, control plane policing, SNMPv3 authentication, SSHv2 or latest version.

    - Should have to support Out of band management through Mgmt Port/ Console.

    - The router shall support online insertion and removal of line cards without requiring the router to be rebooted.

    - Event and System logging: Event and system history logging functions shall be available. The Router shall generate system alarms on events. Facility to put selective logging of events onto a separate hardware here the analysis of log shall be available.

    - Router should have redundant controller cards and should support stateful switch over, nonstop forwarding, nonstop routing and Graceful restart.

    - All slots must be universal, that is, all modules should be interchangeable across every slot in the chassis except for the slots for control card/Switching card which may have fixed slots as per architecture.

    - In case of failure of any single route processor none of the line card traffic should be impacted.

    - Router should have sufficient cooling arrangement through fans or fan tray(s) etc.

    - The router OS should support for Telemetry data for following:

      - Interface operational data

      - Packet/Byte counters for IPv4 & IPv6

      - Packet/Byte rates

      - MPLS Statistics

    - Router should support RSVP inter-area P2MP LSP.

    - Router should support ABR selection for inter area LSP.

    - The routers should support configuration of all timers associated with BGP graceful restart.

    - The router should support IP Prefix Import from Global Table into a VRF Table.

    - The 10G interfaces on router should support Optical capability along with software configurable LAN/WAN functionalities. It should have Multivendor Interoperability.

# Core Router Specifications:

The capacity of the Core routers is as mentioned in Table-II. This is in addition to the General requirements for the Routers mentioned above. These requirements shall supersede the conditions in General Requirements, in case of any contradictory requirements. The proposed Core router should satisfy all the clauses mentioned in Section 2.3 General Router Specifications and should support the clauses mentioned below:

    - The router supplied must have support for dual redundant route processor. It must be supplied with redundant route processor from Day 1.

    - The Core nodes shall be connected in a partial mesh configuration as shown in the above figure over multiple 10G interfaces over the MPLS-TP. MPLS-TP interface specification shall be shared with successful Bidder during detailed engineering

    - The P router shall have the following interface types –

- Total 20 ports line rate 10G Ethernet interfaces uniformly distributed across more than one line cards (Refer slot definition).

- Total 8 ports of line rate 1 Gbps Ethernet interfaces distributed equally across minimum two line cards (Refer slot definition).

    - All these Core routers shall support MPLS VPN “Provider” functionality (or simply “P” routers)

    - Intercity transport of Ethernet traffic across the backbone shall be achieved using Employer existing optical transport infrastructure or IP/MPLS backbone.

    - The proposed router should support  2K VRF instances

    - The router should have 256 Multicast VPN (MVPN)

    - Core routers are used for label operations and therefore 64K LIB scale support.

# Edge Router Specifications:

The proposed Edge router should satisfy all the clauses mentioned in Section 2.3 General Router Specifications and the clauses mentioned below:

    - The router supplied must have support for dual redundant route processor. It must be supplied with 2 route processors from Day 1.

    - The Edge nodes shall be connected in a dual homed configuration with Core routers as shown in the Figure-2-1 above over multiple 10G interfaces to the P network.

    - The PE router shall have the following interface types –

- Minimum 24 nos. of 1G line rate ports distributed across two line cards. (Refer slot definition). Out of total 24 ports 20 ports should be optical SFP (Short Range) based and 04 ports should be of electrical interface.

- Minimum 04 nos. of 10G line rate ports distributed across two line cards. (Refer slot definition).

    - The router shall support MPLS VPNs as per RFC2547bis.

    - Should support minimum 4000 VLAN  per Port

    - Should support minimum 2000 VPN (2k VRF support is asked for both v4 and v6, since, Going forward, customer will be connected on IPv6 also. So, the VRF scale asked is for IPv6 and v4 both.)

    - Should have 256 Multicast VPN (MVPN)

    - Should be able to leverage MPLS & MP-BGP for multicast delivery

# Access Router Specifications:

    - Router should support have 2Gbps of backplane capacity

    - The Router shall be standalone fixed configuration Chassis with redundant power supply.

    - Router should support DC Power supplies

    - A single point failure on the equipment shall not result in network or network management system downtime

    - Router/Switch should support Quality of service for marking, Prioritising and assuring bandwidth Guarantee. Classification can be done based on DSCP, Priority, IP address and 802.1p.

    - Router/Switch should also support RSVP for end to end bandwidth guarantee.

    - Router/Switch should support modular QoS with Multilevel Priority Queue along with weighted fair queuing.

    - Router/Switch should also support Policy for control plane protection

    - The proposed router should be equipped with optimal memory (DRAM & FLASH) to meet all functional and performance requirements stated in Technical Specifications. The route should support wire speed interfaces, routing and MAC tables as asked in TS.

    - The Router should support 1/2/4 Mb buffers.

    - The Router should support multilevel priority scheduling for voice and video applications with minimal jitter, latency and packet loss.

    - The Router shall support fault-tolerant connections to other network or shared media segment to protect against a primary link failure. If the primary link fails, the backup path shall be automatically activated to maintain network connectivity and throughput.

    - The Router should support the following protocols: BGP, MPBGP, OSPF, RFC 3107, OSPFv2 and v3, Loop free alternate, IP FRR, 6PE, 6VPE, VPLS, Layer2 VPN, uRPF, PIMSM and PIM SSM.

    - The Router should support fast convergence protocols like G.8032, IPFRR, MPLS FRR, BGP prefix independent convergence, VRRP or equivalent and BFD for Routing protocols.

    - The Router/Switch should support Point to Point and Point to Multipoint LSP for Unicast and Multicast traffic.

    - The proposed Router shall support Layer 2 and Layer 2 Multicast.

    - It shall support Ethernet Ring protection based on ITU-T G.8032 v2

    - The Router shall support both IPv4 and IPv6.

    - The Router should support Internet Group Management Protocol versions 2 and 3 (IGMPv2 and v3), IP/MPLS, IP FRR, BGP PIC, MPLS LDP, MPLS TE

    - The Router should support layer 2 and layer 3 MPLS VPN.

    - Shall support Frame sizes from 64 bytes to 1600 and to 9216 bytes on all ports including Jumbo frames.

    - Router shall work as DHCP relay agent.

    - Router should support Eight No of hardware queues are required for per port for flow treatment of traffic, Policy Based QOS, WRED, WFQ, Ethernet OAM and Y.1731 performance management.

    - The Router must support the following security features: Security through ACL filters for layers 2 and layer 3 traffic, MAC address limits and storm control for broadcast, multicast and unknown unicast, Authentication, authorization, and accounting (AAA) with TACACS+ or RADIUS, URPF

    - Router should support at least 8x1Gig (Optical) and 4x1Gig (Electrical)  ports distributed along with two slots. All optical ports should be loaded with Short Range SFPs

# Route Reflector Specifications:

- Each RR router shall be dual homed to the core router and a local edge router. The RR router would be connected on GigE port.

- The router shall support Control Plane Protection mechanism.

- The router shall support online insertion and removal of line cards without requiring the router to be rebooted.

- The router shall support at least 64K routing table (RIB).

- Should be fixed/modular chassis based with Redundant, Power supply

- All interfaces on the routers shall provide wire-rate throughput

- The operating system of the router shall have a CPU based architecture.

- Should be supplied with necessary power cards, data cables, connectors, CD’s, manuals, bracket accessories, wire managers and other appropriate accessories

- The router shall have minimum of 10 Gbps routing throughput

- The router should have capability of minimum 64K IPv4 routes in RIB

- The router should have capability of minimum 32K IPv6 routes in RIB.

- The proposed router should be equipped with optimal memory (DRAM & FLASH) to meet all functional and performance requirements stated in Technical Specifications. The route should support wire speed interfaces, routing and MAC tables as asked in TS.

- Route Learning Speed should be at least 10K per second.

- Should have IPv4 Routing, Border Gateway Protocol, and Open Shortest Path First [OSPF]), BGP for IPv4 and Ipv6, MPBGP, Policy Based Routing (PBR), Hot Standby Router Protocol (HSRP)/Virtual Router Redundancy Protocol (VRRP), IPv6 Routing, and BGP Prefix Independent Convergence, DHCPv6 and OSPFv3 for IPv6, 6PE & 6VPE.

- The router should support RFC 3107 of Carrying Label Information in BGP-4.

- Configuration Roll Back to recover the mis-configured router to last good configuration

- Support Access Control List to filter traffic based on Source & Destination IP Subnet, Source & Destination Port, Protocol Type (IP, UDP, TCP, ICMP etc) and Port Range etc., Time based ACL, AAA using radius or TACACS

- Should Support MD-5 route authentication for RIP, OSPF and BGP, URPF, DHCP snooping, control plane policing, SNMPv3 authentication, SSHv2.

- The RR nodes shall be connected in a partial mesh configuration as shown in the above figure over multiple 1 Gig interfaces to the P network.

- The RR router shall have 4 port 01 Gigabit Ethernet interfaces.

# NOC Router:

These router shall be used to connect the network with NOC applications for network management & reporting purpose. The requirement of these routers may be changed during design and engineering.

- Router should have fans/fan tray(s) for cooling.

- Router should have space for minimum fixed/modular configuration 10x1G & 2 x10G interfaces.

- The proposed router should be equipped with maximum memory (DRAM & FLASH) to meet all functional and performance requirements with minimum memory utilization stated in Technical Specifications.

- Should have support for external USB memory/flash memory/4G SD Memory

- Should have 30 Gbps of throughput.

- Should support Static Routing, Default Routing, RIPv2, OSPFv2, BGP.

- Should support BGP4 routing protocol with CIDR and should have all necessary licenses to enable full features of BGP protocol.

- Should support all types IPv4/IPv6 NAT feature/service. In built NAT capability in router or using specific card.

- Should support Static Routing, Default Routing, RIP for IPv6, OSPFv3, BGP for IPv6.

- Should have all necessary licenses for to enable full features of BGP protocol for IPv6

- Router should have full support for dual stack IPv6 on all interfaces and IPv6 over IPv4 tunnelling.

- The router should support Access Control List to filter traffic based on Source & Destination IP Subnet, Source & Destination Port, VLAN, Protocol Type (IP, UDP, TCP, ICMP etc) and Port Range etc.

- The Router shall support QoS for Traffic Management, class based policing, shaping, CBWFQ, WRED, 802.1p and HQOS (All Ethernet ports)

- The router shall support Control Plane Policing to protect the router CPU from attacks.

- Should support IPv6 QoS, ACL, SSH over IPv6.

- The Router should have WAN protocols like PPP/ ML PPP etc.

- Router should support advance generation security protocols (IKEV1/V2) to ensure data confidentiality and integrity. Router should support Network Group Encryption /IPSec Encryption between multiple location any to any without requirement of one to one tunnel creation. Should support at least 250 tunnels.

- The router shall provide MD5 hash authentication mechanism for RIPv2, OSPF, BGP and MPLS routing protocols.

- The router shall support AAA features through Active Directory and/or RADIUS or TACACS+

- The router shall have Network Address Translation (NAT). Router should be able to use NAT feature on all the interfaces. All software licenses should be included from day one.

In addition to the above, they should also conform to the technical specifications listed in the relevant sections.

The routers mentioned above shall support capacities & Interface requirement as mentioned in BoQ (Appendix-B).

# Service Support:

While primary this network will be used for supporting L-3 VPN and Multicast services to NTAMC network, the network should be built in a way to offer the following services. A detailed description on how these services shall be offered to the customer has to be provided by the bidder in the Technical bid:

# Layer 3 VPN Services:

Layer 3 MPLS VPN services will allow creation of Intranet Managed & Un-managed and Extranet Managed and Un-managed services. This service allows customers to connect their offices together over a public infrastructure with services similar to the leased line based Private WAN network.  The MPLS Layer 3 VPN   has the capability to provide managed services to the end customers, wherein the Employer will manage the complete network, including the CPE for the customer.

The various flavours of Layer 3 MPLS VPN services that need to be supported are as under:

- Intranet Managed and Un-Managed – This service allows connectivity between the various offices of a corporate. This service can be offered both as managed service, wherein the Employer will manage the network for the customer and unmanaged service and offer the service up to the customer locations and the customer manages the Customer Premises Equipment.

- Extranet Managed and Un-Managed – This service allows connectivity between organizations and its partners into a single network. This service can again be managed or unmanaged.

- Besides the above features, the proposed Employer’s MPLS Network shall also support the following:

  - Network-based Layer 3 IP-VPN shall be delivered using Multi-Protocol label Switching (MPLS) as defined under RFC2547bis/4364.

  - The proposed solution shall support private and overlapping IP addressing in    accordance with RFC1918.

  - The Contractor shall state and explain the limitations of the network, in terms of   VPN and VPN routes support of the Network in totality. The Contractor shall state and explain method to overcome this limitation if any.

  - Intranet – Managed & Unmanaged

  - Extranet – Managed & Unmanaged

  - Multiservice Managed Leased Line Services

# Layer 2 VPN Services:

Layer 2 MPLS VPN provides the capability to provision layer 2 VPN based on the following encapsulations:

- Ethernet – This is to provide point-to-point transparent connectivity between devices that use Ethernet encapsulation. This service will be Layer 3 independent and shall be capable of carrying any Layer 3 protocol.

- VPLS Service– This service shall be provided to inter-connect customer LANs directly through Ethernet interfaces.

- H-VPLS Support

- Pseudo wire support in VPLS

- The Contractor shall support the VPLS implementation as under:

RFC 4761, Virtual Private LAN Service (VPLS) Using BGP for Auto-Discovery and Signalling

‘OR’

RFC 4762, Virtual Private LAN Service (VPLS) Using Label Distribution   Protocol (LDP) Signalling.

- Ethernet over MPLS

- VPLS (Virtual Private LAN service)

- Multipoint to Multipoint VPLS (Virtual Private LAN service)

- LL (Leased Line) Services with Multi Class PWE3

# Multicast Services:

This is to provide video multicast services (allows certain types of traffic to be delivered to a limited/targeted group of users in the network) for MPLS Layer 3 VPN. The Contractor shall support IGMPv2 and IGMPv3 Force Join over MPLS-VPN. The router shall also support multicast routing and forwarding for Ipv4 and Ipv6 IGMP v2/v3.

# Traffic Engineering

The Employer’s MPLS network shall support the following Traffic Engineering RFCs:

- RFC 3270, Multi-Protocol Label Switching support for differentiated services

- RFC 3097, RSVP cryptographic authentication.

- RFC 3209, RSVP TE extensions to RSVP for LSP tunnels.

- RFC 3630, Traffic Engineering extensions to OSPFv2

- RSVP Graceful Restart

- Draft-ietf-mpls-rsvp-lsp-fastreroute-03.txt, Fast reroute extensions to RSVP-TE for LSP tunnels.

- Section 9 of draft-ietf-mpls-generalized-rsvp-te-09.txt

# Migration Activities:

All existing NTAMC connectivity and services are to be migrated from exiting MPLS network to the proposed IP/MPLS network under this project. The contractor shall submit migration plan in consultation with POWERGRID, and get it approved by the employer before migration activity. Contractor shall be fully responsible for seamless migration without affecting the customer services. The contractor shall ensure service migration without any downtime.

# IPv6 Support:

- The ability to support Ipv6 is crucial for the network to grow and scale, as given below

- Support of IPv6 in the network.

- Support of 6PE in the network.

- Support of 6VPE in the network.

- RFC 2080 – RIPing for IPv6

- RFC 2460 – Internet Protocol Version 6 (IPv6)

- RFC 2461 – Neighbour Discovery for IP Version 6

- RFC 2462 – Ipv6 Stateless Address auto-configuration

- RFC 2463 – ICMPv6 for IPv6

- RFC 2464 – Transmission of Ipv6 Packets over Ethernet Networks

- RFC 2740 – OSPF for IPv6

- RFC 2545 – Use of BGP-4 – Multiprotocol Extensions for IPv6 Inter-Domain Routing

- In view of the migration policy of Government of India from IPv4 to IPv6 customers are required to be provisioned with dual stack IP address assignment (i.e., IPv4 & IPv6 both address assignment on CPE).

- The Contractor shall deploy the network to run on dual stack (i.e. both IPv4 & IPv6) in backbone as well as in Access network, however, the actual implementation requirement & methodology shall be discussed & finalized during detailed engineering.

- As limited public IPV4 addresses are available, POWERGRID may assign IPV6 addresses only.

# Security:

Security forms an integral part of a network design to protect both the end-customers and the network infrastructure. The solution the Bidder proposes shall have the necessary provisions to implement the necessary security measures.

The proposed solution shall support:

- Back-haul filtering: dropping of traffic destined for a specific prefix at wire speed.

- Ingress and egress packet filtering based on L2-L4 criteria at wire speed. The possibility to log the actions on individual filter rules shall be supported.

- Protection of local services (http, small UDP/TCP servers, DHCP, TELNET, SSH, based on L2-L4 criteria.

- AAA support-Accounting, Authorization and Authentication of users and commands. Support of TACACS+ and Radius or equivalent

- Rate-limiting or disabling the generation of ICMP messages triggered by malicious traffic.

- Authentication of routing protocol updates: RIPv2, OSPF, BGP.

- SSH support

# System Availability & performance

The IP/MPLS based Network is envisaged to provide 99.9% network uptime with predictable performance and tight SLAs. The network is proposed to support QoS features associated with MPLS technology and shall also be able to support such features as may be required in future for implementation of new technologies. The platform is intended to be used for convergent services, integrating data, voice & video. It shall comply to the latest ITU/IETF IP based standard protocols supporting data, voice and video applications.

# MPLS System Maintainability

Once a failure or degradation of performance is detected in the communications system, its cause shall be promptly corrected and defective module shall be isolated / replaced. To facilitate performance trending, efficient diagnosis and corrective resolution, the system shall permit in-service diagnostic testing. This can be executed both locally and from remote locations and it can be run manually and initiated under NMS control. Such testing shall not affect the functional operation of the system.

Preventive and problem oriented maintenance of the communications system shall be performed using diagnostics tools of NMS. They shall support complete maintenance of all system elements and shall permit the diagnosis of any fault.  The Contractor shall provide specialized training (as defined in Training section) required to operate above diagnostics tools. For all redundant systems disconnection and repair of any failed device shall not interrupt the operation of the system.

# System Upgradability and Expandability

Contractor supplied IP/MPLS system and its subsystems shall have provisions for equipment expansion accomplished by the addition of plug-in module(s).  The details of the above provision shall be included in the Bid.

Equipment supplied shall be sized (though not necessarily equipped) to support system/ subsystem expansion to full capacity as applicable for each type of equipment. Power supplies shall be sized for maximum equipped system capacity.

# General Equipment Characteristics

The Contractor shall identify in their proposal any redesign, new or modified hardware, software or firmware modules required. All Contractor supplied equipment shall be new and of the finest production quality. The Employer will not accept modules or printed-circuit boards that are modified by appending wires or components. Wired strapping options shall be incorporated in the board design to meet the above requirement.

All applicable requirements stated in this section shall equally apply to the NMS equipment as specified in subsequent section.

# Revision Levels and Modifications

All hardware, firmware and software delivered as part of the communications network shall be field proven and at the most current revision level. All modifications and changes necessary to meet this requirement shall be completed prior to the start of the factory tests or under special circumstances, on written approval by Employer, prior to the completion of SAT.

All field modifications required to update the hardware, firmware and software to the latest revision level occurring after the above specified testing, shall be fully disclosed, documented and presented to the Employer for their consideration. Satisfaction of this disclosure requirement does not obligate the Contractor to implement the changes provided the latest revision date occurs after the above requirements are met. The intent is to provide the Employer with the documentation and opportunity to consider their implementation.

All field modifications of the hardware, firmware and software that is required to meet installation and/or performance specifications, shall be fully documented as part of the deliverables, both as a separate field modifications record and as corrected equipment/configuration documentation.

Version Management procedure of the software shall be described. This information shall include the current version and the schedule of updates in the past and estimate in the future.

# Channel Capacity requirement for MPLS system

Equipment supplied shall be sized and equipped with sufficient capacity to support the network, management and configuration requirements, including spares. The Contractor shall also be responsible for ensuring that sufficient equipped capacity exists to support all alternate routing and redundant equipment requirements defined herein.

Each subsystem supplied shall be sized (to be equipped as specified) to support full subsystem expansion.

The contractor is required to envisage & estimate the total channel requirements for his fully functional MPLS system and to support the NMS function as described in this Technical specification

# Software Upgrades of offered equipment

The Contractor shall provide software upgrades/ updates and patches if any for the proposed equipment for the entire equipment lifespan. The Contractor shall also provide detail procedure to effect the upgrades. It shall be possible to upgrade the software remotely by using supplied NMS.  It shall be possible to upgrade software without disrupting the service.

Up to the expiry of Warranty Period as defined in this TS or extension of the same, the SW upgrades will be provided free of cost in order to fix bugs or improve upon the specified functional requirements.

--- End of Vol II Section 2-A ---