Section 2-B

Transport Network (MPLS-TP) for NTAMC

NETWORK CONFIGURATION and EQUIPMENT CHARACTERISTICS

- Introduction

This section describes the NTAMC communication network configuration and characteristics of the transport layer equipments which offers evolution architecture for TDM-based transport networks, and is optimized to carry packets. It must preserve the OAM and management characteristics of the legacy transport technologies (TDM) and allows a full end-to-end integration with existing and future IP/MPLS infrastructures and associated subsystems. The requirements described herein are applicable to and in support of Equipment/Network configurations depicted in Appendix-C (BOQ). The existing Telecom Network is also attached at Appendix-B (Telecom Map), and Network Augmentation planned under subject package, shall be executed anywhere in India based on requirement.

A Network Management System (Telecommunications Management Network, TMN) is required which offers comprehensive network and service management for end-to-end MPLS-TP networks.  The equipment’s must be based on single platform architecture functional model and should be manageable from the TMN as single network element. Functional and performance requirements of the TMN are defined in Section 3. The minimum subsystem monitoring and control requirements to be provided are specified as part of the subsystem's equipment characteristics.

The Contractor is encouraged to propose any hardware configurations better suited to the characteristics of the Contractor's standard products as long as the equipment characteristic requirements of this specification are met. Such alternatives shall be considered as provided for in Section 1.8 Table of Compliance.

  - General Network Characteristics

    - Description

POWERGRID’s fiber optic network consists of Backbone network providing connectivity across the major & minor cities and Access network to extend the connectivity from the Employer’s PoP (Mostly Sub-Station) to city Centre / customer premises. The PAN India backbone  will be augmented to connect the POWERGRIS Substation and other location such NTAMC, RTAMC etc.

The equipments must able to support packet services, such as MEF CE 2.0 services, and L2 VPN, can be transported using MPLS-TP...

    - Systems Requirements

The Contractor is encouraged to offer standard products and designs. However, the Contractor must conform to the specification requirements and provide any additional equipment necessary to meet the requirements stated herein. It should be noted that preliminary design information and bill of quantity are indicative only; the Contractor shall verify jointly (with Employer) the design data during the site surveys & detail engineering and finalize the BOQ as required for ultimate design & system performance.

Collectively, all of the defined subsystems are considered the communications system. Required characteristics are defined and specified herein.

    - System Availability &Maintainability

The proposed system must be designed for high-availability. All NEs (MPLS-TP& Amplifiers) provided by supplier must be managed on the same NMS.

The proposed system must provide several ways to limit service interruption: Error prevention, Fault tolerance, Error elimination and Error prediction. The “zero downtime upgrade” capacity must be detailed and illustrated by examples taken from live systems implemented by Bidders. The proposed system must guarantee 99.9% availability.

Once a failure or degradation of performance is detected in the communications system, its cause shall be promptly isolated and corrected. To facilitate performance testing, efficient diagnosis and corrective resolution, the system shall permit in-service diagnostic testing to be executed both locally and from remote locations, manually and initiated under TMN control. Such testing shall not affect the functional operation of the system.

Preventive and problem oriented maintenance of the communications system shall be performed using diagnostics tools such as TMN (as defined in chapter 3). They shall support complete maintenance of all system elements and shall permit the diagnosis of any fault.  The Contractor shall provide specialized training (as defined in chapter 6) required to operate above diagnostics tools.

For all redundant systems disconnection and repair of any failed device shall not interrupt the operation of the system.

    - Performance

The proposed system must support high performance. Vendor shall provide all parameters related to system’s performance and explain how they affect to the performance of the system.

    - System Manageability & Security

The proposed system must have manageability. The manageability may include a centralized provisioning interface, a centralized monitoring/management interface and the capacity to easily integrate the system management to a higher level management system.

The supervisory system of the equipment shall be capable of monitoring all cards which it’s available in the equipment.

The proposed system shall support various authentication method (username/ password, TACAC), system and operation logging, user management by user profiles (user classes) offering them different operation/management capability sets (for example: provisioning, reporting, configuration management, etc.), logging their actions, interception from inside and outside

  - General Equipment Characteristics

The Contractor shall identify in their proposal any redesign, new or modified hardware, software or firmware modules required. All Contractor supplied equipment shall be new and of the finest production quality. The Employer will not accept modules or printed-circuit boards that are modified by appending wires or components. Wired strapping options shall be incorporated in the board design to meet the above requirement.

All applicable requirements stated in this section shall equally apply to the TMN equipment as specified in Section 3.

    - Rack & Sub-rack Requirement

- Equipment cabinet must comply with ETSI.

- Equipment sub rack must suitable to install in 19/21-inch rack

- The supplier shall confirm that their solution supports insert, replace and remove of modules while the equipment is powered up without affecting traffic and damaging any module

    - Power Supply

Equipment must support distributed power from two independent external power supply sources. The power supply interface must provide power supply filtering and monitoring as well as surge protection and must be used ETSI environments, with nominal voltages of -48VDC as defined in the description of equipment at para 2.3.

    - Equipment Capacities

Equipment supplied shall be sized and equipped with sufficient capacity to support the circuit, channelization and configuration requirements, including spares, as identified in the appendices. The Contractor shall also be responsible for ensuring that sufficient equipped capacity exists to support all alternate routing and redundant equipment requirements defined herein.

Each subsystem supplied shall be sized (to be equipped as specified) to support full subsystem expansion.

      - Revision Levels and Modifications

All hardware, firmware and software delivered as part of the communications network shall be field proven and at the most current revision level. All modifications and changes necessary to meet this requirement shall be completed prior to the start of the functional test or under special circumstances prior to the completion of SAT.

All field modifications required to update the hardware, firmware and software to the latest revision level occurring after the above specified testing, shall be fully disclosed, documented and presented to the Employer for their consideration. Satisfaction of this disclosure requirement does not obligate the Contractor to implement the changes provided the latest revision date occurs after the above requirements are met. The intent is to provide the Employer with the documentation and opportunity to consider their implementation.

All field modifications of the hardware, firmware and software that is required to meet installation and/or performance specifications, shall be fully documented as part of the deliverables, both as a separate field modification record and as corrected equipment/configuration documentation.

Version Management procedure of the software shall be described. This information shall include the current version and the schedule of updates in the past and estimate in the future. Contractor has to submit an annual report mentioning all the software, their version, updates etc. running on its network

      - Redundancy Requirements and Protection Schemes

The Contractor(s) shall ensure that there shall be no single point of failure due to communications equipment failure. The failure of one element shall not prevent the use of any other that has not failed. Bidders must state detail about redundancy (1+1 Power Supplier & 1+1 Shelf controller cards) mechanism, wherever asked/ specified, of the proposed system at system level, subsystem level, and board level.

The shelf must be equipped with 1+1 redundant system controller (wherever specified) to provide customers with even higher reliability and stability of data plane & control planes. While the system is running, configuration information, run time data, and other files are constantly synchronized from the active to the standby controller card. In the event of a failure for the active card, the standby card will become active and take over all functions the previously active card has taken, allowing continuous control and full NE functionality. Switch from active to standby card can be triggered either manually by the operator or automatically in case of failure.

The shelf must have redundant fan modules. If one fan fails the speed of remaining active fan units will be increased in order to keep operational temperature in the specified range.

Also the supplier shall indicate if all the common parts of the chassis are redundant for example:  Switch fabric redundancy (incase the equipment support centralized switching fabric)

Other protection schemes supported by the proposed equipment shall also be described in the DRS. In case the equipment offered by the Bidder does not support the above mentioned minimum protection methods, the bidder shall have to provide all additional equipment needed to provide same level of flexibility, redundancy and functionality at no additional cost to Employer. The bidders shall provide details of protection scheme(s) in their Bid.

      - Lost Signal Recovery

At any digital signal level, reapplication of a lost signal shall result in automatic resynchronization and full restoration to normal operation availability. All alarms incident to the signal failure, shall be automatically cleared at the equipment, rack and monitoring levels and normal operation indications restored and reported if applicable.

      - Equipment Lifespan

All equipment supplied as a part of the New Generation Equipment Package shall have a minimum expected life of ten (10) years from the date of final acceptance. The Contractor shall ensure the availability of spare parts and cards/items required for expansion/upgradation for the supplied items for a minimum period of ten (10) years from operational acceptance by the Employer or five (5) years after the declaration of withdrawal of equipment from production whichever is earlier. However, the termination of production shall not occur prior to Operational Acceptance of the system by the Employer.

      - Software Upgrades

The Contractor shall provide software upgrades and patches, if any, for the proposed equipment, free of cost, till maintenance period. The Contractor shall also provide detail procedure to affect the upgrades. It shall be possible to upgrade the software remotely by using TMN. It shall be possible to upgrade software without disrupting the service, as far as possible. Any Software upgrade/ patches required in the Equipment/ TMN etc to address any vulnerability related to security of Network in line with the DoT policy, shall be addressed without any financial implication to POWERGRID till the maintenance period.

      - Optical Fiber Characteristics

The existing fibers already installed in last twenty years comprise of DWSM (G.652) and DSSM (G.653) fibers. The exact details/characteristics of the fibers(s) shall be provided during detailed engineering. The intra-city network is based on DWSM (G.652) fiber. The Equipment shall perform satisfactory as per Technical Specification irrespective of the Fiber Type.

      - Supervision and Alarms

ISM (In Service Monitoring) circuitry shall be provided as a function of the supplied system and shall be monitored on TMN. Local visual alarm indicators shall be provided on the equipment, as a rack summary alarm panel. Additionally, F2/Q2/SNMP interfaces or any other standard interface for a local craftsperson terminal interface and remote equipment monitoring is required. The Supervisory system shall monitor at least the following parameters of the Local and Remote equipment-

- Input and Output power of the components

- Laser Temperature

- Total Power at the input/ output of Optical Amplifier

- B-1 Errors

- Hardware failure

- Power Failure

- Automatic Laser Shutdown

- Fan fault, rack alarm for temperature for Equipment

- GFP related alarms for Ethernet

- TCM monitoring and alarms

- ODU/OTU alarms as per G.709

- BER before FEC at line interfaces

There should be proper indication of alarms whenever these parameters cross the limits. The alarms shall be classified as Critical, Major and Minor alarms.

The Equipment shall support external alarms for monitoring and potential free output contacts for control of station associated devices by the TMN as per section 3.2 TMN Architecture. POWERGRID may configure and use these alarms contacts on its own, wherever required. Contractor shall provide technical support for configuration for testing & configuration of alarm contacts wherever required.

  - Network Equipment

The Network equipment comprises of, MPLS-TP Equipment The details are given below:

    - MPLS-TP System

The product/ solution with low footprint (2 RU) & power efficient shall be offered for the access/ aggregation layer in intra-city. All grey optical interfaces upto 10G (Line & client both) shall support Bi-directional pluggable. The system shall support circuit emulation. MPLS-TP (MPLS-Transport Profile) Equipment shall provide the following:

- OAM functions to detect and isolate faults (Like SDH/ SONET)

- Sub-50ms protection and restoration

- Ring & mesh architecture.

- End-to-end QoS

      - Functional Requirement

The offered MPLS-TP platform must be a reliable packet-based network based upon circuit-based transport networking. The architecture of the MPLS-TP platform must be defined as per the IETF RFC 5921& 6215. The requirements for MPLS-TP forwarding, OAM, and network survivability must be in compliance with RFC 5654 and 5860.

The switching fabric plane shall be bidirectional and non-blocking. The Equipment’s shall support a wire speed L2 switching capabilities under full load condition. The platform shall supports enterprise services like Ethernet E-Line & E-LAN services and aggregate & hand over the traffic to next higher Metro Ethernet network Core Layer.

The management of the MPLS-TP networks must be separable from its client networks so that the same means of management can be used regardless of the client. The management functions of MPLS-TP must include fault management, configuration management, performance monitoring, and security management.

The purpose of the management function must be to provide control and monitoring of the MPLS transport profile protocol mechanisms and procedures.  The requirements for the network management functionality must be as per RFC5950 & RFC5951.

MPLS-TP platform must mandatory run BFD (Bidirectional Forwarding Detection) protocol to provide OAM functionality. It should have resiliency to ensure minimal interruption to traffic when a failure occurs within the system or network.

      - Equipment Requirement

  - The size of equipment shall not exceed 2RU in size.

  - Equipment sub rack must suitable to install in 19 inch, 21 inch rack

  - The supplier shall confirm that their solution supports insert, replace and remove of modules while the equipment is powered up without affecting traffic and damaging any module

  - All NEs and cards provided by supplier must be managed on the same NMS

  - Equipment can be divided in two categories (based on their configuration. The details are given below:

| Sl. No. | Particulars | 4x10G | 20x10G |
| --- | --- | --- | --- |
| 1 | Maximum Size (in RU) | 2 RU | 2 RU |
| 2 | Ports (Min) | 4x10G +4x1G (min. 2 optical + min. 2 Elect.) | 20x10G/ 1G (refer Note below) |
| 3 | Coloured interfaces | Only 10G | Only 10G |
| 4 | Bi-Directional SFP/ XFP | Yes | Yes |
| 5 | Redundancy (Power, Fan & other common modules) | Yes | Yes |
| 6 | Redundancy (Line & Client cards) | No | No |
| 7 | Chassis | Any type | Any type |
| 8 | Power (Input Voltage) | -48V DC | -48V DC |

Note: All 20 ports can be configured as 10G or 1G with suitable SXP/ SFP.

      - Technical specification

        - Standard requirement

- All equipment must comply with the Generic requirements mentioned in TEC/GR/TX/CEN-004/03/MAR-17 for MPLS-TP based carrier Ethernet switch for Aggregation & Access Network Applications. A statement of compliance shall be submitted along with the Bid.

- The Tuneable 10G XFP/ SFP+, in the BoQ for all type of MPLS-TP Equipment must support span loss of minimum 22dB without any amplification.

- All Equipment shall support amplifiers solution of 30dB, 35dB, 40dB & 50dB as per BoQ. Kindly refer the detailed BoQ at Appendix B

- The switching fabric plane shall be bidirectional and non-blocking. The equipment shall support a wire speed L2 switching capabilities under full load condition.

- The Equipment shall support Ethernet PW as specified by RFC 3985 and RFC 4385. It shall support Ethernet PW as specified by RFC 4448.

- The MPLS-TP Equipment has UNI (User Network Interface) and NNI (Network Node Interfaces). The MPLS-TP Equipment shall support UNI or NNI mode of operation on all the ports.

- Optical Ethernet interfaces shall be configured as Client or Aggregate interfaces based on requirement.

- All Equipment shall support jumbo frame of 9000 bytes, MDI-X based auto-uplink feature &dynamic online configuration.

- The Equipment shall support buffering of 50ms to take care of momentary congestion and link failures.

- The equipment shall support IPv4 and IPv6 traffic transparently at data plane.

- All Equipment shall be able to function in terminal mode, hub, & mesh and ring topologies.

- The equipment shall provide managed multi-service integration viz., voice, video and data.

- All equipment shall be fully compliant to MEF 9, MEF 14 or CE2.0 standards.

- It shall support built in power diagnostics feature to monitor optical SFP/XFP ports, system diagnostics to detect hardware failures.

        - Layer-2 Switching Features

- It shall support ingress and egress bandwidth profile per User to Network Interface (UNI).

- Equipment shall support Ethernet functions VLAN (IEEE 802.1Q), Q-in-Q (IEEE 802.1ad), VLAN manipulation: stack/switch/strip, Flow control (IEEE 802.3x), IGMP snooping (V1/V2/V3), Link aggregation (IEEE 802.3ad).

- Equipment shall support Link Layer Discovery Protocol as per IEEE 802.1ab, Logical Link control as per IEEE 802.2&Flow control as per IEEE 802.3x.

- The equipment shall supports enterprise services like Ethernet E-Line & E-LAN services and aggregate the traffic and hand over the same to the next higher Metro Ethernet network Core Layer.

- Service multiplexing: A single MPLS-TP Equipment port shall support multiple Ethernet Services

        - VLAN Features

- The MPLS-TP Equipment shall support creation of VLAN among ports of different types as well as on all ports of the interface cards.

- It shall support VLAN bridging (for outer tag only) as per IEEE 802.1adand user isolation per outer VLAN tag. The user isolation shall be configurable on a per port basis.

- The MPLS-TP Equipment shall support VLAN ingress filtering to prevent VLAN leakage & VLAN tag overlapping allowing some ports to be member of more than one VLAN.

        - Port trucking / Link Aggregation

- The MPLS-TP Equipment shall allow Link Aggregation as per IEEE 802.3 ad to allow link resilience.

- Supports load balancing over Aggregated Links.

- Static/LACP LAG on client ports shall be configured in all equipment.

        - Forwarding Support

- The MPLS-TP Equipment shall support to override The MPLS-TP Equipment port MAC address.

- It shall also support VPLS instance static MAC configuration.

        - MAC Address Learning / Limiting:

- The MPLS-TP Equipment shall support L2 Learning parameters: Sources learning per Port/VPLS/VLAN/Source address.

- The MPLS-TP Equipment shall support to set per port/VPLS instance dynamic MAC learning limit.

- The MPLS-TP Equipment shall support to limit the number of source MAC addresses learnt from bridge port/VPLS instance in order to prevent MAC address flooding DoS attack. This limit is configurable per bridged port.

- The MPLS-TP Equipment shall support dropping of Frames with new source MAC-addresses exceeding the configured value.

- The MPLS-TP Equipment shall support per VLAN MAC learning to ensure MAC addresses are learnt only from a VLAN perspective and automatic/manual disabling of MAC addresses learning for the VLAN where there are less than two ports in that VLAN.

- The MPLS-TP Equipment shall support MAC limiting per Ethernet flow point (EFP) or bridge domain

- The MPLS-TP Equipment shall support MAC address limitation and aging

- All static entries shall NOT be aged.

- The MPLS-TP Equipment shall support Hardware based aging of MAC Address Table entries.

- The MPLS-TP Equipment shall support to enable L2 Aging on every port

- The MPLS-TP Equipment shall support MAC address learning disabling

- The MPLS-TP Equipment shall support to filter and discard all Ethernet frames received on bridged ports in the upstream direction with a specific MAC destination address (DA)

- The MPLS-TP Equipment shall support list of allowable MAC destination address

- The MPLS-TP Equipment shall filter the MAC address from bridge port X if the same MAC address appears in the learning table pointing to bridge port Y (port X and port Y on the same The MPLS-TP Equipment and same VLAN), except in the cases where the aggregation network forwards according to MAC Learning table.

- The MPLS-TP Equipment shall support unique MAC address per device to prevent spoofing and provide traceability.

- The MPLS-TP Equipment shall have the capability to prioritize BPDUs in the data plane (by providing dedicated queues) and in the control plane (by providing dedicated CPU queues for BPDUs).

- The MPLS-TP Equipment shall have the capability to drop BPDUs regardless of the BPDU content. This function is configurable on a per port basis.

        - The MPLS-TP Equipment shall support IEEE 802.1Q Tagging

- Tagged only, this is an IEEE 802.1Q trunk.

- Untagged.

- Hybrid, tagged and untagged frames.

- The MPLS-TP equipment shall support the following additional IEEE 802.1Q features

- Tag insertion, removal and swapping.

- Capability of insertion and removal of second tag.

- Encapsulation translation and rewrites Push, Pop and translate for IEEE 802.1Q or Q-in-Q/IEEE 802.1ad tags.

- Local VLAN and ports cross-connect and multipoint or point-to-multipoint with Hierarchical Virtual Private LAN service (H-VPLS bridge topologies with pseudo-wires) or locally defined bridge domains.

- The MPLS-TP Equipment shall allow configuring all 4K VID on all ports and at the same time supporting all 4K VLANs simultaneously. The user/operator shall be able to reuse the same VLAN-ID on a different port on the same. The MPLS-TP Equipment and terminate it into a different PW/VLAN.

- The MPLS-TP Equipment shall accept untagged, priority tagged and C-tagged frames through a IEEE 802.1Q port.

        - Q-in-Q or IEEE 802.1ad mode Requirements

- The MPLS-TP Equipment’s shall perform classification and service delineation based on outer Q tag and outer IEEE 802.1p bits. (i.e. ignore inner tag)

- It shall support VLAN stacking as per IEEE 802.1ad

- It shall have minimum of 4094 S-VIDs. VIDs “0” and “FFFF” is reserved.

- It shall allow only S-tagged frame in ‘.1ad’ ingress ports. It shall be possible to map the traffic to any PW based on SVLAN tag. It shall be possible to keep or pop the SVLAN tag before forwarding it to the PW.

- It shall be possible to set the priority bits in the S-VLAN priority based on the PCP bits of C-tag of the incoming packet in ‘.1ad’ mode.

        - LSP Mode Requirements

  - The MPLS-TP Equipment shall support LSP Mode Scalability Options i.e. Virtual Private LAN Service (VPLS)

- Support Packet Transport Network solution using PW service tunnel.

- TDM and Ethernet traffic shall be emulated into Pseudo-wires and PW label is added for service identification.

- End-to-end transport path LSP shall be created based on MPLS-TP standard and multiple

- The traffic tunnels shall support per LSP queuing/scheduling i.e. the ability to assign and guarantee per LSP per class bandwidth profiles (CIR, EIR, CBS, and EBS)

        - Quality of Service Requirements

- The MPLS-TP Equipment shall support VLAN CoS preservation.

- The MPLS-TP Equipment shall support VLAN CoS differentiation: It shall be possible to configure the classification of the traffic according to the port, VLAN, IEEE 802.1p bits or TOS/DSCP bits.

- The MPLS-TP Equipment shall support creation of VLAN or Flow with DSCP parameters per service for data, video and O&M traffic for service differentiation.

- It shall support measurement of performance bounds for each flow in terms of throughput, loss, delay and delay variation, according to their respective defined service classes as per Y.1731.

- The MPLS-TP Equipment shall support 16, 32, 64,128,256 and 512 k bytes burst sizes.

- The MPLS-TP Equipment shall support wire speed forwarding on all interfaces and all packet sizes even with classification and QoS activated on all interfaces.

- The MPLS-TP Equipment shall support bandwidth management reports and statistics.

- The Edge the MPLS-TP Equipment shall support 8 class of service per flow.

        - Classification/Prioritization:

- The MPLS-TP Equipment shall support classification of ingress traffic for a specific service based on the following mapping:

- IEEE 802.1P Mapping - it shall be possible to reserve parts of the link bandwidth for frames with particular IEEE 802.1p values.

- Customer IEEE 802.1p priority

- Multiprotocol Label Switching (MPLS) Label Stack Entry:"EXP" Field Renamed to "Traffic Class" Field as per RFC 5462

- Source MAC address

- Destination MAC address

- VLAN ID

- IEEE 802.1Q

- Default queue for non-matching traffic

- The MPLS-TP Equipment shall aggregate incoming traffic into Traffic Classes by following characteristics:

- Incoming port (Logical and Physical)

- Source/ destination MAC

- Link layer priority information as per IEEE 802.1p

        - Mapping

- The MPLS-TP Equipment shall support mapping of DSCP/VLAN/Port to MPLS labels.

- Mapping of IEEE 802.1p and IP TOS bits into MPLS EXP bits.

- Mapping of IEEE 802.1q VLAN tags into MPLS labels

- Aggregate incoming traffic into Traffic Classes by MPLS Label EXP bits (E-LSP)

        - Marking/Policing/Shaping:

- The MPLS-TP Equipment shall support the following Marking/Policing/Shaping requirements

- 8 level Priority marking as per IEEE 802.1p

- Filtering

- Broadcast/Multicast suppression

- Bandwidth management policies

- Single rate three colour marking (srTCM)

- Two rate three colour metering (trTCM)

- Trust the colour of the incoming packet.

- Differentiated Service Two-Rate, Three-Color Marker with Efficient Handling of in-Profile Traffic

- Shaping of Burst Traffic

        - Rate Limiting

  - The Equipment shall support Rate limiting of bandwidth per Port and per class (or flow)

  - Configuration of user bandwidth shall be supported, in steps of

- Mbps for 1-1000Mbps

- 100 Mbps granularity for 1-100 Gbps

  - It shall support defining Committed Information Rate (CIR) and an Excess Information Rate (EIR) for each flow in steps of 1Mbps

  - The MPLS-TP Equipment shall support flow based rate limiting method based on per source address, destination address or both.

        - Queuing:

  - The MPLS-TP Equipment shall support the following queuing

- SPQ – Strict Priority Queuing

- WFQ – Weighted fair Queuing

- No of queues per flow treatment of traffic.

- Per service ingress queues are defined on the basis of Peak/Excess burst Size (PBS/EBS), Committed Burst Size (CBS), Peak/Excess Information Rate (PIR/EIR) and committed Information rate (CIR).

- Per service egress queues have distinct parameters defining its operations like Peak/Excess burst Size (PBS/EBS), Committed Burst Size (CBS), Peak/Excess Information Rate (PIR/EIR) and committed Information rate (CIR).

- Alternate priority routing traffic necessary to keep from starving other priority queues.

        - Scheduling:

- The MPLS-TP Equipment shall support scheduling of queues to strict priority with 2 or more priority levels

- The MPLS-TP Equipment’s shall support the following congestion avoidance mechanisms

- Tail Drop

- Deficit Round Robin (DRR)

- Weighted Round Robin (WRR)

- DWRR (Deficit Weighted Round Robin)

- WRED

- Strict Priority (SP)

- SP + Weighted Round Robin (SP + WRR)

- The MPLS-TP Equipment shall support configuring the scheduling as per the

- Per Hop Behaviour (PHB)

- Physical port or logical port basis.

- Up to 8 forwarding class queues can be configured on a per service basis each with its own CIR, PIR, CBS, PBS and Forwarding Class attribute.

- At least three level dropping precedence levels in each queue.

        - Protection Mechanism:

- The MPLS-TP Equipment shall support linear protection (1:1 & 1+1) as specified in ITU-T Rec. G.8131.1 or G.8131.2.

- For all the modes of protection, The MPLS-TP Equipment shall support protection switching within 50ms of expiration of any manually selected hold-off time (with both SF/SD scenario) and shall support all operator commands (Forced Switching [FS], manual Switching [MS], lockout of protection).

- The MPLS-TP Equipment shall support protection switching within 50ms and Forced switching as analogous to SDH systems.

- The MPLS-TP Equipment shall support protection switching triggered by fault detection, such as loss of signal, loss of frame, signal degrade (BER becomes worse than the predetermined threshold), and so on.

- The MPLS-TP Equipment shall support Forced switching activated by administrative events, such as fibre rerouting, fibre replacement, etc.

- The MPLS-TP Equipment shall support Ethernet ring protection for both S-VID and B-VID as per G.8032

- The MPLS-TP Equipment shall support MPLS-TP Linear Protection support as per IETF standards or Ethernet/MPLS SNC based protection as per ITU-T standards.

        - Circuit Emulation Protocols

- The MPLS-TP Equipment shall support PW using Structure-Agnostic Time Division Multiplexing (TDM) over Packet (SAToP) or MEF8.

- The equipment shall support E1/channelized STM-1 circuit emulation mechanism and it shall support Circuit Emulation Services over MPLS as per PWE3 standard to aggregate the TDM PW from the edge and access CEN.

        - Network Synchronization Requirements

- The MPLS-TP Equipment shall be able to synchronize with an external reference clock.

- It also supports Synchronous clock selection algorithm based on the following parameters:

- Quality of Signal

- Signal fail

- Priority

- External Commands

- It shall support SyncE as per ITU-T G.8261, G.6282 and G.8264 or IEEE 1588v2.

- The MPLS-TP Equipment shall support Network Time Protocol (NTP) for synchronizing with a central NTP server.

- The MPLS-TP Equipment shall support timing and synchronization aspects of Packet Networks based on SyncE as per G.8261. The MPLS-TP Equipment shall support timing characteristics of a synchronization slave clock as per G.8262.

- The MPLS-TP Equipment shall support provide a timing-output interface at 2048 KHz for external synchronization. The output shall conform to ITU-T Rec. G.812, as applicable.

        - MPLS-TP STANDARDS

- It shall support the creation of L2VPN solutions using statically configured PWs and tunnels as per RFC 4664

- The MPLS-TP Equipment shall support MPLS-TP requirements as per RFC 5654 or ITU Y.SUP4

- The MPLS-TP Equipment shall support Architecture of MPLS-TP Layer Network as per ITU-T G.8110.1v2 or equivalent IETF standards

- The MPLS-TP Equipment shall support Interfaces for the MPLS-TP Hierarchy as per ITU-T G.8112 or equivalent IETF standards

- The MPLS-TP Equipment shall support Characteristics of MPLS-TP Network Equipment Functional Blocks as per ITU-T G.8121v2 or equivalent IETF standards

- The MPLS-TP Equipment shall support MPLS-TP General Framework as per RFC 5921 or ITU G.8110.1

- The MPLS-TP Equipment shall support MPLS-TP survivability framework as per RFC 6372 or ITU G.8131/G.8132

- The MPLS-TP Equipment shall support MPLS-TP Data plane Architecture as per RFC5960 or ITU Y.SUP4

- The MPLS-TP Equipment shall support MPLS Generic Associated Channel (GAL/G-ACH) as per RFC 5586 or ITU G.8113.1/G.8113.2

- The MPLS-TP Equipment shall support Definition of ACH TLV Structure as per draft-ietf-mpls-tp-ach-tlv-02 or ITU G.8113.1/G.8113.2

- Support Pseudowire Status for Static Pseudowires as per RFC 6478

- It shall support MPLS On-Demand Connectivity Verification and Route Tracing as per RFC 6426 or ITU G.8113.1/G.8113.2

- The MPLS-TP Equipment shall support Proactive Connectivity Verification, Continuity Check, and Remote Defect Indication for the MPLS Transport Profile as per  ITU G.8113.1/G.8113.2

- The Edge MPLS-TP Equipment shall be connection-oriented packet switching model with traffic engineering capabilities that allow deterministic control of the use of network resources. It shall support traffic engineered point to point (P2P).

- The Edge MPLS-TP Equipment shall support packet transport network solution by using PW service tunnel and MPLS-TP transport technology. TDM and Ethernet traffic are emulated into Pseudo-wires and PW label is added for service identification. End-to-end transport path LSP is created based on MPLS-TP standard (ongoing) and multiple PWs are transported over the same LSP end-to-end in both directions.

- In order to support traffic engineered point-to-point MPLS-TP circuits, it shall support all the MPLS-TP specific requirements specified in the RFCs (RFC 3031, 3032, 3443, 5586, 5860, 5960, 6370, 6426, 6428). No explicit support for control plane protocols is required. NMS Support shall be mandatory.

- It shall be possible to rate limit the traffic in MPLS-TP tunnels at minimum 10 Mbps granularity. MPLS-TP tunnels shall support per E-LSP Queuing as defined in RFC 3270.

- In order to support traffic-engineered Pseudo Wire Emulations (PWE) over MPLS-TP, the system shall support the following RFCs (RFC 4448, 5659, 5994). The compliance for these RFCs is required to the extent of supporting MPLS-TP and PWE. No explicit support for control plane protocols is required. NMS support shall be mandatory.

      - Management

The MPLS-TP system shall support the In-Band & Out-Band management communication channel. The management function of MPLS-TP   includes fault management, configuration management, performance monitoring, and security management. The management function must provide control and   monitoring of the MPLS transport profile protocol mechanisms and   procedures.  The requirements for the network management    functionality must be as per RFC5951.

    - Pluggable Optics

      - All 10G pluggable optics shall be used in NNI & UNI ports of MPLS-TP equipment.

      - Bi-Directional XFP/ SFP+ shall be used with 10G (NNI & UNI) interfaces of all MPLS-TP equipment.

    - Optical Amplifiers for Packet Links

      - Optical Amplifiers

Transmit power amplifier and/or a combination of transmit power amplifier & receiver preamplifier shall be offered for 10G Packet Links to meet the link budget requirements stated in ‘Optical Link Performance Requirements’. Amplifier(s) shall be located with the equipment.

The Optical amplifiers shall meet the alarm requirements stated in section 2.2.3.7 and shall be fully manageable from TMN. It should be possible to monitor the optical parameters and also handle alarms of the optical amplifier from the TMN. The Bidders shall provide detail specifications of Optical Amplifiers in the Bid.

Link Solution supporting difference span losses in the BoQ has been asked in the form of ‘Set’. The ‘Set’ consists of all necessary amplifiers, pluggable Optics for equipment, Patch cords, power cables, Communication/ DCN cable accessories and any other items as per TS, for one end of the link i.e. POWERGRID shall order two (02) sets for implementation of one (01) link.

      - Optical Link Performance Requirements

The optical fibre link performance requirements are specified as follows.

      - Link Budget Calculations

The fibre optic link budget calculations shall be calculated based upon the following criteria:

- Fibre attenuation: The actual attenuation figure (including fibre, splice, connector loss) shall be provided during detailed engineering for link budgeting of backbone/ access links.

- Equipment Parameters: The equipment parameters to be considered for link budget calculations shall be the guaranteed “End of Life (EOL)” parameters. In case the End of Life parameters are not specified for the offered equipment, an End of Life Margin of at least 2 dB shall be considered. This parameter is to be considered for all links.

- Optical path Penalty: An optical path penalty of at least 1 dB (or bidder’s standard design value whichever is higher) shall be considered to account for total degradations due to reflections, inter symbol interference, mode partition noise and laser chirp. This parameter is to be considered for all links.

- Maintenance Margin: Appropriate maintenance margin shall be kept towards cabling, repair splicing, cable ageing and temperature variations etc during planning & execution.

- Other losses: Other losses, if any required specifically for system to be supplied shall also be suitably considered for all links.

- Dispersion: The fibre dispersion shall be taken to be the guaranteed maximum dispersion i.e. 18 ps/nm.Km (1550 nm) for G.652 fibre.

- Bit Error Rate: The link budget calculations shall be done for a minimum BER of 10-12 for 10G and 10-15 for 100G links.

- Link solution shall be offered considering the parameters defined above.

Packet links (10G) up to minimum 30dB/ 35dB/40 dB/ 50dB link loss excluding EOL, Path penalty, dispersion, other losses etc. hall be implemented without repeaters. The abovementioned link loss includes fibre loss, splice loss, connector loss and maintenance margin only. The Optical path penalty, EoL margin and other losses (if any) shall be considered by contractor while offering link solution for above requirements.

      - Link Performance

The Link performance for ES, SES and BBER for the fibre optic links shall correspond to National Network as defined in ITU-T G.826, G.828, M.2101, RFC 2544 and Y.1564.

    - Patch Cords:

The Contractor shall provide (i.e. supply and install) connectorised FODP-to-equipment and equipment-to-equipment jumpers (patch-cords) for each equipment with two installed spares. The contractor shall be required to provide armoured patch-cords at some of the locations prone to rat cuts as identified during survey/ or communicated by POWERGRID in BoQ. Fiber jumpers shall be of sufficient lengths as to provide at least 0.5m of service loop when connected for their intended purpose. Contractor shall provide patch cords of appropriate length for interconnection amongst equipment ports and with FODP for proper routing inside the cabinet and to maintain aesthetics. Contractor shall assess the requirement of various lengths of patch cords during survey and keep sufficient quantity of various length of patch cords to cater any changes during Installation. The Patch cords shall comply with the following:

- All the patch cords shall be Single Mode (G657A Fiber), Simplex type.

- The fiber used for making the patch cord shall be Corning or equivalent.

- The aramid yarn used in the cable of the patch cord shall be genuine and not plastic based. Brand of the aramid yarn should only be either Dupont Kevlar or Teijin Twaron.

- The patch cords shall be suitable for use in the temperature range of –30 degree to +70 degree centigrade. Maximum insertion loss for a patch cord should <0.2dB @ 1310 & 1550nm wavelengths.

- All the patch cords/Connector End face should meet cleanliness standard IEC 61300-3-35, and shall be supplied with appropriate end caps for their connectors on either end.

- Return loss: >55db for PC / UPC and >65db for APC @ 1310 & 1550nm wavelengths. Durability: 1000 times, Typical change: <0.1db (Max).

- The manufacturer’s name must be printed on the outer jacket / sheath of the patch cord. Party should have manufacturer certificate from the authorized laboratories / organizations.

- Individual test reports for both connectors of the patch cord should be printed on a sticker and stuck just below the connector boot on both ends. The test report should mention the following:

  - Actual Insertion Loss value in dB.

  - Actual Return Loss value in dB.

  - Wavelength at which both the Insertion Loss and the Return Loss are tested.

  - Unique serial number for each connector with date to ensure traceability. The printing on the sticker should be of durable quality.

- The internal/ lab test reports in support of above mentioned technical parameters shall be provided by the supplier.

The Contractor shall use the following make patch cords along with all Equipment:

1.   FCI Amphenol

2.   TE Connectivity

3.   R&M

4.   Rosenberger

5.   Huber Suhner

6.   Radiall

7.   3M

In case contractor wants to offer patch cord from a different make, it has to take prior approval before commencement of supply, by furnishing documentary proof of successful operation of patch cords of other make.