Section 3-B

NOC Infrastructure

  - Introduction

- The NOCs shall Monitor, Manage the entire supplied IP/MPLS and MPLS-TP Network (including amplifiers) & all its associated devices. The EMS/NMS installed in the NOC shall constantly monitor the IP/MPLS and MPLS-TP Network elements and take care of services like Fault Management, Performance Management, Service Provisioning (provisioning of all type of services envisaged under this project) Management, SLA Management and provide Reporting facilities to operations personnel, business users and end customers directly or through dash boarding/ reporting system.

- The NOC architecture shall be scalable, low latency and shall have high performance. Critical network equipment such as Routers, Switches and firewalls etc. shall be provided on a redundant mode with device level redundancy at DC (Main NOC) and shall be offered with redundant power supply at both DC & DR (Back-up NOC).

- The design shall be according to industry best practices and follow proper Disaster Recovery procedures and emergency response procedures in case of a disaster at the Main site. The Contractor has to provide 99.9% SLA on the operations of the entire NOC.

- Further details with respect to the specific functional requirements stated in the subsequent sections.

  - Hardware, Operating System and other General points

The following shall be necessary parameters that shall be to be taken into consideration:

- The hardware offered by the bidder should be scalable to cater future requirements for both the application and Database Layers.

- The bidder shall propose Hardware infrastructure with any of the following options. However, Hardware requirements as mentioned in clause 3.28.1 of this Technical Specification shall be complied.

- The Bidder shall offer the hardware comprising of Servers, Storage, SAN Switch, DC Switches etc sized as per Software OEM for running the EMS/NMS Applications and Database.

- Alternatively, the bidder can also propose an OEM Supplied prepackaged and tested Integrated System comprising of Servers, Storage and Related Networking instead of separate Servers, Storage, SAN etc.

- Also, the bidder can propose a mix of either of above to provide a complete hardware solution that caters to the total hardware requirement of all the functional blocks including applications, security etc.

- The Integrated System being proposed (for option a-ii & a-iii mentioned above) shall comply to TEC GR: TEC/GR/IT/SRV-002/01/XXX 2016 or latest revision, if released.

- Server & Storage can be from different OEM.

- Operating System shall support latest version of the Applications and Databases. Desktops shall support Microsoft Windows/Linux Operating System for client applications. Servers shall be Windows/Linux/Unix Operating System based.

- All Operating System shall be supplied with required licenses valid throughout project implementation, Warranty & AMC period.

- Operating system shall have Premium support form OEM.

- Ethernet Network Physical Interface shall be minimum 10G.

- Fiber Channel (FC) based physical interface with minimum 16G shall be supported for SAN communication from Server Farm where separate server and storage are proposed.

- Routers and Switches shall be configured in virtual Load Balancing mode at DC to achieve customized load balancing policy for every application.

- The storage should be sized to keep all the system logs/records including operations & maintenance command logs for a period of at least 12 months in online mode, which should include the actual command given, who gave the command, when it was given with date and time and from where etc. & should be easily accessible. For next 2 years the same information shall stored/retained in a non online mode. The software defined storage solution should support both scale up and scale out architecture. The initial capacity shall be 120% and ultimate capacity shall be 200 % of initial capacity.

- Dimensioning of the Solution shall be based on the assumption that current network size would double in the next three (3) years. The NOC Infrastructure and related Software/DB should be dimensioned keeping in view the 5 year network expansion.

    - Generic Hardware Requirements

- Dimensioning of HW (Servers/Storage) shall be based on

  - No. of Network Elements (NE) = Maximum 1000 NEs each for IP/MPLS and MPLS-TP

  - Services/Circuits Growth for five (5) years: Approximately 500 links/services

  - Alarms Rate (Average / Peak)

  - RCA Computation time

  - Data Polling interval

  - Raw data storage/archival duration

  - No. of Users

  - No. of Concurrent Users

  - Application Response time

  - No. of Reports

- Proposed Server/Integrated System shall have the latest version of 64-bit, high performance RISC/X-86 architecture processors.

- The Data Center Connectivity Architecture shall be flexible to meet network design requirements and shall be positioned in several hardware arrangements or be customized to fit specific requirements.

- Separate Servers to be provided for Application and Database in case of both IP/MPLS and MPLS-TP solutions.

- The HDD capacity and RAM shall be sufficient to meet the requirements and shall be sized as per the network/expansion requirements with maximum 50% utilization at project peak.

- The Server access shall support multiple users with authentication. The authorizations and the privileges of the operators (Remote and Local) shall depend upon the Login and Password.

- All hardware devices shall be supplied along with required Transceivers, SFPs (should be SFP+ type) as per system requirement during implementation, Warranty and AMC period.

- Servers quoted shall have a product support roadmap of minimum 6 years as on the date of bid submission and same shall be submitted. Bidder shall submit OEM undertaking for the same.

- The servers shall have connectivity for Ethernet and FC which may be given in separate interfaces or in combined form and should meet the number of interface requirement as specified in Technical Specification.

- The Server shall be provided with Operating System supported by the application.

- Should be supplied with KVM (Keyboard, Video Monitor and Mouse).

- It shall support remote/virtual KVM capability from an external keyboard, video monitor and mouse to all blades installed in the chassis through the management controllers and shall also support virtual media for DVD access.

- The server storage should be RAID 5 supported. All storage are required to be configured in RAID 5 Protection or above.

- The servers shall be sized such that its processing capacity utilization should not exceed 30% of total processing capacity at peak load during GSAT.

- The Proposed NOC infrastructure shall be in one (1) of the following ways for running the NOC Applications and Database to meet the SLA requirements as mentioned in section-7. The proposed infra should be scalable without disrupting the running services:

- The Solution shall be Hardware comprising of Physical Servers for applications and database, however Storage, SAN Switch, DC Switches etc. shall be sized by bidder based on inputs from various OEMs.

- The Solution shall be OEM supplied prepackaged and tested integrated system comprising of Servers, storage, and related networking. The integrated system being proposed shall comply to TEC GR (TEC/GR/IT/SR V -002/01/MAR 2016).

- The Contractor to size the Server Requirement meeting SLA requirement in scope.

    - Reliability, Scalability, Performance and Backup Restore

- Disaster Recovery solution at all level (Applications//Database Servers, All Network/IT Infra components) shall be provided.

- The NOC System shall be able to operate 24x7 with 99.9% availability or better.

- The implementation of the hardware and software configuration shall allow for future expansion and be able to handle maintenance with minimum disruption to the production environment.

- Single point of failure to be taken care and shall have the capacity for fail-over. Any failure in the individual module shall have a minimum impact on the entire system and shall not impact the availability & performance of the services.

- System shall be scalable and it shall be possible to add additional resources/applications/databases with the network (NE/NMS) and user growth.

- System shall be able to handle performance during all parallel activities which includes users performing large reports fetching activity.

- Data consistency checks and cleansing shall be performed on a regular basis to prevent corruption. Solution shall be able to recover point-in-time due to hardware, virtual machine or software failure(s). Data consistency checks and cleansing shall be feature of online & offline storage.

- System shall maintain data integrity in case of failure of a single component.

- Automatic notifications mechanism of single/complete failure shall be provided. Contractor to factor the detail of Mode & Process.

- Hot Swappable and Redundant Power Supply from two different sources (Power sources shall be provided by the Employer) shall be provided.

- System shall provide the single point capability for backup and recover the application and databases. The System shall have suitable Backup mechanism for taking backup of data and the duration shall be configurable. It shall be possible to take back up at any interval either automatically and under operator control. Incremental backup over a configurable period shall be possible. It shall be possible to test the full backup with DR System.

- The system shall provide capability for secured and controlled remote system administration. The system shall provide the ability for system administrators to logon and administer the system from location other than the system console.

- Contractor should provision the Hardware Sizing & Software licenses for each of the EMS/NMSs and (NEs & VNE’s) for at least 120% NEs on day one with maximum licenses for 1000 NEs each for IP/MPLS and MPLS-TP.

  - Storage (SAN/NAS):

The data should be available ONLINE for at least 12 Months and archived data should be available for at least next 2 years.

- First 12 months data should be stored in SSD/SAS drive based volume and after that data shall be archived in SAS volumes of SAN/Software Defined storage.

- Data older than 12 months can be moved from SSD (Cache Tier) to NLSAS storage (Capacity Tier) for later years.

    - SAN Storage:

- Storage array shall comply with technical specifications mentioned on TEC GR No TEC/GR/I/ESI-001/03/MAR12 for Mid-Tier Storage or the latest version.

- Unified Storage System with no single point of failure architecture. The Storage solution shall have NAS & SAN (Unified) as an integrated offering and use the functionality based on performance requirements of the various application. System must support dual-ported 12Gbps SAS Disk Drives & SATA or NL-SAS Disk Drives. NAS should be available in redundant backup mode.

- System to have at least Two (2) controllers, each controller shall have 64 bit CPU or higher. The Cache shall be mirrored and destaged. The storage arrays shall have at least 128GB & above Cache across controllers.

- The Storage should have battery backed up to avoid any data loss in case of power failure or cache module failure.

- The storage array shall support the features to remove redundant data, Data compression, for data reduction & for efficient use of storage capacities, Secure Multi Tenancy for shared storage infrastructure, Space efficient replicas/clones reduced total storage capacity etc.

- Storage System solution to have minimum 8x16Gbps FC Ports per Storage, 8x1GigE IP Ports for NAS functionality & 2x10Gig Ports for ISCSI functionality.

- Storage System solution shall have minimum 4*12Gbps SAS backend disk ports/lane per controller.

- System shall support 600GB & 900GB 10K rpm 12Gbps SAS Disks or higher. System shall also support 2TB 7200 rpm or higher SATA II or 7200 rpm NL-SAS (Nearline SAS) or higher Disk Drives. System shall also support Flash/Solid state Drives. The system shall be configured with minimum of 40 TB raw capacities ( 70%:30% mix between SAS & SSD/Flash) at main DC site & DR site each. The bidder shall provide additional required capacity as per the sizing. The Bidder can choose either 600GB or 900GB HDD meeting the performance criteria as stipulated in this technical specification.

- System shall have the capability to designate hot spares that can be automatically be used to replace a failed drive anywhere in the system. Storage system shall be configured with required Hot-spares shall be 2% over and above the data capacity configured.

- Easy to use GUI based and web enabled administration interface for configuration, storage management. Storage Management software must include both GUI and CLI tools. It must be able to centrally manage the vendor’s same class of arrays over the network. It must support web-based management. It must support event auditing for security.

- Storage system shall be offered with native space saving capabilities for data storage such as compression and de-duplication without impacting the storage performance.

- Storage management software shall be browser-based/web enabled accessible over IP.

- Storage management software shall be able to perform and monitor local and remote replication operations. Storage management software shall be able to configure and manage tiering.

- Storage management software shall provide real time monitoring and historical analysis of storage performance data such as total IOPS, read%, write %, throughput, etc. for analyzing performance of the systems.

- The DR solution would have the Unified Storage Array with same performance as Main DC. DR storage array would have the same capacity of the production volume as the Main DC.

    - SAN Switches:

- The SAN Switch shall support non-blocking architecture with minimum 32 active ports at 16 Gbit/sec full duplex scalable to 64 ports for the main DC & DR sites.

- The switch shall be rack mountable and be supplied with proper rack mount kit to mount in a standard 19”rack.

- All the ports shall provide autosensing 16 Gbit/sec capabilities for backward compatibility.

- The Switch shall be configured with the Zoning and ISL trunking features or equivalent trunking features. License shall be included in the Configuration.

- Support for web based management and shall also support CLI.

- The switch shall be able to support ISL trunking (using 16 Gbit/sec SFPs)

- The switch shall be provided with SFPs for all active ports.

- The switch shall be provided with fibre cables/ cat 5e/6 cables of appropriate lengths for all active ports / switch ports as to connect to servers, storage & Tape library /NAS.

- The SAN switch must support SNMPv3—via Ethernet port.

- The management software must be able to perform

    - Fabric View

    - Summary View

    - Physical View

    - Configuration

    - Monitoring and Alerts

- The SAN switch must support authentication when managing from GUI, console or telnet to prevent unauthorized access

- The SAN switch must support Secure Shell (SSH) encryption to provide additional security for Telnet sessions to the switch.

    - NOC Switches:

- The Layer-3 switch shall be minimum “Low Range Distribution” LAN switch as per TEC specification (TEC-GR No: GR/LSW-01/04 MAR-11 or latest TEC GR as available as on date of bid submission) except MPLS functionality. The port density shall be as per implementation design finalized during detailed engineering & project execution considering expandability.

- The Layer-3 switches shall be deployed in high availability pairs (1+1) configuration at DC.

- Performance of the switch should be non-blocking.

  - Firewall:

- Separate Firewall should be proposed by the bidder for the NOC applications, as per TEC GR no. GR/FWS-01/02 SEP 2006 (or latest TEC GR as available as on date of bid submission). In addition to GR, following features are also required and the same will prevail upon the related Clauses in TEC-GR, in case of any contradiction.

- The proposed firewall shall be installed in HA mode and secure the NOC applications.

- Firewall shall be handling minimum performance criteria as per category “C” of referenced TEC GR with all features enabled without deterioration in performance. However, it shall be overall responsibility of the bidder to demonstrate the capability of supplied Firewall to meet the volumetric of this project at full load at each site; If at any time, during the warranty & AMC period of this project, it is found that the performance is degraded due to undersized Firewall, bidder shall replace it free of cost with suitable high end solution.

- The Firewalls should be Hardware based, Redundant, Reliable, purpose-built security appliance with hardened operating system that eliminates the security risks associated with general-purpose operating systems with 8 Nos. of 10/100/1000 Base TX interfaces, 4 No’s of SFP in interfaces and 1 Fast Ethernet interface.

- The Appliance should be Full-featured, high-performance firewall, and IP Security/Secure Sockets Layer (IP Sec/SSL) VPN technologies deliver robust application security, user- and application-based access control, and remote user/site connectivity.

- Firewall & Integrated IPSEC VPN Applications should be EAL4 / NDPP / FIPS 140-2 / ICSA certified

- The hardware platform & firewall with integrated IPSEC VPN application has to be from the same OEM.

- Should be redundant supporting Active/Active or Active/Standby Firewall for High Availability & Scalability

- The communication between all the components of Firewall System (firewall module, logging & policy management server, and the GUI/Web UI Console) should be encrypted.

- Firewall should support for static and dynamic routing capabilities. The firewall should support open standard protocols like RIP, OSPF and BGP.

- The firewall should have at least local hard-disk or equivalent, in order to keep the event logs in the event of management server connection failure, etc. Provide detailed information.

- Support for Virtual Firewalls so that the Central site can have an individual firewalling for every remote site in future.

- Virtual Interfaces (VLANs) support for at least 250 VLANs for forming secure server Farms and DMZs.

- Scalability through VPN clustering and load balancing.

- VPN Module integrated with the firewall, IP Sec, Dedicated tunnels, Encryption (DES, 3DES, AES), SHA-1 / MD5 authentication, IKE certificate authentication (X.509), Dead peer detection, NAT Traversal supported, Minimum 100 tunnels support.

- Firewall shall be deployed in high availability configuration.

- Firewall shall be provided with performance criteria, minimum no. of ports & other performance parameters as per category “C” of the TEC GR. However any additional ports required for the deployment shall be provided by the contractor.

- Firewall shall have redundant and hot swappable power supplies. Firewall shall be AC powered (220 V _+ 10% -15%) nominal at 50 ± 2 Hz.

- Interface Connectivity:

- The platform should support VLAN tagging (IEEE 802.1q).

- The firewall must be supplied with at least 2x10Gbps SR Fiber

- Firewall should support Active/Active with Stateful Failover for IP Sec VPN Connections.

- Firewall should have a dedicated Console port

- Firewall should have a Management port

- Firewall/IP Sec VPN Management Server requirements

- Firewall Real-Time Monitoring, Management & Log Collection (with storage) and Systems Resource Monitoring should be provided in the Centralized Management.

- Firewall policies can be enforced from centralized management server at DC.

- Firewall Management Systems should support High Availability and support the automatic replication & synchronization of the security policies and objects. Even if the Firewall can be managed through WebGUI, a Separate Firewall management solution shall be proposed.

- Any changes or commands issued by an authenticated user should be logged.

- Firewall Management system should also provide the real time health status of all the firewall modules on the dashboard for CPU & memory utilization, state table, total # of concurrent connections and the connections/second counter.

- Real Time Monitoring, Appliance Health Monitoring, Security Policy Rollout for Firewall Systems on the Firewall appliances and Logs Collection from the Firewall appliances should be from single Management Server/Appliance. Log Analysis solution and Management solution be proposed as two separate server /appliance if both the functionalities are not available through single console

- The Firewall Management system should only support communication to well defined system and all the communication should be encrypted.

- High Availability (HA) in Active-Active

- Management: Support for Centralized Management

- Regulatory: Common Criteria Evaluated Assurance minimum Level 4 (EAL4) / ICSA Labs / Firewall 4.0 Certification or equivalent.

- System shall have API Interface to Integrate with other applications and tools. Such API tools/ Interfaces shall be provided from day one.

  - Backup & Restore Software:

- Backup Software shall be capable of supporting backup /restore functions.

- Ability to backup data from one server platform and restore it from another server platform to eliminate dependence on a particular OS machine and for disaster recovery purposes, as per software application / database to be proposed by bidder please.

- Software shall have full command line support on above mention operating systems.

- The backup software shall be able to encrypt the backed up data using 128-bit / 256-bit AES encryption on the backup client and shall not demand for additional license, any such license if needed shall be quoted for the total number of backup clients asked for.

- Shall support wizard-driven configuration and modifications for backups and devices.

- The backup software shall support industry standard Common Device Interface for advanced device reporting and handling.

- Shall have cross platform Domain Architecture for User management and shall also have role based User management and access control for multitenant environments.

- Shall support de-duplicated backup and recovery.

- Shall support backups for clustered servers and shall have the ability to backup data from clustered servers from the virtual client, backing up data only once and giving consistent backup in case of failover of nodes.

- Shall support backup / recovery of raw SCSI volumes.

- The backup software shall support backup and restore of NDMP data to media server attached Tape/NAS/Network Storage.

- Shall integrate third party VTL / Disk backup solution which has data de-duplication capabilities.

- The backup software shall support for online backup for all the OS / Data Bases supplied with out of box agents. Backup licenses shall be provided based on database being used in the system.

- Must support Hardware and storage array based snapshot backup for off host zero downtime and zero load on the primary backup client.

- The backup software shall support data movement directly from the backup client to the disk target without passing through the backup server.

- Full backup for all applications and databases should not be more than 4 hours.

  - Tape Drive /Tape Library:

- Tape library shall be used for storing archived data and aged data as per POWERGRID archival Policy.

- The tape library unit shall be configured with fiber based tape drives and shall be quoted with requisite number of drives and slots as per the dimensioning parameters given in this document.

- Tape library shall provide web based remote monitoring capability.

- The tape library shall be configured with FC LTO Gen7 or higher tape Drives. The tape library shall be configured with minimum 4 numbers of LTO Gen 7 or higher tape drives with minimum 48 slots. The tape library shall be provided with 16 numbers of LTO 7 cartridges. Each cartridge shall be minimum 6 TB uncompressed & 15TB compressed capacity. Minimum 4 cleaning cartridge shall also be supplied with the tape library.

- All the hardware supplied shall have support life cycle of minimum 7 years from the date of commissioning.

  - LDAP:

- The contractor shall propose, supply, deploy & Configure COTS (Commercial of the shelf) based LDAP with Active directory or equivalent functionality to the scope of project.

- The required licenses & hardware supply & installation shall be in the scope of the contractor, including support till the contract period.

- The proposed solution shall be configured in HA mode at DC.

- The proposed solution software OEM shall submit the 7 years support availability undertaking.

  - AAA/RADIUS/TACACS:

- The proposed solution shall be supplied with AAA support-Accounting, Authorization and Authentication of users and commands.

- Support of TACACS+ and Radius. The AAA sizing shall be done to support the requirement in TS & considering future expandability up to 2000 devices.

- Supply installation, testing of necessary hardware for AAA shall be in scope of the contractor.

  - Antivirus Software:

- The antivirus software shall be enterprise class along with latest version of software.

- Antivirus software should be upgradable to latest virus definitions on periodic basis.

- The solution should provide endpoint security solutions to eliminate the agent footprint from the virtual machines, offload intelligence to a security virtual appliance, and run scans with minimal impact and enhance virtual machine performance.

- The antivirus license should be considered for the contract life time.

  - RAS (Remote Access Server):

- Supply & Installation of necessary hardware for Remote Access Server (RAS)/Terminal Server for RA (Remote Access) requirement as mentioned in section-8 shall be in scope of contractor.

  - Work Stations:

- NOC shall be set-up with Work-Stations and associated network components, cablings etc.

- Remote Workstation solution shall be set-up with Work-Stations and associated network components, cablings etc.

- The work-stations to be supplied for NOC & Remote Workstation Solution should be All-in-one PC with following minimum specifications:

- Processor - Intel i7 or equivalent,

- RAM- 16GB

- HDD- 500GB

- Operating system-Windows 10 or latest version

- Keyboard, Mouse & Monitor (24 Inch)

  - Craft Terminal

Each node on the network shall include provision for connecting a Local Craft Terminal, i.e. a portable (laptop) IBM-compatible personal computer (PC) to support local commissioning and maintenance activities. It shall also be possible to perform maintenance activities on remote stations. Through the use of this Local Craft terminal, the operator shall be able to:

a.	Change the configuration of the station

b.	Perform tests

c.	Get detailed fault information

d.	Download configuration data, software upgrades

e.	Upload alarm data and other system related data from the network element

f.	Monitor performance parameter of optical ports.

The portable computer shall be connected to a V.24/V.28 interface or Ethernet interface or USB available at network element. Portable (laptop) computers, each complete with necessary system and application software to support the functions listed below, shall be supplied as per quantity (tentative) in BoQ to the Employer.

- There shall be a provision at each network element for connecting to a Local Craft Terminal (LCT). This feature shall be available for all network element equipment proposed by the Bidder. The Local Craft Terminal shall be connected through a serial V.24/V.28 or Ethernet interface or USB.

- The Local Craft Terminal software shall be executable on a portable, laptop PC equipped with latest supported Microsoft Windows or Linux (or its variant) operating systems.

- It shall be possible, once connected to one Network Element, to access other network elements by way of addressing.

- It is desired that the Local Craft Terminal and TMN shall have the same Graphical User Interfaces and they shall behave in the same manner regardless of whether they are executing on a Local Craft Terminal or on TMN.

- The Local Craft Terminal shall provide Graphical User Interfaces that enables the user to interact with the embedded software in the network element in such a way that the user can:

  - Inspect all data residing in the network element, being registers, files or database elements;

  - Set all the appropriate data elements, as appropriate, within the network element.

- The Craft Terminal (Hardware & Software) shall be supplied along with associated items like: cable, connectors, licensed software etc.  The Craft Terminal (hardware) shall be a professional grade device complying with MIL-STD-810G reliability & stability, from a prominent Laptop/ Computer manufacturer and shall be supplied along with latest licensed operating system (OS).

The minimum requirement of each craft terminal shall be as follows:

    - Intel 10th Generation Core-i7 processor

    - Licensed MS Window with Anti-Virus provisioning

    - 16GB

    - 250 GB SSD

    - 14-inch Full HD LED

    - Ethernet 10/100 adapter with RJ-45 connector

    - Minimum 6 hours Battery Life

  - Specifications for Patch cords, Connectors and Attenuators:

    - Patch Cords:

- The patch cords shall be suitable for use with the type of connectors as specified in the BOQ.

- All the patch cords with E2000 connectors shall be supplied with Angle Polish type with Maximum Insertion Loss: <0.3db on insertion in equipment.

- All the patch cords shall be Single Mode (G657A Fiber), Simplex type and of lengths as specified in the BOQ unless stated otherwise.

- The connectors shall be securely connected to both the ends of the patch cords without any play / looseness.

- The fiber used for making the patch cord shall be Corning or equivalent.

- The aramid yarn used in the cable of the patch cord shall be genuine and not plastic based. Brand of the aramid yarn should only be either Dupont Kevlar or Teijin Twaron.

- The patch cords shall be suitable for use in the temperature range of –30 degree to +70 degree centigrade.

- Maximum insertion loss for a patch cord should <0.2dB @ 1310 & 1550nm wavelengths.

- All the patch cords/Connector End face should meet cleanliness standard IEC 61300-3-35, and shall be supplied with appropriate end caps for their connectors on either end.

- Return loss: >55db for PC / UPC and >65db for APC @ 1310 & 1550nm wavelengths.

- Durability: 1000 times, Typical change: <0.1db (Max)

- The manufacturer’s name must be printed on the outer jacket / sheath of the patch cord.

- Party should have manufacturer certificate from the authorized laboratories / organizations.

- Individual test reports for both connectors of the patch cord should be printed on a sticker and stuck just below the connector boot on both ends. The test report should mention the following: (a) Actual Insertion Loss value in dB, (b) Actual Return Loss value in dB, (c) Wavelength at which both the Insertion Loss and the Return Loss are tested, and (d) Unique serial number for each connector with date to ensure traceability. The printing on the sticker should be of durable quality.

- The internal/ lab test reports in support of above mentioned technical parameters shall be provided by the supplier.

    - Connectors:

- Material for FC connector body shall be Nickel Plated Brass / Zinc Alloy

- Material for LC connector body shall be PEI or PPS.

- Material for SC connector body shall be Glass filled PBT.

- Material for all connector boots shall be Polyester (Modified Thermo Polyester Ethylene).

- Maximum Insertion Loss shall be <0.2dB when connecting the patch cords through the adapter.

- FC-PC 0dB adapters shall be square in shape such that they can be mounted on standard FODP.

    - Attenuators:

- The attenuation accuracy of the attenuator shall be +/- 0.5 dB of the specified attenuation value.

    - Make of Patch cords and accessories:

The supplier will supply patch cords and accessories from the following manufacturers/vendors only:

- FCI Amphenol

- TE Connectivity

- R&M

- Rosenberger

- Huber Suhner

- Radiall

- 3M

In case contractor wants to offer patch cord from a different make, it has to take prior approval before commencement of supply, by furnishing documentary proof of successful operation of patch cords of other make.

--- End of Vol II Section 3-B ---