| Sl. No. | Clause No. | Existing Clause/item | Amendment |
| --- | --- | --- | --- |
|  | Clause 2.2.1, Section-IX, Vol II, TS | Layout and levels of structures etc. shall be made by the Contractor, at his own cost, from the general grid of the plot and the bench marks given by the Employer. The Contractor shall make his own arrangements, at his own cost, for locating the co-ordinates and position of piles as per approved drawings and for determining the Reduced Level (R.L.) of the locations with respect to the single bench mark indicated by the Employer. Two established reference lines in mutually perpendicular direction shall be indicated to the Contractor. The Contractor shall provide at site all the required survey instruments, materials and men to Employer for verification of the detailed layout and correctness of the layout and levels to the satisfaction of the Employer so that the work can be carried out accurately according to specifications and drawings. The contractor shall be solely responsible for the correctness of layout and levels. | Layout and levels of structures etc. shall be made by the Contractor, at his own cost, from the general grid of the plot and the bench marks given by the Employer. The Contractor shall make his own arrangements, at his own cost, for locating the co-ordinates and position of piles as per approved drawings and for determining the Reduced Level (R.L.) of the locations with respect to the single bench mark indicated by the Employer. Two established reference lines in mutually perpendicular direction shall be indicated to the Contractor. The Contractor shall provide at site all the required survey instruments, materials and men to Employer for verification of the detailed layout and correctness of the layout and levels to the satisfaction of the Employer so that the work can be carried out accurately according to specifications and drawings. The contractor shall be solely responsible for the correctness of layout and levels. Approach road details also to be included in the layout. |
|  | Clause 2.4.2, Section-IX, Vol II, TS | For foundation in creek, exposed to sea water/ coastal environment, or buried aggressive soil areas, tidal zone etc., Portland Pozzolana Cement conforming to IS 1489 shall preferably be used. | For foundation in creek, exposed to sea water/ coastal environment, or buried aggressive soil areas, tidal zone etc., Portland Pozzolana Cement conforming to IS 1489 shall preferably be used. |
|  | Clause 2.5.3, Section-IX, Vol II, TS | Reinforcement steel shall be stored consignment wise and size wise, off the ground and under cover. It shall be protected from rusting, oil grease and distortions. If directed by the Employer, the reinforcement steel may have to be coated with cement wash before stacking, to prevent scale and rust at no extra cost to the Employer. The stacks shall be easily measurable. Only steel needed for immediate use shall be removed from storage. Fabricated reinforcement shall be carefully stored to prevent damage, distortion, corrosion & deterioration. | Reinforcement steel shall be stored consignment wise and size wise, off the ground and under cover. It shall be protected from rusting, oil grease and distortions. If directed by the Employer, the reinforcement steel may have to be coated with cement wash before stacking, to prevent scale and rust at no extra cost to the Employer. The stacks shall be easily measurable. Only steel needed for immediate use shall be removed from storage. Fabricated reinforcement shall be carefully stored to prevent damage, distortion, corrosion & deterioration. For each classification of steel, separate areas shall be earmarked. It is desirable that ends of bars and sections of each class be painted in distinct separate colors. Steel reinforcement shall be stored in such a way as to avoid distortion and to prevent deterioration and corrosion. It is desirable to coat reinforcement with cement wash before stacking to prevent scaling and rusting in case of storage time exceeding one month. In store, reinforcement bar shall be stacked above ground level by at least 150mm either on brick/cement/stone platform or concrete/brick planks. |
|  | Clause 2.6.5.2, Section-IX, Vol II, TS | The proportions of sand, coarse aggregate, cement and water shall be as determined by the mix design. However, in case of nominal mix concrete (for lean concrete only) the proportions of sand, coarse aggregate, cement and water shall be fixed. The proportions, as determined for design mix concrete and shall always be approved by the Employer. The quantities of the cement, sand and coarse aggregates shall be determined by weight. However, for a faster progress at site, quantities of the cement, sand and coarse aggregates can be converted to equivalent volume. The water shall be measured accurately after giving proper allowance for surface water present in the aggregate for which regular check shall be made by the Contractor. | The proportions of sand, coarse aggregate, cement and water shall be as determined by the mix design. However, in case of nominal mix concrete (for lean concrete only) the proportions of sand, coarse aggregate, cement and water shall be fixed. The proportions, as determined for design mix concrete and shall always be approved by the Employer. The quantities of the cement, sand and coarse aggregates shall be determined by weight. However, for a faster progress at site, quantities of the cement, sand and coarse aggregates can be converted to equivalent volume. The water shall be measured accurately after giving proper allowance for surface water present in the aggregate for which regular check shall be made by the Contractor. |
|  | Clause 2.9.4.7, Section-IX, Vol II, TS | The size of cutting tools shall not be less than the diameter of the pile as specified in the drawing and not more than 75mm. | The size of cutting tools shall not be less than the diameter of the pile as specified in the drawing and not more than 75mm. Size of cutting tool shall be such that so as to achieve required diameter of pile as per the design drawing. |
|  | Clause 2.9.8.6, Section-IX, Vol II, TS | Concreting operations shall not proceed if the contaminated drilling mud at the bottom of the pile bore possess density more than 1.25 T/ cum. or sand content more than 7%. The drilling mud sample shall be collected from the bottom of pile bore. This shall be checked at regular intervals, as decided by the Employer thereafter. | Concreting operations shall not proceed if the contaminated drilling mud at the bottom of the pile bore possess density more than 1.25 1.12 T/ cum. or sand content more than 7%. The drilling mud sample shall be collected from the bottom of pile bore. This shall be checked at regular intervals, as decided by the Employer thereafter. |
|  | Clause 1.2.4.5, Section-IX, Vol II, TS | a) Piles shall be designed as per Annexure ‘C’ of IS 2911 (Part 1/Sec 2) for Lateral load. To calculate fixed head moment ‘MF’ of the equivalent cantilever, Pile embedded into pile cap shall be treated as partially fixed (i.e., restrained against rotation, not restraint against position). Accordingly, equivalent length of cantilever piles shall be taken as 0.6 times of total virtual cantilever length (i.e. E + ZF) of pile. | a) Piles shall be designed as per Annexure ‘C’ of IS 2911 (Part 1/Sec 2) for Lateral load. To calculate fixed head moment ‘MF’ of the equivalent cantilever, Pile embedded into pile cap shall be treated as partially fixed (i.e., restrained against rotation, not restraint against position). Accordingly, equivalent length of cantilever piles shall be taken as 0.6 1.2 times of total virtual cantilever length (i.e. E + ZF) of pile. |
|  | Clause 2.6.3.3, Section-IX, Vol II, TS | For foundation in creek, exposed to sea water /coastal environment, or buried aggressive soil areas, tidal zone etc., Grade of Concrete as mentioned in BPS shall be used. In addition, two numbers of coats of bituminous painting of minimum 1.6 kg/m2 per coat shall be applied on all the exposed faces of the foundation (i.e. pedestal/chimney/ tie beam & base slab/ pile cap). Cost of bituminous painting shall be deemed to be included in the unit rate of the concrete | For foundation in creek, exposed to sea water /coastal environment, or buried aggressive soil areas, tidal zone etc., Grade of Concrete as mentioned in BPS shall be used. In addition, two numbers of coats of bituminous painting of minimum 1.6 kg/m2 per coat shall be applied on all the exposed faces of the foundation (i.e. pedestal/chimney/ tie beam & base slab/ pile cap). Cost of bituminous painting shall be deemed to be included in the unit rate of the concrete/ unit rate of pile foundation. |
|  | Clause 2.6.10, Section-IX, Vol II, TS | All concrete surfaces…. Employer. The unit rate concrete work shall be inclusive of the cost of cleaning and finishing exposed surface as mentioned above. | All concrete surfaces…. Employer. The unit rate of pile foundation / unit rate of concrete work shall be inclusive of the cost of cleaning and finishing exposed surface as mentioned above. |
|  | Clause 2.8.4 c), Section-IX, Vol II, TS | The unit rate of concrete work shall include the cost of construction joints. | The unit rate of pile foundation / unit rate of concrete work shall include the cost of construction joints. |
|  | Clause 2.11.1, Section-IX, Vol II, TS | The foundation bolts/ stubs, as required, for the tower structures shall be supplied by the respective tower contractor. These shall be embedded in concrete while the foundation is cast. The Contractor shall ensure the proper alignment of these bolts to match the holes in the base plate and also co-ordinate with the respective tower contractor for its correctness. The final adjustment of these bolts and their grouting are included in the scope of this contract. Grouting of block outs and the gap between the base plate and top of concrete shall be done by the Contractor after finalization of alignments. The unit rate of concreting shall include the cost of above adjustments, grouting, and skins etc. required for this purpose. | The foundation bolts/ stubs, as required, for the tower structures shall be supplied by the respective tower contractor. These shall be embedded in concrete while the foundation is cast. The Contractor shall ensure the proper alignment of these bolts to match the holes in the base plate and also co-ordinate with the respective tower contractor for its correctness. The final adjustment of these bolts and their grouting are included in the scope of this contract. Grouting of block outs and the gap between the base plate and top of concrete shall be done by the Contractor after finalization of alignments. The unit rate of concreting / unit rate of pile foundation shall include the cost of above adjustments, grouting, and skins etc. required for this purpose. |
|  | Clause 3.5.4 e) iv), Section-III, Vol II, TS | No crossing shall be located over a booster transformer, traction switching station, traction sub-station, Overlap Section or a track cabin location in an electrified area. 0 % of the normal span for which the structure is designed whichever is less. | No crossing shall be located over a booster transformer, traction switching station, traction sub-station, Overlap Section or a track cabin location in an electrified area. The crossing span will be limited to 300 meters or 80 % of the normal span for which the structure is designed whichever is less. |
|  | Clause 1.4 A) ii), Section-IA, Vol II, TS | Sag of Top phase circuit shall not be less than 11.69m. | Sag of Top phase circuit shall not be less than 11.69m 9.62m |
|  | Clause 1.5.9, Section-IVF, Vol II, TS | The length of any individual segment shall be such that it can be easily transported and erected. All similar parts shall be made strictly inter-changeable. Pole segments, as far as possible, shall be fabricated in single piece. In case of restriction due to the size of hot dip galvanising bath, pole segments having outer diameter more than 1m may be fabricated in two halves and seamlessly welded after galvanising. | The length of any individual segment shall be such that it can be easily transported and erected. All similar parts shall be made strictly inter-changeable. Pole segments, as far as possible, shall be fabricated in single piece. In case of restriction due to the size of hot dip galvanising bath, pole segments having outer diameter more than 1m may be fabricated in two halves and seamlessly welded after galvanizing. Higher diameter pole segments may be fabricated in two or more parts and kept minimum. The maximum permissible parts shall be as per follows: Sr no; Ouer dia of pole segment (mm); Max permissible no of fabricated parts per pole segment/seams. 1; upto to 600; 1. 2; greater than 600 and upto 1200; 2. 3; greater than 1200 and less than 2000; 3. 4; greater and equal to 2000; 4. |
|  | Clause 1.7.1, Section-IVF, Vol II, TS | The monopole structure parts…..as part of Quality Assurance Plan. In case pole segments having outer diameter more than 1m are fabricated in two halves, each half portion shall be hot dip galvanized. After seamless welding of both halves, surface of the welded portion shall be cleansed and prepared. Two welded portions shall be galvanized by suitably dipping in hot zinc bath or by metallizing using molten zinc technique. For metallizing, the process as per ANSI/AWS WCZ/D19.0-72 or other National/International standard/Guidelines shall be followed. The thickness of zinc coating by metallizing shall not be less than the minimum specified for hot dip galvanizing. | The monopole structure parts….as part of Quality Assurance Plan. In case pole segments having outer diameter more than 1m are fabricated in two halves, each half portion shall be hot dip galvanized. In case of restriction due to the size of hot dip galvanising bath, pole segments having outer diameter more than 1m and upto 2.5m may be galvanized in two halves and then seamlessly welded. For pole segments having outer diameter more than 2.5m galvanizing shall be carried out in maximum four parts, and then seamlessly welded. After seamless welding of both halves above parts, surface of the welded portion shall be cleansed and prepared. Two These welded portions shall be galvanized by suitably dipping in hot zinc bath or by metallizing using molten zinc technique. For metallizing, the process as per ANSI/AWS WCZ/D19.0-72 or other National/International standard/Guidelines shall be followed. The thickness of zinc coating by metallizing shall not be less than the minimum specified for hot dip galvanizing. |
|  | Drawing Section-XI, Vol II, TS |  | Indicative Diagram for M/C Pole structures is attached at Annexure-A1 |
|  | Clause 1.4 B) ii), Section-IA, Vol II, TS | Maximum Nominal mass of complete conductor (kg/km) | Maximum Nominal mass of complete conductor (kg/km) (kg/m) |