| Sl. No. | # Volume/ Section/ Description | # Clauses | # Bidder’s Query | # POWERGRID Reply |
| --- | --- | --- | --- | --- |
| Volume-II: Technical Specifications |  |  |  |  |
|  | Volume-II_Part-1 of Part-16/section project R-01/3.0 Detailed scope of work for Extension of 765/400/220kV Bhadla-II Pooling Substation/ Point no. (b) , Page Nos. 05 of 12. | b. All associated work such as connection to switchyard bay, Extension of Neutral, Delta & Auxiliary Buses, HVWS & Hydrant Protection of Auto transformers, power & control cabling from common Marshaling box of bank, OLTC to RTCC panel (Except special cables), civil work etc. for Three (3) no. 1-Phase, 500MVA, (765/I3)/(400/I3)/33kV kV Auto transformers being supplied under separate package | As per referred clause, please confirm whether 765kV & 400kV BPI and their equipment support structures required for neutral & delta formation and 765kV & 400kV auxiliary bus of auto transformer, are covered under Price Schedule-1&3 Line Item No.-7 – “765 kV, 1 phase Bus Post Insulator (except for Line Traps) – EA – 12” and Line Item No. – 15 – “420 kV, 1 phase Bus Post Insulator (except for Line Traps). – EA – 42” or same are to be separately included along with structure in Sl. No. 81 - Erection Hardware for Neutral/Delta formation. | All 765kV & 400kV BPI and their equipment support structures required for neutral & delta formation and 765kV & 400kV auxiliary bus of auto transformer shall be quoted under BPS, Sl. No. 81 : “Bus post insulators, spacers, equipment support structures, conductors, Altube, clamp, connectors (including765kV, 400kV, tertiary and neutural bushing terminal connectors etc) for arrangement of natural formation for Transformer(s), DELTA formation for one bank (each bank comprises of 3 single phase units of Autotransformers) and making connection arrangement to connect spare unit in place of any unit of the bank without physical shifting”. |
|  | Volume-II_Part-1 of Part-16/section project R-01/3.0 Detailed scope of work for Extension of 765/400/220kV Bhadla-II Pooling Substation/ Point no. (b) , Page Nos. 07 of 12. | j. Bus post insulators, spacers, equipment support structures, conductors, Al tube, clamp connectors for arrangement of neutral formation of transformer, DELTA formation for one bank and making connection arrangement to connect existing spare unit of autotransformer (by means of 765kV, 400kV, Delta and neutral auxiliary buses). |  |  |
|  | Price Schedule Rev-01 Schedule-1 Line Item -71 | Fabrication, galvanising and supply of Std Pipe Structure - 400 kV, 1-Phase, BPI, (Excluding Wave Trap), including nuts, bolts, all type of washers, pack plates, step bolts and gusset plates excluding foundation bolts | Please also provide separate line item for 400kV High BPI structure used in 765/400kV ICT Auxiliary Bus, in addition to subject 400kV BPI structure line item. | Please refer reply at Sl. No. 1. Support structures are inclusive in BPS Item no. 81. |