| Sl. No. | Clause ref. | Existing Provision of the Bidding Document | Query asked by the Bidder | POWERGRID’s Clarification |
| --- | --- | --- | --- | --- |
| 1 | 1.3, Section-IA, Technical specification | Destringing/ Dismantling 1.3.1 The scope of works includes destringing of conductor, earthwire & OPGW, removal of hardware fittings, insulators, conductor, earthwire & OPGW accessories, etc. and dismantling of tower from some sections of existing 220kV/400kV line and transporting of dismantled materials to designated POWERGRID stores. The Employer shall arrange shut down of charged sections of existing transmission lines, if required, before carrying out destringing & dismantling works as per program finalized in co-ordination with site. Appropriate safety measures along with necessary safety tools and equipments to carry out destringing and stringing operations under the above conditions including mechanical/ structural safety of the towers shall be the responsibility of the contractor. The entire quantity of dismantled line materials viz. tower parts, conductor, earthwire, OPGW, hardware fittings, insulators and conductor, earthwire & OPGW accessories removed from the existing line shall be transported to the designated POWERGRID stores by the contractor at his own cost. The Bidder shall submit his offer taking into consideration transportation cost of the dismantled material. 1.3.2 The contractor shall inspect the affected stretch of transmission lines and shall accordingly devise appropriate methodology/procedure of carrying out the destringing/restringing works during detailed engineering & execution stage in consultation with the site-in-charge. | A) TOWER PARTS, BOLTS & NUTS: i) whether the dismantling of tower is to be done through manual process i.e. piece wise dismantling or to cut from stub level with the help of gas cutter. ii) whether full stub has to be dismenttled by braking concrete till base or stub to be cut up to ground level. iii) We understand post dismantling of towers; distmantled tower material has to be returned in Scrap condition and no need to return back in stacked manner i.e. mark number wise to PGCIL store. Please confirm our understanding or else advice. iv) We understand that Dismantled tower material – Unloading at PGCIL store will be done by PGCIL store team and also segregation/staking at PGCIL Store will be carried out by PGCIL store team. Please confirm our understanding or else advice. v) Dismantled Bolt Nuts cannot be return as per BOM weight/Qty as there are chances of misplaced during dismantling work. Please confirm. B) CONDUCTOR: i.) During dismantling of Conductor, in normal circumstances it is not possible to return the dismantled Conductor rewinded in Drum and returned to PGCIL store. Please confirm and advise whether dismantled conductor has to be rewinded in drum and to be returned in good usable condition or it has to be returned in scrap condition i.e bits of random length. ii.) What percentage will be considered towards conductor erosion to work out unit weight of dismantled conductor. iii.) We understand that - Unloading at PGCIL store will be done by PGCIL store team and also segregation/staking at PGCIL Store will be carried out by PGCIL store team. Please confirm our understanding or else advice. C) EARTHWIRE: i) During dismantling of Earthwire/OPGW, in normal circumstances it is not possible to return the dismantled Conductor rewinded in Drum and returned to PGCIL store. Please confirm and advise whether dismantled Earthwire/OPGW has to be rewinded in drum and to be returned in good usable condition or it has to be returned in scrap condition i.e bits of random length. ii) We understand that - Unloading at PGCIL store will be done by PGCIL store team and also segregation/staking at PGCIL Store will be carried out by PGCIL store team. Please confirm our understanding or else advice. D) INSULATOR: i.) Long rod/Disc/polymer insulator must have been used in existing line, while dismantling some of the insulators/Grading rings may get damaged/misplaced and not possible to get all insulators in good conditions. Please confirm that this is acceptable. ii.) We understand that - Unloading at PGCIL store will be done by PGCIL store team and also segregation/staking at PGCIL Store will be carried out by PGCIL store team. Please confirm our understanding or else advice. E) LINE HARDWARE FITTINGS: i.) It might happen, that corona ring, arcing horn, bolts & nuts are missing in existing line hardware fittings, also while dismantling some of the items may get misplaced so whether dismantled material i.e hardware has to be retuned in scrap condition or weight basis, Pls confirm. ii.) We understand that - Unloading at PGCIL store will be done by PGCIL store team and also segregation/staking at PGCIL Store will be carried out by PGCIL store team. Please confirm our understanding or else advice. | Please refer Amendment-II in this regard. |
| 2 | NA | NA | F) RED ZONE: Based on Site visit carried out by us, there is serve problem in RED ZONE in this region where project is to be executed. Please confirm that all issues related to Nexalite will be resolved by PGCIL; as this will affect the Safety of staff working at Project site and also hamper the progress of project. | As per the provisions of Technical Specification, Right of way and way leave clearance shall be arranged by the Employer in accordance with work schedules. Further, it may be noted that as per the information received from site, ROW issue is not envisaged at the worksite. |