SCOPE OF WORK FOR ADDITIONAL EARTHING WORK IN TRANSMISSION LINES

GENERAL

The contractor shall carry out the work to provide additional shield wire earthing work (for pipe type or counterpoise type) in transmission line of voltage level 220 kV (operations & maintenance being done by Kota TL group) in strict adherence to technical specifications (i.e. this document). Before quoting the rate, the contractor shall be totally aware about local conditions, nature of work and areas (i.e. hilly, plain, normal soil, hard or rocky soil etc.) where such work of additional shield wire earthing shall be done. The owner shall not accept rates with respect to different kinds of soil. The contractor shall quote average rates of installation which shall be applicable for all kinds of soil. Except T &P and manpower, all materials like earthing rod for pipe type earthing, PG clamps, down-lead clamps, GS lugs and earth wire of required size and length shall be supplied by the owner at zero cost. The contractor shall transport the materials from the owner store to work locations at his own cost.

WORK AREA

220kV lines under Kota TL group or as decided by the Engineer­ In-Charge of Kota Sub-station.

DETAILED SCOPE OF WORK

The additional shield wire counterpoise earthing shall be adopted for T/L of voltage grade 220 kV and above depending upon the soil resistivity.