Section 2

NETWORK CONFIGURATION and EQUIPMENT CHARACTERISTICS

  - Introduction

This section describes the communication network configuration and characteristics of the DWDM System (with or without OTN multi-service platform), OTN Equipment, MPLS-TP, Switch, amplifiers and associated subsystems. The requirements described herein are applicable to and in support of network configurations depicted in Appendix C along with BoQ. A Network Management System (Telecommunications Management Network, TMN) is required for management of all equipment to be supplied under the project. The DWDM System with OTN multi-service platform shall be based on single platform architecture functional model and should be manageable from the TMN as single network element. Functional and performance requirements of the TMN are defined in Section 3. Minimum subsystem monitoring and control requirements to be provided are specified as part of the subsystem's equipment characteristics.

The Contractor is encouraged to propose any hardware configurations better suited to the characteristics of the Contractor's standard products as long as the equipment characteristic requirements of this specification are met. Such alternatives shall be considered as provided for in Section 1.8 Table of Compliance.

The Contractor is required to enter into legal agreement with Employer for security and access to Employer’s network as per latest DoT guidelines (agreement format enclosed at Appendix-E).  All security related requirements as amended by DoT on time to time basis shall be followed/ complied by Contractor at no additional cost to Employer.

  - General Network Characteristics

    - Description

The existing all India telecom backbone network is depicted in Appendix-B.  The Backbone network shall be built among major & minor cities (Refer Table C-2 at Appendix-C) and Access network is to extend the connectivity from the Employer’s PoP (Mostly Sub-Station) to city centre / customer premises. The backbone and access network is required to be augmented by installing new DWDM links on separate pair of fibre under this project.  The exact link details (Loss & legth) are provided in Table C-3 of Appendix-C. The Lambda matrix and Backbone equipment configuration details have been provided in Table C4, C-5 & C-6 respectively of Appendix-C for reference purpose.

The contractor shall also be required to interface his DWDM system (with or without OTN multi-service platform) to third party DWDM/ SDH/ Router equipment and carry the data on their system. The DWDM system must support alien lambda configuration also.

The requirements provided in BoQ are tentative and it may vary during actual implementation.

    - General Systems Requirements

Collectively, all of the defined subsystems are considered the communications system. Required characteristics are defined and specified herein.

      - System Maintainability

Once a failure or degradation of performance is detected in the communications system, its cause shall be promptly isolated and corrected. To facilitate performance testing, efficient diagnosis and corrective resolution, the system shall permit in-service diagnostic testing to be executed both locally and from remote locations, manually and initiated under TMN control. Such testing shall not affect the functional operation of the system.

Preventive and problem oriented maintenance of the communications system shall be performed using diagnostics tools such as TMN (as defined in chapter 3). They shall support complete maintenance of all system elements and shall permit the diagnosis of any fault.  The Contractor shall provide specialized training (as defined in chapter 6) required to operate above diagnostics tools.

      - System Upgradability and Expandability

The offered DWDM system must be upgradeable by adding adequate modules (Hardware & Software) in Network. The Contractor shall provide In Line Amplifiers (ILA) upgradeable to OADM/ ROADM as offered under this project by addition of necessary modules {Mux/ Demux, Amplifiers, WSS (if required) etc.}, additional chassis and modification of software (if required).  It shall be possible to upgrade in line amplifiers in a span, without requiring additional modules at end OADM/ ROADM or additional In Line Amplifiers or retuning of other amplifiers in the same link etc. The amplifiers used in ILA should be reusable in OADM/ ROADM after upgradation. The amplifiers, to be used in Network must be Variable Gain Amplifiers (VGA) and adjust gain without affecting links.

Terminations for unequipped channels shall be provided as a part of the DWDM system such that channels may be added later on without affecting traffic. Unequipped channels shall support alien wavelength for capacity enhancement without affecting the performance of existing channels. Contractor supplied equipment and subsystems shall have provisions for equipment expansion accomplished by the addition of plug-in module(s). The details of the above provision shall be included in the Bid.

Equipment supplied shall be sized (though not necessarily equipped) to support system/ subsystem expansion to full capacity as applicable for each type of equipment. Power supplies shall be sized for maximum equipped system capacity.

    - General Equipment Characteristics

All applicable requirements stated in this section shall equally apply to the TMN equipment as specified in Section 3.

      - Revision Levels and Modifications

All hardware, firmware and software delivered as part of the communications network shall be field proven and at the most current revision level. All modifications and changes necessary to meet this requirement shall be completed prior to the acceptance of Equipment/ Link/ Network by employer.

All field modifications required to update the hardware, firmware and software to the latest revision level occurring after the above specified testing, shall be fully disclosed, documented and presented to the Employer for their consideration. Satisfaction of this disclosure requirement does not obligate the Contractor to implement the changes provided the latest revision date occurs after the above requirements are met. The intent is to provide the Employer with the documentation and opportunity to consider their implementation.

All field modifications of the hardware, firmware and software that is required to meet installation and/ or performance specifications, shall be fully documented as part of the deliverables, both as a separate field modification record and as corrected equipment/ configuration documentation.

Version Management procedure of the software shall be described. This information shall include the current version and the schedule of updates in the past and estimate in the future. Contractor has to submit an annual report mentioning all the software, their version, updates etc running on its network

      - Equipment Capacities

Contractor has to supply Equipment equipped with necessary common modules and accessories to meet the requirement specified in BoQ. However, all such Equipment, shall be upgradable/ configurable to the ultimate capacity (DWDM Channels), if required in future, by adding additional Sub-racks & necessary modules. Each subsystem supplied shall be sized (to be equipped as specified) to support full subsystem expansion.

Data communications channelization required to support the TMN subsystems specified in Section 3, are not identified in the appendices. Therefore, the Contractor is required to size and equip the system to include all channelization and channel cards required to support the TMN function as described in Section 3.4 Data/ Communication Channel Requirement and Integration.

      - Alternate Routing, Redundancy Requirements and Protection Schemes

The Contractor(s) shall ensure that there shall be no single point of failure of Network due to equipment failure i.e. the failure of one Network Element shall not prevent the use of any other Equipment that has not failed. Equipment redundancy and Automatic Protection Schemes (APS) are specified in the Table 2-1.

Table 2-1 Equipment Redundancy Requirements Summary DWDM System - Power Supply / Converters: 1:1 APS or distributed power supply - Common Control Cards*: 1:1 APS - Cross-connect card**: n+1/ n:1 - Fan cards*: n+1/ n:1 * Common Control cards are defined as those cards, failure of which affects working traffic (including ASON functionality). If Control/ Processor unit failure does not affect the working traffic, no redundancy of this unit is required. Equipment shall have fan redundancy. No single fan failure shall result in a heat dissipation condition that would result in a service outage. ** applicable with OTN (Type-I & II) multi-service platform

The offered DWDM system shall presently be equipped for unprotected (1+0) mode of operation, except as provided in table 2-1 Equipment Redundancy Requirements Summary. The DWDM system shall be transparent to protection mechanism for OTN platform/ MPLS-TP/ SDH equipment. However, the offered DWDM system shall support 1+1 Optical Channel Protection (provide protection against failure of transponders), Optical Fiber Protection (Provide protection on fiber level, if two pairs of fiber exist between adjacent nodes) etc., with switching within 50 ms using any of the protection mechanism. Individual channel(s) shall also switch within 50 ms incase of errors and Signal degrade alarm on the channel(s). POWRGRID shall configure protection schemes in backbone and access links as per requirement. In case the equipment offered by the Bidder does not support the above mentioned minimum protection methods, the bidder shall have to provide all additional equipment needed to provide same level of flexibility, redundancy and functionality at no additional cost to Employer. The bidders shall provide details of protection scheme(s) in their Bid.

      - Equipment Lifespan

All equipment to be supplied under subject package shall have a minimum expected life of Ten (10) years from the date of final acceptance. The Contractor shall ensure the availability of spare parts and cards/ items required for expansion/ upgradation for the supplied items for a minimum period of ten (10) years from operational acceptance by the Employer or five (5) years after the declaration of withdrawal of equipment from production whichever is earlier. However the termination of production shall not occur prior to Operational Acceptance of the system by the Employer.

      - Software Upgrades

The Contractor shall provide software upgrades and patches, if any, for the proposed equipment, free of cost, till maintenance period specified in contract i.e. 5 years after DLP. The Contractor shall also provide detail procedure before applying any upgrade (hardware or Software) in the Network. Software upgrade shall be remotely through TMN with no/ minimum disruption of service.

    - General Repeater Site Considerations

The Employer desires to optimise the number of repeater/ line amplifier locations in order to minimize installation of equipment and provisioning of power supply & associated infrastructure at intermediate unmanned locations. Accordingly, repeater less fibre optic links (defined as links without repeaters/ line amplifiers) are preferable. All fiber optic links are provided in Table C-3 (as per Link details) may be implemented by the Contractor without adding additional repeaters or line amplifiers as far as possible. Bidders are encouraged to offer solution meeting all performance parameters, considering actual length/ losses provided in the table C-3.

The equipment’s can be deployed in new routes based on requirement and readiness of fibre in new routes which shall be finalized during detailed engineering. All new fiber optic link up to 80 km (22 dB, long haul application) /100 km (30 dB, very long haul application) length based on DWDM shall be implemented by the Contractor without repeaters or line amplifiers. For fiber optic links greater than above length, repeater(s)/ line amplifiers shall be allowed. Repeater/ Line amplifiers shall be installed at locations (POWERGRID S/S or enroute village/ town/ city) at Table C-3.

The Employer desires to use existing locations for placement of DWDM Repeater/ Line amplifier for existing backbone links as given in table C-3. As the Employer has to incur additional amount towards making provisions for new repeater/line amplifier locations, at unmanned locations, therefore,  any solution by a bidders proposed with additional  repeater/ ILA shall attract penalty. For the purpose of bid evaluation a cost compensation by an amount of Rs 7,500,000 (Rupees Seventy Five Lakhs) shall be considered for each additional new repeater/ line amplifier location (other than the existing locations mentioned at Table C-3) towards provisioning of suitable facilities by Employer. The details of existing locations have been provided in the Table C-3 of Appendix-C. If any additional new repeater/line amplifier location (than indicated in the bid) is required by the contractor during implementation, a penalty of Rs. 7,500,000 shall be imposed per additional location.

    - Optical Fibre Characteristics

The OPGW installed in last twenty years on various transmission lines, and comprise of DWSM (G.652) and DSSM (G.653) fibres, and span losses has been provided at Table C-3 of Appendix-C. The intracity network is based on DWSM (G.652) fibre.

In case, bidders need any other details/ characteristics of the fibre(s) to finalize their solution, they may be facilitated to test/ measure & collect the data from filed without any financial implication to POWERGRID before submission of their bid. Contractor has to forward for such request of field test/ measurement to POWERGRID within 15 days of publication of NIT, and POWERGRID reserve the right to accept or reject any such request from bidder(s), if received after 15 days of publication of NIT.

  - Backbone & Access Equipment

DWDM Equipment along with OTN multi service platform has been planned for Backbone and intracity core network. Low capacity OTN, MPLS-Tp & Switched has been planned for access Network to connect end customer. The System/ Equipment envisaged under subject package are as below:

    - DWDM System

The details are as below:

      - Functional Requirement

The Equipment for Backbone and Access network shall consist of two parts - DWDM Transport functions and OTN multi-service platform (wherever required) and shall be based on a single platform architecture functional model. The OTN multi-service platform part of the all equipment may not be required to equipped at all backbone & access locations. Accordingly, the bidders are required to offer DWDM system with OTN multi-service platform for Backbone network as well as Access Network.. The requirements for DWDM Transport functions (for Backbone and Access network) and OTN multi-service platform (for Backbone and Access network) are defined below:

        - Reconfigurable Optical Add-drop Multiplexer (ROADM)

Reconfigurable Optical Add-drop Multiplexer (ROADM) based on Flexible grid enabled Wavelength Selection Switch (WSS) technology is required to remotely configure wavelength in DWDM system from TMN system. WSS based ROADM shall support a Colorless (Tunable), Directionless and Flex-grid Mux/ Demux with Dual add/drop architecture. ROADM allows to drop any wavelength at any node, send any wavelengths in any direction (directionless) using any available port on the network node (colorless).

- ROADM should allow individual wavelengths carrying traffic channels to be added and dropped from a transport fibre without the need to convert the signals on all of the DWDM channels to electric signals and back to optical signals. The ROADM system should support multi-degree configurations from 1 to 4/8 degrees with full mesh capabilities between degrees as per requirement in the BoQ. Each side/degree of ROADM node is to be split logically and maximum three degrees may be provisioned in a single subrack/ chassis. ROADM must support adding and dropping of traffic from any of the directions. It should also support automatic power equalisation. ROADM (DWDM) system must not use separate shelf for WSS, Amplifier (EDFA or Raman), Colorless Mux/Demux, Transponder/ Muxponder, OPM etc. System design must be based on Flex-Grid and shall support a wavelength add/drop structure conforming to the N x 12.5 GHz as per ITU-T G.694.1.

- ROADM shall support the add/drop of any channel in resolution of single channel of bi-directional optical wavelengths up to the maximum of 80 nos (100G) of bidirectional optical wavelengths per degree, with the remaining optical wavelengths passed through to any other location without 3R regeneration. There shall be no restrictions on wavelength assignment to the add/drop channels, allowing any or all unique optical wavelengths to be added/dropped at any location. Both symmetric add/drop (whereby no. of dropped channels is equal to no. of inserted channels) and asymmetric add/drop (whereby no. of dropped channels is not equal to no. of inserted channels) shall be supported.

- The system design shall be such that it implements wavelength reuse at Optical Add/Drop Multiplexers, allowing any particular wavelength that has been dropped from a previous optical multiplex section to be reinserted into any subsequent optical multiplex section to carry new traffic. Any insertion losses incurred in the add/drop process shall be compensated within the system.

- ROADM shall support Automatic Switched Optical Network (ASON)/ Generalized Multi-Protocol Label Switching (GMPLS) control plane as per ITU-T Rec. G.8080 or RFC 3945. ASON switching shall take place within 50 msec and restoration within 2 sec. If restoration time is higher than 2 Sec, Contractor shall provide calculations justifying the time taken in restoration in the network during detail engineering/ implementation.

- In general, the 100G or higher wavelengths shall be interfaced to the DWDM system with or without using transponder/ Muxponder.  The actual implementation may vary during detailed engineering.

- The system shall support Gigabit Ethernet (as per IEEE 802.3 and IEEE 802.3ah), 10G Ethernet LAN & WAN PHY as per IEEE 802.3ae and 100G Ethernet as per IEEE 802.3ba. The equipment shall support 10GBASE-LR and 10GBASE-ER optical 10G Ethernet interfaces for LAN application. The client side interfaces shall be 10GBASE-LW for WAN applications. Transport of GE, 10GbE and 100 GbE shall be fully transparent including preamble.

- The DWDM system shall support following types of “bidirectional” transponders/ muxponders based on pluggable optics on client side for 10G & 100G:

- 100G Transponder (tunable) support single carrier based modulation format with client side having SR10 (100m), SR4 (100m), LR4(10km) & ER4 (40 km) options. It shall support OTU4 & 100GE protocol on client port.

- 100G Muxponder (tunable) support single carrier based  modulation format with following  interfaces:

    - STM-64/OC-192/10GbE WAN PHY

    - 10GE  LAN PHY, also a fully transparent

    - OTU2 or OTU2e with standard FEC

- The DWDM system shall allow direct interface with colored DWDM wavelengths from Third party Router/ DWDM/ OTN system to Mux input and the same signal shall be available directly from the Demultiplexer at the other end. The Transponders/ Muxponders shall also accept Third party Router/ DWDM/ OTN system grey wavelengths at input and demultiplex at other end.

- The DWDM Network (Optical layer) shall support transmission of 100G or above line rate or mix of different type of line rates (above 100G). Traffic modules (Transponder & Muxponders) to be supplied under this package shall work on a single carrier channels system with adaptive modulation. The Muxponder & Transponder hardware shall be upgradable to higher rate (above 100G) through a license upgrade and have necessary capacity & ports density on line & client side. The various line rates (100G and higher) shall be software up-gradable through a license, as given in BoQ, without changing the hardware, considering it meets the link budget requirement. Regenerator modules shall also be software upgradable from 100G to 200G through licenses without changing hardware. An item of license upgrade of line rate from 100G to 200G for Muxponder/ Transponder/ Regenerator Modules is given in BoQ, and POWERGRID shall order as per requirement.

- The DWDM system shall support mixed transmission of 100G & 200G channels” in Backbone and Access Network. The ROADM consists of Directionless, colourless, FlexiGrid Mux/Demux, all common cards, power supply cards, other hardware and accessories including sub-rack etc., except the traffic carrying cards.  The ROADM for 8 direction/ 4 direction shall be equipped for terminating, adding/dropping 80 channels/ 40 channels respectively. The transponder/ muxponder shall be 3R type and shall be tunable to any wavelength  as per ITU-T grid. Increasing number of channels in ROADM from present configuration to full capacity of channels (i.e. 80 channels of 100G) shall not require installation of new modules (except transponders/ muxponders, sub rack for housing transponders/ muxponders, if required, Mux/Demux, if required). The supplied equipment shall have additional slots available to house transponders/ muxponders/ regenerators.

- The ROADM for 3/ 2/ 1 direction may not require adding/dropping of channels on day one. In case of any requirement of add/ drop colourless Mux/ Demux under BoQ will be ordered.

- The quantity of sub-rack as part of ROADM base equipment shall be finalized as below:

|  | ROADM Degree |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 8 | 4 | 3 | 2 | 1 |
| Provision to house minimum quantity of Transponder/ Muxponder | 40 | 20 | 8 | 2 | 1 |
| Provision to house minimum quantity of OPM | 40 | 20 | 8 | 2 | 1 |

        - Optical Transport Network (OTN) Multiservice platform

- The system must fully compliant with OTN interface, framing structure and mapping according to ITU-T G.709 and ITU-T G.798. The platform shall be able to converge all legacy as well as new client interfaces and map them on OTN based signals using ODU-k switching, grooming, consolidation and multiplexing for transportation using DWDM technique. The ODU-k (including ODU flex) shall be cross-connected and groomed for consolidation to the colored OTN line signals at OTU-C2 and shall be fed to the DWDM Mux/ Demux without using Transponder or Muxponder. The platform shall support a combination of client interfaces at different rates i.e. STM-16, STM-64, OTU-1, OTU-2, OTU-2e, Gigabit Ethernet (as per IEEE 802.3 and 802.3ah), 10G Ethernet LAN and WAN PHY as per IEEE 802.3ae, 100GE as per IEEE 802.3ba and SAN based Fibre Channel service. The system must compliant with GFP mapping according to ITU-T G.7041, Wavelength Grid and DWDM system capacity as defined in ITU-T recommendation G.694.1. The system can interwork with other equipment (routers and DWDM) base on ITU-T, IETF, IEEE and OIF standards. Support transmissions over G.652, G.653 and G.655 LEAF fiber (for under-ground cable, OPGW cable, aerial cable)

- The cross connect shall be universal switch fabric supporting packet agnostic switching matrix such that it should be able to handle OTN as well as MPLS-TP LSP /PW signals simultaneously. There must not be any blocking/ limitation on use of cross connection capacity for OTN, MPLS-TP, packets & TDM.

- The platform shall comply with ITU-T G.709 and G.959.1 for OTN specifications including support for OTU-4 & OTUC2 frame format on the line side. All client services shall be supervised by processing ODU/OTU overhead bytes. At least path monitoring and section monitoring shall be supported. The system shall provide FEC on the OTUk signal as per ITU-T G.709 and also support & implement HD-FEC as well as SD-FEC as per ITU-T G.975.1. The system shall be provided with FEC enable/ disable facility through LCT/TMN.

- The platform shall support for Automatic Switched Optical Network (ASON) or Generalized Multi-Protocol Label Switching (GMPLS) based control plane at different layers to achieve intelligent functioning and shall be Optical Interworking Forum (OIF) compliant. The system shall support multi-layer protection mechanism. It shall support SNCP and MSP protection at both ODUk and OCh layers. ASON/ GMPLS control plane shall support automatic creation, discovery and recovery to the ODUk and OCh channels. It shall also provide added facilities such as Traffic engineering of optical channels, Mesh network topologies and restoration, Managed bandwidth to core IP connectivity and introduction of new optical services.

- It shall be possible to configure the network as fully protected, partially protected or unprotected.

- The architecture of ASON shall be based on ITU-T G.8080/ Y.1304. The architectural and routing requirements of switched connections (SC) and soft permanent connections (SPC) for ASON based network shall be as per ITU-T G.7715/ Y.1706. The ASON management by TMN shall be compliant to ITU-T G.7718/ Y.1709. The distribution call, connection management, signaling and management of DCM by the system shall be as per G.7713/ Y.1704. The GMPLS architecture shall be as per IETF RFC 3945. The GMPLS traffic engineering, routing, Link Management Protocol, Signaling function, signaling RSVP-TE, signaling procedure etc. shall be as per various IETF  RFCs 4802, 4202, 4361,3471 and 3473.

- Following variants of service level agreements shall be achieved within the ASON/GMPLS architecture:

- Platinum Service:  Permanent 1+1 path restoration + ASON/GMPLS based layer-1 restoration.

- Gold Service:   Permanent 1+1 path protection.

- Silver Service:   ASON/GMPLS based layer-1 restoration.

- Bronze Service:   No Protection mechanism for network restoration.

- ASON switching shall take place within 50 msec and restoration within 200 msec. If restoration time is higher than 200 ms, Contractor shall provide calculations justifying the time taken in restoration in the network during detail engineering/ implementation.

- The system shall support performance parameters relating to ODU-k as specified in ITU-T G.8201 and G.7710. and alarm reporting to TMN related to ODU-k as per ITU-T G.798.

- OTN Equipment has been divided among following categories, as below:

- OTN XC Type-I

The platform is intended to use along with DWDM equipment, and multiple channels from OTN platform may be used along with DWDM channels among major metros routes as specified in Appendix-C. Colored signal/ lambda from OTN Line module will be directly fed into the DWDM optical layer along with DWDM Transponder/ Musponder. The cross-connect module of the platform shall be universal packet/ OTN switch matrix type and the platform shall support at least 4 Tbit/s equivalent ports bi-directional non-blocking matrix with each port configurable for any direction of transmission. The platform shall support 100G/ 100GE (at least 20), 10GE (at least 50) and 1Gbe/ 2.5G (at least 20 ports) or a combination of above.

The OTN platform part of equipment shall have slots available for fully utilizing 4Tbps capacity and slots should not be utilized for placing ROADM/ Amplifier/ OSC or any other passive cards.

- OTN XC Type-II

The platform is intended to use primarily in Access Network in mesh configuration, where 100G Line port (single channel on fiber) will configured in one direction. The cross-connect module of the platform shall be universal packet/ OTN switch matrix type and the platform shall support at least 600Gbps equivalent ports bi-directional non-blocking matrix. The platform shall support 100G (at least 4 Line), 10GE (at least 20) and 1Gbe/ 2.5G (at least 10 ports) or a combination of above.

Amplifier Solution (10G & 100G) up to minimum 30dB (120 Km)/ 35dB (140 Km)/40 dB (160 Km)/ 50dB (200 Km) link loss excluding EOL (2dB), Path penalty (1dB), dispersion, other losses etc shall be implemented without repeaters. The above mentioned link loss includes fibre loss, splice loss, connector loss and maintenance margin only. The Optical path penalty, EoL margin and other losses (if any) shall be considered by contractor while offering link solution for above requirements.

- OTN XC Type-III

The platform is intended to use primarily in Access Network in ring configuration, where 100G Line port will configured in one direction. The cross-connect module of the platform shall be universal packet/ OTN switch matrix type and the platform shall support at least 300Gbps equivalent ports bi-directional non-blocking matrix. The platform shall support 100G (at least 2 Line), 10GE (at least 10) and 1Gbe/ 2.5G (at least 4 ports) or a combination of above.

Amplifier Solution (10G & 100G) up to minimum 30dB (120 Km)/ 35dB (140 Km)/40 dB (160 Km)/ 50dB (200 Km) link loss excluding EOL (2dB), Path penalty (1dB), dispersion, other losses etc shall be implemented without repeaters. The above mentioned link loss includes fibre loss, splice loss, connector loss and maintenance margin only. The Optical path penalty, EoL margin and other losses (if any) shall be considered by contractor while offering link solution for above requirements.

- The configuration of OTN Type-II & OTN Type-III platform is summaries below:

| Sl. No. | Particulars | Type-II | Type-III |
| --- | --- | --- | --- |
| 1 | Maximum Size (in RU) | 12 RU | 12 RU |
| 2 | Ports (Min) | 4x100G+ 20x10G | 2x100G+ 10x10G |
| 3 | Client interfaces | Min 20x10G/ STM-64 & Min 10x1G/ STM-16 | Min 10x10G/ STM-64 & Min 4x1G/ STM-16 |
| 4 | Cross Connect Capacity | Min 600 Gbps | Min 300Gbps |
| 5 | Redundancy | Cross-connect, Power & Fan | Power & Fan |
| 8 | Power (Input Voltage) | -48V DC | -48V DC |
| 9 | Bi-Directional SFP/ XFP | Yes | Yes |

- Equipment Requirement

- All NEs/ modules & associated amplifiers must be managed through the same NMS.

- All optical interfaces shall be based on SFP/SFP+/XFP/CFP2/ CFP4/ QSFP28 optical devices. The SFP/ SFP+/XFP/CFP2/CFP4/QSFP28s shall support Digital Diagnostics tool for managing the optical interface according to Multi-Source Agreement and shall have suitable user interface in TMN and craft terminal. The SFP/ SFP+/XFP/CFP2/CFP4/QSFP28s shall be hot pluggable.

- Client port shall support Bi-Directional SFP/ SFP+/ XFP

- Equipment should support following traffic Client service and all client types should be supported in same or different client card. Client and line should support ODU Flex for CBR and Packet for minimum 4 ports. Purpose of ODU-Flex is Rate limit on client.

  - Ethernet 1GE/nGE

  - Ethernet 10GE LAN/ WAN

  - STM-1, STM-4, STN-16, STM-64

  - ODU0, ODU1, ODU2/2e, ODU4, ODUflex

  - FC100, FC200, FC400

- Feature requirements

- The system architecture must support Point-to-Point, ring and mesh network applications

- The system must support Forward error correction as per the G.709

- The system shall support internal DCN realization via OTN frame overhead (GCC0, GCC1, and GCC2 channel)

- The system must support multi- services: SDH (STM -16/64), Ethernet (1GE/10GE), OTN (ODU0 1, 2, 4, flex) and FC-1G/2G/4G.

- The bidder shall indicate if all the common parts of the chassis are redundant: Switch fabric redundancy (in case the equipment support centralized switching fabric), Fan Power supply redundancy and    Shelf-controller redundancy if traffic affecting.

- The system must support 1+1 Line side protection (APS)

- Wavelength range must be in C-band (1529.16 nm to 1560.61 nm)

- Point to Point link without amplification must have a reach of at least 22dB.

- The Bidder must confirm BER (Bit Error Rate) of network at Begin of life (BoL) is less than 10E-12 with 10G & 10E-15 with100G interface within 24hours. The Link performance for ES, SES and BBER for the Fibre optic links shall correspond to National Network as defined in ITU-T G.826, G.828, M.2101, RFC 2544 and Y.1564.

- All equipment must work in range from -42VDC to -60VDC

- Power connection must be complied with ITU-T Rec.K.27, PR EN 50174-1,2 and EN 50310

- All equipment must be complied with ETS 300 019-1-1, ETS 300 019-1-2 and ETS 300 019-1-3, or equivalent Indian Standards.

- Working temperature range must be from 5 to 50 C degree or larger

- Working humidity range must be from 5% to 90% or larger

- Capacity/ability

- Platform must support Line side & Client interfaces as per BoQ.

- The system should support automatic power reduction (APR) or automatic laser shutdown (ALS) on client side due to following trigger:  Signal fail on line side & Signal degraded on line side

- Interface card must support multiple services e.g. OTU2 (e), STM-64 and 10GE LAN/WAN over single interface card

- The equipment must support Multi-rate interface card (an interface card able to support a range of services with different bit-rates over different ports). In case using 100G DWDM wavelength, the equipment must support 100GE/OTU4 multiplexed into 100G OTN Line interface by ODU4. In case using 10G DWDM wavelength, the equipment must support 10GE/STM64 multiplexed into 10G Line by OTU2.

- The line interfaces must support tuneable wavelength. The equipment must support single-carrier100G coherent solution compliant with OIF recommendation

- The supplier’s solution must support:

        - GbE signal is mapped into ODU0

        - 10GBE LAN PHY signal should be mapped into ODU2

        - STM-64 / 10G POS /10GBE LAN PHY signals should be able to mapped into ODU3.

        - FC400/FC800 signals should be able to mapped into ODU flex

- OTN and SDH traffic must be aggregated into the same wavelength

- Absolute wavelength stability/ Central Frequency deviation ≤ 2.5 GHz

- Chromatic dispersion (CD) ≥ 12000 ps/nm

- Polarization mode dispersion (PMD) ≥ 20 ps

- Management

The OTN system shall support the general communication channel (GCC) defined by, an in-band side channel used to carry transmission management and signaling information within optical transport network elements. Two types of GCC must be available:

- GCC0 – two bytes within OTUk overhead. GCC0 must be terminated at every 3R (re-shaping, re-timing, re-amplification) point and used to carry GMPLS signaling protocol and/or management information.

- GCC1/2 – four bytes (each of two bytes) within ODUk overhead. These bytes must be used for client end-to-end information and shouldn't be touched by the OTN equipment.

        - Optical Amplifier and Repeaters/ Regenerators

Optical Amplifier and Repeaters/ Regenerators are required for DWDM links as per clause 2.3.1.6.

      - Redundancy and Protection

Redundancy and protection requirements have been specified in clause 2.2.3.3.

      - Service Channel

The DWDM system shall support a Optical Supervisory Channel (OSC) as per G.692 in addition to required 80 channels on G.652 fibre. It should be possible to drop OSC information from the DWDM equipment and add it on to existing DWDM/SDH network of Employer for carrying over to control centres for management by TMN.

      - Supervision and Alarms

ISM (In Service Monitoring) circuitry shall be provided as a function of the DWDM system and shall be monitored on TMN. Local visual alarm indicators shall be provided on the equipment, as a rack summary alarm panel. Additionally, F2/Q2/SNMP interfaces or any other standard interface for a local craftsperson terminal interface and remote equipment monitoring is required.

- The Supervisory system shall monitor at least the following parameters of the Local and Remote equipment-

- Input and Output power of the Optical Amplifier(s), Transponder, Muxponder, Insert channel, Dropped channel, individual optical channel, Regenerator, OSC.

- Laser Temperature of Optical Amplifier(s), Transponder, Muxponder and OSC.

- Total Power at the input/ output of Optical Amplifier(s).

- B-1 Errors of individual Transponder/ Muxponder

- Mux and Demux operation

- Hardware failure

- Power Failure

- Automatic Laser Shutdown in the section

- Fan fault, rack alarm for temperature for Equipment

- GFP related alarms for Ethernet

- TCM monitoring and alarms

- ODU/OTU alarms as per G.709

- BER before FEC at OTU-1, OTU-2, OTU-4 and OTU2C line interfaces of Transponder/ Muxponders

There should be proper indication of alarms whenever these parameters cross the limits. The alarms shall be classified as Critical, Major and Minor alarms.

- The Backbone Equipment shall support collection of at least four (04) external alarms for monitoring and four (04) potential free output contacts for control of station associated devices by the TMN as per section 3 TMN Architecture. POWERGRID may configure and use these alarms contacts on its own, wherever required.  Contractor shall provide technical support for configuration for testing & configuration of alarm contacts wherever required.

      - Optical I/O Characteristics, General Parameters and other features

The DWDM system shall have the capability to work on G.652 (80 channels of 100Gbps) at discrete frequencies as specified in ITU-T Recommendation G.692 and flexible DWDM grid as per ITU-T recommendation G.694.1. The DWDM system shall support both 100Gbps and higher bit rates with required number of wavelengths as mentioned above. BoQ of interface cards have been provided in the appendices. The bidder shall provide details of interface and other requirements in the bid.

- The system shall work in Unidirectional mode of operation using dedicated separate fibre for ‘TX’ and ‘RX’ directions in terrestrial long haul/very long haul applications, with or without optical line amplifier before requiring electrical regeneration as per ITU-T Recommendation G.692. The equipment shall use Out of Band Forward Error Correction as per ITU-T G.709 for achieving error free long haul communication without regeneration.

- The DWDM system shall support 20 spans (or more) of 22 dB (80 km) between Line amplifiers & ROADM/ OADM in Long haul application code and shall reach 1600 kms route length before acquiring 3R regeneration for 80 nos channels of 100G each. The DWDM system shall also support 12 spans (or more) of 30 dB (100 km) between Line amplifiers and ROADM/ OADM in Long haul application code and shall reach 1200 kms route length before acquiring 3R regeneration for 100G channels.

- DWDM System along with OTN shall support spans length longer than 150 km & span losses higher than 42 dB, also (Refer span length/ losses in the Network at Appendix-C). The span lengths in Access Network may have 38 dB losses in 40 km, so the supplied system must support such length/ losses without compromising on quality/ performance of network.

- The system shall support optical transport network (OTN) specifications including OTU-1, OTU-2, OTU-4 and OTUC2 frame format on line side and OTU-1, OTU-2, OTU-4 rates at client side as per ITU-T G.709 and G.959.1.  All client services shall be supervised by processing ODU/OTU overhead bytes. At least path monitoring and section monitoring shall be supported.  The system shall support at least two levels of TCM as per ITU-T G.709.

- The DWDM system shall support hybrid transmission of 100G and higher rate (if supported) channels without restriction such as wavelength sub-bands for different line rates. However, link budgeting for each link shall be done for 100G as per lambda matrix requirements provided in Appendix-C. All the routes have been planned using 100G per channels.

- Optical Protection Modules (OPM) shall be used on client as well as line side protection based on the requirement and shall be finalized during detail engineering. It also supports configuration of protection scheme as Revertive/ non revertive. The Optical protection Modules shall be part of base equipment and fully accessible & configurable through NMS. OPM module shall have provision for optical power measurement at all input and output ports. The bidder may also offer alternate solution subject to meeting the same functionality.

- The DWDM system (including ROADM/ OADM and Repeaters and excluding In Line amplifiers) shall have in-built capability to measure optical power and wavelength for each of the wavelengths of DWDM system. The DWDM system shall also provide in-service path measurements that include span loss, amplifier gain, wavelength power, pre-FEC BER, CD and latency. In case the equipment offered by the Bidder does not support the in-built capability, the bidder shall have to provide all additional equipment needed to provide same level of functionality at no additional cost to Employer. The Muxponder cards and Transponder card shall have 100G link testing feature using pseudorandom bit sequence (PRBS) patterns.

- All Transponder/ Muxponder card shall support both Hard Decision-FEC or Soft Decision-FEC.

- There should be a provision of built-in software controlled Optical equalizer or Optical pre-emphasis in the equipment to meet the gain flatness and noise requirements automatically with the insertion or deletion of channels. The removal/ insertion/ failure of any Transponder/ Muxponder card shall not disturb the working or deteriorate the BER of the other working channels in the system. The individual optical channels power must be automatically adjustable by system without any manual intervention, for channel power balancing, in order to achieve an optimum OSNR. The DWDM System shall support “channel power balancing” for all optical channels at the output of a node for adjustment at the proper power level for optimized sub-sequent transmission. The offered system shall support the option to automatically adjust the launched power with specified limits with the addition or removal of wavelengths keeping the total optical power constant. Channel power balancing shall apply to all channels, independently from the channel path in the node whether band pass through or channel pass through.

- The equipment shall have the provision of Automatic Laser Shut-Down (ALSD)/ APR in the case of fibre-plant breakdown and automatic re-start on restoration of fault in accordance with ITU-T  G.692, G.681, G.664 and IEC 60825-1 (latest edition). The system shall restore within the restoration time as per ITU-T G.664 & G.681 after restoration of the fibre-plant breakdown or a faulty amplifier. The optical access ports should be designed to protect themselves against the entry of dust when they are not occupied by an external fibre-optic connection. The optical access port shall be easy to clean by the user.

- The same DWDM system shall also work on G.653 fiber, incase requirement arises in future. Table 2-2 provides the electrical and optical characteristics as well as other general parameters for DWDM system.

| Table 2-2 Optical I/O Characteristics and General Parameters |  |
| --- | --- |
| Specifications and Parameters | Per ITU-T G.692, G.694.1 |
| Optical Source Lifespan | Better than 5 X105 hours |
| Optical Fibre Type | G.652, G.653 & G.655 |
| Optical Connectors | Type FC-PC/ SC-PC at FODP/FDF side |
| Transmission Quality | Per ITU-T G.825.1, G.783, IEEE 802.3 |
| Source Primary Power | -48 Vdc |
| Optical Interfaces | Per ITU-T G.707,G.957, G.691, G.783, G.825, IEEE 802.3, IEEE 802.3z, IEEE 802.3ah, IEEE 802.3ae, IEEE 802.3ba |
| Forward Error Correction | As per G.975, G.709 |
| OTN Interfaces | Per ITU-T G.709, G.959.1, GR-2918-CORE |
| SAN Interfaces | Per ANSI X3.296, X3.230, X3.303 |

      - DWDM Optical Amplifiers and Repeaters/ Regenerators

        - Optical Amplifiers

DWDM Amplifiers (EDFA) and Raman Amplifiers has been asked in BoQ. Individual amplifier or a combination of Amplifiers (EDFA & Raman) shall be used to meet the link budget requirements stated in clause 2.3.1.8 Optical Link Performance Requirements. Both the transmitter booster amplifier and the receive preamplifier shall be co-located at the terminal stations and shall be an integral part of DWDM Terminals whereas the line amplifier shall be located somewhere along the transmission line away from the terminal stations.

- The Optical Amplifier shall be bit rate independent i.e. it should support all 100Gbps and above bit rates. Addition/ provisioning of new channels (wavelengths) shall not require retuning of optical amplifiers or addition of new optical amplifiers. The optical amplifiers shall be equipped for the full capacity of channels.

- The Optical Amplifiers should have the following mechanism to maintain error free system operation under dynamic conditions:

              - Fast gain control loop: to protect against short term transient conditions such as sudden loss of signal.

              - Slow output power control loop: to protect against long term conditions such as fibre aging.

- Raman Amplifier, if offered, shall comply to ITU-T standard G.664 for eye safety and having features of supporting additional independent mechanisms for detecting a fiber break or open connector, allowing automatic shut-down of the Raman pump module and should be able to automatically restart once the disruption has been corrected and should be monitored centrally from the TMN.

- The optical amplifiers shall respond automatically to changes in the number of channels without the need for manual intervention or realignment. Variable Gain Amplifier (VGA) units to automatically compensate for variations in span attenuation due to aging and splicing etc.

- There shall be provisioning of monitoring points for monitoring of channel power, OSNR, Pre-FEC BER and wavelengths at the output points of Booster Amplifier, In Line Amplifier and Pre-amplifier using external OSA/ DWDM analyzer. These points shall be suitably connectorised and on connecting the measurement devices shall not affect the transmission on the main path.

- The Optical amplifiers shall meet the service channel and alarm requirements stated in Sections: 2.3.1.3 and 2.3.1.4. The Bidders shall provide detail specifications of Optical Amplifiers in the Bid.

- EDFA & Raman Amplifier will also be used in Access Network.

- All types of supplied optical amplifiers shall be suitable for operation at 100 Gbps and higher rate.

- All other hardware & software at In-Line amplifier (ILA) location shall be 100G (and above) rate, and must not be required to upgrade, in case of upgrade of line rate from 100G to a higher rate.

      - Repeaters/ Regenerators

Repeaters/ Regenerators are required for DWDM links. The Repeaters/Regenerators shall be configurable to become ROADM, by addition of some modules (if required).

        - Regenerator Base Equipment

- Regeneration Bases Equipment must be a 4 degree ROADM (Colourless, Directionless, Flex-grid, 40 Channel Mux/ Demux).

- The Base equipment to be required at all locations, where lambda/ channels will be regenerated using Regenerator modules. The base Equipment configuration should be standard in nature, and consists of all necessary modules except Regenerator Modules.

- The quantity of sub-rack to be supplied as part of every Regenerator Base Equipment shall accommodate at least regeneration of 30 channels.

        - Regenerator Modules

- Regenerator Modules shall be used to regeneration of DWDM Channel as per link budget requirement during detailed engibeering.

- The regenerator modules shall be auto configurable to different line rate based through on the Software license i.e. same hardware can be used for both 100G and higher line rates. In case of upgrade of line rate of Transponder/ Muxponder at both ends from 100G to higher, the regenerator locations in between shall be upgraded from 100G to higher rate through a license, if it meets the link budget. All other hardware/ software at regenerator locations must be compatible to 100G and higher line rate.

      - Optical Link Performance Requirements

The optical fibre link performance requirements are specified as follows.

        - Link Budget Calculations

The fibre optic link budget calculations shall be calculated based upon the following criteria:

  - Fibre attenuation: The actual attenuation figure (including fibre, splice, connector loss) have been specified in appendices. These attenuation figures have to be considered for link budgeting of these links.

  - Equipment Parameters: The equipment parameters to be considered for link budget calculations shall be the guaranteed “End of Life (EOL)” parameters. In case the End of Life parameters are not specified for the equipment, an End of Life Margin of at least 2 dB shall be considered. This parameter is to be considered for all links.

  - Optical path Penalty: An optical path penalty of at least 1 dB shall be considered to account for total degradations due to reflections, inter symbol interference, mode partition noise and laser chirp. This parameter is to be considered for all links.

  - Maintenance Margin: A maintenance margin of at least 2.5 dB/ 100Km shall be kept towards cabling, repair splicing, cable ageing and temperature variations etc. This parameter is to be considered for all links.

  - Other losses: Other losses, if any required specifically for system to be supplied shall also be suitably considered for all links.

  - Dispersion: The fibre dispersion shall be taken to be the guaranteed maximum dispersion i.e. 20 ps/nm.Km (1550 nm) for G.652 fibre and 2.7 ps/nm.Km (1525-1575nm) for G.653 fibre. PMD coefficient shall be taken as 0.5 ps/km^½ for G.652 fibre and 0.5 ps/km^½ for G.653 fibre.

  - Bit Error Rate: The link budget calculations shall be done for a BER of 10-15 for 100G channels of DWDM system.

  - In some fibre spans, variation of fibre loss has been observed due to variation in day/ night temperature.  The total span loss (i.e. fibre, splice & connector loss) may vary +/- 1dB (i.e. total 2 dB) within a day period in a span of ~100 km (e.g. if a span loss is 27.5 dB then it may vary between 26.5dB to 28.5dB). Therefore additional margin of 1dB (due to variation in temperature) per 100 km is to be considered per section for backbone network.

  - For finalising the Network design & BOQ, abve methodology shall be adopted during the detail engineering considering fibre attenuation, dispersion, PMD and splice loss determined during the detailed engineering/ test data measured in survey.

        - Link Performance

The Link performance for ES, SES and BBER for the fibre optic links shall correspond to National Network as defined in ITU-T G.826, G.828, M.2101, RFC 2544 and Y.1564.

      - FODP to Optical Equipment Connectivity

The Contractor shall be responsible for connectivity between the FODP and the optical equipment (DWDM/ OTN system, Repeaters and Optical Amplifiers). The Contractor shall provide FC coupled patch-cords (at FODP/FDF side) of suitable length as per the clause 2.4. The patch-cord length between the FODP & equipment rack shall be suitably protected from abrasion, crush or mechanical damage otherwise by flexible conduits or equivalent Employer approved techniques.

    - MPLS-TP System

The product/ solution with low footprint (2 RU) & power efficient shall be offered for the access/ aggregation layer in intra-city. All grey optical interfaces uptoSTM-64/ 10G (Line & client both) shall support Bi-directional pluggable. The system shall support circuit emulation. MPLS-TP (MPLS-Transport Profile) Equipment shall provide the following:

- OAM functions to detect and isolate faults (Like SDH/ SONET)

- Sub-50ms protection and restoration

- Ring & mesh architecture.

- End-to-end QoS

      - Functional Requirement

The offered MPLS-TP platform must be a reliable packet-based network based upon circuit-based transport networking. The architecture of the MPLS-TP platform must be defined as per the IETF RFC 5921& 6215. The requirements for MPLS-TP forwarding, OAM, and network survivability must be in compliance with RFC 5654 and 5860.

- The switching fabric plane shall be bidirectional and non-blocking. The Equipment’s shall support a wire speed L2 switching capabilities under full load condition. The platform shall supports enterprise services like Ethernet E-Line & E-LAN services and aggregate & hand over the traffic to next higher Metro Ethernet network Core Layer.

- The management of the MPLS-TP networks must be separable from its client networks so that the same means of management can be used regardless of the client. The management functions of MPLS-TP must include fault management, configuration management, performance monitoring, and security management.

- The purpose of the management function must be to provide control and monitoring of the MPLS transport profile protocol mechanisms and procedures.  The requirements for the network management functionality must be as per RFC5950 & RFC5951.

- MPLS-TP platform must mandatory run BFD (Bidirectional Forwarding Detection) protocol to provide OAM functionality. Itshould have resiliency to ensure minimal interruption to traffic when a failure occurs within the system or network.

      - Equipment Requirement

  - The size of equipment shall not exceed 4RU in size.

  - Equipment sub rack must suitable to install in 19 inch, 21 inch rack

  - The supplier shall confirm that their solution supports insert, replace and remove of modules while the equipment is powered up without affecting traffic and damaging any module

  - All NEs and cards provided by supplier must be managed on the same NMS

  - Equipment can be divided in three categories (Type-1, 2 & 3) based on their configuration. The details are given below:

| Sl. No. | Particulars | Type-1 | Type-2 | Type-3 |
| --- | --- | --- | --- | --- |
| 1 | Maximum Size (in RU) | 4 RU | 2 RU | 2 RU |
| 2 | Ports (Min) | 2x100G+ 10x10G | 4x10G +4x1G (min. 2 optical + min. 2 Elect.) | 4x10G + 4x1G (min. 2 optical + min. 2 Elect.) + 6xE1+ 1xSTM-1 |
| 3 | Coloured interfaces | Yes (Both 100G & 10G) | Only 10G | Only 10G |
| 4 | Bi-Directional SFP/ XFP | Yes (only 10G) | Yes (10G & 1G optical) | Yes (10G & 1G optical) |
| 5 | Redundancy (Power & Fan modules) | Yes | Yes | Yes |
| 6 | Power (Input Voltage) | -48V DC | -48V DC & 220V AC | -48V DC & 220V AC |

      - Technical specification

        - Standard requirement

- All equipment must comply with the Generic requirements mentioned in TEC/GR/TX/CEN-004/03/MAR-17 for MPLS-TP based carrier Ethernet switch for Aggregation & Access Network Applications. A statement of compliance shall be submitted along with the Bid.

- The Tuneable 100G CFP/ CFP2/CFP4 & Tuneable 10G XFP/ SFP+, in the BoQ for all type of MPLS-TP Equipment must support span loss of minimum 22dB without any amplification.

- All Equipment (100G & 10G) shall support amplifiers solution of 30dB, 35dB, 40dB & 50dB as per BoQ. Kindly refer the detailed BoQ at Appendix B

- The switching fabric plane shall be bidirectional and non-blocking. The equipment shall support a wire speed L2 switching capabilities under full load condition.

- The Equipment shall support Ethernet PW as specified by RFC 3985 and RFC 4385. It shall support Ethernet PW as specified by RFC 4448.

- The MPLS-TP Equipment has UNI (User Network Interface) and NNI (Network Node Interfaces). The MPLS-TP Equipment shall support UNI or NNI mode of operation on all the ports.

- Optical Ethernet interfaces shall be configured as Client or Aggregate interfaces based on requirement.

- All Equipment shall support jumbo frame of 9000 bytes, MDI-X based auto-uplink feature &dynamic online configuration.

- The Equipment shall support buffering of 50ms to take care of momentary congestion and link failures.

- The equipment shall support IPv4 and IPv6 traffic transparently at data plane.

- All Equipment shall be able to function in terminal mode, hub, & mesh and ring topologies.

- The equipment shall provide managed multi-service integration viz., voice, video and data.

- All equipment shall be fully compliant to MEF 9, MEF 14 or CE2.0 standards.

- It shall support built in power diagnostics feature to monitor optical SFP/XFP ports, system diagnostics to detect hardware failures.

        - Layer-2 Switching Features

- It shall support ingress and egress bandwidth profile per User to Network Interface (UNI).

- Equipment shall support Ethernet functions VLAN (IEEE 802.1Q), Q-in-Q (IEEE 802.1ad), VLAN manipulation: stack/switch/strip, Flow control (IEEE 802.3x), IGMP snooping (V1/V2/V3), Link aggregation (IEEE 802.3ad).

- Equipment shall support Link Layer Discovery Protocol as per IEEE 802.1ab, Logical Link control as per IEEE 802.2&Flow control as per IEEE 802.3x.

- The equipment shall supports enterprise services like Ethernet E-Line & E-LAN services and aggregate the traffic and hand over the same to the next higher Metro Ethernet network Core Layer.

- Service multiplexing: A single MPLS-TP Equipment port shall support multiple Ethernet Services

        - VLAN Features

- The MPLS-TP Equipment shall support creation of VLAN among ports of different types as well as on all ports of the interface cards.

- It shall support VLAN bridging (for outer tag only) as per IEEE 802.1adand user isolation per outer VLAN tag. The user isolation shall be configurable on a per port basis.

- The MPLS-TP Equipment shall support VLAN ingress filtering to prevent VLAN leakage & VLAN tag overlapping allowing some ports to be member of more than one VLAN.

        - Port trucking / Link Aggregation

- The MPLS-TP Equipment shall allow Link Aggregation as per IEEE 802.3 ad to allow link resilience.

- Supports load balancing over Aggregated Links.

- Static/LACP LAG on client ports shall be configured in all equipment.

        - Forwarding Support

- The MPLS-TP Equipment shall support to override The MPLS-TP Equipment port MAC address.

- It shall also support VPLS instance static MAC configuration.

        - MAC Address Learning / Limiting:

- The MPLS-TP Equipment shall support L2 Learning parameters: Sources learning per Port/VPLS/VLAN/Source address.

- The MPLS-TP Equipment shall support to set per port/VPLS instance dynamic MAC learning limit.

- The MPLS-TP Equipment shall support to limit the number of source MAC addresses learnt from bridge port/VPLS instance in order to prevent MAC address flooding DoS attack. This limit is configurable per bridged port.

- The MPLS-TP Equipment shall support dropping of Frames with new source MAC-addresses exceeding the configured value.

- The MPLS-TP Equipment shall support per VLAN MAC learning to ensure MAC addresses are learnt only from a VLAN perspective and automatic/manual disabling of MAC addresses learning for the VLAN where there are less than two ports in that VLAN.

- The MPLS-TP Equipment shall support MAC limiting per Ethernet flow point (EFP) or bridge domain

- The MPLS-TP Equipment shall support MAC address limitation and aging

- All static entries shall NOT be aged.

- The MPLS-TP Equipment shall support Hardware based aging of MAC Address Table entries.

- The MPLS-TP Equipment shall support to enable L2 Aging on every port

- The MPLS-TP Equipment shall support MAC address learning disabling

- The MPLS-TP Equipment shall support to filter and discard all Ethernet frames received on bridged ports in the upstream direction with a specific MAC destination address (DA)

- The MPLS-TP Equipment shall support list of allowable MAC destination address

- The MPLS-TP Equipment shall filter the MAC address from bridge port X if the same MAC address appears in the learning table pointing to bridge port Y (port X and port Y on the same The MPLS-TP Equipment and same VLAN), except in the cases where the aggregation network forwards according to MAC Learning table.

- The MPLS-TP Equipment shall support unique MAC address per device to prevent spoofing and provide traceability.

- The MPLS-TP Equipment shall have the capability to prioritize BPDUs in the data plane (by providing dedicated queues) and in the control plane (by providing dedicated CPU queues for BPDUs).

- The MPLS-TP Equipment shall have the capability to drop BPDUs regardless of the BPDU content. This function is configurable on a per port basis.

        - The MPLS-TP Equipment shall support IEEE 802.1Q Tagging

- Tagged only, this is an IEEE 802.1Q trunk.

- Untagged.

- Hybrid, tagged and untagged frames.

- The MPLS-TP equipment shall support the following additional IEEE 802.1Q features

- Tag insertion, removal and swapping.

- Capability of insertion and removal of second tag.

- Encapsulation translation and rewrites Push, Pop and translate for IEEE 802.1Q or QinQ/IEEE 802.1ad tags.

- Local VLAN and ports cross-connect and multipoint or point-to-multipoint with Hierarchical Virtual Private LAN service (H-VPLS bridge topologies with pseudo-wires) or locally defined bridge domains.

- The MPLS-TP Equipment shall allow configuring all 4K VID on all ports and at the same time supporting all 4K VLANs simultaneously. The user/operator shall be able to reuse the same VLAN-ID on a different port on the same. The MPLS-TP Equipment and terminate it into a different PW/VLAN.

- The MPLS-TP Equipment shall accept untagged, priority tagged and C-tagged frames through a IEEE 802.1Q port.

        - Q-in-Q or IEEE 802.1ad mode Requirements

- The MPLS-TP Equipment’s shall perform classification and service delineation based on outer Q tag and outer IEEE 802.1p bits. (i.e. ignore inner tag)

- It shall support VLAN stacking as per IEEE 802.1ad

- It shall have minimum of 4094 S-VIDs. VIDs “0” and “FFFF” is reserved.

- It shall allow only S-tagged frame in ‘.1ad’ ingress ports. It shall be possible to map the traffic to any PW based on SVLAN tag. It shall be possible to keep or pop the SVLAN tag before forwarding it to the PW.

- It shall be possible to set the priority bits in the S-VLAN priority based on the PCP bits of C-tag of the incoming packet in ‘.1ad’ mode.

        - LSP Mode Requirements

  - The MPLS-TP Equipment shall support LSP Mode Scalability Options i.e. Virtual Private LAN Service (VPLS)

- Support Packet Transport Network solution using PW service tunnel.

- TDM and Ethernet traffic shall be emulated into Pseudo-wires and PW label is added for service identification.

- End-to-end transport path LSP shall be created based on MPLS-TP standard and multiple

- The traffic tunnels shall support per LSP queuing/scheduling i.e. the ability to assign and guarantee per LSP per class bandwidth profiles (CIR, EIR, CBS, and EBS)

        - Quality of Service Requirements

- The MPLS-TP Equipment shall support VLAN CoS preservation.

- The MPLS-TP Equipment shall support VLAN CoS differentiation: It shall be possible to configure the classification of the traffic according to the port, VLAN, IEEE 802.1p bits or TOS/DSCP bits.

- The MPLS-TP Equipment shall support creation of VLAN or Flow with DSCP parameters per service for data, video and O&M traffic for service differentiation.

- It shall support measurement of performance bounds for each flow in terms of throughput, loss, delay and delay variation, according to their respective defined service classes as per Y.1731.

- The MPLS-TP Equipment shall support 16, 32, 64,128,256 and 512 k bytes burst sizes.

- The MPLS-TP Equipment shall support wire speed forwarding on all interfaces and all packet sizes even with classification and QoS activated on all interfaces.

- The MPLS-TP Equipment shall support bandwidth management reports and statistics.

- The Edge the MPLS-TP Equipment shall support 8 class of service per flow.

        - Classification/Prioritization:

- The MPLS-TP Equipment shall support classification of ingress traffic for a specific service based on the following mapping:

- IEEE 802.1P Mapping - it shall be possible to reserve parts of the link bandwidth for frames with particular IEEE 802.1p values.

- Customer IEEE 802.1p priority

- Multiprotocol Label Switching (MPLS) Label Stack Entry:"EXP" Field Renamed to "Traffic Class" Field as per RFC 5462

- Source MAC address

- Destination MAC address

- VLAN ID

- IEEE 802.1Q

- Default queue for non-matching traffic

- The MPLS-TP Equipment shall aggregate incoming traffic into Traffic Classes by following characteristics:

- Incoming port (Logical and Physical)

- Source/ destination MAC

- Link layer priority information as per IEEE 802.1p

        - Mapping

- The MPLS-TP Equipment shall support mapping of DSCP/VLAN/Port to MPLS labels.

- Mapping of IEEE 802.1p and IP TOS bits into MPLS EXP bits.

- Mapping of IEEE 802.1q VLAN tags into MPLS labels

- Aggregate incoming traffic into Traffic Classes by MPLS Label EXP bits (E-LSP)

        - Marking/Policing/Shaping:

- The MPLS-TP Equipment shall support the following Marking/Policing/Shaping requirements

- 8 level Priority marking as per IEEE 802.1p

- Filtering

- Broadcast/Multicast suppression

- Bandwidth management policies

- Single rate three colour marking (srTCM)

- Two rate three colour metering (trTCM)

- Trust the colour of the incoming packet.

- Differentiated Service Two-Rate, Three-Color Marker with Efficient Handling of in-Profile Traffic

- Shaping of Burst Traffic

        - Rate Limiting

  - The Equipment shall support Rate limiting of bandwidth per Port and per class (or flow)

  - Configuration of user bandwidth shall be supported, in steps of

- Mbps for 1-1000Mbps

- 100 Mbps granularity for 1-100 Gbps

  - It shall support defining Committed Information Rate (CIR) and an Excess Information Rate (EIR) for each flow in steps of 1Mbps

  - The MPLS-TP Equipment shall support flow based rate limiting method based on per source address, destination address or both.

        - Queuing:

  - The MPLS-TP Equipment shall support the following queuing

- SPQ – Strict Priority Queuing

- WFQ – Weighted fair Queuing

- No of queues per flow treatment of traffic.

- Per service ingress queues are defined on the basis of Peak/Excess burst Size (PBS/EBS), Committed Burst Size (CBS), Peak/Excess Information Rate (PIR/EIR) and committed Information rate (CIR).

- Per service egress queues have distinct parameters defining its operations like Peak/Excess burst Size (PBS/EBS), Committed Burst Size (CBS), Peak/Excess Information Rate (PIR/EIR) and committed Information rate (CIR).

- Alternate priority routing traffic necessary to keep from starving other priority queues.

        - Scheduling:

- The MPLS-TP Equipment shall support scheduling of queues to strict priority with 2 or more priority levels

- The MPLS-TP Equipment’s shall support the following congestion avoidance mechanisms

- Tail Drop

- Deficit Round Robin (DRR)

- Weighted Round Robin (WRR)

- DWRR (Deficit Weighted Round Robin)

- WRED

- Strict Priority (SP)

- SP + Weighted Round Robin (SP + WRR)

- The MPLS-TP Equipment shall support configuring the scheduling as per the

- Per Hop Behaviour (PHB)

- Physical port or logical port basis.

- Up to 8 forwarding class queues can be configured on a per service basis each with its own CIR, PIR, CBS, PBS and Forwarding Class attribute.

- At least three level dropping precedence levels in each queue.

        - Protection Mechanism:

- The MPLS-TP Equipment shall support linear protection (1:1 & 1+1) as specified in ITU-T Rec. G.8131.1 or G.8131.2.

- For all the modes of protection, The MPLS-TP Equipment shall support protection switching within 50ms of expiration of any manually selected hold-off time (with both SF/SD scenario) and shall support all operator commands (Forced Switching[FS], manual Switching [MS], lockout of protection).

- The MPLS-TP Equipment shall support protection switching within 50ms and Forced switching as analogous to SDH systems.

- The MPLS-TP Equipment shall support protection switching triggered by fault detection, such as loss of signal, loss of frame, signal degrade (BER becomes worse than the predetermined threshold), and so on.

- The MPLS-TP Equipment shall support Forced switching activated by administrative events, such as fibre rerouting, fibre replacement, etc.

- The MPLS-TP Equipment shall support  Ethernet ring protection for both S-VID and B-VID as per G.8032

- The MPLS-TP Equipment shall support MPLS-TP Linear Protection support as per IETF standards or Ethernet/MPLS SNC based protection as per ITU-T standards.

        - Circuit Emulation Protocols

- The MPLS-TP Equipment shall support PW using Structure-Agnostic Time Division Multiplexing (TDM) over Packet (SAToP) or MEF8.

- The equipment shall support E1/channelized STM-1 circuit emulation mechanism and it shall support Circuit Emulation Services over MPLS as per PWE3 standard to aggregate the TDM PW from the edge and access CEN.

        - Network Synchronization Requirements

- The MPLS-TP Equipment shall be able to synchronize with an external reference clock.

- It also support Synchronous clock selection algorithm based on the following parameters:

- Quality of Signal

- Signal fail

- Priority

- External Commands

- It shall support SyncE as per ITU-T G.8261, G.6282 and G.8264 or IEEE 1588v2.

- The MPLS-TP Equipment shall support Network Time Protocol (NTP) for synchronizing with a central NTP server.

- The MPLS-TP Equipment shall support timing and synchronization aspects of Packet Networks based on SyncE as per G.8261. The MPLS-TP Equipment shall support timing characteristics of a synchronization slave clock as per G.8262.

- The MPLS-TP Equipment shall support provide a timing-output interface at 2048 KHz for external synchronization. The output shall conform to ITU-T Rec. G.812, as applicable.

        - MPLS-TP STANDARDS

- It shall support the creation of L2VPN solutions using statically configured PWs and tunnels as per RFC 4664

- The MPLS-TP Equipment shall support MPLS-TP requirements as per RFC 5654 or ITU Y.SUP4

- The MPLS-TP Equipment shall support Architecture of MPLS-TP Layer Network as per ITU-T G.8110.1v2 or equivalent IETF standards

- The MPLS-TP Equipment shall support Interfaces for the MPLS-TP Hierarchy as per ITU-T G.8112 or equivalent IETF standards

- The MPLS-TP Equipment shall support Characteristics of MPLS-TP Network Equipment Functional Blocks as per ITU-T G.8121v2 or equivalent IETF standards

- The MPLS-TP Equipment shall support MPLS-TP General Framework as per RFC 5921 or ITU G.8110.1

- The MPLS-TP Equipment shall support MPLS-TP survivability framework as per RFC 6372 or ITU G.8131/G.8132

- The MPLS-TP Equipment shall support MPLS-TP Data plane Architecture as per RFC5960 or ITU Y.SUP4

- The MPLS-TP Equipment shall support MPLS Generic Associated Channel (GAL/G-ACH) as per RFC 5586 or ITU G.8113.1/G.8113.2

- The MPLS-TP Equipment shall support Definition of ACH TLV Structure as per draft-ietf-mpls-tp-ach-tlv-02 or ITU G.8113.1/G.8113.2

- Support Pseudowire Status for Static Pseudowires as per RFC 6478

- It shall support MPLS On-Demand Connectivity Verification and Route Tracing as per RFC 6426 or ITU G.8113.1/G.8113.2

- The MPLS-TP Equipment shall support Proactive Connectivity Verification, Continuity Check, and Remote Defect Indication for the MPLS Transport Profile as per  ITU G.8113.1/G.8113.2

- The Edge MPLS-TP Equipment shall be connection-oriented packet switching model with traffic engineering capabilities that allow deterministic control of the use of network resources. It shall support traffic engineered point to point (P2P).

- The Edge MPLS-TP Equipment shall support packet transport network solution by using PW service tunnel and MPLS-TP transport technology. TDM and Ethernet traffic are emulated into Pseudo-wires and PW label is added for service identification. End-to-end transport path LSP is created based on MPLS-TP standard (ongoing) and multiple PWs are transported over the same LSP end-to-end in both directions.

- In order to support traffic engineered point-to-point MPLS-TP circuits, it shall support all the MPLS-TP specific requirements specified in the RFCs (RFC 3031, 3032, 3443, 5586, 5860, 5960, 6370, 6426, 6428). No explicit support for control plane protocols is required. NMS Support shall be mandatory.

- It shall be possible to rate limit the traffic in MPLS-TP tunnels at minimum 10 Mbps granularity. MPLS-TP tunnels shall support per E-LSP Queuing as defined in RFC 3270.

- In order to support traffic-engineered Pseudo Wire Emulations (PWE) over MPLS-TP, the system shall support the following RFCs (RFC 4448, 5659, 5994). The compliance for these RFCs is required to the extent of supporting MPLS-TP and PWE. No explicit support for control plane protocols is required. NMS support shall be mandatory.

      - Pluggable Optics

      - All 100G/ OTU4 (80km & 40 km) pluggable optics in the BoQ is for Line side application with their respective equipment (MPLS-TP Type-I). Contractor has to supply pluggable optics (CFP/ CFP2/ CFP4/ QSFP28/ Coherent DCO etc) meeting the distance requirement and compatible with the respective equipment.

      - All 10G pluggable optics shall be used in OTN (10G ports) and NNI & UNI ports of MPLS-TP equipment.

      - Bi-Directional XFP/ SFP+ shall also be used as 10G or STM-64 in OTN ports to connect MPLS-TP (10G) Equipment or SDH (STM-64) equipment as per requirement.

      - Bi-Directional XFP/ SFP+ shall be used with 10G (NNI & UNI) interfaces of all MPLS-TP equipment.

      - 1G pluggable supporting up to 40 Km, 80 km, 120 km and 180 km under BoQ, are envisaged to use with MPLS-TP equipment also.

      - Optical Amplifiers for Packet Links

        - Optical Amplifiers

Transmit power amplifier and/or a combination of transmit power amplifier & receiver preamplifier shall be offered for 10G Packet Links and 100G Packet links to meet the link budget requirements stated in ‘Optical Link Performance Requirements’. Amplifier(s) shall be located with the equipment.

The Optical amplifiers shall meet the alarm requirements and shall be fully manageable from TMN. It should be possible to monitor the optical parameters and also handle alarms of the optical amplifier from the TMN. The Bidders shall provide detail specifications of Optical Amplifiers in the Bid.

Link Solution supporting difference span losses in the BoQ has been asked in the form of ‘Set’. The ‘Set’ consists of all necessary amplifiers, pluggable Optics for equipment, Patch cords, power cables, Communication/ DCN cable accessories and any other items as per TS, for one end of the link i.e. POWERGRID shall order two (02) sets for implementation of one (01) link.

        - Optical Link Performance Requirements

The optical fibre link performance requirements are specified at 2.3.1.5.4 as follows.

        - Link Budget Calculations

The fibre optic link budget calculations shall be calculated based upon the following criteria:

- Fibre attenuation: The actual attenuation figure (including fibre, splice, connector loss) shall be provided during detailed engineering for link budgeting of backbone/ access links.

- Equipment Parameters: The equipment parameters to be considered for link budget calculations shall be the guaranteed “End of Life (EOL)” parameters. In case the End of Life parameters are not specified for the offered equipment, an End of Life Margin of at least 2 dB shall be considered. This parameter is to be considered for all links. EOL shall be considered for Single Channel, in case of OTN

- Optical path Penalty: An optical path penalty of at least 1 dB (or bidder’s standard design value whichever is higher) shall be considered to account for total degradations due to reflections, inter symbol interference, mode partition noise and laser chirp. This parameter is to be considered for all links.

- Maintenance Margin: Appropriate maintenance margin shall be kept towards cabling, repair splicing, cable ageing and temperature variations etc during planning & execution.

- Other losses: Other losses, if any required specifically for system to be supplied shall also be suitably considered for all links.

- Dispersion: The fibre dispersion shall be taken to be the guaranteed maximum dispersion i.e. 18 ps/nm.Km (1550 nm) for G.652 fibre.

- Bit Error Rate: The link budget calculations shall be done for a minimum BER of 10-12 for 10G and 10-15 for 100G links.

- Link solution shall be offered considering the parameters defined above.

- Packet links (10G & 100G) up to minimum 30dB (120 Km)/ 35dB (140 Km)/40 dB (160 Km)/ 45dB (180 Km) link loss excluding EOL, Path penalty, dispersion, other losses etcs hall be implemented without repeaters. The above mentioned link loss includes fibre loss, splice loss, connector loss and maintenance margin only. The Optical path penalty, EoL margin and other losses (if any) shall be considered by contractor while offering link solution for above requirements.

        - Link Performance

The Link performance for ES, SES and BBER for the fibre optic links shall correspond to National Network as defined in ITU-T G.826, G.828, M.2101, RFC 2544 and Y.1564.

      - Management

The MPLS-TP system shall support the In-Band & Out-Band management communication channel. The management function of MPLS-TP   includes fault management, configuration management, performance monitoring, and security management. The management function must provide control and   monitoring of the MPLS transport profile protocol mechanisms and   procedures.  The requirements for the network management functionality must be as per RFC5951.

    - L2 Switch

The Layer 2 switch is a type of network switch or device that works on the data link layer (OSI Layer 2) and utilizes MAC Address to determine the path through where the frames are to be forwarded. It must use hardware based switching techniques to connect and transmit data in a local area network (LAN).

Layer 2 switches must be primarily responsible for transporting data on a physical layer and in performing error checking on each transmitted and received frame. A layer 2 switch must require MAC address of NIC on each network node to transmit data. They must learn MAC addresses automatically by copying MAC address of each frame received, or listening to devices on the network and maintaining their MAC address in a forwarding table. This must also enables a layer 2 switch to send frames quickly to destination nodes. The switch must work with 220V AC power.

      - Technical Specification

  - Port type/density/architecture

- L2 managed switch with minimum Ethernet 4 port 10/100/1000Mbps ports (min 2 optical + min 2 Electrical). Optical & Electrical pluggable SFPs shall be ordered as per BoQ.

- Auto negotiate/manual setting

- IEEE 802.3af PoE delivered over any of 10/100/1000 ports.

- All ports on the switch should be of non-blocking architecture

- Switch should be equipped with Variable speed Fan for cooling

- Supports Layer 2 features such as Port Trunking, SNMP, IGMP, VLAN and other required protocols

- High performance architecture of switch that is capable for providing the non-blocking switch fabric and wire-speed throughput of min 10Gbps

  - Features:

- SNMPv3, NTP, HTTPS, SSHv2,TELNET , FTP/TFTP

- Minimum Mac address learning 4K

- Should support IPv6 in hardware and software from day 1

- Switch should have a console port for configuration and diagnostic purposes

- Simplified QoS management using 802.1p, Differentiated Services (DiffServ), or type of service (ToS) traffic prioritization specifications

- Fully resilient stacking for optimized growth with simplified management

- ACLs for granular security and QoS implementation

- Can be configured and monitored from a standard web browser

- Secure remote management of the switch via Secure Shell (SSH) and SSL encryption

- 802.1Q-based VLANs enable segmentation of networks for improved performance and security

- Automatic configuration of VLANs across multiple switches through Generic VLAN Registration Protocol (GVRP) and Generic Attribute Registration Protocol (GARP)

- User/network port-level security via 802.1X authentication and MAC-based filtering

- Increased bandwidth and added link redundancy with Link Aggregation Control Protocol (LACP)

- Enhanced rate-limiting capabilities, including back pressure and multicast and broadcast flood control

- Port mirroring for noninvasive monitoring of switch traffic

- Jumbo frame support up to 10KB

- Simple Network Management Protocol (SNMP) versions 1, 2c, and 3 and Remote Monitoring (RMON) support

- Fully rack mountable using the included rack-mounting hardware

  - Management Features:

- SNMPv3, NTP, HTTPS, SSHv2, SCP2, SFTP, TELNET

- L2 Ping & L2 trace route, and TFTP, LLDP

- Switch should have OS API/CORBA interfaces to integrate with 3rd party applications

- Should have automated port profile technology for smart plug and play and seamless deployment

- Switch should support 4 groups of embedded RMON (history, statistics, alarm and 3events)

  - Operating Condition

- Operating Temp: 0° C to minimum 50° C

- Operating Humidity: 10% to 95% non- condensing

- Miscellaneous Feature Set:

- The switch act as a RADIUS/TACACS+ client

- IEEE802.1X: RADIUS authentication, authorization and accounting, MD5 hash, guest VLAN, single/multiple host mode and single/multiple sessions

- The Layer-2 switch should support:

  - Storm control

  - DHCP Snooping

  - Loop Protection

  - Cable Diagnostics

  - Persistent Power: The switch should allow the PD’s (POE Devices) to retain power in case the switch undergoes a reboot. This allows PoE Devices with built in storage to continue operation and sync up on network availability.

      - Pluggable Optics

- Contractor has to supply pluggable optics meeting the distance requirement and compatible with the respective equipment. 1G pluggable supporting up to 40 Km, 80 km, 120 km and 180 km has been envisaged under this Package and kept as a separate item in BoQ.

- All type of 1G pluggable optics in BoQ shall be used with Switch.

      - Optical Amplifiers for Packet Links

        - Optical Amplifiers

Transmit power amplifier and/or a combination of transmit power amplifier & receiver preamplifier shall be offered for 1G Packet Links to meet the link budget requirements stated in ‘Optical Link Performance Requirements’. Amplifier(s) shall be located with the equipment.

The Optical amplifiers shall meet the alarm requirements and shall be fully manageable from TMN. It should be possible to monitor the optical parameters and also handle alarms of the optical amplifier from the TMN. The Bidders shall provide detail specifications of Optical Amplifiers in the Bid.

Link Solution supporting difference span losses in the BoQ has been asked in the form of ‘Set’. The ‘Set’ consists of all necessary amplifiers, pluggable Optics for equipment, Patch cords, power cables, Communication/ DCN cable accessories and any other items as per TS, for one end of the link i.e. POWERGRID shall order two (02) sets for implementation of one (01) link.

        - Optical Link Performance Requirements

The optical fibre link performance requirements are specified as follows.

        - Link Budget Calculations

The fibre optic link budget calculations shall be calculated based upon the following criteria:

- Fibre attenuation: The actual attenuation figure (including fibre, splice, connector loss) shall be provided during detailed engineering for link budgeting of backbone/ access links.

- Equipment Parameters: The equipment parameters to be considered for link budget calculations shall be the guaranteed “End of Life (EOL)” parameters. In case the End of Life parameters are not specified for the offered equipment, an End of Life Margin of at least 2 dB shall be considered. This parameter is to be considered for all links.

- Optical path Penalty: An optical path penalty of at least 1 dB (or bidder’s standard design value whichever is higher) shall be considered to account for total degradations due to reflections, inter symbol interference, mode partition noise and laser chirp. This parameter is to be considered for all links.

- Maintenance Margin: Appropriate maintenance margin shall be kept towards cabling, repair splicing, cable ageing and temperature variations etc during planning & execution.

- Other losses: Other losses, if any required specifically for system to be supplied shall also be suitably considered for all links.

- Dispersion: The fibre dispersion shall be taken to be the guaranteed maximum dispersion i.e. 18 ps/nm.Km (1550 nm) for G.652 fibre.

- Bit Error Rate: The link budget calculations shall be done for a minimum BER of 10-9 for 1G.

- Link solution shall be offered considering the parameters defined above.

- Packet links (1G) up to minimum 58dB (230 Km) link loss excluding EOL, Path penalty, dispersion, other losses etcs hall be implemented without repeaters. The above mentioned link loss includes fibre loss, splice loss, connector loss and maintenance margin only. The Optical path penalty, EoL margin and other losses (if any) shall be considered by contractor while offering link solution for above requirements.

        - Link Performance

The Link performance for ES, SES and BBER for the fibre optic links shall correspond to National Network as defined in ITU-T G.826, G.828, M.2101, RFC 2544 and Y.1564.

  - Patch Cords:

The Contractor shall provide (i.e. supply and install) connectorised FODP-to-equipment and equipment-to-equipment jumpers (patch-cords) for each equipment with two installed spares. The contractor shall be required to provide armoured patch-cords at some of the locations prone to rat cuts as identified during survey/ or communicated by POWERGRID in BoQ. Fiber jumpers shall be of sufficient lengths as to provide at least 0.5m of service loop when connected for their intended purpose. Contractor shall provide patch cords of appropriate length for interconnection amongst equipment ports and with FODP for proper routing inside the cabinet and to maintain aesthetics. Contractor shall assess the requirement of various lengths of patch cords during survey and keep sufficient quantity of various length of patch cords to cater any changes during Installation. The Patch cords shall comply with the following:

- All the patch cords shall be Single Mode (G657A Fiber), Simplex type.

- The fiber used for making the patch cord shall be Corning or equivalent.

- The aramid yarn used in the cable of the patch cord shall be genuine and not plastic based. Brand of the aramid yarn should only be either Dupont Kevlar or Teijin Twaron.

- The patch cords shall be suitable for use in the temperature range of –30 degree to +70 degree centigrade. Maximum insertion loss for a patch cord should <0.2dB @ 1310 & 1550nm wavelengths.

- All the patch cords/Connector End face should meet cleanliness standard IEC 61300-3-35, and shall be supplied with appropriate end caps for their connectors on either end.

- Return loss: >55db for PC / UPC and >65db for APC @ 1310 & 1550nm wavelengths. Durability: 1000 times, Typical change: <0.1db (Max).

- The manufacturer’s name must be printed on the outer jacket / sheath of the patch cord. Party should have manufacturer certificate from the authorized laboratories / organizations.

- Individual test reports for both connectors of the patch cord should be printed on a sticker and stuck just below the connector boot on both ends. The test report should mention the following:

  - Actual Insertion Loss value in dB.

  - Actual Return Loss value in dB.

  - Wavelength at which both the Insertion Loss and the Return Loss are tested.

  - Unique serial number for each connector with date to ensure traceability. The printing on the sticker should be of durable quality.

- The internal/ lab test reports in support of above mentioned technical parameters shall be provided by the supplier.

The Contractor shall use the following make patch cords along with all Equipment:

1.   FCI Amphenol

2.   TE Connectivity

3.   R&M

4.   Rosenberger

5.   Huber Suhner

6.   Radiall

7.   3M

In case contractor wants to offer patch cord from a different make, it has to take prior approval before commencement of supply, by furnishing documentary proof of successful operation of patch cords of other make.

  - Inside Plant

For the purposes of the specification, Inside Plant shall include cabling, wiring etc. Equipment and material components of the Inside Plant are part of this procurement.