ANNEXURE - "A"

# Detailed Scope of Work

Bypassing and restoration of 11kV & LT lines for the construction work of 765kV D/C Mandsaur-Khandwa Transmission Line PKG-1 (TL-04) [AP 34/0 to AP 65/0] under POWERGRID SIROHI KHANDWA TRANSMISSION LIMITED

Scope of Work:

  - The scope of work includes:

    - Bypassing of 11kV/LT lines with (cables will be provided by POWERGRID) pulling down 11kV/LT conductor. Restoration of line after completion of stringing work on confirmation by POWERGRID. Bypass cable to be laid down with proper safety measures below ground as per guidelines of MPPKVVCL.

    - Restoring the line after completion of stringing/erection of 765kV D/C Mandsaur-Khandwa Transmission Line PKG-1 (TL-04) [AP 34/0 to AP 65/0] under POWERGRID SIROHI KHANDWA TRANSMISSION LIMITED

    - Bypassing of 11kV/LT lines will be carried out for smooth and timely execution of conductor stringing/tower erection of 765kV Double circuit Mandsaur-Khandwa Transmission line (TL04) as per instruction of EIC. One or more 11kV/LT lines can be bypassed simultaneously as per site requirement using suitable cables. Agency has to ensure deployment of sufficient manpower as per site condition.  These bypass arrangements will remain in service for a period of minimum 7-10 days or as per instruction of EIC.

    - The work should be done as per enclosed BOQ and as per guidelines of local administration, MPEB, MPPKVVCL and any other authorities.

    - Executing agency has to mobilize all the resources within 07 days for bypass arrangement of 11/LT line on received of NOA/LOA.

    - After completion of conductor stringing/ tower erection activities of 765kV line, restoration of bypass arrangement as per instruction of EIC.

    - POWERGRID will provide 11kV /LT cable at KPIL store, Kachhanariya, near Nagda, Dist Ujjain (MP). Agency will transport required cable from KPIL store to various site location between AP 34/0 to AP 65/0 (Maximum distance 85-90 Kms one side).

    - All other items/accessories for bypass arrangement and restoration of line shall be in the scope of agency.

    - Storage of cable and accessories and watch & ward in the scope of agency.

    - Any damage occurred during the execution of work to distribution agency will be repaired/rectified by Bypass agency without no extra cost to POWERGRID.

    - Necessary insurance/ labour license will be in the scope of agency.

    - Fooding/lodging/transportation of labour will be in the scope of agency.

    - Meggering of cables is to be performed before installation of each crossing.

    - Necessary coordination & liasoning with electricity department (MPEB) for availing of shutdown will be in the scope of agency.

  - POWERGRID shall issue letter for required shutdown and extend necessary support.

  - The work shall be executed, as per direction of Engineer-in-Charge, In accordance with the description in the Bill of Quantity and other tender documents including the detailed Scope of Work as mentioned at point no. 1.0.

  - All such items which are not specially mentioned but are necessary for successful completion of the work are to be deemed to be included in the scope of the work unless otherwise specifically excluded.

  - The agency shall visit the site and acquaint with all the feature affecting tender.

  - Any temporary measures made by agency for their machinery movement to workplace shall be at their own cost.

2.0 The tender documents forming the contract are to be taken as mutually explanatory to one another & in case of any varying or conflicting provisions in any of the documents following order of preference shall be adopted: -

- Item Description in Bill of Quantities

- Scope of work

- Conditions of Contract

  - Engineer-in-Charge

  - The Engineer-in-Charge for the package shall be:

Sr. General Manager, Ujjain will be the Engineer-in-charge for the subject work.

The address of the Engineer-in-charge is as follows:

General Manager,

Power Grid Corporation of India Limited, D14/29, Kshipra Vihar, Ujjain,

Ujjain, Ujjain, Madhya Pradesh – 456010

The Engineer-in-Charge may authorise any of his officers as his representative for the execution of the subject work.

  - The entire scope of work shall be carried out as per the instructions of the Engineer-in- Charge and/or his representative.

4.0 Location of Work

4.1 The address of site where the work is to be executed is as follows: Construction site of 765kV D/C Mandsaur-Khandwa Transmission Line PKG-1 (TL-04) [AP 34/0 to AP 65/0] under POWERGRID SIROHI KHANDWA TRANSMISSION LIMITED under Malhargadh, Mandsaur tehsil of Mandsaur District, ,Jaoraa, tehsil of Ratlam District and Khachrod,Badnagar Tehsil of Ujjain District.

As per Scope of Work in section 1.0 above and Price Schedule