# Construction of Truss less GIS Store at POWERGRID Namsai SS.

TECHNICAL SPECIFICATION

- The bidder should	visit the site and ascertain the availability of material, site conditions and should quote their rates taking into account all the factors affecting the work.

- Fly Ash bricks and cement concrete brick of at average compressive strength of 50 kg/cm2 shall be used as per CPWD specification/ relevant IS code.

- CPWD specification shall be applicable for all schedule items.

- All non-scheduled items shall be strictly as per item description of respective BOQ and instruction of Engineer-in-Charge. CPWD specifications shall be applicable for all non- schedule items similar to schedule items. Where CPWD Specification of similar item(s) are not available TS attached by POWERGRID/Manufacturer shall be applicable.

- The record for consumption of anti termite solution shall be maintained at every  stage and duly got certified by the Engineer-in-Charge.

fi. All tests on materials and finished products as required in the field qualify plan shall be arranged without any extra cost to POWERGRID.

7 All schedule items are based on DSR'2021. Any discrepancy in item description shall be corrected accordingly.

- No materials shall be supplied by POWERGRID. Item rates are inclusive of materials cost.

- Wherever MS structural members are used it should be properly rolled & anti corrosive paint should be applied prior to applying Synthetic Enamel Paint.

- Prior to concreting & removal of shuttering for any type of RCC work contractor has to inform the Engineer-in-charge well in advance.

- All bought out items shall be approved by Engineer in charge prior to its procurement. All  material should be of 1st quality .

- Where ever wood is to be used, it should be well seasoned and chemically treated

- TMT reinforcement steel may be procured as per the provision given in SFQP for civil works.

- OPC/PPC cement shall be used in all activities, however her special care shall be taken in case of PPC cement as per requirement of CPWD Specification.

- TS of Rolling shutter is attached & work shall be carried out strictly in accordance with TS.

- Truss less Roof should be as per BOQ, Indicative drawing and relevant Standard.

- Specification of rolling shutter is as tabulated below :

| Components of Rolling Shutter | Specifications |
| --- | --- |
| Slat material (Shutter curtain): | min. 1.0 mm steel slate (Galvalume) |
| Slat profiles | min. 100mm wide |
| Side guide | GI side guides min. 75mm Especially design guides with wind locks for high wind velocity exposed shutters. Nylofelt seals fitted on the shutter skin for frictionless movement within the guides. |
| Bottom profile | Aluminium bottom rail is provided with additional rubber seal for tight closing for prevention of dust entry. |
| Roller shaft | Roller shaft shall be heavy duty mild steel strong suspension tubular shaft without springs. |
| Side brackets | M.S. plate 3 mm to 10 mm as per the opening dimension. |
| Surface Finish & Painting | Primer and Duco paint (Matt finish) |
| Operations | Electrically operated with motor of suitable capacity along with manual operation by hand chain or hand crank for emergency |

Sr.	COMPONENTS OF

SEPCIFICATIONS

No.  ROLLING SHUTTER

1	'Sla1 iralerial shall be nsiii I .t)1riii lhk. steel slate fgal attune). Th

Slat	Malerial	(Shutte Cui‘tain)

3	Slat Profiles

Side Guide

hulter curtain shall be construcled fToin single skinned Cal alume leel laths. Each lalh shall be retained b; interlocking \\11h each! other. The material shall be finished » ilh 1» o coats o1 Polyurethane spi’ay paint o er one coal of e}0ox pi iiner.

100 mm to 120nzv

GI Side Guide shall   be nd. 75mni   or as per manufacture 1 ecification def ending on sixe of rolling shutler and shall b especially design guides i ith  ind locks for lñgh » ind velocin

exposed shutters. PVC/n›’lofelt seals fitted on the shutter skin fo friction less mo ement » ithin the guides. Thickness of side guides hall be min. 2.0mm thick Gal anised Steel i› ilh ktegrated seals

fitted to_he li}is of lhe guides to pre ent metahto-melal contact.

5

Boltom Profile

7

Roller Shaft

8	Side Brackets

9

Bend-resislant	double-skinned	extruded	Aluminium	bottom. x‘ofile or Gal anised Steel bottom profile suitable for the locking

arrans ement shall be pro ided.	The bottom rail shall be filled with extruded rubber bottom seal IO j3re ent the entn of dust

hen it is full closed.

Roller shaft shall be proi’ided as per manufactui‘e specification Y\1lh hea •y-duty mild steel strong suspension tubular shaft

i ithout s rin   suit ble r en	as re u ied. _  	 		 Should be of MS Plate 3nun to l0mm as per manufacturer specification.		_		_

All the exposed surfaces shall be finished with I o coats o.‘ Epox¿• paint o er one coat of approx ed primer as recommended

Surface Finish & Painting by manufacturer.	However. contractor may pro ide shutte finished ii ith 2 coats of PU paints »sthout any financia

|implicalion to POWERGRID.

- he shutter shall be electrically operated with motor of suitabl

Operations

apacip along i with hand chain or hand crank for manual operations. The shutter should have both facilities i.e. moto operated & manual operated.

- Hood & Motor Co her	Shall be of Pressed gal °alume steel hood box complete »•itL

Control Si itch

Dri e Type

Dri e Uliit

12

reinforced bracket as pei’ manufacturer specification. This should be minimum 1 mm thick Gale alume sheet.	 	 Electric Motor should be either Single Phase or Three Phase! depending	upon lhe size of opening as per manufacturer specification.	_

- Indirect /Direct Connecting Dri e selection mode as peI irariu(acturer specificalion. Dri e should be with integral breaking sj stem with Long life. System pot er supply shall be wired »’ilh indUStfi i3lug l3ush button integrated in the liousins lid

- Three station push button »°ilh dead man control sit itch vhicl a1loi›°s lhe shutter to mo e as long as the operating bulton is pressed.

Emergenc;	manual Emergenc;	operation b; means (manual o *eration) sUCh as: l3eral ion		mechdillCal de ice. chaui and crank operalion for oper at url

’oiling shutlers.