| Sl. No. | # Volume/ Section/ Description | # Clauses | # Bidder’s Query | # POWERGRID Reply |
| --- | --- | --- | --- | --- |
| Volume-II: Technical Specifications |  |  |  |  |
|  | Section Project/ 2. Scope/Cl. No. 2.4/ Pg. No. 03 | Based on actual destination as decided by Employer prior to scheduled dispatch of the Individual Transformer/ Transformer Banks (as applicable), the conditions of roads, capacity of bridges, culverts etc. in the route shall be assessed by the contractor. The scope of any necessary modification/ extension/ improvement to existing road, bridges, culverts etc. shall be included under present scope. The contractor shall carry out the route survey along with the transporter and submit the detail proposal and methodology for transportation of Individual Transformer/Transformer Banks(As applicable), for approval of Purchaser within Three(03) months from the date of finalization of location, as intimated by POWERGRID. | Transformer dispatch is contingent upon the real destination, as determined by the employer upon award. In this regard, we understand that any necessary modification/ extension/ improvement to existing road, bridges, culverts etc are excluded from present scope and the same shall be assessed by contractor post award and necessary report shall be submitted to employer. Please kindly confirm/clarify. | Please refer to Clause 1.3 of Section -Project of the technical specification. Local Transportation, In-transit insurance, loading and unloading charges, referred to as F&I charges, shall be finalized with the Contractor based on actual destination decided by Employer prior to scheduled dispatch of the equipment. |
|  | Section Project/ 5. Schedule of Quantities/ Pg. No. 04 | The bill of quantity is indicated in the Bid price Schedules. Bidder should indicate all such items in the bid proposal sheets which are not specifically mentioned but are essential for execution of the contract. Items which explicitly may not appear in various schedules and required for successful commissioning of the Reactors shall be included in the bid price and shall be provided at no extra cost to Employer. | We understand that Erection of Reactors is not in bidder's scope. Please confirm. | Please refer Amendment No-01 of Bidding document |
|  | Section Project/ 2. Scope/Cl. No. 2.4/ Pg. No. 03 | As per circular from Ministry of Road Transport & Highways – Dt: 17.01.2024 | Please find attached circular from MoRTH and request letter from Bidder as Attachment-I. | Bidder shall quote to meet the requirement of Technical Specification. |
|  | General | Storage at site | We understand that free space based on the employer requirement (Indoor/Outdoor) at respective site locations within plant boundary shall be arranged by M/s PGCIL. Kindly Confirm. | On availability of Land, free space for Contractor's indoor store and outdoor store at respective plot areas will be provided by Employer within plant boundary. However, arrangement of land for indoor store and outdoor store shall be in the scope of the bidder. Storage Platform/Foundation for Transformer is not envisaged under the present scope. |
|  | General | Tools and tackles for erection testing and commissioning. | We have considered that tools & Tackles as required for erection, testing and commissioning of Transformers shall be mobilized by bidder and de-mobilize after successful completion of works. Please confirm. | Any special tools, tackles, testing equipment etc required for both commissioning and O&M purpose, which are proprietary in nature, shall be supplied by bidder and the price of the same is deemed to be included in the price of transformer. |