## QUALIFICATION OF THE BIDDER Qualification of bidder will be based on meeting the minimum pass/fail criteria specified below regarding the Bidder’s Technical Experience and Financial Position as demonstrated by the Bidder’s responses in the corresponding Bid Schedules. Technical experience and financial resources of any proposed sub-contractor shall not be taken into account in determining the bidder’s compliance with the qualifying criteria, except for requirement stipulated herein below for Erection Experience at Clause 1.2. The bid can be submitted by (i) A Manufacturer meeting the specified requirements given under para 1.1(a) or 1.1(b), 1.1d) or 1.2 & 1.4 or (ii) An individual firm meeting the specified requirements given under para 1.3 & 1.4, or (iii) Licensee of a Manufacturer(Licensor), meeting the specified requirements given under para 1.1(c) & 1.4 or (iv) A Joint Venture of manufacturer & Erector (Requirements specific to Joint Ventures are given under Para 1.5 below). The Employer may assess the capacity and capability of the bidder to ascertain that the bidder can successfully execute the scope of work covered under the package within stipulated completion period. The assessment shall inter-alia include (i) document verification; (ii) bidder’s works/ manufacturing facilities visit; (iii) manufacturing capacity, details of work executed, works in hand, anticipated in future & balance capacity available for the present scope of work; (iv) details of plant and machinery, manufacturing and testing facilities, manpower and financial resources; (v) details of quality system in place; (vi) past experience and performance; (vii) customer feedback; (viii) Banker’s feedback etc.

1.1 Technical Experience 1.1(a) The Bidder should have manufactured, tested and supplied at least one hundred (100) km of High temperature low sag (HTLS) conductor of same technology as that of the conductor being offered in this package having minimum *thirty seven (37) number of strands or 200 sq. mm. aluminum cross section area in last Seven(7) years as on the originally scheduled last date of bid submission (soft copy) i.e. 04/08/2025 and the same should have been in satisfactory operation$ for a period of at least one (1) year as on the originally scheduled last date of bid submission (soft copy) mentioned above. $ Satisfactory operation means Certificate issued by the Employer certifying the operation without any adverse remark. OR

1.1(b) The Bidder should have manufactured, tested and supplied at least one hundred (100) km of High temperature low sag (HTLS) conductor of same technology as that of the conductor being offered in this package having minimum *thirty seven (37) number of strands or 200 sq. mm. aluminum cross section area in last Seven(7) years as on the originally scheduled last date of bid submission (soft copy) mentioned above and the same should have been in satisfactory operation$ as on the originally scheduled last date of bid submission (soft copy) mentioned above. $ Satisfactory operation means Certificate issued by the Employer certifying the operation without any adverse remark. Note: In case of clause 1.1(b) above, the warranty obligations in terms of 10% of the ex-work cost of the HTLS conductor for additional period of two (2) years over and above the warranty period as specified in the bidding documents shall be applicable. OR

1.1(c) The Bidder, not meeting the qualification requirements stipulated at para 1.1(a) or 1.1(b) above, can also submit bid as a Licensee of a Licensor meeting the requirement stipulated at para 1.1(a), provided that:- - The Bidder/Licensee has manufactured, tested and supplied at least cumulative one thousand (1000) km of ACSR/ AAAC/ ACAR/ AACSR/ AL59 conductor having at least same or more number of strands as that of the conductor being offered in the package during last seven (7) years as on the originally scheduled scheduled last date of bid submission (soft copy) mentioned above and - The Bidder/Licensee should have established manufacturing facility & developed High temperature low sag (HTLS) conductor of same technology as that of the conductor being offered in the package having minimum thirty seven *(37) number of strands or 200 sq. mm aluminum cross section area and should have successfully carried out following tests as on the originally scheduled last date of bid submission (soft copy) mentioned above:- - On complete Conductor - DC resistance test on stranded conductor - UTS test on stranded conductor at ambient & at designed elevated temperature (minimum 150 Deg C design temperature). - On Conductor Strand/ core - Heat resistance test on Aluminium Alloy strands (not applicable for annealed aluminium) - Torsion and Elongation tests on core strands#/composite core# - Breaking load test on core strands#/ Composite core# and Aluminium#/ Aluminium Alloy# strands - Conductivity test on Aluminium#/ Aluminium Alloy# strands - Glass transition temperature test (For composite core only) - Flexural Strength test (For composite core only) # as the case may be Note: The tests indicated at B) above should have been carried out by the Bidder/Licensee on their own or by their supplier of aluminium alloy strands, core/ core strands. - The Bidder/Licensee shall furnish a legally enforceable joint undertaking by the licensor along with the Licensee in its bid (Form enclosed at Section-VI, Sample Forms & Procedures, Volume-I of Bidding Documents) to guarantee following requirement: - a) Any design undertaken by the Licensee shall be approved by the Licensor. b) Manufacturing by the Licensee shall be done with the approval of the Licensor under a quality assurance programme approved and monitored by the Licensor. c) In addition to the Contract Performance Security to be furnished by the Licensee, the Licensor shall furnish back up performance security in the form of bank guarantee for 5% of the Ex-works cost of the HTLS conductor as per format provided in the bid documents for successful performance of HTLS conductor to be manufactured and supplied by the Licensee under the contract. d) The Licensor must guarantee sequential and timely supply of materials and submission of technical information and data as desired by the Employer so as to meet the overall construction schedule and e) The agreement between the Licensee and the Licensor (copy to be submitted along with the bid) shall be valid for a period of at least two (2) years after the guarantee period of equipment and materials under supply is over. - Bidders participating individually as the Licensee should meet the qualification requirement stipulated at para 1.1d) or 1.2. Note: In case of clause 1.1 (c) above, the Bidder participating as a Licensee shall provide warranty obligations in terms of 10% of the ex-work cost of the HTLS conductor for additional period of two (2) years over and above the warranty period as specified in the bidding documents.

1.1 (d) Erection Experience - The bidder should have Successfully completed# stringing of not less than 60 km for Package OH01 of cumulative route length of transmission lines of *345/400 kV or higher voltage class as a prime contractor or as a partner in +Joint Venture within the last Seven (7) years as on the originally scheduled last date of bid submission(soft copy) mentioned above. and (ii) out of the above cumulative route length, at least one (1) transmission line project shall have route length of 25 km or more for Package OH01 +Note: In case the bidder has executed the work under a contract that had been awarded on a Joint Venture wherein the bidder was one of the partners, the experience of the bidder shall be considered limited to the scope executed by the bidder as a partner under the said contract. # Successfully completed means issue of provisional or final taking over certificate (TOC) by the Employer to the contractor for the referred contract.

1.2 In case, the Bidder, do not meet the Erection Experience criteria stipulated at para 1.1 (d) above on its own, the bidder should have in its bid, propose a Sub-Contractor meeting the above criteria.

1.3 (i) The bidder should have successfully completed# physical construction of transmission line project(s) involving stringing of not less than following cumulative route length of *345/400 kV or higher transmission line as a prime contractor or as a partner in a joint venture+ within the last seven (7) years as on the originally scheduled last date of bid submission (soft copy) mentioned above. For Package- OH01} : 60 km and (ii) out of the above cumulative route length, at least one (1) transmission line project shall have route length of 25 km or more for Package OH01 +Note: In case the bidder has executed the work under a contract that had been awarded on a Joint Venture wherein the bidder was one of the partners, the experience of the bidder shall be considered limited to the scope executed by the bidder as a partner under the said contract. # Successfully completed means issue of provisional or final taking over certificate (TOC) by the Employer to the contractor for the referred contract. (iii) The bidder shall either meet requirements stipulated at 1.1 (a) or 1.1 (b) or 1.1 (c) on its own or they should have assured access from conductor manufacturer meeting the requirement at 1.1 (a) or 1.1 (b) or 1.1 (c). In such a case, the bidder shall furnish Joint Deed of Undertaking (Format Enclosed) along with the conductor manufacturer in the bid to guarantee quality & timely supply of conductor and confirming to furnish a performance guarantee of 2% of the Ex-works cost of such conductor from the conductor manufacturer. This will be in addition to contract performance guarantee to be submitted by the contractor (bidder) on award of contract. Note: In case conductor is proposed from conductor manufacturer not meeting requirements specified at Clause 1.1(a) but meeting requirements specified at Clause 1.1 (b) or 1.1 (c), the bidder shall provide warranty obligations in terms of 10% of the ex-work cost of the HTLS conductor for additional period of two (2) years over and above the warranty period as specified in the bidding documents.

In case bidder is a holding company, the technical experience referred to in clause 1.1, 1.2 & 1.3 above shall be of that holding company only (i.e. excluding its subsidiary/ group companies). In case bidder is a subsidiary of a holding company, the technical experience referred to in clause 1.1, 1.2 & 1.3 above shall be of that subsidiary company only (i.e. excluding its holding companies).

1.4 Financial Position For the purpose of the particular bid, bidders shall meet the following minimum criteria: - Net worth for last 3 financial years should be positive. - Minimum Average Annual Turnover* (MAAT) for best three years i.e. 36 months out of last five financial years of the bidder should be Indian Rs. 94.78 Cr. for Package OH01 * Note: Annual Gross Revenue from operations/ Gross operating income as incorporated in the profit & loss account excluding other operative Income/other income. - Bidder shall have liquid assets (L.A) or/and evidence of access to or availability of credit facilities of not less than Indian Rs. 15.80 Cr. for Package OH01 d) In case Bid is submitted by the Licensee of a Qualified Manufacturer (licensor), the requirements stipulated at Clause 1.4 a), b) & c) above shall be individually met by the Licensee as well as the Licensor. In case bidder is a holding company, the financial position criteria referred to in clause 1.4 above shall be of that holding company only (i.e. excluding its subsidiary/group companies). In case bidder is a subsidiary of a holding company, the financial position criteria referred to in clause 1.4 above shall be of that subsidiary company only (i.e. excluding its holding company).

NOTE: RELAXATION FOR MSEs/ STARTUPs i) MSEs^/START-UPs^^ meeting the specified requirements at para 1.4(a) above, shall also be considered qualified if they meet 80% (Eighty percent) of the requirements specified at para 1.4(b) & para 1.4(c) above. ^MSEs as defined in bidding documents. ^^START-Ups as defined by DPIIT, applicable as on the originally scheduled date of bid submission (soft copy).

1.5 Requirements specific to Joint Ventures between conductor manufacturer & Erector In case, bid is submitted by a Joint Venture (JV) of conductor manufacturer & Erector as partners, the JV shall meet collectively the complete requirement mentioned at clause 1.1 & 1.4 above and they must meet the following requirements: (i) One of the partner(s) of JV shall meet the Technical Experience criteria for conductor manufacturer stipulated under para 1.1(a) or 1.1 (b) or 1.1(c) and, (ii) The other partner of JV shall be an Indian entity and should meet the Erection Experience criteria specified at 1.1 (d). iii) All the partners of the JV shall meet individually the Financial Position criteria given at 1.4 (a) above. iv) The lead partner shall meet not less than 40% of the Financial Position minimum criteria given at Para 1.4 (b) & 1.4 (c) v) The other partner shall meet not less than 25% of the minimum Financial Position criteria given at Para 1.4 (b) & 1.4 (c) The figure of average annual turnover and liquid assets/credit facilities for each of the partners of the JV shall be added together to determine the JV’s compliance with the minimum qualifying criteria set out in para 1.4(b) & 1.4(c) above.

1.6 The bidder shall submit documentary evidence in support of qualification requirement stipulated above.

*** End of Annexure-A (BDS) ***