| Sl. No. | Clause No./sections | Existing Clause/item | Amendment |
| --- | --- | --- | --- |
|  | Clause 1.10 of Section IB of Technical Specification | - Stringing ------- d) For transmission line with Bundle Conductor per phase: The entire stringing work of conductor shall be carried out by tension stringing technique. The bidder shall deploy requisite number of TSEs as per following so as to meet the completion schedule -Minimum 8Tonne capacity for 220kV Twin Bundle, -Minimum 8Tonne capacity for 400kV Twin Bundle, -Minimum 8Tonne capacity for 400kV Triple Bundle, -Minimum 15Tonne capacity for Quad Bundle and, -Minimum 24Tonne capacity or double no. of TSEs of 15Tonne capacity for Hexa Bundle Conductor. ----- | - Stringing ------- d) For transmission line with Bundle Conductor per phase: The entire stringing work of conductor shall be carried out by tension stringing technique. The bidder shall deploy requisite number of TSEs as per following so as to meet the completion schedule -Minimum 8Tonne capacity for 220kV Twin Bundle, -Minimum 8Tonne capacity for 400kV Twin Bundle, -Minimum 8Tonne capacity for 400kV Triple Bundle, -Minimum 15Tonne capacity for Quad Bundle and, -Minimum 18Tonne capacity for Hexa Zebra Bundle and, -Minimum 24Tonne capacity or double no. of TSEs of 15Tonne capacity for Hexa Lapwing Bundle Conductor. ----- |