| Sl. No | Clause No | Existing clause | Amended as |
| --- | --- | --- | --- |
| 1 | Voll.II, Insert a New Chapter Appendix-G & Price Schedule | - | Insert a New Chapter in Vol-II. Part- F: Centre of Excellence (CoE) system. The Scope of Work, BOQ & Architecture shall be as per Annexure-A. |
| 2 | Vol-II, Part-C, Clause-1.0 Pg. 4 of 24. | - Introduction This Technical specification is for the Automated Fault Analysis System (AFAS) and Remote Accessibility system (RAS) implementation at control centres, integration of the existing substation devices like Station Data Concentrators (SDC), AutoDR systems used for Disturbance Record (DR) collection and for accessing the substation relays/IEDs from the various control centres under the NTAMC upgradation project. The following are the functional requirements of this system: (c) Remote access of the Relays/IEDs in the Substation from Control centres (RTAMC and NTAMC) and substation Client PCs for collecting the relay parameters & settings, configuration files, internal logs, event lists etc. (d) Remote parameterization and configuration of the Relays/IEDs in the Substation from the Control centres | 1.0 Introduction Replace with This Technical specification is for the Automated Fault Analysis System (AFAS) and Remote Accessibility system (RAS) implementation at control centres, integration of the existing substation devices like Station Data Concentrators (SDC), AutoDR systems used for Disturbance Record (DR) collection under the NTAMC upgradation project. Scope of Remote Access of relays/IEDs in RAS Scope is removed. The following are the functional requirements of this system: (c) deleted. (d) deleted. |
| 3 | Vol-II, Part-C, Clause-2.1 Pg. 6 of 24. | 2.1. RAS Client PC Requirements RAS Client PCs are to be supplied by contractor for all the existing substations and control centres. The RAS client PCs shall be able to run both thick clients and web portals of RAS and AFAS systems over a secure network. The client PCs shall be able to configure the existing and new SDC and Terminal Server (TS). The PCs shall support installation of different make relay software for accessing the IEDs through existing SDC. | 2.1. RAS Client PC Requirements Replace with RAS Client PCs are to be supplied by contractor for all the existing substations and control centres. The RAS client PCs shall be able to run both thick clients and web portals of RAS and AFAS systems over a secure network. The client PCs shall be able to configure the existing and new SDC and Terminal Server (TS). |
| 4 | Vol-II, Part-C, Clause-3.0 Pg. 7 of 24. | 3.0 Functional Requirements for RAS System The Remote Accessibility System shall provide access to the remote device i.e. protection relays. These remote devices shall be having either serial port or Ethernet port. These devices shall be accessed from client PCs at control centres and substations through RAS servers using the existing RAS equipment at substation for relay parameterization, configuration changes, internal logs & database downloads, and diagnosis. Relay configuration software shall be installed in RAS client PC and these shall be used to access the devices from remote for configuration and settings changes. Relay configuration software shall be supplied by employer. The project files and configuration files shall be kept in a centralized location. The user interface for RAS shall meet the following requirements in addition to the requirements in Sec 4.2. (b) Add, modify and remove access rights to the IEDs from a central location. (d) Manage project files/ device configuration files, IED software versions, passwords and access history centrally. | 3.0 Functional Requirements for RAS System Deleted the paragraphs Scope of Remote Parameterization in RAS system is removed “The Remote Accessibility to the remote_________________________________________________________________ files shall be kept in a centralized location. (b) Deleted (d) Deleted. |
| 5 | Appendix-G & Price Schedule | RAS APPLICATION SOFTWARE FOR REMOTE PARAMETERIZATION | Scope of Remote Parameterization in RAS system is removed. Based on the above, the Price Schedule has been revised and is enclosed herewith, which shall replace the previous Price Schedule of the Bidding Documents. Note- Bidders are requested to consider the revised excel sheet(s) titled “Price Schedule_rev02” for bid preparation/submission for the subject package. It may be noted that while submitting the bid(s), bidders are required to change the excel file name to ‘Price_Schedule’. As per the provisions of the portal, it is mandatory to upload excel file with titled ‘Price_Schedule’ (Bidders may refer user manuals at the portal https://etender.powergrid.in regarding submission of bid). |