PART-F

CENTER OF EXCELLENCE (COE)

Table of Contents

1.0 Introduction	3

2.0 Bill of Quantity (BOQ)	4

2.1. BOQ for Supply, Installation & F&I:	4

2.1. BOQ for AMC:	4

3.0 Technical Specifications:	4

3.1. Server for hosting custom application:	4

3.2. Storage Server NAS:	4

3.3. Managed LAN L3 Type-1:	5

3.4. Workstation As Per TS:	5

4.0 Architecture	5

CENTER OF EXCELLENCE (CoE) SYSTEM

# Introduction

POWERGRID is implementing a “Centre of Excellence (CoE)” centralized management of the substation level IEDs & SAS gateways through a Software based relay database management & calculation system. The system includes Servers at Main NTAMC, Clients at all RTAMCs and Workstation at each substation. The requisite hardware for establishment of COE is envisaged under this project. The Hardware/Software system in this section shall be referred as CoE Subsystem.

The Scope of work shall include:

- Bidder Scope:

- Supply & Installation of Hardware items mentioned in BOQ (refer Sr.no 2.0).

- Shall provide Operating System for CoE as specified below:

  - Servers: Microsoft Windows Server 2019 Standard Edition

  - Workstation at Control Centre: Microsoft Windows 11 Professional (64-bit), (RDS CAL)

  - Workstation at Substation: Linux.

- Shall provide necessary support for Commissioning.

- Clients of EDR, Antivirus, Application whitelisting, HIDS, patch management shall be installed as specified in Technical Specifications for similar hardware items.

- Shall provide all IT support services like Backup of Servers, user authentication, etc.

- Shall integrate with NMS, Inventory management, SIEM, patch management etc. as specified in Technical Specifications for similar hardware items.

- Cyber Security requirements mentioned in technical specifications are to be complied by Bidder.

- Comprehensive AMC for Hardware defects.

- The CoE system availability shall be maintained at 99.95% as per Part-A, Section-4 of Technical Specifications. However, failure due to application software or software not supplied by Contractor shall not be considered for measurement of System Availability criteria.

- Employer Scope:

- Supply & Installation of Application Software for CoE system.

- Configuration of Application Software & Application maintenance.

There shall be separate LAN for the COE items (Server, Storage & Workstations etc.) at Main NTAMC. The COE LAN shall be connected to Type-1 External Firewall for accessing the Substation end workstations installed for COE.

At RTAMC, the client workstation shall be connected to Data LAN for accessing the Servers at Main NTAMC.

# Bill of Quantity (BOQ)

## BOQ for Supply, Installation & F&I:

| ITEM DESCRIPTION | UNIT | CoE Function | Main NTAMC | RTAMC | Substation |
| --- | --- | --- | --- | --- | --- |
| CONTROL CENTRE ITEMS |  |  |  |  |  |
| HARDWARE FOR CENTRE OF EXCELLENCE(COE) |  |  |  |  |  |
| SERVER FOR HOSTING CUSTOM APPLICATION | EA | Application Server- 2nos Database Server - 2nos Virtual Desktop Infrastructure Server - 2nos | 6 | - |  |
| STORAGE SERVER NAS | EA | Storage Server | 1 | - |  |
| WORKSTATION AS PER TS | EA | To Perform CoE functions at Main NTAMC & RTAMCs | 10 | 11 |  |
| TYPE-3 TFT MONITOR -TFT | EA |  | 10 | 11 |  |
| MANAGED LAN L3 TYPE-1 | EA | For Connecting CoE system at Main NTAMC | 3 | - | - |
| HARDWARE FOR SUBSTATION |  |  |  |  |  |
| WORKSTATION AS PER TS | EA | Console at S/s shall be connected to existing SAS switch and existing S/s Router cum Firewall. | - | - | 300 |

## BOQ for AMC:

| ITEM DESCRIPTION | UNIT | QUANTITY (For 6 Years) |
| --- | --- | --- |
| Main NTAMC |  |  |
| QUARTERLY COMPREHENSIVE MAINTENANCE CHARGES OF ITEMS SUPPLIED UNDER CENTRE OF EXCELLENCE (COE)* | Lot | 24 |

*This line Item includes all the Components, Hardware and Software supplied as per Sr.no 2.0 (BOQ) under COE system. The availability, Severity level & response time of CoE system shall be measured as per Part-A, Section-4 of TS.

# Technical Specifications:

## Server for hosting custom application:

This Server Shall be used to deploy the various “Application Software” envisaged under Centre of Excellence (COE). Three different applications are envisaged for deploying in these servers in Redundant Configuration.

The minimum hardware configuration shall be as per Vol-II, Appendix-P of Part-B of Main NTAMC specifications.

## Storage Server NAS:

NAS shall be provided for storing user defined File data of all COE servers and workstations. The minimum hardware configuration shall be as per Vol-II, Appendix-P of Part-B of Main NTAMC specifications.

## Managed LAN L3 Type-1:

Servers, NAS and Workstations of COE at Main NTAMC shall be connected to dedicated LAN. The LAN of COE devices shall be separate and connected to Firewall. The minimum hardware configuration shall be as per Vol-II, Appendix-P of Part-B of Main NTAMC specifications.

## Workstation (Jump Server) as Per TS:

Workstation console shall be provided at Main NTAMC, Back up NTAMC & RTAMCs to perform various activities related to COE. A dedicated one (1) no. “Workstation (Jump Server)” shall be supplied at each substation. Workstation consoles shall be capable of driving three monitors, a single keyboard and a mouse with dual LAN interfaces.

The Interfaces of Workstations shall be as follows:

- Main NTAMC				: 2 x 10G FO Ports

- Back up NTAMC, RTAMCs & Substations	: 2 x 1G CU Ports

The minimum hardware configuration of Workstations & Type-3 TFT monitor shall be as per Vol-II, Appendix-P & Section-5 of Part-B.

# Architecture

# The indicative architecture of CoE system is given below. The components of CoE are marked with ‘*’(asterisk). The other components shall be shared with the NTAMC infrastructure.

-----------End of the Section-----------