NAME OF WORK:

Dismantling of Old A.C. sheets and replacing with  Precoated galvanized profile sheets including providing of new steel trusses , fixing of false ceiling with frame, fixing of vitrified tiles for floor of painting for both external and internal wall and other allied work at 1 no B-Type Quarters at 220/132 kV Salakati S/S, POWERGRID.

TECHNICAL SPECIFICATIONS

Dismantling of Old A.C. sheets and replacing with Precoated galvanized profile sheets including providing of new steel trusses , fixing of false ceiling with frame, fixing of vitrified tiles for floor of painting for both external and internal wall and other allied work at 1 no B-Type Quarters at 220/132 kV Salakati S/S, POWERGRID.

1.0 Introduction:

    - The intent of this technical specification covers construction of all civil work as covered in the scope of contract, BOQ description and as per drawings supplied by Owner/Consultant/vendor for Dismantling of Old A.C. sheets and replacing with Precoated galvanized profile sheets including providing of new steel trusses , fixing of false ceiling with frame, fixing of vitrified tiles for floor of painting for both external and internal wall and other allied work at 1 no B-Type Quarters at 220/132 kV Salakati S/S, POWERGRID under O&M budget.

    - All civil works shall be carried out as per design/drawings standardized by the Owner and the specifications provided by the Owner. All standard drawings are enclosed with the tender documents. In case any item is not covered under specification then the same shall be carried out as per BOQ description and then CPWD specification and applicable Standards and Codes. Any item for which specification is not provided herein and is not covered under CPWD specification shall be executed as per manufacturer guidelines or best industrial practices as instructed by the EIC. All materials shall be of best quality conforming to relevant Indian Standards and Codes. In case of any conflict between Standards/ Code and Technical Specification, the provisions of Technical Specification shall prevail.

    - The Contractor shall furnish all labours, tools, equipment, materials, temporary works, constructional plant and machinery, fuel supply, transportation and all other incidental items not shown or specified but as may be required for complete performance of the Works in accordance with drawings, specifications and direction of Owner.

    - All materials including cement, reinforcement steel and structural steel etc. shall be arranged by the Contractor. All testing required shall be arranged by the Contractor at his own cost. The contractor shall execute the work as per the Field Quality Plan (FQP) attached with this document.

    - The bidder shall fully apprise himself of the prevailing conditions at the proposed site. Climatic conditions including monsoon patterns, local conditions and site specific parameters and shall include for all such conditions and contingent measures in the bid, including those which may not have been specifically brought out in the specifications.

    - The location of the work shall be decided by the EIC. The type of toilet to be constructed shall also be decided by POWERGRID.

# Scope of Works:

The scope of this specification covers Dismantling of Old A.C. sheets and replacing with Precoated galvanized profile sheets including providing of new steel trusses , fixing of false ceiling with frame, fixing of vitrified tiles for floor of painting for both external and internal wall and other allied work at 1 no B-Type Quarters at 220/132 kV Salakati S/S, POWERGRID under O&M budget in accordance with Bill of Quantities Specifications, approved drawings, Conditions of Contract, Quality Assurance Programs and proper safety measures during construction and to the satisfaction of the Engineer in charge. The scope of work is indicated in details hereunder-

    - Dismantling of existing of Old A.C. sheets and replacing with Precoated galvanized profile sheets etc. at 220 kV Salakati SS..

    - providing of new steel trusses , fixing of false ceiling with frame, fixing of vitrified tiles for floor of painting for both external and internal wall and other allied work related work etc.

The above scope of work mentioned is not exhaustive and works is to be carried out as per the BOQ attached and will also include any additional works that may occur to complete the intent of the above works and as suggested by the employer.

# Schedule Items:

    - The Items of works considered based on Delhi Schedule of Rates are termed as Schedule Items.

    - The work shall be executed in accordance with the specification stipulated in the Bill of Quantity and other bidding documents read along with CPWD (Central Public Works Department) specifications-2019 for civil works and IS codes with up to date corrections. For non-schedule items specification as given along with tender document and similar items of CPWD shall be applicable.

    - The list of references for civil works are CPWD specifications, relevant IS codes and best practices.

# 4.0.	NON-SCHEDULE ITEMS:

The items not available in Delhi Schedule of Rates and are considered based on market rates of materials & works are termed as Non-Schedule Items. The specification for the non- schedule items is attached with the tender documents. For any item if specification is not provided herein, the item is to be executed as per Bill of Quantity given or available specification of CPWD for similar items. Any item for which specification is not covered under CPWD specification shall be executed as per manufacturer guidelines or as instructed by EIC.

# GENERAL:

    - All RCC work including material properties, proportioning, batching, mixing, transporting, pouring, compacting, finishing, curing, sampling, testing, acceptance criterion etc. shall be as per IS : 456-2000 (or latest).

    - All RCC work shall be of Nominal Mix 1:1.5:3 as specified in the drawing and BOQ.

    - In case of volumetric mixes, Standard measuring boxes of 35cmX40cmX25cm (35 liters capacity as per CPWD) shall be used for measuring fine and coarse aggregates.

    - However, full 50 kg of OPC/PPC/Slag cement shall be directly unloaded into the mixer hopper to ensure that cement consumption is 400kg per cum of 1:1.5:3 concrete irrespective of different types of cements with different densities.

    - Ordinary Portland cement (OPC-43 Grade) of IS 8112/ IS 12269 and Portland Pozzolana Cement (PPC) of IS 1489(part-1) shall be used for all RCC works and for all other cement works. This will supersede other specifications of cement to be used for the works.

    - Use of chemical admixtures shall be permitted in accordance, with the provisions of IS 456 and IS 9103. It shall be the responsibility of the producer to establish compatibility and suitability of any admixture with the other ingredients of the mix and to determine the dosage required to give the desired effect. The amount of admixture added to mix shall be recorded in the production record by the producer. Admixtures should be stored in a manner that prevents degradation of the product and consumed within the time period indicated by the admixture supplier. Any vessel containing an admixture in the plant or taken to site by the producer shall be clearly marked as to its content. When offering or delivering a mix to a purchaser it should be indicated if such a mix contains an admixture or combination of admixtures or not. The admixtures may be identified generically and should be declared on the delivery ticket.

    - For ready mixed cement concrete, in addition to the CPWD specification, and relevant IS codes, the design mix shall be submitted to Engineer in Charge for approval. Design mix concrete may be procured from approved RMC plant as per latest IS: 4926 or may be prepared by using a portable/mini/compact weigh batch plant conforming to latest IS: 4925. As per provisions of IS: 456, fly ash conforming to grade I of IS: 3812 (part-1) may be used as part replacement of cement, when OPC cement is used for concrete works. Uniform blending with cement is to be ensured.

    - Materials shall be of approved quality and the best of their kind available and shall generally conform to I.S. Specifications. All hard ware fittings shall be of either ISI marked best quality or as per the brands and quality mentioned in the BOQ or list of approved makes and shall be selected as per the Instructions of Engineer in Charge.

    - All the finishing items are to be executed in line with the BOQ, specification and also as per the instruction of EIC. The material is to be procured as per the brand and shades decided by the Engineer in Charge. The Bills / invoices are to be submitted to POWERGRID.

    - Reinforcement steel shall be thermo-mechanically treated (TMT) or HYSD reinforcement bars of grade Fe 500D or higher grade conforming to IS: 1786. Reinforcement shall be arranged by contractor.

    - The Contractor shall bring samples of all materials; finished or raw to the site and shall get them approved from the Engineer-in-Charge before using them in the work. Any material not conforming to approved samples shall be removed within 48 hours of the Engineer-in-Charge’s instruction to the Contractor to do so, failing which it shall be removed by the Engineer-in-Charge or his representative from the site at the risk and cost of the Contractor.

    - Samples of materials to be used with original/colour catalogue with specification shall be brought by contractor well in advance and shall be displayed and kept in site. Samples of all kinds of materials, machineries to be supplied under the scope of the work shall require approval of the Owner.

    - Before placement of award for supply & installation of equipment or materials, the contractor shall submit a proposal furnishing all the catalogs & specifications for approval of the Engineer in charge, well in advance.

    - Double pipe scaffolding shall be used for all the works and as per industry best practices. All the health and safety precautions as per the relevant IS codes and approved standard guidelines for constructions works to be ensured by the executing agency and holds complete responsibility.

    - Where equivalent is indicated, it shall mean material with equivalent quality and technical specification with the vendors indicated. In case it is not possible to identify similar materials with other approved vendors, the material shall be selected based on equivalent quality & Technical specification & cost from other reputed vendors with the approval of the EIC.

    - The work shall be executed at all levels unless otherwise specified.

    - The contractor must take precautions to protect all existing structures, pipelines and utilities from damage. This applies not only to the area within the limits of work but also along any local access roads or roadways internal to the project.

    - The Contractor shall work in Co-ordination of the other Contractors already engaged in the project area and resolve all the matters with mutual understanding.

    - The Contractor shall be sole responsible for making/ repairing of carriage ways required if any.

    - The Contractor shall be responsible for the keeping safe custody of all the construction material and T&P and shall store the material as per the standard industrial norm.

# 6.0.	DRAWINGS:

All major drawings are enclosed with this document. These drawings are tentative drawings and may change a little during detailed engineering. Drawings that are to be submitted by Contractor shall be submitted well in advance for approval of Engineer In- Charge. POWERGRID will supply only those drawings which are major structural/ Architectural drawings only. Other detail/working drawings which are required as per site condition shall be either submitted by the agency or mutually decided with POWERGRID as per BOQ item and best practice and put up for approval of E-I-C. Drawings shall not be issued for items for which detailed specification is available in TS or BOQ and specific vendor for the item is mentioned. In such cases, shop drawings if required shall be arranged by the agency from the vendor and approval shall be taken from E-I-C.

# FIELD QUALITY PROGRAM:

    - The work shall be executed as per the Standard Field Quality Document No. C/FQA/SFQP/SITE/-CIVIL-2024 Rev- 05 dtd.10.07.2024 pertaining to building civil works along with the other quality plans indicated in the Technical specifications. All tests on materials and finished products as required in field quality plan shall be carried out by the Contractor at his own cost. The following guide lines shall govern for carrying various lab tests:

    - The material testing shall be carried out in any of the NABL accredited POWERGRID’s approved laboratories. In case Testing facilities are not available for any specified test given in FQP/Tech Spec/BOQ item description in these laboratories, the same shall be carried out in any other laboratory where such facilities are available with the specific approval of Engineer in Charge.

    - The contractor shall arrange at his own cost, packing, forwarding and other incidentals for conducting the tests.

    - All test reports shall be submitted to POWERGRID within 7 days from the date of dispatch of test specimens to the laboratory.

# 8.2	REQUIREMENT FOR FILL MATERIAL UNDER FOUNDATION

The thickness of fill material under the foundations if required, shall be such that the maximum pressure from the footing, transferred through the fill material and distributed onto the original undisturbed soil will not exceed the allowable soil bearing pressure of the original undisturbed soil. The filling shall be done in accordance with clause 4.10 of IS: 1080. For expansive soils the fill materials and other protections etc. to be used under the foundation is to be got approved by the Employer. Cohesive Non Swelling (CNS) soil, if required, for filling under / around the foundations, cable trenches, drains, roads etc. shall confirm to IS: 9451: 1994 reaffirmed 2004.

# SCHEDULE ITEMS:

All the schedule works shall be done as per the CPWD specification. The work shall be carried out as per mentioned in BOQ and all materials shall be of POWERGRID approve brand or as

per SFQP of POWERGRID. Such deviation shall be taken approval from E-I-C prior to execution.

# 9.1.1	MODE OF MEASUREMENTS:

The of measurements for schedule items shall be followed as per the procedure mentioned in the latest CPWD specification and its revisions. For Non-schedule items, measurements shall be done as per the unit mentioned against the items and BOQ description following standard practices.

****