Section 1

INTRODUCTION and GENERAL INFORMATION

  - Introduction

This Volume II of the Bid Document describes the technical specifications for Customer Premises Equipment for Aggregation & Access network being procured by the Employer ‘PowerGrid Teleservices Limited’ (PowerTel). This Specification describes functional and performance requirements of MPLS-TP CPE, TMN System and associated Equipment/ System.

The purpose of this section of the specification is to provide general information and define the scope of this project.

  - Project Overview

PowerTel have a country wide Telecom network connecting the major cities of India. The details of the Telecom network have been given in Appendix-B.  The access network in cities is a mix of DWDM, OTN, MPLS-TP and SDH. A pan Indian IP-MPLS Network is also running over the existing DWDM, OTN, MPLS-TP and SDH Network.

PowerTel has planned to augment its Telecom access network using MPLS-TP Equipment on separate pairs of fibre, however limited capacity (Single channel 10G/ 100G) requirement in backbone network may also be considered, if required. The planned Network expansion under this package will coexist with the current MPLS-TP, OTN & SDH network.

Equipment under package shall be installed & commissioned all over India based on requirement, within the existing Network (as shown in Appendix-B) as well as in new cities/ region with expansion of Network anywhere in India.

  - Scope and General Requirements

This specification defines the design, engineering, performance, testing and implementation requirements under subject Package. Sections 1 through 8 define the general requirements of the installation, testing & commissioning and maintenance of MPLS-TP and associated subsystems under subject Package-B1.

The equipment shall be deployed in Access as well as Backbone Network. However, the requirement/ quantity of equipment laid out in the BoQ (Annexure-C) is tentative, and individual quantity of any of equipment/ module, Pluggable and any other sub-system listed there in the BoQ, can be increase/ decrease to any value, keeping the increase/ decrease in overall value of contract as per applicable limits defined in SCC. The Equipment will be ordered in ‘LOT’, where approximate quantity in each of the Lot is defined at Annexure-C. The Actual quantity in each of the Lot may vary by ±10% of the quantity specified for a respective lot under Annexure-C.

All MPLS-TP Equipment and amplifier solution required under the project shall be offered by the bidder from the same manufacturer and accessible through the single NMS supplied along with Equipment.

    - Scope of Work

The scope of work under the subject Package includes survey, planning, design, engineering, supply, installation, termination, testing, commissioning, training, and maintenance of:

- All types of MPLS-TP Equipment (100G, 10G, 1G, FE,  E1 etc.)

- Amplifier solution supporting long distance solution @ 10G link capacity with MPLS-TP Equipment

- Equipment Cabinets/ Racks

- DC Power System (DCPS)

- Mux/ Demux

- All associated hardware, cables, Power Cables, accessories and fittings

- Telecommunication Management Network (TMN).

- Support (Documentation as well as manpower/ licenses) for integration of TMN system with GIS Mapping System, OSS, Network Orchestrator etc. in future without any financial implication to PowerTel.

- Remote Access Server solution meeting DoT requirement.

- Mandatory Testing and Certification of Telecommunication Equipment (MTCTE) of MPLS-TP Equipment and amplifier solution

- Security Testing of all Equipment/ elements as per Common Criteria Certification.

- Vulnerability Assessment of Network from a certified third party on completion of project.

- All other associated works/items described in the technical specifications for a viable and fully functional System.

- The scope of work under this subject package shall cover supply, installation and other services anywhere in India, Defect liability Period (01 Year) of complete system and subsequently AMC of 5 years for all MPLS-TP equipment, TMN hardware & Software.

- All offered equipment must be upgraded to the latest software and feature version to have a better performance and security.

- Replacement of all Desktop Computers and TMN (server, other hardware, Software etc.) during AMC period.

    - General Requirements

- The contractor shall supply ‘Trusted Products’ from ‘Trusted Sources’ in line with the ‘National Security Directive on Telecommunication Sector (NSDTS)’ under subject package. Trusted Products are products whose critical components and the products themselves are sourced from Trusted Sources. The National Cyber Security Coordinator (NCSC) is the Designated Authority (DA) for the determination of inclusion of a vendor as a Trusted Source, of a Telecom product as a Trusted Product and the methodology for the said inclusion. Contractors are advised to do their due diligence while selecting an OEM and offering the model/ type equipment. The Operational Acceptance of Network shall be provided to the contractor only after clearance of the Telecom Equipment as ‘Trusted Products’ from ‘Trusted Sources’ as per NCSC. It’s a mandatory compliance.

- ‘Mandatory Testing and Certification of Telecom Equipment (MTCTE)’ under the provision of Indian Telegraph (Amendment) Rules 2007 shall apply on all equipment to be supplied under subject Package. Contractor has to supply only certified equipment as per MTCTE. Please refer the weblink https://www.mtcte.tec.gov.in for more details. It’s a mandatory compliance.

- Contractor shall comply with Indian Telecom Security Assurance Requirements (ITSAR), notified by NCCS for the product(s) to be supplied under the package. The ITSAR compliance shall only be applicable, if notified before the last date of bid submission. It’s a mandatory compliance, if notified for the product.

- Contractor has to sign NDAs (Non-Disclosure Agreement) with employer as per the format attached at Appendix-F and undertakes commitment to provide malware free software and patches for Software upgrades. Contractor has to sign NDA with respective OEM(s) wherever applicable to ensure the security of assents of PowerTel & POWERGRID

- Contractor shall comply with Indian Telecom Security Assurance Requirements (ITSAR), notified by NCCS for the product(s) to be supplied under the package. The ITSAR compliance shall only be applicable, if notified before the last date of bid submission.

- The Contractor is encouraged to offer standard products and designs. However, the Contractor must conform to the specification requirements and provide any additional equipment necessary to meet the requirements stated herein.

- It should be noted that bill of quantity is indicative only, and the equipment shall be ordered based on requirement and the solution/ design finalized during detail engineering.

- The Bidder's proposal shall address all functional and performance requirements within this specification and shall include sufficient information and supporting documentation in order to determine compliance with this specification without further necessity for inquiries. Brochure & Data sheets of all equipment shall be submitted, in this regard.

- The Bidder's proposal shall clearly identify all features described in the specifications or in any supporting reference material that will not be implemented; otherwise, those features shall become binding as part of the final contract.

- An analysis of the functional and performance requirements of this specification and/or site surveys, design, and engineering may lead the Contractor to conclude that additional items are required that are not specifically mentioned in this specification. All such additional items necessary for a viable and fully functional Network to be implemented, that meets or exceeds the capacity and performance requirements specified, shall be provided/ supplied by Contractor without any additional cost to Employer. Such materials shall be considered to be within the scope of the contract. The Bidders shall identify and include all such additional items in their proposal, to the extent possible.

- The offered items shall be designed to operate in varying environments across India. Adequate measures shall be taken during manufacturing of Equipment to provide protection against contaminants, pollutants, water & moisture, lightning & short circuit, vibration and electro-magnetic interference etc. The Contractor shall demonstrate a specified level of performance of the offered items during well-structured factory and field tests, and necessary certification (if any).

- The bidders are advised to visit sites (at their own expense), prior to the submission of a proposal, and make surveys and assessments as deemed necessary for proposal submission. The Contractor is required to visit sites, wherever asked by Employer, to record information necessary in finalization of quantity of accessories, length of cables & patch cords etc for finalization of BoQ. The site visits shall include all necessary surveys to allow the contractor to perform the design and implementation functions. Contractors are encouraged to utilize the available rack space in existing racks at existing locations for intended Equipment under this package, wherever available and permitted by Employer. The Contractor shall submit the site survey format for Employer’s approval. The site survey schedule shall be finalized in consultation with the Employer. The Employer or its representative may be associated with the Contractor during their site survey activities.

- After the site/ route survey the Contractor shall submit to the Employer a survey report as per the approved survey format. This report shall include at least the following items:

- Proposed layout of Equipment in the existing rooms and buildings/ shelters.

- Proposed Length of power, earthing, signal cables and patch cords etc.

- Confirmation of adequacy of Space, air conditioning and AC/DC Power supply requirements.

- Utilization of existing rack space

- Measurement of fibre attenuation by power meter, in case of backbone links.

- Any other test performed to record fiber parameters, necessary for their proposed Equipment/ Solution.

  - General Responsibilities and Obligations

This section describes the general responsibilities and obligations of the Contractor and the Employer.

    - Responsibilities for the Implementation Plan

The Bidder's technical proposal shall include a project implementation plan and schedule that is consistent with the implementation plan detailed in Section 6 and Appendix-A of this specification. The implementation plan shall be modelled such that it provides the phase wise supply, installation & activation of this Project. The Implementation plan shall include the activities of both the Contractor and the Employer, showing all key milestones and clearly identifying the nature of all information and project support expected from the Employer. The Employer and Contractor shall finalize the detailed Implementation plan following award of the contract.

    - Contractor's Responsibilities and Obligations

The Contractor shall be responsible for the implementation of the Project.

Contractor's obligations include, but are not limited to, the following:

          - Provide a working system that meets the functional and performance requirements of this specification.

          - Survey of the locations wherever required, and submission of survey report as per agreed formats.

          - Site visits and studies are necessary to identify and provide all equipment needed to implement the Solution.

          - Engineering and design specific to each location including review of, and conformance with local environmental and earthing requirements.

          - Define source power requirement for each cabinet/ rack of equipment provided at each location.

          - Development of installation and safety guidelines and procedures for the mechanical installation including testing and documentation.

          - Appointment of Project Manager, Design Engineer & other resources for successful & timely completion of project.

          - Project management, project scheduling, including periodic project reports documenting progress during the contract period.

          - Planning and implementation for a smooth transition from existing operations to new upgraded network operations including new equipment, wherever applicable.

          - Submit earlier carried out type test reports or conduct type tests & functional tests as per requirement.

          - Factory and site acceptance testing of all hardware, software and firmware provided as per approved procedures.

          - Shipment of all equipment and documentation to the Employer designated locations and/or staging areas.

          - Staging, maintenance and security of the staging area including the full responsibility for protection from fire and theft.

          - Supply, installation, testing and commissioning of all hardware, software and firmware required to satisfy this specification.

          - All cabling wiring includes supply, laying and termination of cables (signal, power supply & earthing etc.), DC/AC distribution boards with MCBs required for powering equipment.

          - Interfacing and integration of all supplied equipment with third party SDH/DWDM/Router equipment and with other equipment being procured by Employer in future.

          - Installation and integration of Telecommunications Management Network (TMN) system software, hardware and firmware.

          - Configure all circuits/ Services (end to end) in the network as indicated by the Employer during detail Engineering and implementation.

          - System integration & commissioning of the overall telecommunications network and also coordination with other Project Contractors for phased implementation.

          - All documentation and drawings as specified.

          - All necessary information, documents, manpower support, licenses for enabling interfaces etc required for integration of TMN system with OSS, GIS System, Network Orchestrator etc, planned to be procured in future.

          - Training of the Employer personnel.

          - Maintenance, debugging and support of the items through final acceptance, and maintenance (including deployment of resources) throughout the defect liability period and after defect liability period as defined in Section 7.

          - All required spare parts and maintenance aids

          - Ensuring availability of service and spares of the equipment to meet availability requirement specified at Section 6 & 7.

          - Provide spare modules/ cards/ base equipment for MPLS-TP and TMN system.

          - Compliance of all security related requirements as per latest DoT/ MoP/ GoI guidelines with subsequent amendments including security certification of equipment as defined in Section-8.

          - Taking over of project in Lots.

          - Network Security Audit cum Vulnerability Assessment Test, and implementation of mitigation plan.

          - Handing Over of complete Network after Acceptance testing and completion of Vulnerability Assessment testing & its mitigation plan.

          - Defect liability period (DLP) of 01 year after TOC.

          - AMC of the Equipment/ system for at minimum 5 years.

          - Deployment of Technical Manpower as per the request of Employer.

          - Replacement of all Work Stations as well as Severs during AMC.

- Detailed descriptions of the Contractor's obligations, in relation to individual items and services offered, are delineated in other sections of this specification.

    - The Employer Responsibilities and Obligations

The Employer will provide the following items and services as part of this Project:

          - Overall project management of the project.

          - Source power at (nominal) -48 volts DC and/or AC power at (nominal) 230 volts, 50 Hz as required by the equipment to be supplied.

          - Providing support and access to facilities at the sites.

          - Implement the major civil works such as expansion or construction of rooms, trenches necessary for installation of proposed equipment.

          - Provide to the extent possible drawings for existing sites and facilities for which equipment installations are planned.

          - Obtaining statutory clearances from regulatory bodies, if required.

          - Approval of key personnel for the project.

  - Systems To Be Supported

The fibre optic cables consist of DWSM (G.652) & DSSM (G.653) fibres for backbone sections and DWSM (G.652) fibres for Access network.

  - General Bidding Requirements

The Bidder shall be responsive to the Employer's technical requirements as set forth in this specification. The Bidder's proposal shall include the following:

- The Technical Proposal including the documents listed in the Table 1-1: Bid Documents Checklist, shall be provided in the bid.

- A detailed project implementation plan and schedule that is consistent with the scope of the project and Employer's specified objectives.  The plan shall include the activities of both the Contractor and Employer, show all key milestones, and clearly identify the nature of all information and project support to be provided by Employer. Manpower resources proposed to be deployed by the Contractor during the execution phase shall be clearly indicated.

| Table 1-1: Bid Documents Checklist |  |  |  |
| --- | --- | --- | --- |
| Sl. No. | Description: | Enclosure Reference |  |
| 1 | MTCTE Certificates/Reports for offered Equipment/Sub-systems as per specification requirement. | Page no. Ref no. |  |
| # 2 | # Trusted Vendor Certificates pertaining to OEM of the Equipment & TMN | Page no. Ref no. |  |
| # 3 | # Trusted Equipment Certificates/Reports for offered Equipment/Sub-systems as per specification requirement. | Page no. Ref no. |  |
| 4 | Completed Data Requirement Sheets (As per Appendix- D of Technical Specs Volume II ) | Page no. Ref no. |  |
| 5 | Network design document including link budgeting for Amplifier Solution, Pluggable etc as per specification requirements. | Page no. Ref no. |  |
| 6 | Detailed Project Implementation Plan (As per Section 1.7 General Bidding requirements of Technical Specs Volume II) | Page no. Ref no. |  |

  - Table of Compliance

Bidder shall use one copy of Volume I, "Conditions of Contract," and Volume II, "Technical Specifications" to indicate compliance status with those volumes. Within the right-hand margin, Bidder shall indicate compliance status to each paragraph along with a cross-reference to its proposal and an index key for any explanation or comment.

In addition, the Bidder shall annotate the Table of Contents of each of the above stated volumes to provide a high-level summary of compliance status. In both cases, the following symbols, and no others, shall be used:

| C | Bid complies with all requirements in the adjacent paragraph. |
| --- | --- |
| A | Bid is not compliant with the requirements in the adjacent paragraph, but a functional alternative is proposed. |
| X | Bid takes exception to the requirements of the adjacent paragraph and no functional alternative is proposed. |

Only one symbol shall be assigned to a paragraph and shall indicate the worst case level of compliance for that paragraph. This annotation may be hand written.

Bidder shall also underline, on the compliance copy, all requirements to which exceptions have been taken (X) or to which alternatives have been proposed (A).

Each alternative shall be clearly and explicitly described. Such descriptions shall use the same paragraph numbering as the bid document sections addressed by the alternatives. All alternative descriptions shall be in one contiguous section of the Bidder's proposal, preferably in the same volume, and titled "Alternatives." A separate section titled "Exceptions" should be provided containing any discussion or explanation Bidder chooses to provide concerning exceptions taken. Alternatives which do not substantially comply with the intent of the bid documents will be considered exceptions.

The Employer will assess the merits of each alternative and exception and will be the sole judge as to their acceptance.

  - Organization of the Technical Specification Document

Sections 2 through 8 provide the project requirements of the Fibre Optic Transmission System to be provided.

| Section 2 | contains the hardware description and specifications for the Telecom Equipment |
| --- | --- |
| Section 3 | contains telecommunications management network requirements |
| Section 4 | contains environment, Power Supply, Cabling and Earthing Requirements |
| Section 5 | contains the inspection and testing requirements. |
| Section 6 | details requirements for project management, Quality Assurance, Training and Documentation |
| Section 7 | Maintenance services requirements |
| Section 8 | Security Requirements |

The following is a list of the Volume II Appendices:

| Appendix A | Implementation Schedule |
| --- | --- |
| Appendix B | Existing Telecom Network, |
| Appendix C | Bill of Quantity (BoQ) |
| Appendix D | Data Requirement Sheets |
| Appendix E | Template of the Agreement between TSP and Vendor as per DoT guidelines |
| Appendix F | Non-Disclosure Agreement (NDA) |

  - Applicable Standards

Equipment must comply with the MTCTE essentials requirements for required equipment and must comply with latest essential requirement and amendment if any. The equipment must attain the MTCTE certificate for the offered equipment.

Specifications and codes shall be the latest version, inclusive of revisions, which are in force at the date of the contract award. Where new specifications, codes, and revisions are issued during the period of the contract, the Contractor shall attempt to comply with such, provided that no additional expenses are charged to the Employer without Employer’s written consent.

In the event the Contractor offers to supply material and/or equipment in compliance to any standard other than Standards listed herein, the Contractor shall include with their proposal, full salient characteristics of the new standard for comparison.

In case the requirements stipulated in the specifications are more stringent than those specified by the standards, the specification shall override the standards.