Section 2

NETWORK CONFIGURATION and EQUIPMENT CHARACTERISTICS

  - Introduction

This section describes the communication network configuration and characteristics of the MPLS-TP, Amplifier Solutions, DC Power System, Mux/ Demux and associated subsystems. The requirements described herein are applicable to and in support of network configurations depicted in Appendix C along with BoQ. A Network Management System (Telecommunications Management Network, TMN) is required for the management of all MPLS-TP Equipment and Amplifier Solution to be supplied under the project. Functional and performance requirements of the TMN are defined in Section 3. Minimum subsystem monitoring and control requirements to be provided are specified as part of the subsystem's equipment characteristics. Equipment must comply with the MTCTE essentials requirements for required equipment and must comply with latest essential requirement and amendment if any. The equipment must attain the MTCTE certificate for the offered equipment.

The Contractor is encouraged to propose any hardware configurations better suited to the characteristics of the Contractor's standard products as long as the equipment characteristic requirements of this specification are met. Such alternatives shall be considered as provided for in Section 1.7 Table of Compliance.

The Contractor is required to enter into a legal agreement with the Employer for security and access to Employer’s network as per latest DoT guidelines (agreement format enclosed at Appendix-E).  All security related requirements as amended by DoT on a time-to-time basis shall be followed/ complied by Contractor at no additional cost to Employer.

  - General Network Characteristics

    - Description

PowerTel’s fiber optic network consists of a Backbone network providing connectivity across the major & minor cities and Access network to extend the connectivity from the Employer’s PoP (Mostly Sub-Station) to city Centre / customer premises. The Access & Aggregation network in cities is a mix of DWDM/MPLS-TP/SDH & is required to be augmented by installing new MPLS-TP equipment over separate pairs of fiber.

New MPLS-TP equipment platforms must be able to support packet and TDM transport. Packet services, such as MEF CE 2.0 services, L3 VPNs, and L2 VPN, can be transported using MPLS-TP giving the service provider the freedom to choose the most appropriate technology to PowerTel. New MPLS-TP equipment must support native TDM & circuit emulation and facilitate PowerTel to build a packet network parallel to the existing legacy TDM networks.

The requirements provided in BoQ are tentative and they may vary during actual implementation.

    - General Systems Requirements

The Contractor is encouraged to offer standard products and designs. However, the Contractor must conform to the specification requirements and provide any additional equipment necessary to meet the requirements stated herein. It should be noted that preliminary design information and bill of quantity are indicative only; the Contractor shall verify jointly (with Employer) the design data during the site surveys & detail engineering and finalize the BOQ as required for ultimate design & system performance. Collectively, all of the defined subsystems are considered the communications system. Required characteristics are defined and specified herein.

      - System Maintainability

All NEs (MPLS-TP & Amplifiers) provided by the supplier must be managed on the same NMS.

The proposed system must provide several ways to limit service interruption: Error prevention, Fault tolerance, Error elimination and Error prediction. The “zero downtime upgrade” capacity must be detailed and illustrated by examples taken from live systems implemented by Bidders. The proposed system must guarantee 99.999% availability means that total downtime of the system must be less than 5 minutes per year.

Once a failure or degradation of performance is detected in the communications system, its cause shall be promptly isolated and corrected. To facilitate performance testing, efficient diagnosis and corrective resolution, the system shall permit in-service diagnostic testing to be executed both locally and from remote locations, manually and initiated under TMN control. Such testing shall not affect the functional operation of the system.

Preventive and problem-oriented maintenance of the communications system shall be performed using diagnostics tools such as TMN (as defined in chapter 3). They shall support complete maintenance of all system elements and shall permit the diagnosis of any fault.  The Contractor shall provide specialized training (as defined in chapter 6) required to operate the above diagnostics tools.

      - System Manageability & Security

The proposed system must have manageability. Manageability may include a centralized provisioning interface, a centralized monitoring/management interface and the capacity to easily integrate the system management to a higher-level management system.

The supervisory system of the equipment shall be capable of monitoring all cards which are available in the equipment.

The proposed system shall support various authentication method (username/ password, TACAC,...), system and operation logging, user management by user profiles (user classes) offering them different operation/management capability sets (for example: provisioning, reporting, configuration management, etc.), logging their actions, interception from inside and outside.

    - General Equipment Characteristics

All applicable requirements stated in this section shall equally apply to the TMN equipment as specified in Section 3.

      - Revision Levels and Modifications

All hardware, firmware and software delivered as part of the communications network shall be field proven and at the most current revision level. All modifications and changes necessary to meet this requirement shall be completed prior to the acceptance of Equipment/ Link/ Network by employer.

All field modifications required to update the hardware, firmware and software to the latest revision level occurring after the above specified testing, shall be fully disclosed, documented and presented to the Employer for their consideration. Satisfaction of this disclosure requirement does not obligate the Contractor to implement the changes provided the latest revision date occurs after the above requirements are met. The intent is to provide the Employer with the documentation and opportunity to consider their implementation.

All field modifications of the hardware, firmware and software that is required to meet installation and/ or performance specifications, shall be fully documented as part of the deliverables, both as a separate field modification record and as corrected equipment/ configuration documentation.

Version Management procedure of the software shall be described. This information shall include the current version and the schedule of updates in the past and estimate in the future. Contractor has to submit an annual report mentioning all the software, their version, updates etc running on its network.

      - Equipment Capacities

Equipment supplied shall be sized and equipped with sufficient capacity to support the circuit, channelization, and configuration requirements, including spares, as identified in the appendices. The Contractor shall also be responsible for ensuring that sufficient equipped capacity exists to support all alternate routing and redundant equipment requirements defined herein.

Each subsystem supplied shall be sized (to be equipped as specified) to support full subsystem expansion.

Data communications channelization required to support the TMN subsystems specified in Section 3, are not identified in the appendices. Therefore, the Contractor is required to size and equip the system to include all channelization and channel cards required to support the TMN function as described in Section 3.4 Data/ Communication Channel Requirement and Integration.

      - Alternate Routing, Redundancy Requirements and Protection Schemes

The Contractor(s) shall ensure that there shall be no single point of failure of Network due to equipment failure i.e. the failure of one Network Element shall not prevent the use of any other Equipment that has not failed. Bidders must state details about redundancy (1+1 Power Supplier, FANs) mechanism, wherever asked/ specified, of the proposed system at system level, subsystem level, and board level.

Other protection schemes supported by the proposed equipment shall also be described in the DRS. In case the equipment offered by the Bidder does not support the above-mentioned minimum protection methods, the bidder shall have to provide all additional equipment needed to provide same level of flexibility, redundancy, and functionality at no additional cost to Employer. The bidders shall provide details of protection scheme(s) in their Bid.

      - Equipment Lifespan

All equipment to be supplied under the subject package shall have a minimum expected life of Ten (10) years from the date of final acceptance. The Contractor shall ensure the availability of spare parts and cards/ items for a minimum period of ten (10) years from operational acceptance by the Employer or five (5) years after the declaration of withdrawal of equipment from production whichever is earlier. However, the termination of production shall not occur prior to Operational Acceptance of the system by the Employer.

      - Software Upgrades

The Contractor shall provide software upgrades and patches, if any, for the proposed equipment, free of cost, till maintenance period specified in contract i.e. 5 years after DLP. The Contractor shall also provide detail procedure before applying any upgrade (hardware or Software) in the Network. Software upgrade shall be remotely through TMN with no/ minimum disruption of service.

    - Optical Fibre Characteristics

The OPGW has been installed in last twenty years on various transmission lines and comprise of DWSM (G.652) and DSSM (G.653) fibers, and the intracity network is based on DWSM (G.652) fibre.

In case bidders need any other details/ characteristics of the fibre(s) to finalize their solution, they may be facilitated to test/ measure & collect the data from field without any financial implication to PowerTel before submission of their bid. The contractor has to forward for such request of field test/ measurement to PowerTel within 15 days of publication of NIT, and PowerTel reserve the right to accept or reject any such request from bidder(s), if received after 15 days of publication of NIT.

  - MPLS-TP Equipment for Aggregation & Access Network

    - MPLS-TP System

The product/ solution with low footprint & power efficient shall be offered for the access/ aggregation layer in intra-city. All optical interfaces up to 10G (Line & client both) shall support Bi-directional pluggable. Also, PowerTel reserves the right to use third party pluggable & interfaces shall not have any kind of restriction from Bidder/OEM for the offered equipment. The system shall support circuit emulation MPLS-TP (MPLS-Transport Profile) Equipment shall provide the following:

- OAM functions to detect and isolate faults (Like SDH/ SONET)

- Sub-50ms protection and restoration

- Ring & mesh architecture.

- End-to-end QoS

      - Functional Requirement

The offered MPLS-TP platform must be a reliable packet-based network based upon circuit-based transport networking. The architecture of the MPLS-TP platform must be defined as per the IETF RFC 5921& 6215. The requirements for MPLS-TP forwarding, OAM, and network survivability must be RFC 5654 and 5860.

- The switching fabric plane shall be bidirectional and non-blocking. The Equipment’s shall support a wire speed L2 switching capabilities under full load condition. The platform shall support enterprise services like Ethernet E-Line & E-LAN services and aggregate & hand over the traffic to next higher Metro Ethernet network Core Layer.

- The management of the MPLS-TP networks must be separable from its client networks so that the same means of management can be used regardless of the client. The management functions of MPLS-TP must include fault management, configuration management, performance monitoring, and security management.

- The purpose of the management function must be to provide control and monitoring of the MPLS transport profile protocol mechanisms and procedures.  The requirements for the network management functionality must be as per RFC5950 & RFC5951.

- MPLS-TP platform must mandatorily run BFD (Bidirectional Forwarding Detection) protocol to provide OAM functionality. It should have resiliency to ensure minimal interruption to traffic when a failure occurs within the system or network.

      - Equipment Requirement

  - Equipment sub rack must be suitable to install in 19 inch and 21-inch rack

  - The supplier shall confirm that their solution supports insert, replace and remove of modules while the equipment is powered up without affecting traffic and damaging any module.

  - All NEs and cards provided by supplier must be managed on the same NMS.

  - Equipment can be divided in Five categories (Type-1, 2,3,4 &5) based on their configuration. The details are given below:

| Sl. No. | Particulars | Type-1 (6x100G) | Type-2 (2x100G) | Type-3 (10G) | Type-4 (10G with E1) | Type-5 (10G) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Maximum Size (in RU) | 7 RU or less | 2 RU | 2 RU | 2 RU | 2 RU |
| 2 | Ports (Min) | 6x100G+ min 16x10G | 2x100G+ min 10x10G | 4x10G +4x1G (min. 2 optical + min. 2 Elect.) | 4x10G + 4x1G (min. 2 optical + min. 2 Elect.) + 3xE1+ 1xSTM-1 | 2x10G (optical), 2x1G (min Optical + min 1 Electrical) |
| 3 | Coloured interfaces | Yes (Both 100G & 10G) | Yes (Both 100G & 10G) | Only 10G | Only 10G | Only 10G |
| 4 | Bi-Directional SFP/ XFP | Yes (only 10G) | Yes (only 10G) | Yes (10G & 1G optical) | Yes (10G & 1G optical) | Yes (10G & 1G optical) |
| 5 | Redundancy (Power, Fan & other common modules) | Redundant PSU, FTU | Redundant PSU, FTU | Redundant PSU, FTU | Redundant PSU, FTU | Redundant PSU, FTU |
| 7 | Chassis | Any type | Any type | Any type | Any type | Any Type |
| 8 | Power (Input Voltage) | -48V DC | -48V DC | -48V DC & 220V AC | -48V DC & 220V AC | -48V DC & 220V AC |

      - Technical specification

        - Standard requirement

- All equipment must comply with the Generic requirements mentioned in TEC/GR/TX/CEN-004/03/MAR-17 for MPLS-TP based carrier Ethernet switch for Aggregation & Access Network Applications. A statement of compliance shall be submitted along with the Bid.

- All Equipment (100G & 10G) shall support amplifiers solution of 30dB, 35dB, 40dB & 45dB as per BoQ. Kindly refer the detailed BoQ at Appendix B

- The switching fabric plane shall be bidirectional and non-blocking. The equipment shall support a wire speed L2 switching capabilities under full load condition.

- The Equipment shall support Ethernet PW as specified by RFC 3985 and RFC 4385. It shall support Ethernet PW as specified by RFC 4448.

- Equipment shall support minimum number of statically configured LSP as mentioned below:

          - Type-I: 2K

          - Type-II: 600

          - Type-III&IV: 200

          - Type-V: 100

- The MPLS-TP Equipment has UNI (User Network Interface) and NNI (Network Node Interfaces). The MPLS-TP Equipment shall support UNI or NNI mode of operation on all the ports.

- Optical Ethernet interfaces shall be configured as Client or Aggregate interfaces based on requirement.

- It shall support minimum Multicast group and members per multicast group as per below: -

a. Type-1 – MCAST Group per system – 1500, L2 multicast group members 256

b. Type-2 – MCAST group per system- 600, L2 multicast group members 256

c.  Type-3 – MCAST Group per system – 100, L2 multicast group members 256

d. Type-4 – MCAST Group per system – 100, L2 multicast group members 256

e. Type-5 – MCAST Group per system – 50, L2 multicast group members 256

- All Equipment shall support jumbo frame of 9000 bytes

- The Equipment shall support buffering of 50ms to take care of momentary congestion and link failures.

- The equipment shall support IPv4 and IPv6 traffic transparently at data plane.

- All Equipment shall be able to function in terminal mode, hub, & mesh and ring topologies.

- The equipment shall provide managed multi-service integration viz., voice, video and data.

- All equipment shall be fully compliant to MEF 9, MEF 14 or CE2.0 & MEF3.0 standards.

- It shall support built in power diagnostics feature to monitor optical SFP/XFP ports, system diagnostics to detect hardware failures.

        - Layer-2 Switching Features

- It shall support ingress and egress bandwidth profile per User to Network Interface (UNI).

- Equipment shall support Ethernet functions VLAN (IEEE 802.1Q), Q-in-Q (IEEE 802.1ad), VLAN manipulation: stack/switch/strip, IGMP snooping (V1/V2/V3), Link aggregation (IEEE 802.3ad).

- Equipment shall support Link Layer Discovery Protocol as per IEEE 802.1ab, Logical Link control as per IEEE 802.2&

- The equipment shall support enterprise services like Ethernet E-Line & E-LAN services and aggregate the traffic and hand over the same to the next higher Metro Ethernet network Core Layer.

- Service multiplexing: A single MPLS-TP Equipment port shall support multiple Ethernet Services handover from one port through ENNI

        - VLAN Features

- The MPLS-TP Equipment shall support creation of VLAN among ports of different types as well as on all ports of the interface cards.

- It shall support VLAN bridging (for outer tag only) as per IEEE 802.1adand user isolation per outer VLAN tag. The user isolation shall be configurable on a per port basis.

- The MPLS-TP Equipment shall support VLAN ingress filtering to prevent VLAN leakage & VLAN tag overlapping allowing some ports to be member of more than one VLAN.

        - Port trucking / Link Aggregation

- The MPLS-TP Equipment shall allow Link Aggregation as per IEEE 802.3 ad to allow link resilience.

- Supports load balancing over Aggregated Links.

- Static/LACP LAG on client ports shall be configured in all equipment.

        - Forwarding Support

- The MPLS-TP Equipment shall support to override The MPLS-TP Equipment port MAC address.

- It shall also support VPLS instance.

        - MAC Address Learning / Limiting:

- The MPLS-TP Equipment shall support L2 Learning parameters: Sources learning per Port/VPLS/VLAN/Source address.

- The MPLS-TP Equipment shall support to set per port/VPLS instance dynamic MAC learning.

- The MPLS-TP Equipment shall support to limit the number of source MAC addresses learnt from bridge port/VPLS instance in order to prevent MAC address flooding DoS attack. This limit is configurable per bridged port.

- The MPLS-TP Equipment shall support dropping of Frames with new source MAC-addresses exceeding the configured value.

- The MPLS-TP Equipment shall support per VLAN MAC learning to ensure MAC addresses are learnt only from a VLAN perspective and automatic/manual disabling of MAC addresses learning for the VLAN where there are less than two ports in that VLAN.

- The MPLS-TP Equipment shall support MAC limiting per Ethernet flow point (EFP) or bridge domain.

- The MPLS-TP Equipment shall support MAC address limitation and aging.

- All static entries shall NOT be aged.

- The MPLS-TP Equipment shall support Hardware based aging of MAC Address Table entries.

- The MPLS-TP Equipment shall support to enable L2 Aging on every port.

- The MPLS-TP Equipment shall support MAC address learning disabling.

- The MPLS-TP Equipment shall support to filter and discard all Ethernet frames received on bridged ports in the upstream direction with a specific MAC destination address (DA)

- The MPLS-TP Equipment shall support list of allowable MAC destination address.

- The MPLS-TP Equipment shall filter the MAC address from bridge port X if the same MAC address appears in the learning table pointing to bridge port Y (port X and port Y on the same the MPLS-TP Equipment and same VLAN), except in the cases where the aggregation network forwards according to MAC Learning table.

- The MPLS-TP Equipment shall support unique MAC address per device to prevent spoofing and provide traceability.

- The MPLS-TP Equipment shall have the capability to prioritize BPDUs in the data plane (by providing dedicated queues) and in the control plane (by providing dedicated CPU queues for BPDUs).

- The MPLS-TP Equipment shall have the capability to drop BPDUs regardless of the BPDU content. This function is configurable on a per port basis.

- The Access and Aggregation Switch shall support the following size of FDB table entries:

Type-1: shall support dynamic MAC entries 150K with static MAC – 2K.

Type-2: shall support dynamic MAC entries 150 K with static MAC – 2K

Type-3: shall support dynamic MAC entries 32K.

Type-4: shall support dynamic MAC entries 32K.

        - The MPLS-TP Equipment shall support IEEE 802.1Q Tagging.

- Tagged only, this is an IEEE 802.1Q trunk.

- Untagged.

- Hybrid tagged and untagged frames.

- The MPLS-TP equipment shall support the following additional IEEE 802.1Q features.

- Tag insertion, removal, and swapping.

- Capability of insertion and removal of second tag.

- Encapsulation translation and rewrites Push, Pop and translate for IEEE 802.1Q or QinQ/IEEE 802.1ad tags.

- Local VLAN and ports cross-connect and multipoint or point-to-multipoint with Hierarchical Virtual Private LAN service (H-VPLS bridge topologies with pseudo-wires) or locally defined bridge domains.

- The MPLS-TP Equipment shall allow configuring all 4K VID on all ports and at the same time supporting all 4K VLANs simultaneously. The user/operator shall be able to reuse the same VLAN-ID on a different port on the same. The MPLS-TP Equipment and terminate it into a different PW/VLAN.

- The MPLS-TP Equipment shall accept untagged, and C-tagged frames through a IEEE 802.1Q port.

        - Q-in-Q or IEEE 802.1ad mode Requirements

- The MPLS-TP Equipment’s shall perform classification and service delineation based on outer Q tag and outer IEEE 802.1p bits. (i.e. ignore inner tag)

- It shall support VLAN stacking as per IEEE 802.1ad

- It shall have minimum of 4094 S-VIDs. VIDs “0”

- It shall allow only S-tagged frame in ‘.1ad’ ingress ports. It shall be possible to map the traffic to any PW based on SVLAN tag. It shall be possible to keep or pop the SVLAN tag before forwarding it to the PW.

- It shall be possible to set the priority bits in the S-VLAN priority based on the PCP bits of C-tag of the incoming packet in ‘.1ad’ mode.

        - LSP Mode Requirements

  - The MPLS-TP Equipment shall support LSP Mode Scalability Options i.e. Virtual Private LAN Service (VPLS)

- Support Packet Transport Network solution using PW service tunnel.

- TDM and Ethernet traffic shall be emulated into Pseudo-wires and PW label is added for service identification.

- End-to-end transport path LSP shall be created based on MPLS-TP standard and multiple

- The Service tunnels shall support per LSP queuing/scheduling i.e. the ability to assign and guarantee per LSP per class bandwidth profiles (CIR, EIR, CBS, and EBS)

        - Quality of Service Requirements

- The MPLS-TP Equipment shall support VLAN CoS preservation.

- The MPLS-TP Equipment shall support VLAN CoS differentiation: It shall be possible to configure the classification of the traffic according to the service, VLAN, IEEE 802.1p bits or TOS/DSCP bits.

- The MPLS-TP Equipment shall support creation of VLAN or Flow with DSCP parameters per service for data, video and O&M traffic for service differentiation.

- Equipment shall support in-built feature of measurement of performance bounds for each flow in terms of throughput, loss, delay and delay variation, according to their respective defined service classes as per Y.1731.

- The MPLS-TP Equipment shall support 16, 32, 64,128,256 and 512 k bytes burst sizes.

- The MPLS-TP Equipment shall support wire speed forwarding on all interfaces and all packet sizes even with classification and QoS activated on all interfaces.

- The MPLS-TP Equipment shall support bandwidth management reports and statistics.

- The Edge the MPLS-TP Equipment shall support 8 class of service per flow.

        - Classification/Prioritization:

- The MPLS-TP Equipment shall support classification of ingress traffic for a specific service based on the following mapping:

- IEEE 802.1P Mapping - it shall be possible to reserve parts of the link bandwidth for frames with particular IEEE 802.1p values.

- Customer IEEE 802.1p priority

- Multiprotocol Label Switching (MPLS) Label Stack Entry:"EXP" Field Renamed to "Traffic Class" Field as per RFC 5462

- Source MAC address

- Destination MAC address

- VLAN ID

- IEEE 802.1Q

- Default queue for non-matching traffic

- The MPLS-TP Equipment shall aggregate incoming traffic into Traffic Classes by following characteristics:

- Incoming port (Logical and Physical)

- Source/ destination MAC

- Link layer priority information as per IEEE 802.1p

        - Mapping

- The MPLS-TP Equipment shall support mapping of DSCP/VLAN/Port to MPLS labels.

- Mapping of IEEE 802.1p and IP TOS bits into MPLS EXP bits.

- Mapping of IEEE 802.1q VLAN tags into MPLS labels

- Aggregate incoming traffic into Traffic Classes by MPLS Label EXP bits (E-LSP)

        - Marking/Policing/Shaping:

- The MPLS-TP Equipment shall support the following Marking/Policing/Shaping requirements.

- 8 level Priority marking as per IEEE 802.1p

- Broadcast/Multicast suppression

- Bandwidth management policies

- Trust the colour of the incoming packet.

- Differentiated Service Two-Rate, Three-Color Marker with Efficient Handling of in-Profile Traffic

- Shaping of Burst Traffic

        - Rate Limiting

  - The Equipment shall support Rate limiting of bandwidth and per class (or flow)

  - Configuration of user bandwidth shall be supported, in steps of

- Mbps for 1-1000Mbps

- 100 Mbps granularity for 1-100 Gbps

  - It shall support defining Committed Information Rate (CIR) and an Excess Information Rate (EIR) for each flow in steps of 1Mbps.

  - The MPLS-TP Equipment shall support flow-based rate limiting method based on per source address, destination address or both.

        - Queuing:

  - The MPLS-TP Equipment shall support the following queuing

- SPQ – Strict Priority Queuing

- WFQ – Weighted fair Queuing

- No of queues per flow treatment of traffic.

- Per service ingress queues are defined on the basis of Peak/Excess burst Size (PBS/EBS), Committed Burst Size (CBS), Peak/Excess Information Rate (PIR/EIR) and committed Information rate (CIR).

- Per service egress queues have distinct parameters defining its operations like Peak/Excess burst Size (PBS/EBS), Committed Burst Size (CBS), Peak/Excess Information Rate (PIR/EIR) and committed Information rate (CIR).

- Alternate priority routing traffic necessary to keep from starving other priority queues.

        - Scheduling:

- The MPLS-TP Equipment shall support scheduling of queues to strict priority with 2 or more priority levels.

- The MPLS-TP Equipment’s shall support the following congestion avoidance mechanisms.

- Weighted Round Robin (WRR)

- DWRR (Deficit Weighted Round Robin)

- WRED

- Strict Priority (SP)

- The MPLS-TP Equipment shall support configuring the scheduling as per the

- Per Hop Behaviour (PHB)

- Physical port or logical port basis.

- Up to 8 forwarding class queues can be configured on a per service basis each with its own CIR, PIR, CBS, PBS and Forwarding Class attribute.

- At least three level dropping precedence levels in each queue.

        - Protection Mechanism:

- The MPLS-TP Equipment shall support linear protection (1:1 & 1+1) as specified in ITU-T Rec. G.8131.1 or G.8131.2.

- For all the modes of protection, The MPLS-TP Equipment shall support protection switching within 50ms of expiration of any manually selected hold-off time (with both SF/SD scenario) and shall support all operator commands (Forced Switching [FS], manual Switching [MS], lockout of protection).

- The MPLS-TP Equipment shall support protection switching within 50ms and Forced switching as analogous to SDH systems.

- The MPLS-TP Equipment shall support protection switching triggered by fault detection, such as loss of signal, loss of frame, signal degrade (BER becomes worse than the predetermined threshold), and so on.

- The MPLS-TP Equipment shall support Forced switching activated by administrative events, such as fibre rerouting, fibre replacement, etc.

- The MPLS-TP Equipment shall support Ethernet ring protection for both S-VID and B-VID as per G.8032

- The MPLS-TP Equipment shall support MPLS-TP Linear Protection support as per IETF standards or Ethernet/MPLS SNC based protection as per ITU-T standards.

        - Circuit Emulation Protocols

- The MPLS-TP Equipment shall support PW using Time Division Multiplexing (TDM) over Packet (SAToP) or MEF8.

- The equipment shall support E1/channelized STM-1 circuit emulation mechanism and it shall support Circuit Emulation Services over MPLS as per PWE3 standard to aggregate the TDM PW from the edge and access CEN.

        - Network Synchronization Requirements

- The MPLS-TP Equipment shall be able to synchronize with an external reference clock.

- It also supports Synchronous clock selection algorithm based on the following parameters:

- Quality of Signal

- Signal fail

- Priority

- External Commands

- It shall support SyncE as per ITU-T G.8261, G.6282 and G.8264 or IEEE 1588v2.

- The MPLS-TP Equipment shall support Network Time Protocol (NTP) for synchronizing with a central NTP server.

- The MPLS-TP Equipment shall support timing and synchronization aspects of Packet Networks based on SyncE as per G.8261. The MPLS-TP Equipment shall support timing characteristics of a synchronization slave clock as per G.8262.

- The MPLS-TP Equipment shall support provide a timing-output interface at 2048 KHz for external synchronization. The output shall conform to ITU-T Rec. G.812, as applicable.

        - MPLS-TP STANDARDS

- It shall support the creation of L2VPN solutions using statically configured PWs and tunnels as per RFC 4664

- The MPLS-TP Equipment shall support MPLS-TP requirements as per RFC 5654 or ITU Y.SUP4

- The MPLS-TP Equipment shall support Architecture of MPLS-TP Layer Network as per ITU-T G.8110.1v2 or equivalent IETF standards.

- The MPLS-TP Equipment shall support Interfaces for the MPLS-TP Hierarchy as per ITU-T G.8112 or equivalent IETF standards.

- The MPLS-TP Equipment shall support Characteristics of MPLS-TP Network Equipment Functional Blocks as per ITU-T G.8121v2 or equivalent IETF standards.

- The MPLS-TP Equipment shall support MPLS-TP General Framework as per RFC 5921 or ITU G.8110.1

- The MPLS-TP Equipment shall support MPLS-TP survivability framework as per RFC 6372 or ITU G.8131/G.8132

- The MPLS-TP Equipment shall support MPLS-TP Data plane Architecture as per RFC5960 or ITU Y.SUP4

- The MPLS-TP Equipment shall support MPLS Generic Associated Channel (GAL/G-ACH) as per RFC 5586 or ITU G.8113.1/G.8113.2

- The MPLS-TP Equipment shall support Definition of ACH TLV Structure as per draft-ietf-mpls-tp-ach-tlv-02 or ITU G.8113.1/G.8113.2

- Support Pseudowire Status for Static Pseudowires as per RFC 6478

- It shall support MPLS On-Demand Connectivity Verification and Route Tracing as per RFC 6426 or ITU G.8113.1/G.8113.2

- The MPLS-TP Equipment shall support Proactive Connectivity Verification, Continuity Check, and Remote Defect Indication for the MPLS Transport Profile as per ITU G.8113.1/G.8113.2

- The Edge MPLS-TP Equipment shall be connection-oriented packet switching model with traffic engineering capabilities that allow deterministic control of the use of network resources. It shall support traffic engineered point to point (P2P).

- The Edge MPLS-TP Equipment shall support packet transport network solution by using PW service tunnel and MPLS-TP transport technology. TDM and Ethernet traffic are emulated into Pseudo-wires and PW label is added for service identification. End-to-end transport path LSP is created based on MPLS-TP standard (ongoing) and multiple PWs are transported over the same LSP end-to-end in both directions.

- In order to support traffic engineered point-to-point MPLS-TP circuits, it shall support all the MPLS-TP specific requirements specified in the RFCs (RFC 3031, 3032, 3443, 5586, 5860, 5960, 6370, 6426, 6428). No explicit support for control plane protocols is required. NMS Support shall be mandatory.

- It shall be possible to rate limit the traffic in MPLS-TP tunnels at minimum 10 Mbps granularity. MPLS-TP tunnels shall support per E-LSP Queuing as defined in RFC 3270.

- In order to support traffic-engineered Pseudo Wire Emulations (PWE) over MPLS-TP, the system shall support the following RFCs (RFC 4448, 5659, 5994). The compliance for these RFCs is required to the extent of supporting MPLS-TP and PWE. No explicit support for control plane protocols is required. NMS support shall be mandatory.

      - Pluggable Optics

      - All 100GGrey pluggable optics in the BoQ for MPLS-TP (Type-I & II) shall be QSFP28. In case the 100G ports on the equipment are of legacy physical size (CFP/ CFP2 etc), QSFP28 pluggable with appropriate adaptor shall be supplied. The 100G Tuneable Pluggable may be supplied as per the physical interfaces (CFP/ CFP2/ CFP4/ QSFP28/ Coherent DCO etc) on the Equipment.

      - The Tuneable 100G & Tuneable 10G Pluggable in the BoQ for all type of MPLS-TP Equipment must support span loss of minimum 22dB without any amplification.

      - All 10G pluggable optics shall be used in NNI & UNI ports of MPLS-TP equipment.

      - Bi-Directional XFP/ SFP+ shall also be used as 10G or STM-64 in MPLS-TP (10G) Equipment or existing SDH (STM-64) equipment of Tejas make, as per requirement.

      - Bi-Directional XFP/ SFP+ shall be used with 10G (NNI & UNI) interfaces of all MPLS-TP equipment.

      - 1G pluggable supporting up to 40 Km, 80 km, 120 km and 180 km under BoQ, are envisaged to use with MPLS-TP equipment also.

      - Amplifier Solution for Packet Links

        - Optical Amplifiers

Transmit power amplifier and/or a combination of transmit power amplifier & receiver preamplifier shall be offered for 10G Packet Links and 100G Packet links to meet the link budget requirements stated in ‘Optical Link Performance Requirements’. Amplifier Solution shall be located with the equipment.

The Optical amplifiers shall meet the alarm requirements and shall be fully manageable from TMN. It should be possible to monitor the optical parameters and also handle alarms of the optical amplifier from the TMN. The Bidders shall provide detail specifications of Optical Amplifiers used in the Amplifier Solution, in the Bid.

Link Solution supporting difference span losses in the BoQ has been asked in the form of ‘Set’. The ‘Set’ consists of all necessary amplifiers, pluggable Optics for equipment, Patch cords, power cables, Communication/ DCN cable accessories and any other items as per TS, for one end of the link i.e. PowerTel shall order two (02) sets for implementation of one (01) link.

        - Optical Link Performance Requirements

The optical fibre link performance requirements are specified at 2.3.1.5.4 as follows.

        - Link Budget Calculations

The fibre optic link budget calculations shall be calculated based upon the following criteria:

- Amplifier solution for Packet links (10G) up to minimum 30dB (120 Km)/ 35dB (140 Km)/40 dB (160 Km)/ 45dB (180 Km) link budget has been calculated with attenuation factor 0.25dB per km and considering factor such as optical path penalty, temperature etc. However, Bidder must consider addition 1 dB EOL margin and 1 dB connector margin for all required amplifier solutions.

- Dispersion: The fibre dispersion shall be taken to be the guaranteed maximum dispersion i.e. 18 ps/nm.Km (1550 nm) for G.652 fibre.

- Bit Error Rate: The link budget calculations shall be done for a minimum BER of 10-12 for 10G and 10-15 for 100G links.

        - Link Performance

The Link performance for ES, SES and BBER for the fibre optic links shall correspond to National Network as defined in ITU-T G.826, G.828, M.2101, RFC 2544 and Y.1564.

      - Management

The MPLS-TP system shall support the In-Band & Out-Band management communication channel. The management function of MPLS-TP   includes fault management, configuration management, performance monitoring, and security management. The management function must provide control and   monitoring of the MPLS transport profile protocol mechanisms and   procedures.  The requirements for the network management functionality must be as per RFC5951.

  - Patch Cords:

The Contractor shall provide (i.e. supply and install) connectorized FODP-to-equipment and equipment-to-equipment jumpers (patch-cords) for each equipment with two installed spares. The contractor shall be required to provide armored patch-cords at some of the locations prone to rat cuts as identified during survey/ or mentioned while finalization PowerTel of BoQ. Fiber jumpers shall be of sufficient lengths as to provide at least 0.5m of service loop when connected for their intended purpose. The contractor shall provide patch cords of appropriate length for interconnection amongst equipment ports and with FODP for proper routing inside the cabinet and to maintain aesthetics. Contractor shall assess the requirement of various lengths of patch cords during survey and keep sufficient quantity of various length of patch cords to cater any changes during Installation. The Patch cords shall comply with the Comply with the TEC No. TEC/GR/TX/OFJ-01/05/NOV-09 and latest release and amendments.

  - Multiplexer-Demultiplexer Unit - 8 channels passive module (With Self):

    - MDU modules must support adding and dropping of minimum of 8 no’s of DWDM channels with 100GHz spacing in C-band (191 to 196 THz). The required frequency band will be shared during the details engineering.

    - The module must support multiplexing of minimum of 8 wavelengths in the transmit direction and de-multiplexing of minimum 16 wavelengths in receive direction.

    - The offered module must have an Upgrade/ Express port provided so that two (02) such MDU8 cards can be cascaded to build a 16-channel system with 100GHz spacing.

| Parameters | Units | Value |
| --- | --- | --- |
| Number of Channels | ---- | 8 |
| Channel Spacing | GHz | 100 |
| Pass band | GHz | ITU+/-15 |
| Upgrade port/Express Port | ---- | Yes |
| Maximum optical Insertion Loss | dB | 5 |
| Maximum IL from Express RX/TX port to Com Tx/Rx |  | 2 |
| Isolation: Adjacent channel | dB | >30 |
| Isolation: Non-Adjacent channel | dB | >40 |
| Power Handling | mW | 500 |
| Type of Connectors |  | LC Type |

  - DC Power System & Power Cables:

DC Power System of 48Vdc 100A shall be supplied to power the MPLS-TP Equipment. The DCPS should be suitable for use in Customer Premises, compact in size and can be installed in the Racks to be supplied under the subject package. The size of proposed DCPS shall not be more than 2RU.

    - Scope of Work

- Supply of DC Power supply system to suit specified requirements as detailed below in this document and conforming to the relevant standards.

- Delivery will be done as per BOQ.

- The Technical documents also includes information related to product specification/`user guide/ installation guide, including safety guidelines etc.

- The equipment supplied shall be under warranty for 1 year from date the material is supplied at Owner premises and after joint verification of material. The Owner will send, at his own cost, the defective material to the address indicated by the Contractor within India. The Contractor will replace or repair the defective material and deliver the same to Owner's store/premises at designated location within India at his own cost.

- Providing routine test results or certificate indicating quality acceptance by manufacturer's internal inspection as performed at the manufacturer's factory/ works before delivery. warranty certificate, user/ installation manuals for the SMPS based modules and other control modules.

    - Standards:

The SMPS based Power System shall comply with the Comply with the TEC No. No.: TEC/GR/FA/SMP-001/07/MAR-17 and latest release and amendments.

The SMPS based power system (including shelf and all accessories) shall conform to EN 55022 (or CISPR 22); class A or equivalent for radiated electromagnetic emission generated by the system and conducted electromagnetic emission on AC power lines.

    - System design parameters:

- -48V DC output power supply.

- Modular design based on SMPS.

- Load current: Sufficient to supply 100A @ -48V DC as per Bill of Quantity

- Protection: N+1 for modules.

- Single shelf for SMRs, metering, alarm, and DC switches & disconnections suitable for 19" and 21” rack.

- The height should not be more than 2U size for 100Amp system and the depth of the converter should be maximum of 450mm.

- The design of offered system such that there should not be any requirement to access it from back side for I&C as well as during O&M of the device.

- 100Amp load current requirement shall be met using minimum 2 nos of SMR modules.

    - Technical requirement:

- Input AC range: 150V to 275V phase-to-neutral AC, 48 to 52 Hz.

- DC Output voltage: 48V (nominal), -48 to -56V DC (Adjustable).

- Operating mode: Float/Charge

- SMR Modules to be hot plug-in.

- Load & line regulation: < 1%

- Load sharing between SMR modules (variation up to +/- 5%).

- The Peak-to-Peak Ripple: Voltage at the output of the rectifier module, without battery connected, shall not exceed 300 mV at the Switching Frequency measured by an Oscilloscope of 50/60 MHz bandwidth (Typical).

- Efficiency: > 85% at full load, 230V AC.

- Protection: (i) AC High/Low voltage cut-off. (ii) Short circuit (iii) Overload (iv) EMI/RFI and Line surge protection.

- Metering: Digital meter to read (i) Output voltage, (ii) Load Current

- LED Indications: (i) Mains healthy (Front panel) (ii) SMR module fail.

- Minimum 06 Nos of MCBs for DC Load distribution shall be provided in configuration of 2x63A + 2x32A + 2x20A.

    - POWER Cables

    - The contractor shall supply, install and commission all power cables, grounding cables, control cables, network interface cables, MCBs, fuses etc. of required size and capacity including associated hardware (lugs, glands, cable termination boxes, PVC pipe etc.) required to mechanically and electrically complete the installation of equipment and their successful operations.

    - The Contractor shall be responsible for cable laying and termination at both ends of the cable. The Contractor shall also be responsible for termination of feeder cables at Contractor’s equipment end including supply of suitable lugs, glands, terminal blocks, PVC pipe, etc.

    - The requirement of quantities of AC & DC cables for Equipment and DCPS have been identified in BoQ and for other accessories including grounding cables, control cables, network interface cables, PVC pipe, MCBs (AC & DC) etc and others associated hardware (lugs, glands etc) shall be supplied by the contractor as per actual requirement without any extra financial implication to PowerTel. The AC & DC cables as per BoQ will be ordered for following:

      - Connection from Equipment to existing DCPS of PowerTel/ Customer (Only Equipment is supplied).

      - Connection from Rack (DCDB) to existing DCPS of PowerTel/ Customer (Equipment along with Rack is supplied).

      - Connection from DCPS to AC mains (Equipment along with Rack & DCPS is supplied)

    - The DC cable for s.no.(a)&(b) of suitable rating out the cable in BoQ, shall be supplied as per the quantity mentioned in BoQ. The cables shall be suitable for laying in racks, cable trays, ducts, trenches and conduits.

    - Earthing connection for Equipment/ DCPS/ Rack will be Extend from Site existing Earth pit.

    - All AC power cables shall be stranded Aluminium conductor armoured XLPE/ PVC insulated and sheathed conforming to IS 1554 Part-1/IS 7098. The cables shall be suitable for buildings and underground buried installations with uncontrolled backfill and chances of flooding by water.

    - All DC power cables shall be stranded Copper conductor, armoured XLPE/PVC insulated and sheathed, 1100V grade as per IS 1554 Part-1/IS 7098.

    - All wires used for manufacturing the cables shall be true circular in shape before stranding and shall be uniformly good quality and free from defects. The filler and inner sheath shall be of non-hygroscopic and shall be soften than insulation. Outer sheath shall be suitable for operating temperatures of cable.  Repaired cables shall not be accepted.

    - All cables shall conform to relevant IS.

    - Each length of complete cable between electrical and mechanical assemblies, enclosures and distribution frames shall be identified at each end with an indelibly marked shrink-on sleeve which indicates the cable number, and the near-end and far-end destination.

    - All the required MCBs, fuses etc. required for safe operation of the equipment shall also be supplied and installed by the Contractor. Separate cables shall be laid for AC and DC. The cables shall be suitable for laying in cable trays, ducts, trenches, and conduits.

  - Cabinet

    - Equipment cabinet must comply with ETSI 300 119 RACK Dimensions (2200mm x 600mm x 600m) and must be supply with power distribution panel.

    - The rack shall have a DCDB(PDP) with sufficient number of MCBs to support installation of minimum of 3 equipment’s/Sub-racks.

    - Dust and moisture protection shall meet or exceed IP20 standards.

    - Rack should be equipped with a fiber management tray or fiber tray rack mount to organize and access patch cord ports.

    - Finish colours for enclosures shall be gloss white enamel on the inside, and semi-gloss medium grey enamel on the outside.

Refer para 4.3 of Sec-4 of the Technical Specification for details.

  - Inside Plant

For the purposes of the specification, Inside Plant shall include cabling, wiring etc. Equipment and material components of the Inside Plant are part of this procurement.